;General definition for PC87363/364/366 serial

;Default function is 2S, 1P, 1G, 1FDD, 1Midi

PC87363				EQU	1	;Use PC87363

PC87364				EQU	1	;Use PC87364

PC87366				EQU	1	;Use PC87366

Superio_Port_4E			EQU	1	;If doesn't define, 2Eh will be

;used

No_Use_KB			EQU	1	;No use I/O keyboard controller

NO_KB_CLOCK_ITEM		EQU	1	;No Keyboard clock item

FDC_WR_PROTECT_SELECTABLE	EQU	1	;FDD write protection item

Superio_Support_Floppy_Drive_B	EQU	1	;Support Drive B functionality

Superio_Irq_High 	EQU	00011110b	;This is which IRQ can be use

;in win95/98

;bit 0 is IRQ8 and bit 1 is

;IRQ9, and so on.

Superio_Irq_Low		EQU	10111000b	;This is which IRQ can be use

;in win95/98

;bit 0 is IRQ0 and bit 1 is

;IRQ1, and so on.

Superio_Dma		EQU	00001010b	;This is which DMA can be use

;in win95/98

;bit 0 is DMA0 and bit 1 is DMA1

;, and so on.

No_Superio_Support_Game		EQU	1	;Doesn't Support game

No_Superio_Support_Midi		EQU	1	;Doesn't Support midi

Superio_Support_Msg_Led		EQU	1	;1 = ACPI Message use Led1

;2 = ACPI Message use Led2

;MSG = 0 light out, otherwise

;light on

Superio_Support_ACPI_Led	EQU	1	;1 = ACPI State use Led1

;2 = ACPI State use Led2

;S0 depend on led setting

;S1 blinking at 1Hz

;S3 blinking at .5Hz

;S4 invert S0

;S5 invert S0

Superio_Support_APM_Led		EQU	1	;1 = APM State use Led1

;2 = APM State use Led2

Superio_APM_Led_Not_blinking	EQU	1	;led not blinking at D3	state

Superio_Led1			EQU	1	;0 = led1 normal is low

;1 = led1 normal is high

;the same as Led2

Use_Superio_Fan			EQU	1	;Use super I/O fan controller

Superio_Fan0			EQU	0	;0 = fan low active

;1 = fan high active

;the same as fan 1, fan2

No_show_Fan_Speed		EQU	1	;Doesn't show fan speed

Superio_Support_Power_Loss	EQU	1	;Support power loss

Superio_On_Now_Function		EQU	1	;Support on now function

Superio_Runtime_Port		EQU	800h	;available address at run-time

;if not defined, default is 800h

GEN1_port			EQU	800h	;Enable ICH GEN1 at 800h

;Superio_Runtime_Port at the

;same time

Use_Superio_GPIO		EQU	1	;Support Simple I/O

Superio_GPIO01		EQU	0000XXXXb	;bit 1 0						;    | |

;    0 0 = output low

;    0 1 = output high

;    1 0 = input

;---------------------

;bit 2 = 0 open-drain

;	 1 push-pull

;---------------------

;bit 3 = 0 internal pull-up

;	 1 no internal pull-up

;---------------------

;GPIO00 - GPIO07

;GPIO10 - GPIO17

;GPIO20 - GPIO27

;GPIO30 - GPIO34

Flash_Write_By_Superio_GPIO	EQU	11h	;Use GPIO11 to control flash

;writing

Flash_Write_By_GPIO_High	EQU	1	;Flash write if GPIO is high

Use_Superio_ACB			EQU	1	;Support Access	Bus

;below is about sensor

Superio_Support_Sensor		EQU	1	;Support Voltage/Thermal

;Device

Use_Pin47_49_By_VLM		EQU	1	;Not use cpu defuse

Superio_Use_External_VREF	EQU	1	;Sensor use external reference

;value

CPU_Sensor			EQU	0	;0 : CPU use temperature 1

;1 : CPU use temperature 2

;2 : CPU use temperature 3

No_Use_Thermal_1		EQU	1	;Not use Temperatue 1

No_Use_Thermal_2		EQU	1	;Not use Temperatue 2

No_Use_Thermal_3		EQU	1	;Not use Temperatue 3

Sensor_Volt0_Str	EQU	'Voltage 0'	;Input what you want to

;show in voltage 0 item.

;the same as 1 to 6

Sensor_Temp1_Str	EQU	'Remote Diode 1 (CPU 0)'

;Input what you want to

;show in Temperature 1 item.

;the same as 2 to 3

Sensor_Volt0_Negative		EQU	1	;Voltage 0 is negative

;the same as 1 to 6

Superio_No_Support_Keylock	EQU	1	;No support keylock