TSRs

A number of issues in the design of Terminate-and-Stay-Resident

programs (TSRs) need to be handled differently in protected mode than

in real mode.  The total process is complicated enough so that we have

created a number of special functions to make developing protected-mode

TSRs easier.  All of these are included in the standard DOS/4G API

libraries.  These functions are designed to deal with the issues of

protected-mode TSR development described for this example.

Critical issues for TSRs that require the use of the helper fuctions

include:

Installation: Protected-mode TSRs can be installed in the same way as

real-mode TSRs, or they can be installed using D32TsrExit.  Either way,

the TSR will remain in extended memory.  Use D32SegMoveLow to move code

which will execute in real mode down to low memory during installation.

If you install the TSR via the standard DOS or library functions, the

parameter which indicates how much of the TSR to preserve on exit will

be ignored.  The Tenberry kernel will automatically determines how much

memory space your TSR requires.

Activation: The TSR can be signaled (activated) by means of a software

interrupt.  If a protected-mode TSR is signaled through a software

interrupt, this raises the usual issues of protected-mode interrupt

handling plus one additional major complication: another protected-mode

program will not share either the protected-mode Interrupt Vector Table

(IDT) or the protected-mode descriptor table (GDT or LDT) with the TSR.

This means that both protected-mode and real-mode programs will only be

able to signal the TSR through a real-mode interrupt, and will only be

able to pass data in low memory (or by using absolute physical

addresses).

I/O Conflicts: Use D32TsrSavePSP during installation to save the I/O

environment needed by the TSR.  Then use D32TsrSwitchPSP during TSR

execution to switch from the initial I/O environment to the TSR's I/O

environment on entry and back to the initial I/O environment on exit.

Removal: Removing a TSR from memory (uninstalling it) will cause

serious problems unless the interrupt handler chain and/or keyboard

handler is restored to its original state (so that you don't try to

signal the TSR after it has been removed from memory).  This is more

difficult in protected mode, and not possible in some protected-mode

environments.  Everything which has been installed has to be removed

before the TSR itself can be removed (all keyboard hooks, real-mode

callbacks and interrupt vectors); interrupt vectors in particular have

to be restored to their original state.  This can't happen if another

interrupt handler has been chained to your interrupt handler after the

TSR was installed, and it can't happen in certain DPMI environments

where the host does not permit real-mode interrupt handlers to be

removed.  At present there is no API function to check whether the TSR

can be safely uninstalled.  Use D32DPMIRmSetVector to restore the

original vectors for any real-mode interrupts hooked by your TSR.  If

all these operations succeed, use D32TsrExit to actually remove (and

exit from) the TSR.

This example shows how to write and install a protected-mode Terminate-

and-Stay-Resident program, and how to signal that TSR by means of a

software interrupt from either real mode or protected mode.

The example consists of the following files in the EXAMPLES\TSRDEMO

directory:

TSRDEMO.C      The main TSR program.

TSFEXECA.ASM   The assembly source for the execution context switcher.

TSRDEMOA.ASM   The assembly source for the TSR real-mode interrupt

handler.

SIGRM.C        The source for the real-mode signal program.

SIGRMA.ASM     The assembly source for the real-mode catcher when the

TSR exits.

SIGPM.C        The source for the protected-mode signal program.

MAKEFILE       Polymake-configured makefile to build all the example

executables using Microsoft VC 4.x and/or Watcom.