DIALING A NON-HAYES-COMPATIBLE MODEM

** Addendum to Appendix A of the Link Users Manual **

The following shows an example of a session you might write if your modem is

not Hayes Smartmodem compatible and you've determined that there is no Dialer

Prefix that will work.

The goal is to write a session that performs automatic dialing. However, this

still won't let you use the Dial Modem command to dial your modem. You'll have

to dial it by performing the session from the Sessions List or Playback

Sessions menu.

To write such a session, you need to know what commands you send to the modem

to dial a telephone number, and what responses the dialer will send back

indicating the status of the call. The following is an example of such a

session named "DIALXYZ" for use with an XYZ modem.  This modem requires

the phone number to be sent in a string "dial#<phone number>#, and the dialer

responses with either: "connect", "busy", "no answer" or "error".

Session Statement                           Explanation

-----------------------------------         ------------------------------------------------------

XYZ modem dialer			    comment for use with XYZ modem.

ASK Phone number?			    Displays a menu to enter phone number.

IF ERROR DONE			    If the user presses the ESC key, the session is done.

TIMEOUT 30				    Sets the wait timeout to 30 seconds.

LOOP BEGIN 5                              Sets the loop counter to 5.

SEND dial#<REPLY>#                     Sends to the modem the string "dial#" followed by the users

reply to the ASK followed by a "#".

IF WAIT connect<THEN>DONE              If the dialer sends back "connect" then the session is done.

IF WAIT busy<THEN> GOTO retry          If the dialer sends back "busy" the session will continue at the label err.

IF WAIT no answer<THEN> GOTO retry     If the dialer sends back "no answer" the session will contine at the label err.

WAIT error                             If the dialer sends back "error" the session contines with no error.

IF ERROR SEND hangup                   If the dialer does not send back one of the above after 30 seconds,

the session sends a hang up string.

retry:                                    Label statemnt.

LOOP END                                  If loop counter <5 jumps up to statement after LOOP BEGIN.

DISPLAY Unable to connect                 After 5 retries, a message is displayed.

DONE                                      Session is done.

Modify this example session to create a session tailored for your

modem's requirments.  Then you can even assign a DESQview keyboard

macro to one of the function keys to playback that session.

Example: {Learn {F10} "Dial XYZ modem"}

{F1}pDIALXYZ{Enter}

{Finish}

The above macro for the F10 key will playback "F1" key, "P",

"DIALXYZ" in the fill-in, then Enter key.