SUPERVGA AND EXTRA PRINTER GRAPHICS USING FTN77/486

(This note supplied with version 2.75 of FTN77/486)

Graphics  support  has  been  added to FTN77 for a number of SuperVGA

boards.  Also, support has been extended  from  16  colour  modes  to

cover  256  colour  modes.  Only  4-plane  16  colour modes and 8-bit

packed pixel 256  colour  modes  will  be  supported.  There  are  no

plans to support 2 or 4 colour modes on any adapter.

Three new powerful features have been added:

o    Auto recognition of graphics board

o    An  inquiry  routine  to  return supported modes with resolution

and colour information has been added

o    A  parametric  mode  change  routine  has  been  added.   Simply

specify  the  resolution and the number of colours and if a mode

is supported for your board then that mode is entered.

The need for display memory banking

A single 64k segment is allowed  in  the  PC  memory  map  for  video

memory.  However,  substantially  more than this is required for some

video modes.  For example, a  1024x768x256  colour  mode  requires  a

memory  space  of  768k.  The  solution  to this problem is to divide

the display memory in to banks and `bank in' sections  of  it  in  to

the  PC's  memory  space  as required.  This has a marginal effect on

performance  in  most  cases  but  for  some  boards,  notably   VESA

compliant,  the  effect can be noticeable.  Note that a diagonal line

may  require  12  -  13  bank  switches.  DBOS,  for   this   reason,

maintains  a  record  of  the current 64k graphics bank and will only

switch to a new bank if it is different  than  the  old.  This  is  a

good   scheme,   but   has   the  disadvantage  that,  under  special

circumstances, the DBOS record of the current  graphics  bank  number

may  become  invalid  resulting  in graphics drawing occurring at the

wrong screen position.  This occurs when BIOS is  used,  directly  or

indirectly  to  calculate  a  new  screen  address,  for example when

character output to the screen via  DOS  or  BIOS  is  performed.  In

this  case  use  the special routine provided, RESTORE_GRAPHICS_BANK@

after any such output.  If in doubt, use the routine  -  it  is  very

quick  and  will  guarantee  that  DBOS  and  the  board use the same

graphics bank.  There is no need  to  use  RESTORE_GRAPHICS_BANK@  if

the mode is unbanked.

The  need  for  graphics banking is mode specific.  Some modes do not

require it.  It is possible to calculate  whether  there  is  a  need

for  graphics  banking.  For all FTN77 supported modes perform one or

the other of these calculations:

16 Colour modes

MapSize=(xResolution/8)*yResolution

Copyright (C) Salford Software Ltd 1991                  Page 1 of 10

FTN77 and SUPERVGA

256 Colour modes

MapSize=xResolution*yResolution

These calculations give the display memory map size  in  bytes,  note

NOT the size of the display memory.

If MapSize exceeds 64kbytes then banking is required:

e.g.  800x600x16 colours

MapSize  =  (800/8)*600  =  60,000  bytes  <  64kbytes,  banking  not

required

e.g.  640x400x256 colours

MapSize= 640x400 = 256,000 bytes > 64kbytes, banking required.

Considerations when using 16 colour modes

On the  EGA  card  the  palette  registers  determined  the  physical

colours.  Each  gun  was  allocated  2  bits  thus giving 4 levels of

intensity per gun  (OFF  1/3  2/3  FULL)  and  hence  6  bits  or  64

possible  different  colours  in  all.  On the VGA card, the physical

colours are  specified  by  the  video  DAC  registers.  The  palette

registers  now  extend  to  8 bits and give the video DAC register to

use for  that  particular  `logical'  colour.  To  make  this  scheme

compatible  with  the EGA card, the first 64 video DAC registers will

produce the all the possible EGA  physical  colours  in  the  correct

order.  As  with  the  256  colour  modes, be very careful about what

you assume with regard to  the  video  DAC  contents.  The  first  64

video  DAC  values  will  be  the same in all 16 colour modes, unless

you have changed them.  The other video DAC registers  may  be  unset

or  contain  random  values.  Even if they contain sensible values on

your card, do not assume that they will do on others.

Considerations when using 256 colour modes

Physical colours  are  set  using  the  video  DACs.  The  16  colour

palette  registers  are supposedly bye passed in these modes.  Do not

rely on this however, on some boards setting the  16  colour  palette

registers  CAN  alter  the  screen colours in 256 colour modes.  Some

cards do not properly initialise the  video  DACs  when  not  in  the

standard  320x200x256  colour VGA mode.  They may also have different

video  DAC  settings  for  every  graphics   mode   on   the   board.

Certainly,  do  not  assume  that different cards have the same video

DAC settings in equivalent modes.

A  new  routine,  LOAD_STANDARD_COLOURS@,  has  been  provided.  This

will  load  the video dac values with the standard 300x200x256 colour

VGA video dac values.  Use this routine only  in  256  colour  modes,

the standard 16 colour video dac values are different.

Currently supported boards use the following chipsets

+ AHEAD Wizard/Deluxe              AHEAD V5000

Page 2 of 10                  Copyright (C) Salford Software Ltd 1991

FTN77 and SUPERVGA

+ AST VGA Plus                     Western Digital chipset

+ ATI VGA WONDER                   ATI I8800

* Boca Research 1024VGA            C&T 82C452

* Chips & Technologies             C&T 82C452

+ Cirrus                           Cirrus 510/520 610/620

+ EVEREX                           Trident

GENOA SuperVGA 6000 Series       GENOA

+ Headland Technology VIDEO 7      Headland

+ HP                               Paradise chipset

+ MaxLogic MaxVGA                  Cirrus 510/520 610/620

+ Oaktech OTI-067                  OAK Technologies

+ ORCHID ProDesigner               Tseng ET3000

ORCHID ProDesigner II            Tseng ET4000

+ Phoenix                          Paradise chipset

+ Quadram Spectra                  Tseng ET3000

* Sigma VGA Legend                 Tseng ET4000

+ Sota VGA/16                      Tseng ET3000

+ STB VGA EXTRA                    Tseng ET3000

STB Powergraph                   Tseng ET4000

+ Tecmar VGA ADGM                  Tseng ET3000

+ TRIDENT                          Trident 8800CS, 8800BR (Note)

+ Truetech Hires VGA               Tseng ET3000

+ TSENG ET3000                     Tseng ET3000

* TSENG ET4000                     Tseng ET4000

+ Western Digital                  Paradise

+ Willow Publishers' VGA           Tseng ET3000

# VESA compliant boards

+ - chip set and board untested with FTN77 - may not work.

* - untested on this board, but works with FTN77 on different board.

# - tested on OLIVETTI OVC, using C&T 82C452 chipset - works

Note: Trident 8800BR

Only the mode not requiring bank switching is supported i.e.

mode 5Bh 800x600x16 colours

Copyright (C) Salford Software Ltd 1991                  Page 3 of 10

FTN77 and SUPERVGA

SUBROUTINE GET_GRAPHICS_MODES@(XRES,YRES,COLOURS,MODE,BANKED)

INTEGER*2 XRES(*),YRES(*),MODE(*)

INTEGER*4 COLOURS(*)

LOGICAL*2 BANKED(*)

Returns  arrays  containing  the horizontal and vertical resolutions,

the number of colours, the corresponding mode  and  a  logical  array

stating  whether  or  not the mode uses `banked' memory.  Ensure that

your arrays are large enough to hold  the  returned  information.  An

array length of 20 will usually be safe.

This  routine has the side effect of configuring FTN77 to your board.

SUBROUTINE GRAPHICS_MODE_SET@(IXRES,IYRES,NCOLOURS,ERROR_CODE)

INTEGER*2 IXRES,IYRES,ERROR_CODE

INTEGER*4 NCOLOURS

Finds  a  graphics  mode  with  IXRES  horizontal  resolution,  IVRES

vertical  resolution  and  the  number  of  colours  NCOLOURS.  If no

suitable mode exists on your board or the specified  mode  could  not

be  entered  then  ERROR_CODE  is  non zero.  Always check ERROR_CODE

after calling this routine.  Reasons for  failure  to  enter  a  mode

might be:

1.  Insufficient memory on the board for the mode.

Some modes require 1Mb of graphics memory.

2.  You have an early version of the board that

does not support the mode.

3.  You have an incorrect monitor type. Some

boards detect the monitor type and will not

enter a mode that needs a monitor different

than that which is attached.

4.  FTN77 does not support the mode. Only modes

returned by GET_GRAPHICS_MODES@ are supported.

*************************************************************

* NOTE entering a mode for which your monitor is unsuitable *

*      is likely to damage your monitor and graphics board. *

*      It is YOUR responsibility to check with the board    *

*      and monitor manufacturers for suitability.           *

*************************************************************

SUBROUTINE RESTORE_GRAPHICS_BANK@

If  any  action has been taken which directly or indirectly uses BIOS

to calculate a screen  address  whilst  in  graphics  mode  then  the

current   graphics  64k  bank  number  maintained  by  DBOS  will  be

invalidated.  This may result in graphics drawing  occurring  at  the

wrong   place   on   the   screen.   To   remedy   this  effect  call

RESTORE_GRAPHICS_BANK@ after any such action.  This  routine  is  not

needed  if  the  current  graphics  mode  is  any of the standard VGA

graphics  modes  ie  640x350x16  colours,   640x480x16   colours   or

320x200x256 colours.  This routine is needed, for example,

Page 4 of 10                  Copyright (C) Salford Software Ltd 1991

FTN77 and SUPERVGA

1) Any operation that involves standard output to the

screen eg PRINT, WRITE, READ or PAUSE statements. Use

of COU@, COUA@, COUP@, SOU@ or SOUA@. Use of NEWLINE@.

Use of PRINT_I1@, PRINT_I2@, PRINT_I4@, PRINT_HEX1@,

PRINT_HEX2@, PRINT_HEX4@. Use of READ_EDITED_LINE@.

2) Direct or indirect use of BIOS to set the screen position

eg, SET_CURSOR_POS@.

3) Use of BIOS to effect screen output eg writing characters

or plotting points. Use the DBOS facilities provided

instead.

SUBROUTINE GET_DACS_FROM_SCREEN_BLOCK@(BUFFER,PALETTE,DACS,

+IERR)

INTEGER*1 PALETTE(17),DACS(3,256)

INTEGER*2 IERR

INTEGER*4 BUFFER

This  routine  returns  the PALETTE and DAC information stored with a

screen block.  BUFFER is the pointer to the screen  block.  If  there

is no palette or DAC information stored then IERR is non zero.

Screen  blocks  created  with  CREATE_SCREEN_BLOCK@  are  stored with

default palette and DAC information.

Screen blocks created by loading PCX files with  PCX_TO_SCREEN_BLOCK@

return  a  different set of palette information.  To see the image at

its    best    use     the     palette     values     returned     by

GET_DACS_FROM_SCREEN_BLOCK@    and   set   the   video   DACs   using

SET_VIDEO_DAC_BLOCK@.  You may use the  palette  values  returned  by

loading  a  PCX  file  but the image may lack the tonal graduation of

the palette/DAC values returned by this routine.

256 colour screen modes  bypass  the  palette  registers  so  the  16

colour palette information is irrelevant.

SUBROUTINE SET_VIDEO_DAC_BLOCK@(IFIRST,NREGS,IRGB)

INTEGER*2 IFIRST,NREGS

INTEGER*1 IRGB(3,NREGS)

Sets  an  NREGS  block  of  video DACs starting at video DAC register

IFIRST.  IRGB is a 2 dimensional array  containing  RGB  information.

IFIRST      first video DAC to set

NREGS       number of video DAC registers to set. Each video

DAC register requires an RGB triple.

IRGB(1,i)   Red gun level for video DAC register i

IRGB(2,i)   Green gun level for video DAC register i

IRGB(3,i)   Blue gun level for video DAC register i

RGB values should be in the range 0-63.

Copyright (C) Salford Software Ltd 1991                  Page 5 of 10

FTN77 and SUPERVGA

SUBROUTINE GET_VIDEO_DAC_BLOCK@(IFIRST,NREGS,IRGB)

INTEGER*2 IFIRST,NREGS

INTEGER*1 IRGB(3,NREGS)

Reads  an  NREGS  block  of video DACs starting at video DAC register

IFIRST.  IRGB is  a  returned  2  dimensional  array  containing  RGB

information.

IFIRST      first video DAC to read

NREGS       number of video DAC registers to read. Each video

DAC register returns an RGB triple.

IRGB(1,i)   Red gun level for video DAC register i

IRGB(2,i)   Green gun level for video DAC register i

IRGB(3,i)   Blue gun level for video DAC register i

RGB values are in the range 0-63.

SUBROUTINE LOAD_STANDARD_COLOURS@

Loads  the  video DACs with the same standard 256 colour values as in

the 320x200x256 colour standard VGA mode.  Use this routine  only  in

256  colour  modes,  the video DAC values for the 16 colour modes are

different.

Video DAC values  0 to 15 are the standard EGA colours 0 - 15

16 to 31 are a grey scale of increasing intensity

There follows three 72 colour  structures  of  decreasing  intensity.

Each  72 structure itself is three 24 colour structures of decreasing

saturation.  The 24 colour structures can be  regarded  as  a  colour

wheel  going  from  blue  to red to green and back to blue again with

all the intermediate hues.

The effect is like this:

high       medium       low

saturation  saturation  saturation

high

intensity      32-55        56-79      80-103

medium

intensity     104-127      128-151    152-175

low

intensity     176-199      200-223    224-247

colours 248 to 255 are black.

Graphics Drawing modes

DBOS.LIB provides support for two basic drawing  modes  for  graphics

primitives  REPLACE  and XOR drawing.  With the extension of graphics

primitives to 256 and the forthcoming extension  to  16  bit  colours

the existing scheme has been extended.

Page 6 of 10                  Copyright (C) Salford Software Ltd 1991

FTN77 and SUPERVGA

REPLACE drawing

Use the pixel colour you require unmodified.

ie

CALL DRAW_LINE@(0,0,639,479,15)

will  draw  an intense white line diagonally across a standard screen

in 640x480 mode.

XOR drawing

The exact method used to obtain XOR drawing varies  with  the  number

of bit planes in the current graphics mode.

16 Colour (4 bit planes)

Add 128 (Z'80') to the basic colour required.

256 Colour (8 bit planes)

Add 32768 (Z'8000') to the basic colour required.

The scheme is simpler than at first appears.

e.g.

CALL DRAW_LINE@(0,0,639,479,OR(Z'80',15))

CALL DRAW_LINE@(0,0,639,479,OR(Z'8000',15))

will  draw  an  intense  white  line  in  xor drawing mode diagonally

across  the  screen  in  6480x480  in  a  16  and  256  colour   mode

respectively.

XOR  mode  drawing  is  necessary  in  extended modes so that a cross

hair mouse cursor can be  drawn  as  mouse  drivers  do  not  support

these modes.  An example program is given below.

PROGRAM MOUSEGIN

C  C  This  program  will  demonstrate  using a mouse to move a cross

around the C screen. C CTRL & BREAK to exit the program. C

EXTERNAL M_TRAP,B_TRAP

INTEGER*2 MX,MY,X,Y,XMAX,YMAX

INTEGER*4 Q

COMPLEX*16 LAB

COMMON /C1/ MX,MY

COMMON /C2/ X,Y,XMAX,YMAX

COMMON /C3/ LAB

C Get address of label to jump to on BREAK

CALL LABEL@(LAB,*2)

C Trap mouse activity

CALL SET_TRAP@(M_TRAP,Q,4)

C Trap CTRL BREAK combination

CALL SET_TRAP@(B_TRAP,Q,0)

C Set mouse interrupt to trap mouse movement

Copyright (C) Salford Software Ltd 1991                  Page 7 of 10

FTN77 and SUPERVGA

CALL SET_MOUSE_INTERRUPT_MASK@(1)

C Enter VGA graphics mode

CALL VGA@

XMAX=639

YMAX=479

C Get current mouse position

CALL GET_MOUSE_POS@(MX,MY)

X=MX

Y=MY

C Draw cross at that point

CALL GIN

C Loop until mouse position changes 1 CONTINUE

IF ((X.EQ.MX).AND.(Y.EQ.MY)) GOTO 1

C Remove the old cross

CALL GIN

X=MX

Y=MY

C Draw new cross,

CALL GIN

C READ '(A)' C and loop.

GOTO 1

C Program will get here on CTRL BREAK C Go to text  mode  and  finish

2 CALL TEXT_MODE@

END

INTERRUPT SUBROUTINE M_TRAP

C Subroutine called when mouse is moved

INTEGER*2 MX,MY,MASK

COMMON /C1/ MX,MY

CALL GET_MOUSE_EVENT_MASK@(MASK)

C Get the new mouse position

CALL GET_MOUSE_POS@(MX,MY)

END

INTERRUPT SUBROUTINE B_TRAP

C Subroutine called when CTRL BREAK is pressed

COMPLEX*16 LAB

COMMON /C3/ LAB

C Jump to label in main program

CALL JUMP@(LAB)

END

SUBROUTINE GIN

C Subroutine to draw the cross on the screen

EXTERNAL DRAW_LINE@

COMMON /C2/ X,Y,XMAX,YMAX

INTEGER*2 X,Y,XMAX,YMAX,Y1

C  Xor  mode  drawing is obtained by adding 128 to the desired colour

C Draw the Vertical line

CALL DRAW_LINE@(X,0,X,YMAX,5+128)

C Draw the Horizontal line

Y1=(INTL(YMAX)*INTL(Y))/199

CALL DRAW_LINE@(0,Y1,XMAX,Y1,5+128)

RETURN

END

Extra graphics modes supported

Page 8 of 10                  Copyright (C) Salford Software Ltd 1991

FTN77 and SUPERVGA

In addition to the boards noted above in the 2.51  release  note  the

Trident  8900  and  the  ET4000AX  Rev  E  are  now  supported.  This

support includes a  1280x1024x16  colour  mode  on  the  Tseng  based

boards.

The  GENOA  7800 board has been tested (with 1280x1024x16 colour 1Mb)

Support and detection of the OAK OTI-037,  OTI-067  and  the  Trident

8800CS  as  been  corrected  and  improved.  Display memory detection

has been provided for Tseng ET4000 boards  and  TVGA  8800  and  TVGA

8900 boards.

Printer support

DBOS  has  enhanced  its  graphics capabilities by the provision of a

PCL LJ2 driver. This driver should work with  all  HP  PCL  printers.

The  driver  has  been  tested  with  the  HP  DeskJet 500 and the HP

LaserJet 3+

Two new routines have been added.  These routines  should  be  called

before   the   opening   of   the   graphics   printer   with  either

OPEN_GPRINT_DEVICE@, or  OPEN_GPRINT_FILE@  to  specify  the  printer

type and resolution required.

SUBROUTINE SELECT_DOT_MATRIX@(ITYPE,IXRES,IYRES)

INTEGER*2 ITYPE,IXRES,IYRES

This makes the provision of arbitrary size printer maps possible.

Input arguments:

ITYPE       Printer type. Should be 0.

IXRES       Horizontal resolution of printer map

IYRES       Vertical resolution of printer map

SUBROUTINE SELECT_PCL_PRINTER@(ITYPE,PAPER_SIZE,IDPI,IXRES,

+                                 IYRES)

INTEGER*2 ITYPE,IDPI,IXRES,IYRES

CHARACTER*(*) PAPER_SIZE

Specifies  the printer to be of PCL type and specifies the resolution

of the printer map via a character string PAPER_SIZE and a  dots  per

inch specifier IDPI.

Note  that,  in  contrast  to  SELECT_DOT_MATRIX@  the  resolution in

pixels is an OUTPUT argument.  Do not assume that these  values  will

remain fixed as they may vary for compatibility with new devices.

Input arguments:

ITYPE       Printer type. Should be 2 or 0.

PAPER_SIZE  Should be one of 'LETTER', 'LEGAL',

'EXECUTIVE', or 'A4'

IDPI        Pixel size in dots per inch, should be one

of 75, 100, 150, 300

Copyright (C) Salford Software Ltd 1991                  Page 9 of 10

FTN77 and SUPERVGA

Output arguments:

IXRES       Horizontal resolution of printer map

IYRES       Vertical resolution of printer map

Page 10 of 10                 Copyright (C) Salford Software Ltd 1991