LAN Driver Statistics        Introduction

By comparing information about the LAN drivers installed on

the NetWare server, you can tell which cabling system is

handling the most traffic.

If errors occur frequently on a high-traffic system, you

may want to switch some of the workstations on the busy

system to a new cabling system or to a less busy cabling

system.

For information on viewing LAN driver statistics, see "

Viewing LAN Driver Statistics" in Chapter 6 of Supervising

the Network.

The generic statistics common to most of the current

drivers are listed in the following tables:

Table B-1, "LAN Driver statistics common to all topologies,"

Table B-2, "Generic statistics for Ethernet drivers,"

Table B-3, "Generic statistics for RX-Net(tm) drivers,"

Table B-4, "Generic statistics for Token-ring drivers,"

Table B-5, "Generic statistics for baseband pcn2l drivers,"

CUSTOM STATISTICS vary, depending on the LAN driver installed. For statistical

information about third-party drivers, check the documentation that comes with

the driver.  In most cases the programmer put this information in to give added

statistical information that is in most cases only useful to the programmer.

NOTE: If Card is not listed in the custom statistics section contact

manufacturer/Driver writer of LAN Card in server for custom statistics

documentation.

Table B-6, "Custom statistics for Ethernet drivers,"

Table B-7, "Custom statistics for RX-Net(tm) drivers,"

Table B-8, "Custom statistics for Token-ring drivers,"

Table B-9, "Custom statistics for IBM baseband pcn2l drivers,"

----------------------------------------------------------

Table B-1, "LAN Driver statistics common to all topologies,"

Statistics for LAN Drivers

LAN Driver statistics

Statistic Explanation

Driver Name                      The driver name and parameters

that correspond to the  hardware

settings on the network board.

Version                          The current version of the driver.

Node Address                     The station or node address of the

network board in the  NetWare

server.

Protocols                        The communication protocols bound

to the driver with BIND.

Network Address                  The network number assigned to the

cabling system the LAN  driver is

operating on. Appears only if the

IPX protocol has  been bound to

the board.

Generic Statistics

Total Packets Sent               The number of packets sent from

the NetWare server through  this

LAN driver. (By comparing this

figure with the figures for  other

LAN drivers, you can see which

driver is handling the  most

traffic.)  (Maintained by TSM).

Total Packets Received           The number of packets received by

the NetWare server since  it was

last booted (includes file service

requests, packets  routed to

another network, and packets sent

to other IPX  sockets in the

NetWare server). (Maintained by TSM).

No ECB Available Count           A counter that increments when a

device sends a packet to  your

NetWare server, but no packet

receive buffer is available.  The

NetWare server allocates more

packet receive buffers  after each

incident until it reaches its

maximum limit  (configured with a

SET parameter). If you are using

an EISA or  microchannel bus

master board (such as the NE3200),

you  will probably need to

increase both the minimum and

maximum number of packet receive

buffers. See the SET parameters.

(Maintained by TSM).

Send Packet Too Big Count        A counter that increments when the

NetWare server tries to  transmit

a packet that is too large for the

hardware to handle. (Maintained by

TSM).

Reserved                         This field is reserved for use by

the msm, but should be initialized

to zero.

Receive Packet Overflow Count    The HSM (lan driver) may user this

counter to indicate the number of

times the adapter's receive buffers

overflowed causing subsequent

incoming packets to be discarded.

(?? A counter that increments when

the NetWare server receives  a

packet that is too big to store in

a cache buffer. This happens

rarely, unless you are running a

software program that does  not

negotiate packet size. Contact the

vender for an updated  version of

the software. ??). (Maintained by

the HSM).

Receive Packet Too Big Count     The TSM increments this counter

whenever a packet is received that

is too large for the provided

receive buffer(s). (Maintained by

TSM).

Receive Packet Too Small Count   Some TSMs increment this counter if

a packet is received that is too

small for media definitions.

Currently only the RX-Net TSM

Maintains this counter.

Send Packet Miscellaneous        A counter that increments when

Errors                           errors occur with send packets.

(Maintained by HSM).

Receive Packet Miscellaneous     A counter that increments when

Errors                           occur with receive

packets. (Maintained by HSM).

Send Packet Retry Count          A counter that increments when the

NetWare server tries to  send a

packet but fails because of a

hardware error. Based on  the

number of retries in the retry

counter, the NetWare server  will

try to send the packet until it is

sent or the retry setting is

reached. If you receive a large

number of these errors, use  LOAD

LAN driver to increase the maximum

retry count, or  check the cabling

and workstation hardware.

(Maintained by HSM).

Checksum Errors                  The HSM may use this counter to

increment when the checksum byte at

the  end of the  packet does not

match the sum of the bytes

contained in the packet. This

indicates a data error. (Maintained

by HSM).

Hardware Receive Mismatch count  A counter that increments when the

packet length received  by the

hardware and the length specified

by the packet do  not match.

Currently only the Ethernet TSM

maintains this counter. (Maintained

by the TSM).

Total Send OK Byte Count Low     The number of bytes including low

level headers successfully

transmitted.  (Maintained by the

MSM).

Total Send OK Byte Count High    Upper 32-bits of the Total Send OK

Byte Count Low.  Basically this

will increment to 1 when the above

counter reaches 4 gigabytes

(Maintained by the MSM).

Total Receive OK Byte Count Low  The number of bytes including low

level headers successfully

received.  (maintained by the MSM).

Total Receive OK Byte Count High Upper 32-bits of the Total Receive

OK Byte Count Low.  Basically this

will increment to 1 when the above

counter reaches 4 gigabytes.

(maintained by the MSM).

Total Group Address Send Count   The number of packets transmitted

with a group or multicast

destination address. (maintained by

MSM).

Total Group Address Receive      The number of packets received

Count                            with a group or multicast

destination address. (maintained by

MSM).

Adapter Reset Count              The number of times the adapter

was reset because of  internal

failures or other calls to the

Driver Reset routine.  (maintained

by the HSM).

Adapter Operating Time Stamp     The time stamp indicating when the

adapter last changed  operational

state (such as load, shutdown, or

reset). (Maintained by the MSM).

Adapter Queue Depth              The number of transmit packets

(transmit ECBs) that are queued for

the  adapter.  This is an

indication of throughput overload

on  transmits. (maintained by the

TSM).

---------------------------------------------------------------

Table B-2, "Generic statistics for Ethernet drivers,"

Generic statistics for Ethernet drivers (Using ETHERTSM.NLM).

Statistic Explanation

Send OK Single Collision Count   The number of frames involved in a

single collision but are

subsequently transmitted

successfully. When the Ethernet

controller detects a collision, it

backs off and then will retry the

transmission.

Send OK Multiple Collision Count The number of frames involved in

more than one collision but

transmitted successfully. This

happens when the Ethernet

controller had to back off more

than once due to collisions.  See

also TxAbortExcessCollisions.

Send OK But Deferred             The number of frames whose

transmission was delayed  because

of a busy medium. This happens

when another  station is

transmitting on the wire when the

adapter receives  the command to

transmit a packet.

Send Abort From Late Collision   The number of transmits that had a

collision occur after 512 bits of

the packet had been transmitted.

This can be caused by faulty

adapters, faulty network

equipment, cable lengths not to

specifications, or faulty

termination.

Send Abort From Excess           The number of transmits that were

Collisions                       aborted because of too  many

collisions. This usually indicates

that a board in the network is bad

or jabbering.  This condition could

also possibly occur in very heavy

traffic  conditions.

Send Abort From Carrier Sense    The number of transmits aborted

because of loss of carrier  sense

while transmitting without any

collisions occurring. This usually

indicates a bad board in the

network (shorting the cable),

faulty cable, un-terminated cable

or faulty repeater.

Send Abort From Excessive        The number of transmits aborted

Deferral                         because of excessive  deferrals.

This is usually caused by a faulty

adapter or repeater in the network

that is jabbering on the wire.

This condition could also possibly

occur in very heavy traffic

conditions.

Receive Abort From Bad Frame     The number of received frames that

Alignment                        were misaligned. This occurs when

the number of octets in the frame

is not correct or it does not pass

the FCS check.  These  bad packets

are usually caused by a faulty

adapter or repeater in the system.

They can also be caused by a

collision.

-------------------------------------------------------

Table B-3, "Generic statistics for RX-Net(tm) drivers,"

Generic statistics for RX-Net(tm) drivers (Using RXNETTSM.NLM).

Statistic Explanation

No Response to Free Buffer       A counter that increments each

Enquiry                          time there is no response  from

the receiving node to Free Buffer

Enquiry.

Network Reconfiguration Count    A counter that increments each

time a Network  Reconfiguration

occurs.

Invalid Split Flag in Packet     A counter that increments each

Fragment                         time the Split Flag in the  packet

fragment is not the value

expected. Packet fragments

received out of order can cause

this value to increase.

Orphan Packet Fragment Count    A counter that increments each

time a packet fragment is

received that is not a part of a

previously received packet and

therefore cannot be appended.

Receive Packet Timeout          A counter that increments each

time a received packet times  out

while waiting for the rest of the

packet fragments to arrive.

Free Buffer Enquiry NAK Timeout A counter that increments each

time a transmit packet times  out

while waiting for an

acknowledgment to a Free Buffer

Enquiry from the receiving node.

Total Send Packet Fragments OK  A counter that increments each

time a packet fragment is sent

successfully. This number will be

high because packets are made up of

multiple fragments. For example a

4K packet is actually eight 512

byte packet fragments.

Total Receive Packet Fragments  A counter that increments each

OK                              time a packet fragment is

received successfully. This number

will be high because packets are

made up of multiple fragments. For

example a 4K packet is actually

eight 512 byte packet fragments.

---------------------------------------------

Table B-4, "Generic statistics for Token-ring drivers,"

Generic statistics for TOKEN-RING drivers (Using TOKENTSM.NLM).

Statistic Explanation

AC Error Counter             This counter increments when a ring

station receives a "Standby Monitor

Present" MAC frame with the A/C bits

in the Frame Status field equal to

zero without first receiving an

"Active Monitor Present" MAC frame.

Abort Delimiter              This counter increments when a ring

station transmits an abort delimiter.

An abort delimiter is transmitted when

a ring station receives a frame in

which the token bit of the access

control field is set to show "Token"

and not "Frame."  A ring station can

also transmit an abort delimiter in an

internal hardware error has occurred.

Burst Error Counter          This counter increments when a ring

station detects the absence of "five

half-bit times" (a burst-five error).

Other stations will detect a burst-

four error followed by idles.

Frame Copied Error Counter   This counter increments when a ring

station recognizes (receives or

repeats) a frame addressed to its

specific address and detects that the

FS field A bits are set to 1

indicating a possible line hit or a

duplicate address.

Frequency Error Counter      This counter is incremented when the

frequency of the incoming signal

differs from the expected frequency by

more than that specified in Section 7

(IEEE Std 802.5-1989).

Internal Error Counter       This counts the times a ring station

has a recoverable internal error,

which means a ring station is probably

marginal.

Last Ring Status             This code changes each time the ring

status changes.  Status codes, are

reported by the physical hardware.

See the IBM Token-Ring Network

Architecture Reference for the Status

Code, Function and Meaning.

Line Error Counter           This counter increments when a frame

or token is repeated by the ring

station.  A frame is repeated when a

"Frame Check Sequence" error occurs or

a code violation exists between the

starting and ending delimiters of the

frame.

Lost Frame Counter           This counts instances when a ring

station transmits a frame that does

not return to the station.  The active

monitor sends a new token.

Token Error Counter          This counter is incremented when a

station acting as the active monitor

recognizes an error condition that

needs a token transmitted. This occurs

when the TVX timer expires.

Up Stream Node High Dword:   The first 8 digits of the Up Stream

node address of next node up stream on

the ring.

Up Stream Node Low Dword:    The next 4 digits of the Up Stream

node address of next node up stream on

the ring.

Last Ring ID                 This contains the value of the local

ring.

Last Beacon Type             This contains the value of the last

beacon type.

---------------------------------------------

Table B-5, "Generic statistics for baseband pcn2l drivers,"

Generic statistics for PCN2L IBM BASEBAND drivers (Using PCN2LTSM.NLM).

Statistic Explanation

Send OK Single Collision Count   The number of frames involved in a

single collision but subsequently

transmitted successfully.

Send OK Multiple Collision Count The number of frames involved in

more than one collision but

subsequently transmitted

successfully.

Send OK But Deferred             The number of frames whose

transmission was delayed  because

of a busy medium. This happens

when another  station is

transmitting on the wire when the

adapter receives  the command to

transmit a packet.

Send Abort From Excess           The number of transmits that were

Collisions                       aborted because of too  many

collisions. This is usually caused

by a faulty adapter in  the

system. It can also occur under

very heavy traffic  conditions.

Send Abort From Carrier Sense    The number of transmits aborted

because of loss of carrier  sense

while transmitting any collisions.

This is usually caused  by a

faulty adapter in the system,

faulty cabling, an  unterminated

cable, or a faulty repeater.

Receive Abort From Bad Frame     The number of received frames that

Alignment                        were misaligned. These  bad

packets are usually caused by a

faulty adapter or repeater  in the

system. They can also be caused by

a collision.

-----------------------------------------------------

Table B-6, "Custom statistics for Ethernet drivers,"

NE2000.LAN, NE2.LAN, and NE2_32.LAN, NE1000.LAN

UnderrunErrorCount               This counter increments when the

RAM buffer on the network board is

full; the board cannot accept any

more packets until the RAM buffer

is cleared.

TransmitTimeoutCount             This counter increments when a

network board interrupts the file

server with the message that the

"send bit" is lost.  This is a

hardware problem caused by faulty

cabling, a bad network board, or a

missing terminator.

RxPagingErrorCount               This is a count of the errors that

occur when internal buffers on the

card are corrupted.

ReceiveFIFOOverrunErrorCount     This counter increments when an

incoming packet causes an overflow

because FIFO was not serviced.

ReceiverMissedPacketCount        This counter increments when a

packet is sent to a network board

that cannot accept the packet

because all its receive buffers are

full.

GotNothingCount                  This counter increments when the

file server receives an interrupt

from a network board that is not

transmitting or receiving anything.

This is not serious.

UnsupportedFramePacketCount      This counter increments when a

packet is received by the lan

driver with a frame type that

hasn't been loaded for the given

card.

UnsupportedMulticastCount        This counter increments for each

multicast packet received by the

card that is not registered with

the driver.

BackToBackSendCount              This counter increments each time

the driver can buffer a send packet

onto the network board while the

board is sending a previous buffer.

Use this counter to track

congestion on the network board.

See also "EnqueuedSendsCount".

EnqueuedSendsCount               This counter increments when the

driver is unable to transmit a

packet and must put the packet in a

que until the transmitter is

available.  Use the counter to

track congestion on the network

board.  See also "BackToBack

SendCount".

In32BitModeCount (NE1000 only)   This counter increments if a

network board is ever found in 32-

bit mode. (Currently, workstations

run in 8-bit mode.) Occasionally,

older ne1000 boards have bad ships

that make the boards go into 32-bit

mode.  If you ever see this counter

incremented, you probably have one

of those older boards on the

network.

NE2100.LAN, NE1500T.LAN

HeartBeatError                   This counter increments when there

is a collision after transmission.

This function is also known as

heartbeat or SQE (Signal Quality

Error) test.  This counter would

indicate a hardware problem.

MemoryTimeout                    This counter increments when there

is contention on the bus.  This

counter incrementing is indicative

of a bus contention problem.  If

this counter is incrementing then

more than likely you have either

have multiple NE2100 or NE1500T

cards in the server or another Bus

Mastering Device in the server (ie.

disk channel).

TxBabblingError                  This counter increments when there

is excessive length in the transmit

buffer.  It will increment after

1519 data bytes have been

transmitted from the buffer.  It

indicates that the transmitter has

been on the channel longer than the

time required to send the maximum

length packet.  This counter would

indicate a hardware NIC card

problem in the server.

TxUnderflowError                 This counter increments when

something else on the bus takes

control of the bus while the LAN

driver is putting the data on the

wire.  The packet would then have

to be retransmitted.

TXBufferError                    This counter increments when there

is a problem with the transmit

buffer.  This counter will also

commonly increment when

TxUnderflowError increments.  This

counter could be indicative of

hardware problem in the server.

RxECBsOver16MegCount             This counter will increment when a

TxECBsOver16MegCount             transmit or receive occurs and the

driver has had to use the reserved

buffers below 16 meg double buffer

the ECB because the card can't

access above 16 Meg of memory

(physical limitation).  The NE1500T

and NE2100 cards are not able to

access memory above 16 MB.

Therefore, if the operating system

issues an Event Control Block (ECB)

with an address above 16MB of

memory, the board uses some of the

reserved buffers below 16MB to

queue the request.  These are not

errors.  They are only keeping

track of how many ECBs were

redirected to the buffers below

16MB.   In many cases, this counter

can be as high as the total packets

sent and received.  Because of this

double buffering that occurs it

does cause a performance hit.  Any

board that is busmastering or uses

DMA that is not a 32 bit adapter

will potentially experience a

performance hit if you have more

than 16 MB of ram.

PacketUsed2ECBs                  This counter will increment if the

Server Maximum Physical Receive

Packet Size is set to 1514 (default

for 3.11 servers, it can be changed

in the STARTUP.NCF), and we need an

extra 4 bytes to receive a near

full size packet to include the CRC

on the end of the packet we will

use 2 ECBs.  This is a slight

performance hit to use 2 ECBs

instead of one but will function

properly. NOTE: Netware 3.12 and

4.X default the Maximum Physical

Receive Packet Size to 4202.

NE3200.LAN

Transmit Retry Failure:       This counter increments when the

driver is unable to transmit a packet

a reset number of times.  This may

indicate a hardware problem.

Tx Clear To Sends Errors:     Attempt to catch an 82586 errata.

There are some conditions when the

clear to send signal from the 82586

chip is incorrect.  This indicates the

number of times the corrective code on

the adapter was executed to work

around this condition in the 82586.

Tx DMA Underrun Errors:       Attempt to catch an 82586 errata.

This can occur when contention between

the BMIC, 80186 and 82586 occurs on

the adapter.  The 82586 thinks it did

not get all of the packet for

transmission. The transmit operation

must be retried in this case.  This

indicates the number of times the

corrective code on the adapter was

executed to work around this

condition.

Rx DMA Overrun Errors:        Attempt to catch an 82586 errata.  If

two packets are received back to back

at near the minimum ethernet

inter-frame spacing of 9.6

microseconds then the chip may report

an overrun.  If so, the frames are

lost by the chip and the source will

have to re-transmit.

Rx Packet Slide Errors:       Attempt to catch an 82586 anomaly.  In

some strange conditions, the 82586 may

get off by two bytes in the receive

packet descriptors. This number

indicates the number of times this

condition occurred. The sending

station must retransmit the packet in

this case.

Rx Dummy RCB Used Errors:     Attempt to catch an 82586 errata.  In

some cases, the 82586 may attempt to

receive data into a non-existent

receive buffer off the end of its

receive buffer list.  To catch for

this condition and avoid internal data

corruption, a dummy receive buffer is

added to the end of the list.  This

variable counts the number of times

the 82586 attempted to write into the

dummy buffer.

Internal Adapter Reset:       This is count of the number of resets

in that occurred on the adapter (by

the 80186) due to failures on the

adapter.  This counter increments when

the software corrects itself for minor

problems or if the card gets into an

unknown state. This counter commonly

increments.  Under normal conditions,

more of these errors should occur

during idle time than when the driver

is busy.  For this counter to warn you

of potential hardware problems, you

must receive thousands of these errors

when the network is busy.

Mondo Fragment Length Errors: An NLM on the Server has passed the

NE3200 driver an ECB whose logical

memory address we couldn't change to a

physical memory address. You should

check other NLMs on the system and

upgrade them.  If you are still

experiencing problems identify which

NLM is causing the problem and contact

the third party manufacturer of the

NLM.

Polling Timeout:              The adapters request was put on the

que but because the server was busy it

was not able to be serviced within 800

nanoseconds (default).  If this occurs

the adapter will then fire an

interrupt to get serviced.

Reset Because Hardware Died Errors:     If the adapter gets in an

unknown state or stops transmitting on

the host side the driver will

increment this counter will

reset/restart the adapter.

Number Of Interrupts Fired:   This counter increments each time the

adapter had to fire an interrupt to

service a request because the polled

request wasn't serviced.

NE32HUB.LAN

FIFOUnderRunCount             This counter increments usually during

high utilization or high BUS usage and

we are unable to send the packet.

This should occur rarely.

ByteCountMismatchCount        This counter increments when the

transmit packet size is not = to sum

of fragments passed to the SONIC chip

on board.  This should occur rarely.

HardTransmitErrorCount        This counter increments when a CRC,

Excessive Deferral, ByteCount

MismatchCount, or FIFOUnderRunCount

occurs on a transmit.  This is more of

a general transmit error counter.

TransmitCollisionsCount       This counter increments when the card

can't transmit on the wire.  If the

card takes 5 times to get the packet

transmitted this counter will be

incremented by 4.

OutOfWindowCollisionCount     This counter increments when an

illegal collision occurs.  The

collision should normally be during

the preamble.  If the collision

incorrectly occurs after the preamble

this counter will increment.  This

should be rare.

CRCErrorCount                 This counter increments when a packet

is received by the card with a bad

CRC.

FIFOOverrunCount              This counter increments when a packet

is received and there are errors in

the packet. The driver looks at the

FIFO overrun counter to determine if

there are errors.  The packet is not

received if there are errors.

RDAExhaustCount               This counter increments when the

driver can't take the data out of the

buffer fast enough to process it and

the Receive Descriptor Area is

overflowed.  This shouldn't happen

very often but is more likely under

high utilization.

RRAExhaustCount               This counter increments when the

driver can't take the data out of the

buffer fast enough to process it and

the Receive Resource Area is

overflowed.  This shouldn't happen

very often but is more likely under

high utilization.

RBAExceedCount                This counter increments when the

driver can't take the data out of the

buffer fast enough to process it and

the Receive Buffer Area is overflowed.

This shouldn't happen very often but

is more likely under high utilization.

FlagFoundCount                This counter increments when the RIC

chip on the card reports an error such

as (Jabber, Out of window Collision

etc.)

PacketsCompressedCount        This counter increments when the

driver is in promiscuous mode and is

looking at every packet.  To get the

management information efficiently the

driver only copies the first 50 bytes

of the packet into the buffer and then

adds on the 7 bytes of management

information on the end of the packet.

This occurs only on the packets that

are not destined for the server when

the driver is loaded with PCOMP=ENABLE

(default).

RICAddressWasInvalidCount     This counter increments when the RIC

address was corrupt on a receive

management packet.  The driver expects

this number to be correct.  That is

why the driver checks the RIC address.

TxTimeOutCount                This counter increments when a

transmit doesn't occur in 2 seconds.

Port0ErrorCount               This counter increments when the port

address is set to 0. The port address

should always be between 1 and 12.

This will not occur frequently.

PortBigErrorCount             This counter increments when the port

address is above 13. The port address

should always be between 1 and 12.

This will not occur frequently.

ValidCRCCount                 This counter increments when the

Management Information overlaying the

CRC is correct.  It should be

incorrect.  This should happen rarely.

------------------------------------------------------

Table B-7, "Custom statistics for RX-Net(tm) drivers,"

Custom statistics for Arcnet server driver:

TRXNET.LAN Statistic Explanation

NONE

------------------------------------------------------

Table B-8, "Custom statistics for Token-ring drivers,"

Custom statistics for token-ring cards:

TOKEN.LAN, NTR2000.LAN

Bad Correlator Count          This counter increments when a network

board responds with a request for data

from the file server that the file

server does not have.  The ECB or some

other code may be corrupted.

Eventually, this type of error will

down the server.  If this counter is

non-zero, you should try to find the

software that is corrupting the data.

Unknown ARB requests          This counts bad ARBs (Adapter Request

Blocks).  Normally the network board

(adapter) uses one of four known

commands to communicate with the

driver.  If a network board sends a

command that is NOT one of the four,

the driver does not recognize the

request.  This is not a catastrophic

error.   Sometimes old adapters send

bad ARB requests because of software

problems resident on the board.

NetWare responds to the network board

so that the board will not hang.

TOKENDMA.LAN

MicroChannel Error Count:     Adapter ran into a problem

transmitting on the Bus. The Adapter

Interrupt occurred from the firmware

on the card.

ECBs Over 16Meg:              The number of packets received that

had to use an ECB over 16 meg.  This

number should only increment when

using more than 16 meg of ram in the

F/S.

DMA Bus Errors Count:         This counter is incremented when a DMA

transfer completes with a bus error.

If this increments it could be

indicative of a hardware problem.

DMA Parity Errors Count:      This counter is incremented when a DMA

transfer completes with a parity

error. If this increments it could be

indicative of a hardware problem.

Command Reject Count:         This counter is incremented when the

Driver sends a command to the card and

the command is either invalid or the

card is still busy processing the

previous command.  This number should

be 0 or low.

Tx Timeout Count:             This counter is incremented and the

adapter is reset if 2 seconds elapses

and the driver hasn't heard back from

the firmware that the transmit was or

wasn't successful.  This counter shows

the driver successfully recovering

from the lost hardware transmit.  It

is OK to see this number increment.

Transmit Late Count:          This counter will increment when the

driver thought that the card was

transmitting more than it really was

because of what was reported by the

firmware.  The data that wasn't

transmitted will then be sent in the

next packet.  This is a latency type

of issue.  You will likely see more of

these errors on a busier network.

Transmit Defragment Count:    The IBM Token-Ring DMA LAN boards are

not able to access memory above 16 MB.

Therefore, if the operating system

issues an Event Control Block (ECB)

with an address above 16MB of memory,

the board uses some of the reserved

buffers below 16MB to queue the

request.  These are not errors.  They

are only keeping track of how many

ECBs were redirected to the buffers

below 16MB.   In many cases, this

counter can be as high as the total

packets sent and received.  Because of

this double buffering that occurs it

does cause a performance hit.  Any

board that is busmastering or uses DMA

that is not a 32 bit adapter will

potentially experience a performance

hit if you have more than 16 MB of

ram.

------------------------------------------

Table B-9, "Custom statistics for IBM baseband pcn2l drivers,"

Custom statistics for PCN2L IBM baseband server driver:

PCN2L.LAN

HotCarrierInterruptCount      This counter increments when the board

detects a carrier longer than expected

without a transmit.  This Generally

indicates that some board on the

network has failed or is starting to

go bad.

No82588InterruptCount         This counter increments each time it

receives an interrupt from the card

but not the 82588 chip.  This should

never happen.

WeirdInterruptCount           This counter increments when the file

server has received an interrupt from

the card, but the card claims not to

have sent one.  This should never

happen.

BadTransmitCompleteInterruptCount       This counter increments for

each complete transmission with no

transmit active.

HardTransmitErrorCount        This counter increments when a

transmit fails and the driver retries

the transmit.

GotNothingCount               This counter increments when the

driver receives an interrupt from the

card indicating that it has completed

a receive but there is no data in the

card's receive buffer.  This is not

serious.

ReceiveUnderrunErrorCount     This counter increments when the

driver finds less data in the card's

buffer than the card said was there.

ReceivedShortPacketCount      This counter increments when a packet

of less than 17 bytes is received.

BadReceiveConditionCodeCount  This counter increments when the

buffer is flushed because the card

hasn't received the incoming packets

properly.