Microsoft® Windows NT™ TP4/CLNP/ES-IS

User’s Guide

µChapter 1 TP4/CLNP/ES-IS Networking Concepts	3

Chapter 2 Installation and Configuration	6

Installation	6

Configuration	8

Verifying the Installation	10

Chapter 3 Network Administration	11

Administration Tasks	11

Trouble Shooting	11

Advanced TP4/CLNP/ES-IS Parameters	12

## Chapter 1 TP4/CLNP/ES-IS Networking Concepts

#### Overview of Windows NT TP4/CLNP/ES-IS

Transport Layer Protocol class 4 / Connectionless Network Layer Protocol (TP4/CLNP) is a networking transport protocol stack that provides communication across interconnected networks, between machines with diverse hardware architecture and multiple computer operating systems. In Microsoft Windows NT, TP4/CLNP can be used with Windows Networking Services to connect to non-Microsoft hosts running similar protocol stack.

#### The  OSI Architecture

The OSI standards were developed to allow computer systems built by different vendors to exchange data. These computer systems may have different operating systems and vary in the way data is processed, stored or represented internally. But as long as the information that is exchanged between the systems conforms to OSI standards, information can be interpreted upon receipt and  communication is possible.

The first step in the OSI standards development was  the creation of OSI reference model. This model was developed by International standards Organization (ISO), is divided into seven layers. Each layer provides a well defined set of functions based on published standards. The rules and conventions of that conversation is called a protocol. At each layer N, there is a N-Layer protocol.

Service primitives are special messages which define the services that a layer provides. The details of how the services are implemented are transparent to the service user, usually the next upper layer. The communication between adjacent layers is via service access point (SAP). There may be multiple SAPs available to the service user.

The protocol can be connection oriented or connection-less. In the case of connection oriented protocol like TP4, the service user must establish a connection between peer entities. Conversely, the service user does not setup a connection when using a connectionless protocol like CLNP

TP4/CLNP is part of the family of protocols that are defined under ISO/OSI ( International Standards Organization / Open System Interconnection). TP4 refers to the set of standards in the 4th layer of the 7 layer architecture defined in OSI whereas, CLNP refers to the 3rd layer. ISO protocols suite mostly has peer to peer type of protocol. This does not restrict the design of upper layer or application layer protocols like File Transfer Access and Management (FTAM) or Virtual Terminal Protocol( VTP ) which are of client - server type. This OSI model uses connection oriented as well as connectionless protocols. The session layer and up protocols are exclusively connection oriented. Transport layer and down protocols are either connectionless or connection oriented. With proper choice of protocol one can run a connection oriented protocol like TP4, over a connectionless protocol providing a unreliable network service ( unreliable ) for example,  in a Local-Area-Network ( LAN ) environment, as well as over a connection oriented network service like X.25 typically seen in a Wide-Area-Network ( WAN ) environment.

#### Transport Class 4 (TP4)

Transport protocol class 4 is based on international standard ISO 8073.  It is a recommended standard for the transport layer for communication systems under US GOSIP (Government OSI Profile) standard. This protocol uses network service provider like CLNP and interface based on ISO 8348 to provide end to end reliable data transfer capability. TP4 guarantees the reliable delivery of packets by using sequence numbers, ensures data integrity by use of checksum, and in the case CLNP does corrupt a packet, it will retransmitt the packet. This reliability built into the TP4 allows it to run over a unreliable CLNP protocol based network service provider.

This reliability comes at a price. TP4 headers require use of additional bits to provide proper sequencing information, mandatory checksum, acknowledgment of successful receipt of data. This generally results in increased network traffic and decreased throughput or performance. To reduce the load on network, TP4 allows configuration of  various parameters. These provide for proper flow

#### CLNP

Connectionless Network Layer protocol is based on ISO 8473  standard. It's main functionality is to provide the routing of the data packet in the subnetwork so that it may reach it's destination. There is neither a guarantee that the packets will reach their destination nor in the sequence they were sent. Thus it is the responsibility of CLNP user to make sure they take proper measures. CLNP has a matrix that allows it to make decisions regarding routing data packets based on security, priority, quality of Service etc. This is generally referred to as the routing table. Since the entries do not change i.e.. they are static, this table is also referred to as static routing table. CLNP protocol may  also uses a routing table that is maintained by ES-IS protocol module.

#### ES-IS

End System to Intermediate System protocol is based on ISO 9542 standard. It is designed to operate in close injunction with CLNP (ISO 8473) and its addenda. It is a dynamic routing protocol, that operates over a point to point links or broadcast subnetworks.

ES-IS enables Intermediate system (IS) to dynamically find ESs that are attached to the same network. Conversely it allows ESs to dynamically find ISs that are attached to the same network. ESs can also locate each other on a single subnetwork and when multiple ISs are attached to the same network, enables ISs to redirect ESs to the ISs representing the most efficient route to a given destination thus allowing ES to dynamically configure the OSI routing.

#### OSI Addressing

In order to communicate, it is necessary to identify the objects involved in communication. The objects have names and  addresses. A name is a collection of attributes that identify the object  in a domain. As address is a name that is used to specify the location of an object. The names must be registered with a registration authority, without which a chaos will result and systems would not be able to identify themselves uniquely  in a global environment.

## Chapter 2 Installation and Configuration

### Installation

This chapter describes the installation process and discusses the details you need to understand to make decisions during the installation and configuration process.

TP4/CLNP/ES-IS networking transport is not installed as part of the Windows NT Setup. Once Windows NT is installed, the TP4/CLNP/ES-IS networking transport can be installed via the Networks in the Control Panel.

Note: You must have administration privileges to install TP4/CLNP/ES-IS.

The Networks utility allows you to install and configure networking software, including the TP4/CLNP/ES-IS.

1.	From the control panel, select Networks. This display the Networks dialog box.

2.	Choose Add Software... button. This displays Add Networks Software dialog box.

3.	Choose  <Other > from this dialog box, using the scroll bar, as shown in Figure 2.1.

Figure 2.1	 Add Network Software Dialog

4.	Click on Continue button.  A subwindow  Insert Diskette appears as shown in Figure 2.2. Insert the Installation diskette in appropriate drive, or enter the full path of where the TP4 installation files are located.

5.	Choose OK

Figure 2.2 Insert Diskette Dialog Box

6.	Upon inserting appropriate diskette in the drive, Select OEM Option dialog box appears as shown in Figure 2.3.

Figure 2.3 Select OEM Option Dialog Box.

7.	Choose ISO TP4/CLNP stack option, and click on OK button. This will copy all necessary drivers and utilities in appropriate directories.

### Configuration

After the TP4/CLNP/ES-IS transport is installed, configure it as follows:

1.	From the Control panel, open Networks, if it isn't already open as shown in Figure 2.4.

Figure 2.4. Networks Dialog Box

Figure 2.5 ISO TP4/CLNP Configuration Dialog Box

1.	Choose TP4/CLNP Transport from the Network Software list.

2.	Choose the Configure button. The TP4/CLNP/ES-IS Configuration dialog box shown in Figure 2.5 appears

3.	Choose the Network card you would like to run OSI protocol stack on by using "adapter" list box.

4.	This dialog box includes the following parameters.

Local Address

Specifies the value of an user friendly alias to the local machine NSAP value. This is typically used in the Windows Sockets interface bind() call for the network address portion of the transport address.  Default is your computer name.

NSAP

Specifies the globally unique Network Service Access Point address value for the machine. This goes in the protocol and seen by other machines on the network. It is typically used in the Windows Sockets interface connect() and accept() calls for the network address portion of the transport address.

Any ASCII string can be used to specify it's value. This ASCII address string that begins  with a '$' character is interpreted as an hex address string, else literal ASCII string would be taken as NSAP. Default is your computer name

5.	Choose Yes to reboot the system now.

### Verifying the Installation

The installation diskette has the  necessary utilities to verify the installation in the loop back mode. The steps are

1.	Create a directory named sample in your area.

2.	Copy client.exe and server.exe in to this directory.

3.	From a MS-DOS command prompt type server. Type your computer name in upper case when prompted for an NSAP. Type 01 when prompted for an TSel ( Transport Selector )

4.	From another MS-DOS command prompt, type client. Type your computer name in upper case when prompted for an NSAP. Type 02 when prompted for a TSel ( Transport selector ). When  prompted for remote NSAP address , type the NSAP and then at TSel prompt for  remote application,  type 01. ( You are providing information about server that is running in other command window)

5.	Now  the some data should be exchanged between client and server. This is evident from the  output in respective windows.

## Chapter 3 Network Administration

### Administration Tasks

Once TP4/CLNP/ES-IS transport service is installed, normally, little additional effort is required.

This chapter describes the configuration of advanced parameters to fine tune the performance of the protocol stack. It also instructs how to instruct OSI to run on multiple cards and then add static routing entries.

### Trouble Shooting

This transport service once installed in the Windows-NT kernel, is configured to start, by default, automatically at the boot time. All the parameters, used by the deamons to make this service available to a  transport service user, reside in the registry database. An administrator can change the values of these parameters to fine tune the performance of this transport stack. If a mistake is made, the deamon may fail to get the service up and running.

To find out if the service is available or not, following steps can be followed.

1.	From the control panel, open Services, if it isn't  already open.

2.	Use the scroll bar to find TP4/CLNP Protocol entry from a list of services as shown in figure 3.1.

3.	Make sure it is started.

4.	If it is shown to be not started, Select the TP4/CLNP Protocol. Choose "Start" button.

5.	If it fails, check the Windows NT event log for error reports, and also check the parameters in the Windows NT registry data base.

Figure 3.1: Windows-NT services Dialog Box

### Advanced TP4/CLNP/ES-IS Parameters

All the parameters that affect TP4/CLNP operation are stored in the registry. The parameters most commonly accessed can be modified via the dialogs discussed in chapter 2. These parameters are a subset of the entire set of TP4/CLNP/ES-IS related registry parameters. Other TP4/CLNP/ES-IS related parameters, which may be modified with the registry editor (regedt32.exe), are included in this discussion.

Note: Be very cautious about editing any parameters with regedt32. You should modify values that you fully understand, or unpredictable results may occur.

#### Transport Layer (TP4) Parameters:

MaxTpStreams

Specifies the maximum limit on the number of transport streams that can be opened.

MaxTSDUSize

Specifies the maximum TSDU size value. This parameter is used only if you plan to use SOCK_SEQPACKET device type of Windows Sockets interface. If you use SOCK_STREAM device type, this value has no effect.

The SOCK_SEQPACKET device type supports message boundary concept, where as SOCK _STREAM does not.

MaxTIDUSize

Specifies the maximum amount of data, in number of octets, that can be sent or received at a time to the transport layer. A request to send data more than this limit would cause a fatal error on the connection.

TimerRetransmission

Specifies the value of the Retransmission Timer in units of 100 msecs.

TimerKeepAlive

Specifies the value of the Keep Alive Timer in units of 100 msecs. It is usually about 3 times the Retransmission Timer value.

TimerInactivity

Specifies the value of the Inactivity Timer in units of 100 msecs. It is usually about 3 times the Keep Alive Timer value.

TimerFrozenRef

Specifies the value of the Frozen References Timer in units of 100 msecs. It represents the amount of time the transport layer connection reference numbers and sequence numbers need to be frozen before they can be reused. Generally it should be more than the multiple of TransmissionCount and TimerRetransmission value.

TimerTS2

Specifies the value of the TS2 Timer in units of 100 msecs. Generally it can be the same as Retransmission timer value.

TransmissionCount

Specifies the maximum number of times a TPDU will be transmitted before declaring that there is a connection failure.

WindowSendMax

Specifies the maximum send window size.

WindowRecvMax

Specifies the maximum receive window size. Generally the performance is better with bigger window sizes. However, when working with EtherLink II network adapters it is better to use a window size of "1".

WindowRecvResHiWat

Specifies the resource limit hi water mark for receive window. It should be greater than WindowRecvMax.

WindowRecvResLoWat

Specifies the resource limit low water mark for receive window. It should be lower than the WindowRecvResHiWat.

PropFormat

Specifies the Sequence Number Format that need to be proposed when initiating a connection. Takes the values  Normal Format (0), Extended Format (1).

SelFormat

Specifies the Sequence Number Format that need to be selected when the connection initiator proposes to use the Extended Format. Takes the values Normal Format (0), Extended Format (1).

PropTPDUSize

Specifies the TPDU size that need to be proposed during connection initiation. It can take values from 7 to 13. A value of "x" represents a TPDU size of 2 power "x".

SelTPDUSizeLimit

Specifies the TPDU size maximum limit in accepting a TPDU size proposal during connection establishment. It can take values from 7 to 13. A value of "x" represents a TPDU size of 2 power  "x".

PropChksumNonUse

Specifies whether to propose non use of checksum during connection initiation. Takes the values Yes (1), No (0).

SelChksumNonUse

Specifies whether to select non use of checksum when it is proposed in the incoming call. Takes the values Yes (1), No (0).

#### Network Layer ( CLNP/ES-IS) Parameters

MulticastES

All End Systems multicast SNPA for IEEE802.3 subnetworks. For US GOSIP, this parameter is defined in the 1990 NIST Agreements for OSI. For special needs, the parameter is left exposed for user configurability.

MulticastIS

All Intermediate Systems multicast SNPA for IEEE802.3 subnetworks. For US GOSIP, this parameter is defined in the 1990 NIST Agreements for OSI.For special needs, the parameter is left exposed for user configurability.

#### OSI on multiple cards

In server systems there may be more than one communications card on which you may want to turn on the OSI capability. By default OSI LAN protocol stack will run on all the possible network adapter cards, it is capable of.

In order to reduce the boards you want to run OSI LAN services on, reduce the number of strings in the Bind subkey value in the registry database parameter as described below.

HKEY_LOCAL_MACHINE->System->CurrentControlSet->Services->IsoTp->Linkage->Bind

#### Static Routing Table

In the OSI model, ES-IS protocol module provides a capability to the end systems to find out the existence of each other by using this protocol. This obviates a need to constantly update the database of known systems as in the case of TCP/IP protocol, where hosts file has to be updated to reflect any changes to  LAN.

Not all systems that have OSI capability, can be assumed to have ES-IS capability. In that case, if your systems wishes to communicate with such a system, one has to explicitly inform the local system a way to reach that remote system. This is one of reasons to update Static routing table. It is also possible that your server may want to direct the traffic to the remote sites via a hard coded route, based on any of the reasons like security, economics, backup route etc.

The file osirte in %SystemRoot%\drivers\etc directory describes the static routing table entries for the network layer. The format of the entries is as shown below:

NSAP:MAC/X.121:Priority: NDIS_device_Name

Every line has three fields, separated by ':'.

The first field is the NSAP of the application on the remote system.

The second field is the MAC or X.121 DTE address of the remote system. if it is present in the local LAN or MAC address of the router you want to direct the PDUs to. In the case where, the remote system can be reached X.25 and your system has the capability turned on for OSI LAN over X.25, enter the X.25 address using X.121 formatting convention. The values in this field are hex encoded nibbles of the address. Now if the X.121 address contains odd number of digits, even it out by using 'f'. For example, if the DTE address is '123456789' then the actual value in the osirte file for second field will be $123456789f.

The third field is the priority field. The only values allowed are 0 and 1. The route with priority 0 is attempted first. If that fails, then one with priority 1 will be attempted. The precedence for the routing is as follows:

Static route with priority 0: 1st attempt

Static route with priority 1: 2nd attempt

Note: Following routes are recorded automatically in the routing tables through various exchanges between 2 end systems or an end systems and a Intermediate system (router)

·	Route recorded using RD packet from a router (IS) : priority 2: 3rd attempt

·	Route recorded using ES-IS H packet between ESes.: priority 3: 4th attempt

·	Others: priority 4: Last attempt.

The fourth field is the NDIS device name for local communications board you want to route this PDU on. It must match NDIS device name for one of the boards present in the local system. The easiest way to find out the NDIS device name for a particular board is looking at Bind subkey value,  which may look as follows:

\Device\Streams\Elnk31

\Device\Streams\SkFddiFe2..

Then there are 2 communications boards. The NDIS device names are \Device\Elnk31 and \Device\SkFddiFe2 for the respective boards.

Restart the TP4/CLNP service for the changes to take effect.