==PCBPLUS.DOC================TANGO-PCB PLUS==============Version 2.30==

INTRODUCTION

Tango-PCB  PLUS Version 2.30 contains the enhancements and  bug  fixes,

described below.

INSTALLATION INSTRUCTIONS

ACCEL  has standardized on an automatic installation program,  INSTALL,

which  uses compressed files on the distribution diskettes.  The Tango-

PCB PLUS Reference Manual contains complete instructions for using this

utility.   The minimum free disk space required for a full installation

of this release is 3.2 MB.

NEW FEATURES AND ENHANCEMENTS

The  following  features have been added or improved in  this  release.

Please  consult the Release Notes for a detailed description  of  these

enhancements.

OPTIMIZING  NETS  RECONNECT - Before this release, the  Nets  Reconnect

command  would  create bluelines in a daisy-chain fashion--without

regard  for  connection  lengths.  Now  Nets  Reconnect  optimizes

bluelines  as it creates them.  There is no choice of optimization

methods; Nets Reconnect always optimizes for total (x + y) length.

ENHANCED GERBER INPUT FOR PANELIZATION - Tango-PCB PLUS makes it easier

to  panelize your Gerber output.  Before this release,  loading  a

photoplot  file  would  clear the current design.   This  made  it

tedious  to create a layout with many panels, either of  the  same

layer  or  different ones.  Now loading a photoplot file does  not

clear the design, and you can choose the destination layer.   Note

that all holed apertures will produce through-hole pads; non-holed

apertures  will  produce  surface-pads  on  the  TOP  layer  only,

regardless of destination.

MORE GERBER  APERTURES  -  The  number of  Gerber  apertures  has  been

increased: the highest-numbered aperture is now 250.

NOTE:  version  2.30  .DFN  files are not  fully  compatible  with

previous versions.  Versions before 2.30 will not process aperture

definitions for D-Codes over 99, but they will process assignments-

-reassign these by hand, or clear them and use the Auto button.

OUTPUT  REPORTS FILE VIEWER - Another output option has been  added  to

the  Output Reports destination group: the View option sends the

output  directly to the Viewer.  From the viewer, the entire  file

can be inspected, or output to a file or port.

DRIVERS  -  Several  graphics drivers and a printer  driver  have  been

added:

VESA800.DRV     The  VESA800 graphics  driver  supports  VESA

graphics at 800x600 resolution.

VESA1024.DRV    The  VESA1024 graphics driver  supports  VESA

graphics at 1024x768 resolution.

VESA1280.DRV    The  VESA1200 graphics driver  supports  VESA

graphics in 1280x1024 resolution.

VESA8G.DRV      The  VESA8G  graphics  driver  supports  VESA

graphics in 800x600 resolution, for VESA implementations with

Red and Green switched (GRB instead of RGB).

VESA10G.DRV     The  VESA10G graphics  driver  supports  VESA

graphics  in  1024x768  resolution, for VESA  implementations

with Red and Green switched (GRB instead of RGB).

VESA12G.DRV     The  VESA12G graphics  driver  supports  VESA

graphics  in  1280x1024 resolution, for VESA  implementations

with Red and Green switched (GRB instead of RGB).

See VESA.DOC for more information.

BJ180.DRV A Bubble Jet printer driver is now available.  This

driver  supports A and B sized Black/White output at  180x180

dpi for Cannon Bubble Jet printers.

KNOWN PROBLEMS

1.   DXF OVAL PADS - Oval pads show up as lines in DXF output, when the

lines/arcs/circles option is selected.

2.   NETS  OPTIMIZE - This command currently does not support partially

routed  nets,  and  may  drop connections from  them  rather  than

optimizing.   If this happens, use Nets Reconnect to  replace  the

connections.

3.   OUTPUT  DRIVERS  -  Even though the driver input  control  in  the

second  Output  Plot/Print dialog will accept manual  entry,  this

entry  is  ignored; drivers can only be selected from the  drivers

list box.

4.   MOVE ENDPOINT - It is sometime possible to fool Move Endpoint  and

leave  a  dangling blueline.  If a line connected  to  another  is

deleted and replaced with a blueline, then the line is replaced by

Place  Line  (not Nets Route), Move Endpoint at that  intersection

may move the lines but leave the blueline.  This can be fixed with

Nets  Reconnect,  but it is always best to route connections  with

Nets Route.

5.   SELECT  LINE  - Some diagonal lines cannot be selected  except  at

their  endpoints;  others can be selected even if  the  cursor  is

close  to but not over the line.  These problems never occur  with

lines  at  90-  or 45-degree angles, but only with  non-orthogonal

lines.

6.   SQUARE APERTURES - If you use a Square Aperture draw aperture  you

may  see some 'chunks' missing from corners of your polygons  when

you load your photoplot file.

7.   BLUE  LINES  -  Deleting Polygons or Arcs, or Lines that  are  not

routed  to  Pad Centers, will not give 'blue lines'  if  they  are

deleted.

8.   MACROS - Under extreme low-memory conditions, you may receive  two

out-of-memory  error  messages while  recording  a  macro.   After

acknowledging the first, the program may appear to be hung but  it

is  actually  waiting for another acknowledgment: hit  <Enter>  to

return to normal processing.

9.   POLYGONS  -  Complicated polygons are those with many simultaneous

vertical edges; e.g., a polygon that looks like a horizontal  saw,

with  all  of  its  points at the same y-coordinate.   Complicated

polygons  outline  correctly, but do not  fill  correctly  on  the

display.   Also,  there  may  not  be  enough  memory  to   output

complicated  polygons,  depending on the  size,  scaling  factors,

available memory, and driver used (the Postscript drivers are  not

affected).  If you run out of memory while outputting, you can try

purging  the Undo information (File DOS will do this),  break  the

polygon into two or more smaller polygons, or rotate the image  to

change  the vertical edges into horizontal ones.  Also,  extremely

narrow polygons may not be displayed at some zoom levels.

ADDITIONAL CONSIDERATIONS

1.   NETS  RECONNECT  WITH  MULTIPLE  PLANE  NETS  -  When  using  Nets

Reconnect with a split-plane design, you must make sure that  free

pads  and vias are disconnected from the power and ground  planes.

If  they  are connected, Nets Reconnect (and Nets Verify and  DRC)

will detect a short, and stop without filling primitives with  net

information.  If you wish to reload the netlist for  a board  with

free  pad  or via plane-connections, you must follow these  steps:

1)  disconnect  all free pads and vias from the power  and  ground

planes,  keeping track of them for future reference; 2)  load  the

netlist; 3) Nets Reconnect; 4) use Nets Plane Assign for the plane

nets  (this should replace all of your plane connections); 5)  run

Nets  Reconnect  again  if superfluous bluelines  remain;  and  6)

referring to your notes from step 1), reconnect any leftover  pads

and  vias to the power or ground planes.  Once the primitives have

net  information, you should never have to disconnect the pad  and

via plane connections in order to load your netlist.

2.   NETS  ROUTE  SUSPENSION  - You can use "set TANGOSLASH=string"  in

your  AUTOEXEC.BAT file to assign alternate keys  for  suspension;

there  are  several  things to keep in mind.  You  must  use  only

printable characters in your string.  These characters can have no

other  meanings: hotspot and macro keys will not work.  Using  the

TANGOSLASH  variable  at all replaces the  slash  keys  with  your

string; the slash keys will no longer work unless included in your

string.

3.   POSTSCRIPT  CARTRIDGES  - Some PostScript  cartridges  cause  port

timeout errors; this usually happens because they are not finished

processing the current page when sent more data, and they have not

set  the  port busy-flag. Some cartridges redefine the  page  size

regardless of instructions received to the contrary; these devices

print out an A4-size output on two A4-size sheets instead of  one.

Neither of these problems occur on any true PostScript printer  we

have tested or on other cartridges.

4.   KEYWORDS  - The .DATE, .TIME, or .FILENAME keywords are  refreshed

whenever  output--whether  the  output  is  to  the  screen  or  a

peripheral device.  Only the keyword strings are stored, not their

resultant  text.  This means that selecting them for movement  may

create "text placed on text" for the keyword.  This does not occur

for  .FILENAME.  It can occur for .DATE if placed today and  moved

tomorrow,  and occurs often for .TIME.  Zoom Redraw  corrects  the

display.  When printing, if the .TIME keyword crosses two or  more

printing passes, its output may change between them.

5.   ROUTING WITH MULTIPLE PLANE NETS - Tango-Route PLUS fully supports

designs with one power and one ground net, but has limited support

for  the  Multiple  Plane Nets feature in  this  version:   unless

surface-mount pads are fanned out to vias in Tango-PCB PLUS, Tango

Route  PLUS will not route them or otherwise connect them  to  the

plane;  if fanned out to vias, the vias will be connected  to  the

plane.  Tango RoutePRO supports a maximum of one power net and one

ground net.

6.   DESIGN-BASED  .INI  FILES  -  If the program  is  started  without

specifying  a design file, the PCB.INI file is always  loaded  and

processed  first, before the design-based .INI file referenced  in

PCB.INI.    The   colors  and  settings  (including  speed-palette

display) stored in PCB.INI will appear momentarily, to be replaced

immediately  by  the design-based .INI colors and settings.   This

will  occur when you start the program without specifying a design

file.

7.   DESIGN-BASED INI FILES - Remember that design-based INI files  are

only  saved  if the design file is explicitly saved.   This  means

that  Setup  DRC and Output Reports information and  changes,  for

instance, will be lost unless you save the design file.  The  rule

of  thumb  is  now more valid than ever:  when you  want  to  keep

something, save!  PCB.INI always contains the current setting when

you exit the program.

8.   DESIGN-BASED  DFN FILES - When Tango-PCB PLUS finds  and  loads  a

design-based  DFN file, it first clears all definitions.   If  you

have line, pad, via, text, custom speed-palette or CAM definitions

or  assignments  that you want to keep, make sure they  are  saved

into  a  design-based DFN file (you will be prompted to do  this),

because  they  may be overwritten when you load  your  next  file.

Clearing the DFNs keeps your environment free from the clutter  of

unused DFNs.  For instance, before this release it was quite  easy

to  accumulate hundreds of unused pad definitions, which you would

have to prune by hand (using the Current Pad command) just to keep

them   manageable.   If  you  wish  to  load  additional   non-CAM

definitions, use the File Load command to load the desired file as

a  block, then delete the block:  new DFNs are always created  for

new  items  placed in the workspace.  PCB.DFN always contains  the

current DFNs when you exit the program.

9.   OUTPUT  TYPE  SETTINGS  - Remember that each  output  type  (Plot,

Print,  PostScript, and DXF) has its own complete  set  of  output

parameters including scaling values and the destination  port  (or

file).   When  you  change types (using the  driver  List  button,

driver names can be typed into the input control but they will not

register  unless selected using List), the program  remembers  the

parameters for the old type, and replaces them with the remembered

parameters for the new type--the changes have not been lost.

10.  DXF-IN  DIMENSIONS - When working with dimensions in  AutoCAD,  be

sure to use the PURGE command before output to DXF.  Each time you

move  or  otherwise change a dimension, AutoCAD generates  another

image  for the dimension; these leftover images are output to  DXF

unless you the PURGE command beforehand.

11.  DXF-IN VALUES - Remember that DXF items that exceed Tango-PCB PLUS

specifications  cannot be loaded into the workspace.   Items  that

fall  outside  the workspace boundary, are too big, or  too  thick

will be discarded after an appropriate error message.  If you need

such  items to be loaded, you must alter them in your CAD  program

before  loading the DXF file.  Remember too that many CAD programs

use external utilities for DXF Export and Import:  changes made in

the program itself may not be reflected in the DXF file unless you

run the utility.

12.  GERBER  FILE LOAD - Remember that in order to load and  display  a

photoplot file correctly, your current aperture descriptions  must

match  those  used  to  create the file.  If  you  have  loaded  a

different design file, the current aperture data will be  replaced

if  there is a DFN file corresponding to that design file.  If you

are  unsure about your aperture descriptions, the safest procedure

is  to  reload the design file to get its DFN file, then load  the

photoplot file; do a File Clear before loading the photoplot  file

if  you  don't want the clutter--DFNs are cleared only by  loading

another DFN file.

13.  AUTOSAVE - If an AutoSave hits during a Zoom command, the save  is

canceled.

14.  BLUE  LINES/DELETE  VIA  -  Deleting  a  Via  that  contains   Net

information  will not generate a connection line  (or  consolidate

connection  lines).  If you are deleting 2 line segments  and  the

Via  that connects them, be sure to delete the Via before deleting

both  line  segments.  If you delete the Via after  deleting  both

line segments, the connection lines will not be consolidated,  and

Nets  Route will force you to route to the location of the deleted

Via.   If  you use Delete Highlight or Delete Block to remove  the

Lines and Via you will not see this problem.

15.  .DRV  FILES  -  Do not place any non-supplied .DRV  files  in  the

directory with your Tango .DRV files.  Tango software looks at the

.DRV files to present the list of output drivers to the user.   If

non-supplied files exist in the directory, unexpected behavior may

result.

16.  LISTING  FILES - Due to the changes we made in filename  filtering

in  the  File Load list box, you can no longer use X*.PCB to  list

all  of  your  .PCB  files that start with X.   You  can  use  the

technique to filter files in the second list box.

17.  BINARY  FILES COMPATIBILITY - The current versions of Tango  Route

PRO  will  not support binary files.  You must save off  the  file

using ASCII format before routing.

18.  METRIC GRIDS - Because the resolution of our database is 1 mil, we

can  only   display or save items in 1 mil increments (not  .1mm).

The metric grid coordinates that display on the Status Line should

be thought of as an on-screen calculator that converts an imperial

location to a metric location.  The visible grid only displays  in

imperial units.

19.  MACROS  -  Because some commands have been changed, and  some  new

commands have been added, you may find that some Macros that  were

recorded  in versions prior to 2.30 may no longer work  correctly.

You  will  have to re-record any macros that no longer work  using

version 2.30.

20.  MACROS  -  If  a  macro  contains two  or  more  separate  editing

commands,  UNDO will only affect the last one; it will not  affect

the entire macro.

21.  VERSION  1.16 TO 2.30 CONSIDERATIONS - 2.30 requires more  netlist

information than is contained in version 1.XX files;  be  sure  to

reload your netlist file to insure proper processing.

22.  PIN  DESIGNATOR MISMATCH - If the pin designators of  a  component

you  are  auto-placing are different from those  of  the  netlist,

those  connections  cannot be made.  Also,  if  you  have  already

placed  a component whose pin designators disagree with a  netlist

that  you  load,  connections cannot be made, and  no  warning  is

given.   Run  the  Unplaced  Nodes  DRC  Report  to  detect  these

potential problems.

23.  OBSOLETE OVERLAY FILES - Overlay files (.OV1 and .OV2) are no

longer used. Delete PCB.OV1 and PCB.OV2 (left over from 1.16) from your

PCB directory.

GRAPHICS CARDS/TANGO DRIVER LIST

RESOLUTION COLORS   GRAPHICS CARD         Tango DRIVER

640 X 480      IBM MCGA (Monochrome)         MCGA

720 X 348      Hercules Graphics (Mono)      HERCULES

640 X 350  16  ATI EGA WONDER                EGA

640 X 350  16  ATI VGA WONDER                EGA

640 X 350  16  EIZO MD-B07 VGA               EGA

640 X 350  16  EIZO MD-B10 VGA               EGA

640 X 350  16  GENOA SuperVGA                EGA

640 X 350  16  IBM EGA or Compatible         EGA

640 X 350  16  QUAD GTI SuperVGA             EGA

640 X 350  16  Sigma VGA/H                   EGA

640 X 350  16  SOTA  VGA/16                  EGA

640 X 350  16  Tecmar VGA/AD                 EGA

640 X 350  16  Video7 VEGA Deluxe            EGA

640 X 480  16  ATI VGA Wonder                VGA

640 X 480  16  Colorgraphics EGA             EGA

640 X 480  16  EIZO MD-B07 VGA               VGA

640 X 480  16  EIZO MD-B10 VGA               VGA

640 X 480  16  EVEREX Micro Enhancer EGA     EGA

640 X 480  16  GENOA SuperEGA                EGA

640 X 480  16  GENOA SuperVGA                VGA

640 X 480  16  IBM VGA                       VGA

640 X 480  16  IBM PS/2 VGA                  VGA

640 X 480  16  NEC GB1                       VGA

640 X 480  16  Paradise EGA                  EGA

640 X 480  16  PCG VGA                       VGA

640 X 480  16  QUAD EGA ProSync              EGA

640 X 480  16  QUAD GTI SuperVGA             VGA

640 X 480  16  Sigam VGA/H                   VGA

640 X 480  16  SOTA VGA/16                   VGA

640 X 480  16  Tecmar VGA/AD                 VGA

640 X 480  16  Tseng Labs EGA 480            EGA

640 X 480  16  Video7 VEGA Deluxe            VGA;VSEVDLX

640 X 528  16  GENOA SuperEGA                CUSTOM/SVGA

720 X 540  16  Video7 VGA                    VSEV720

800 X 600  16  Ahead VGA Wizard+/Wiz Deluxe  CUSTOM/SVGA

800 X 600  16  AST VGA Plus                  CUSTOM/SVGA

800 X 600  16  ATI ULTRA                     ATI800

800 X 600  16  ATI VGA Wonder                ATI800

800 X 600  16  BOCA Super VGA                CUSTOM/SVGA

800 X 600  16  Cardinal VGA600               CUSTOM/SVGA

800 X 600  16  Chips & Tech. Super VGA       CUSTOM/SVGA

800 X 600  16  Diamond Speedstar             TSENG800

800 X 600  16  Diamond Speedstar 24X         PARA800

800 X 600  16  Diamond Stealth               ORCS3800

800 X 600  16  EIZO MD-B10 VGA               CUSTOM/SVGA

800 X 600  16  EVEREX Micro Enhancer EGA     CUSTOM/SVGA

800 X 600  16  EVEREX ViewPoint 256 / 512    CUSTOM/SVGA

800 X 600  16  GENOA SuperVGA 5400/6400      GEN800

800 X 600  16  GENOA SuperVGA 7400 Series    TSENG800

800 X 600  16  GENOA SuperVGA 8000 Series    ORCS3800

800 X 600  16  Hewlett-Packard VGA           CUSTOM/SVGA

800 X 600  16  OAK OTI-067 VGA               OAK800

800 X 600  16  Orchid Fahrenheit 1280        ORCS3800

800 X 600  16  Orchid ProDesigner Plus       ORC800

800 X 600  16  Orchid ProDesignerII/IIs      ORC800

800 X 600  16  Paradise EGA or VGA           PARA800

800 X 600  16  PCG PHOTON MEGA PLUS VGA      CUSTOM/SVGA

800 X 600  16  QUAD GTI SuperVGA             CUSTOM/SVGA

800 X 600  16  Renaissance RVGA I/II         CUSTOM/SVGA

800 X 600  16  Sigma VGA/H                   CUSTOM/SVGA

800 X 600  16  SOTA VGA/16                   CUSTOM/SVGA

800 X 600  16  STB VGA                       CUSTOM/SVGA

800 X 600  16  STB Powergraph VGA            TSENG800

800 X 600  16  Tatung TVGA-16                CUSTOM/SVGA

800 X 600  16  Tatung VGA                    CUSTOM/SVGA

800 X 600  16  Tecmar VGA/AD                 CUSTOM/SVGA

800 X 600  16  TIGA Hercules Graph Station   TIGA

800 X 600  16  Trident TVGA VGA / 8900       TRI800

800 X 600  16  TSENG 4000                    TSENG800

800 X 600  16  Super VGA                     CUSTOM/SVGA

800 X 600  16  VESA                          VESA800

800 X 600  16  VESA (GRB Color Palette)      VESA8G

800 X 600  16  Video7 VGA                    CUSTOM/SVGA

800 X 600  16  Video7 Fastwrite              VSEV800

800 X 600  16  Wizard Deluxe EGA             CUSTOM/SVGA

800 X 600  16  ZyMOS POACH 51 VGA            CUSTOM/SVGA

1024 X 768 16  ATI VGA WONDER XL             ATI1K

1024 X 768 16  ATI ULTRA                     ATI1K

1024 X 768 16  Diamond Speedstar             TSENG1K

1024 X 768 16  Diamond Stealth               ORCS31K

1024 X 768 16  EIZO B10                      GEN1024

1024 X 768 16  GENOA SuperVGA 5400/6400      GEN1024

1024 X 768 16  GENOA SuperVGA 7400 Series    TSENG1K

1024 X 768 16  GENOA SuperVGA 8000 Series    ORCS31K

1024 X 768 16  OAK OTI-067 VGA               OAK1K

1024 X 768 16  Orchid ProDesigner VGA Plus   ORC1024

1024 X 768 16  Orchid ProDesigner II/IIs     TSENG1K

1024 X 768 16  Orchid Fahrenheit 1280        ORCS31K

1024 X 768 16  Paradise VGA1024              GEN1024

1024 X 768 16  Sigma Designs VGA/X16         GEN1024

1024 X 768 16  SOTA VGA/16                   GEN1024

1024 X 768 16  STB Powergraph                TSENG1K

1024 X 768 16  TIGA Hercules Graph Station   TIGA

1024 X 768 16  Trident 8900 Series           TRI1024

1024 X 768 16  TSENG 4000 Series             TSENG1K

1024 X 768 16  VESA                          VESA1024

1024 X 768 16  VESA (GRB Color Palette)      VESA10G

1024 X 768 16  Video7 VRAM                   VSEV1024

1024 X 768 16  Video7 VRAM II                VSEV1024

1024 X 768 16  Western Digital 90C00         ORC1024

1280 X 1024 16 VESA                          VESA1280

1280 X 1024 16 VESA (GRB Color Palette)      VESA12G