SCOPE 1.0 - an example real-time web device

Copyright (c) 1990, 1999 Erick Engelke

Introduction

============

SCOPE is a demo program for developers who wish to create web-controlled

devices.  When you run this program on any DOS PC, and then point a Web

browser at it, any browser becomes a console for the device.

This program was written for the low priced, credit card sized,

uFlash computers from JK Microsystems.  They also sell eRTOS, the

TCP/RTOS/Webserver development system which does all the hard work.

The source code to this sample is available in the eRTOS kit.

See: http://www.jkmicro.com

Installation

============

1. Unzip the various files.

2. All the .htm files should be in a subdirectory named web under this

directory.  Most unzip programs will put them there, but you must

fix that if yours does not.

3. Get and install a packet driver for your Ethernet card.  (ODI and

NDIS drivers do not work, you need a packet driver.)

4. Edit TCP.CFG and insert your network values.

5. Run the packet driver, then run scope.exe

6. On another machine, point your web browser at the scope machine

and be amazed.  Note that you can ftp to it with any userid

and the password specified in tcp.cfg file - great for field

updates.

Description

===========

In our example, the Title frame across the top of the screen contains

several buttons which bring up various  +---------------------+

displays in the text/input frame on     | Title               |

the lower left.                         | BUTTON BUTTON BUTTON|

+--------+------------+

In real time, a graph is created and    | text   | real time  |

displayed on the lower right.  It gets  | and    | graphical  |

updated every few seconds.              | input  | display    |

+--------+------------+

Using web forms and CGI scripting, the one panel can supply new values

to the graph creation function.