GALDIAG - Galacticomm Worldgroup Diagnostic Utility

Copyright (c) 1996 Galacticomm, Inc.  All rights reserved.

OVERVIEW

--------

GALDIAG provides you with the ability to log important

technical aspects of your system and the Worldgroup software

to a diagnostic file that Tech Support can use to help debug

any problems that might arise within Worldgroup.

INSTALLATION

------------

Simply copy the file GALDIAG.EXE into your main WG directory.  GALDIAG.EXE

must be launched from your main WG directory.

USAGE

-----

It is recommended you create a BBSCLEAN.BAT file, or update your

current BBSCLEAN.BAT to include GALDIAG so there is always current

information produced by GALDIAG that Tech Support can use.  Here's

an example of a new BBSCLEAN.BAT:

BBSCLEAN.BAT

------------

BBSRMB        <-- Removes Btrieve for more memory

GALDIAG /a    <-- Runs the Diagnostic program in "auto" mode

------------

Running GALDIAG in "auto" mode (indicated by the '/a' command line)

requires no user interaction.

When GALDIAG is run manually, it will prompt you with a pick-list

of diagnostic options - you have the ability to toggle these on

or off before beginning the actual logging process.

The Port Diagnostic will prompt you for a "port scan method".  Your

choices are:

1) Standard ports in DOS.

2) All ports in Hardware setup along with standard ports.

3) All POSSIBLE ports.

The third option can cause lock-ups due to the fact that a large

range of addresses are being tested.  Certain "unfriendly"

devices/drivers may reside at one of these addresses, thus

causing a lock-up.  Use the third port scan method with caution.

All information is logged to the ASCII file GALDIAG.TXT in the

main WG directory.

DISCLAIMER

----------

This module is provided "AS IS" and is not officially supported by

Galacticomm, Inc.  Galacticomm, Inc. is not responsible for any

damage or loss sustained due to installing or running this module.