Rlogin Client - connecting to a remote computer on the Internet

-------------

Rlogin allows you to make a connection to a remote computer across the

Internet.  The Worldgroup server acts as a relay station, ferrying data

in both directions between the remote site and your computer.  The basic

format of the command is:

/rlogin <hostname>

This is sufficient for most uses of rlogin, for example when

connecting to another Worldgroup on the Internet.  Here is the complete

format of the /rlogin command, with all its options:

/rlogin   <hostname> | <ipaddress> | last   [-l <username>]

[-a]

[-e <abortstr>]

[-p <port #]

<hostname>     An Internet host domain name, such as gcomm.com

or ds.internic.net.

<ipaddress>    32-bit Internet IP address, such as 199.227.15.16

or 129.40.2.59.

last           Reuses the last site from your most recent

/host, /rlogin, or /ftp command.

-l <username>  Specifies login as <username>.

-a             Specifies an ASCII connection, which may be

aborted.

-e <abortstr>  Specifies string to abort rlogin session.

Only valid when used in conjunction with -a

option.

-p <port #>    Specifies the port number to connect to

on the outbound address.

You can type the /rlogin command from almost any terminal-mode prompt.

From some prompts on Worldgroup, however, the /rlogin command may

automatically return you to a parent menu when you're done.

Here are some simple examples of the /rlogin command:

/rlogin gcomm.com

/rlogin 199.227.15.16

/rlogin last

/rlogin gcomm.com -l stryker

/rlogin gcomm.com -a -e !!!

/rlogin 199.227.15.16  -p 2200

You'll usually specify Internet domain names for rlogin connections,

but you can specify an IP address too (the four numbers separated by

dots).  Using "last" instead of a host name or IP address will

reconnect to the last host you specified in a /ftp, /rlogin, or

/host command.

Rlogin always connects to port 513 on the specified host

unless a specified port number is entered.

Exiting from rlogin:

To disconnect from the remote computer and return to your

Worldgroup server:

wait 3 seconds

type --- (3 dashes or <abortstr> specified in -e option)

If this doesn't work, wait a little longer, and type 3 dashes

(or <abortstr> specified in -e option) again.

Upper/Lower case:

When connecting to a host on the Internet using its domain name, case

is unimportant.  For example, all of these commands would connect you

to the Galacticomm Demonstration System:

/rlogin gcomm.com

/rlogin GCOMM.COM

/rlogin GcOmM.CoM