TIPX - TBBS/IPAD IPX protocol            Rev: 01/23/95

This protocol was designed by eSoft, Inc. and is released into the

public domain.  Feel free to use it for any purpose you wish.

Rather than use SPX to obtain a connection between computers, the TBBS/IPAD

IPX protocol uses only IPX itself.  It uses the minimal portion of the IPX

protocol in order that it may operate on the widest range of IPX network

hardware and software.  Note:  This protocol DOES count on any bridges or

internetwork IPX routers implementing SAP (Service Advertising Protocol).

A TBBS/IPAD server (i.e. a node which will accept one or more incoming

connections from other nodes) will advertise itself as a TYPE=9191h server.

Each server is located on the network (or internetwork) using the Service

Advertising Protocol (broadcast messages to the special socket 0452h) and

posts a server name which may be up to 48 characters in length.

A TBBS/IPAD client who wishes to locate a specific server name, should

broadcast a SAP query packet with a Query Type=3 and a Server Type=9191h.

It should then examine each SAP response packet it receives to find the

name of the server it wishes to connect with.  When it finds the name,

the same packet will give the full 12 byte IPX address (net:node:socket)

of that server and the client may proceed to attempt to open a circuit

directly with that IPX address.

The TBBS/IPAD IPX protocol uses standard IPX packets (type=4) with a maximum

data field length of 520 bytes (total packet size of 550 bytes).  The format

of the data within these packets is as follows:

TBBS/IPAD IPX packet data block format

Offset

dec hex

�����������������������������������������������Ŀ

0   0 �       # of bytes in data field (High Order)   �

�����������������������������������������������Ĵ

1   1 �       # of bytes in data field (Low Order)    �

�����������������������������������������������Ĵ

2   2 �       Sequence Number (High Order)            �

�����������������������������������������������Ĵ

3   3 �       Sequence Number (Low Order)             �

�����������������������������������������������Ĵ

4   4 �       Packet Type                             �

�����������������������������������������������Ĵ

5   5 �       Circuit Number                          �

�����������������������������������������������Ĵ

6   6 �       Checksum (High Order)                   �

�����������������������������������������������Ĵ

7   7 �       Checksum (Low Order)                    �

�����������������������������������������������Ĵ

8   8 �       First byte of data field (may be absent)�

�����������������������������������������������Ĵ

�                . . .                          �

�����������������������������������������������Ĵ

n   n �       Last byte of data field (512 bytes max) �

�������������������������������������������������

The checksum is calculated on all bytes in the data packet and IPX

header excepting the checksum bytes themselves.  It is a 16 bit number

formed by adding the bytes in the packet and ignoring carry out of

the 16 bits.  Note:  Data packets run a separate sequence number series

from all other packets to allow certainty as to how many data packets

have been received.

TBBS/IPAD Protocol Data Packet Types

Packet Type 0 - Request Connection

Bytes 8-n: Text of Server Name a connection is requested with.

This packet is always sent to the server's circuit 0.  The server will

respond with a TYPE=1 packet to either grant or refuse the connection.

Packet Type 1 - Connection Request Response

Byte 8: Circuit Number Opened (1-254).  If this circuit number is

zero, then your request for a connection was refused.

Byte 9-10: Sequence number of the Request Connection packet this

is a response to (hi-lo order).

Note:  This response is always sent to the client's circuit 0.

Packet Type 2 - ACK Packet(s)

Byte 8-9: Sequence number of packet being acknowledged.

Byte 10-11: Control Info per type 4 packet.  Bit 0 of byte 11

has no meaning here.

This packet indicates that the packet with the indicated sequence number

has been received from the other end and has been processed.

Note:  Receipt of an Ack for a sequence number, also acks all packets

with a lower sequence number that are outstanding.

Packet Type 3 - NAK Packet

Byte 8-9: Sequence number of packet being negatively acknowledged.

Byte 10-11: Reason for rejection.

0=data error.  Packet was malformed or failed checksum.

8000h=data error.  Packet reassembly timed out.

1=closed circuit.  Data, Control, or ACK/NAK packet was

sent to a circuit > 0, and that circuit was not open

to the sender's IPX address.

2=Protocol Error.  The packet was valid, but was not

valid here based on the protocol.

3=Data Packet outside of window.  The "naking" program

is holding unacknowledged data which makes its window

too small to accept this packet.

This packet indicates that the packet with the indicated sequence number

has been received from the other end, but had an error and should be resent.

Note:  Receipt of an NAK for a sequence number, implies Acks for all packets

with a lower sequence number that are outstanding.

Packet Type 4 - Circuit Control

Byte 8: Control Flags as follows:

Bit 7: 1 = may send more data, 0 = flow control halt data (RTS)

Bit 6: 1 = Circuit active, 0 = Circuit inactive (CD)

Bit 5: 1 = Circuit connected.

Bits 4-0: reserved, always 0.

Byte 9: Bits 7-1: Reserved, Always 0.

Bit 0: 0=original, 1=ACK of rcvd control packet.

Note:  Receipt of a flow control packet is always given an ACK packet.

Receipt of a circuit inactive packet when the circuit is active

is always "acknowledged" with a return circuit inactive packet,

indicating your end is changing state to disconnected.  If you

already see the circuit as disconnected, respond with an ACK.

Packet Type 5 - Data Packet

Up to 512 bytes of data to be transferred on the indicated circuit.

Note:  Data packets should be responded to with an ACK packet.

Protocol State Tables

The TBBS/IPAD protocol state tables presume the following:

1.  That general IPX initialization has been done.

2.  That received packets are processed via an ESR at interrupt

level, while outgoing packets are sent from a pool of ECBs

which are not released until the ACK or Time Out tasks

do so.  That is, the packets are sent, and the sending task

is finished with all IPX duties.

3.  The overall protocol transaction follows this sequence.

a.  A client will locate a server it wishes to connect to, using

the Service Advertising Protocol (SAP) "Find Sever" process.

b.  The client will then request of that server to open a

data circuit using the "Open Circuit" process.

c.  The client and server will then use the data circuit

until one of them closes it, at which time the total

client/server transaction has ended.  A new transaction

between this client and server may begin at either

(1) or (2) above as required.

4.  Any packets received by either a client or server from an IPX

address not currently assigned to this circuit will be NAK'd

with "circuit not open" responses if they are valid TBBS/IPAD

protocol format packets, or silently discarded if they are not

TBBS/IPAD protocol format packets.

5.  Although the protocol is designed with an eye towards allowing

sliding windows, at this time all implementations have a

window size of 1.

6.  Security items such as verifying that the incoming source address

matches the address assigned to an open circuit before processing

a packet are assumed to occur in ESR entry, and are not reflected

in these state tables.

"Find Server" Client SAP query transaction

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� FS0 � SQInit   �                         � Open any IPX socket,    � FS1 �

�     �          �                         � Set retry count.        �     �

��������������������������������������������������������������������������Ĵ

� FS1 � SndQuery �                         � Set T/O timer, Send     � FS2 �

�     �          �                         � SAP query packet to     �     �

�     �          �                         � FFFF:FFFFFF:0452 (from  �     �

�     �          �                         � socket opened in FS0).  �     �

��������������������������������������������������������������������������Ĵ

� FS2 � WtServer �1� Rcv SAP response Pkt  � store name/address,     � exit�

�     �          � � with desired server's � set "server found"      �     �

�     �          � � name and address      �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Rcv SAP response Pkt  � discard packet          � FS2 �

�     �          � � with another server's �                         �     �

�     �          � � name and address      �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� Rcv packet that is    � discard packet          � FS2 �

�     �          � � not SAP response pkt  �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �4� T/O Timer expires     � Decrement Retry Counter � FS3 �

��������������������������������������������������������������������������Ĵ

� FS3 � QryAgain �1� Retry counter > 0     �                         � FS1 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Retry counter = 0     � Set "server not found"  � exit�

����������������������������������������������������������������������������

"SAP server" - Server SAP response transaction

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� SS0 � SSInit   �                         � Open Socket 0452h       � SS1 �

��������������������������������������������������������������������������Ĵ

� SS1 � SSInit2  �1� Socket "in use" open  � Bridge/Router doing     � exit�

�     �          � � failure               � this function for us    �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Socket Opened         � Post SAP receive ECBs   � SS2 �

�     �          � �                       � and buffers to listen   �     �

�     �          � �                       � on socket 0452h         �     �

��������������������������������������������������������������������������Ĵ

� SS2 � SSResp   � Rcv SAP query on socket � Send our SAP Advert pkt � SS2 �

�     �          � 0452h                   � to the IPX address in   �     �

�     �          �                         � the query's IPX header  �     �

�     �          �                         � "source" field.         �     �

����������������������������������������������������������������������������

Note:  Every 60 seconds a server should broadcast its SAP Advertising

Packet to FFFF:FFFFFF:0452h from its own socket (obtained in FS0).

This will tell any bridges or routers about the server's presence.

This should be done unconditionally, whether socket 0452h can be

opened or not.

Opening a Data Circuit (Client makes request)

Client:  Sends Request for Connection (Type 0) packet to server on

Circuit 0.

Server:  Sends Connection Request Response (Type 1) to client on

Circuit 0.

Client:  Sends an ACK to the server on Circuit 0.

1. If Circuit Number given in response is 0, client's request

has been refused.  The client cannot have a connection

with the server at this time.

2. If the Circuit Number is > 0, then the server is inviting

the client to open the indicated circuit.  In this case

the client has 5 seconds to complete the following transaction:

Client:  Sends a Circuit Control (Type 4) packet with "Circuit connected".

"May send data" and "Circuit Active" Bits indicate initial state

of the circuit, and may be set to any initial conditions the

client program desires. This packet is sent to the circuit number

just granted.

Note:  All packets on this circuit now must have the circuit

number in the packet header.

Server:  Sends a Circuit Control (Type 4) packet with "Circuit connected".

"May send data" and "Circuit Active" Bits indicate initial state

of the circuit from the server's perspective.

Client:  Sends ACK for the Server's Circuit Control packet.

The circuit is now open.  Normal traffic on an open circuit is

data packets, control packets and acks or naks.

Closing a Data Circuit Disconnection

If the Client initiates the circuit close:

Client:  Sends Circuit Control packet (type 4) with "Circuit Connected" Bit

set to zero (circuit # in header = this circuit).

Note:  Until the circuit is shut down, the client should continue

to ACK data packets normally.

Server:  Responds with Circuit Control packet (type 4) with "Circuit

Connected" bit set to zero (circuit # in header = this circuit).

Client:  Responds with ACK for server's Circuit Control packet and the

circuit is disconnected (circuit # in header = this circuit).

If Server initiates the circuit close:

Server:  Sends Circuit Control packet (type 4) with "Circuit Connected" Bit

set to zero (circuit # in header = this circuit).

Note:  Until the circuit is shut down, the server should continue

to ACK data packets normally.

Client:  Responds with Circuit Control packet (type 4) with "Circuit

Connected" bit set to zero (circuit # in header = this circuit).

Server:  Responds with ACK for Client's Circuit Control packet and the

circuit is disconnected (circuit # in header = this circuit).

Receive a packet on Circuit 0 - client

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� CR0 � WtC0Rcv  �1� Rcv Type 0 pkt        � Send NAK, protocol err  � CR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Rcv Type 1 pkt        � Post for CO2            � CR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� Rcv Type 2 (ACK) pkt  � Discard Pkt             � CR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �4� Rcv Type 3 (NAK) pkt  � Discard Pkt, post reason� CR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �5� Rcv Type 4 (CTL) pkt  � Send NAK, protocol err  � CR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �6� Rcv Type 5 (DATA) pkt � Send NAK, protocol err  � CR0 �

����������������������������������������������������������������������������

Receive a packet on Circuit 0 - server

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� SR0 � WtS0Rcv  �1� Rcv Type 0 pkt        �                         � SO0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Rcv Type 1 pkt        � Send NAK, protocol err  � SR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� Rcv Type 2 (ACK) pkt  � Post for SO4            � SR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �4� Rcv Type 3 (NAK) pkt  � Discard Pkt, post reason� SR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �5� Rcv Type 4 (CTL) pkt  � Send NAK, protocol err  � SR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �6� Rcv Type 5 (DATA) pkt � Send NAK, protocol err  � SR0 �

����������������������������������������������������������������������������

Note:  In a combination client/server, these two tables are combined

into a single Circuit 0 receive packet handler.

"Open Circuit" - Client transaction

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� CO0 � COInit   �                         � Set retry counter       � CO1 �

��������������������������������������������������������������������������Ĵ

� CO1 � COQuery  �                         � Set T/O timer, Send     � CO2 �

�     �          �                         � Circuit Request Pkt     �     �

�     �          �                         � to server (from socket  �     �

�     �          �                         � opened in FS0).         �     �

��������������������������������������������������������������������������Ĵ

� CO2 � WtCirc   �1� T/O timer expires     � Decrement Retry counter � CO3 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Rcv circuit response  � Send ACK packet (on     � CO4 �

�     �          � � packet                � circuit 0)              �     �

��������������������������������������������������������������������������Ĵ

� CO3 � COQrtry  �1� Retry counter > 0     �                         � CO1 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Retry counter = 0     � Set "server timed out"  � exit�

��������������������������������������������������������������������������Ĵ

� CO4 � CORcvd   �1� Rcv circuit 0 in the  � Set "server busy"       � exit�

�     �          � � response pkt          �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Rcv circuit "n" in    � Set Retry counter       � CO5 �

�     �          � � the response pkt      �                         �     �

��������������������������������������������������������������������������Ĵ

� CO5 � COAsgn   �                         � Set Circuit "n" seq #   � CO6 �

�     �          �                         � to 0001, Set T/O timer, �     �

�     �          �                         � Send circuit control    �     �

�     �          �                         � pkt with "circuit       �     �

�     �          �                         � connected" set          �     �

��������������������������������������������������������������������������Ĵ

� CO6 � COWtOpn  �1� T/O timer expires     � Decrement Retry counter � CO7 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Rcv circuit control   � Mark Circuit Open       � exit�

�     �          � � pkt w/"circuit        �                         �     �

�     �          � � connected" set        �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� Rcv circuit control   � Mark Circuit Closed,    � exit�

�     �          � � pkt w/"circuit        � set "circuit closed by  �     �

�     �          � � connected" reset      � server"                 �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �4� Rcv NAK, circuit not  � Mark Circuit Closed,    � exit�

�     �          � � open                  � set "circuit closed by  �     �

�     �          � �                       � server"                 �     �

��������������������������������������������������������������������������Ĵ

� CO7 � COWtRty  �1� Retry counter > 0     �                         � CO5 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Retry counter = 0     � Set "server timed out"  � exit�

����������������������������������������������������������������������������

"Open Circuit" - Server transaction

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� SO0 � SOInit   � Rcv Circuit Request pkt � Set retry counter       � SO1 �

��������������������������������������������������������������������������Ĵ

� SO1 � SOAssign �1� Circuit not avail     � Send Response Pkt with  � SO2 �

�     �          � �                       � Circuit=0,              �     �

�     �          � �                       � Set T/O timer           �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Circuit is avail      � Send Response Pkt with  � SO4 �

�     �          � �                       � Circuit="n",            �     �

�     �          � �                       � Mark circuit "opening", �     �

�     �          � �                       � Set T/O timer           �     �

��������������������������������������������������������������������������Ĵ

� SO2 � SOWtRej  �1� Rcv ACK pkt           �                         � exit�

�     �          ���������������������������������������������������������Ĵ

�     �          �2� T/O timer expires     � Decrement retry counter � SO3 �

��������������������������������������������������������������������������Ĵ

� SO3 � SORetry1 �1� Retry counter > 0     � Re-send Response pkt,   � SO2 �

�     �          � �                       � Set T/O Timer           �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Retry counter = 0     � Mark circuit avail      � exit�

��������������������������������������������������������������������������Ĵ

� SO4 � SOWtACK  �1� Rcv ACK and circuit   � Mark circuit "assigned",� SO6 �

�     �          � � marked "opening"      � Set T/O timer           �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Rcv ACK and circuit   � Circuit now active      � exit�

�     �          � � marked "open"         �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� T/O timer expires     � Decrement retry counter � SO5 �

��������������������������������������������������������������������������Ĵ

� SO5 � SOWAT0   �1� Circuit marked "open" � Lost ACK, but all is OK � exit�

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Retry counter > 0     � Resend response pkt,    � SO4 �

�     �          � �                       � Set T/O timer           �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� Retry counter = 0     � Mark Circuit "avail"    � exit�

��������������������������������������������������������������������������Ĵ

� SO6 � SOWAT1   � T/O timer expires       �                         � SO7 �

��������������������������������������������������������������������������Ĵ

� SO7 � SOMBO    �1� Circuit marked "open" �                         � exit�

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Circuit not marked    � Mark circuit "avail"    � exit�

�     �          � � "open"                �                         �     �

����������������������������������������������������������������������������

Note:  A circuit marked "avail" on a server, is also assumed to be marked

"closed" for any test of condition.

Receive a packet on Circuit "n" - client or server (n > 0)

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� NR0 � WtCnRcv  �1� Rcv Type 0 or 1 pkt   � Send NAK, protocol err  � NR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Rcv Type 2 (ACK) pkt  �                         � RA0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� Rcv Type 3 (NAK) pkt  �                         � RN0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �4� Rcv Type 4 (CTL) pkt  �                         � RC0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �5� Rcv Type 5 (DATA) pkt �                         � RD0 �

����������������������������������������������������������������������������

Rcvd Control Packet on circuit "n" (client or server)

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� RC0 � RcvCtl0  �1� Circuit "n" not closed�                         � RC1 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Circuit "n" closed,   � Send NAK, circuit not   � NR0 �

�     �          � � & "Circuit Connected" � open                    �     �

�     �          � � in rcvd pkt = set     �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� Circuit "n" closed,   � Send Circuit Control    � NR0 �

�     �          � � & "Circuit Connected" � pkt w/circuit connected �     �

�     �          � � in rcvd pkt = reset   � reset, ACK=1            �     �

��������������������������������������������������������������������������Ĵ

� RC1 � RcvCtl1  �1� "Circuit Connected"   � Mark circuit "open" and � RC2 �

�     �          � � in rcvd packet = set  � post other packet       �     �

�     �          � �                       � control bits.           �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Circuit Connected in  � Mark circuit "closed"   � RC2 �

�     �          � � rcvd packet = reset   �                         �     �

��������������������������������������������������������������������������Ĵ

� RC2 � RcvCtl2  �1� "ACK" bit = 1 in      �                         � NR0 �

�     �          � � rcvd packet           �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� "ACK" bit = 0 in      � Send Circuit Control    � NR0 �

�     �          � � rcvd packet           � pkt w/circuit connected �     �

�     �          � �                       � same as rcvd packet,    �     �

�     �          � �                       � other bits per current  �     �

�     �          � �                       � states, ACK=1           �     �

����������������������������������������������������������������������������

Rcvd ACK Packet on circuit "n" (client or server)

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� RA0 � RcvACK0  �1� Circuit "n" not closed�                         � RA1 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Circuit "n" closed    � discard ACK pkt         � NR0 �

��������������������������������������������������������������������������Ĵ

� RA1 � RcvACK1  �1� ACK seq # is within   � Mark all pkts sent with � NR0 �

�     �          � � the "window" of sent  � this sequence number or �     �

�     �          � � packets               � older seq number as     �     �

�     �          � �                       � sent, release their     �     �

�     �          � �                       � ECBs and buffers        �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� ACK seq # is outside  � discard ACK packet      � NR0 �

�     �          � � the "window" of sent  �                         �     �

�     �          � � packets               �                         �     �

����������������������������������������������������������������������������

Rcvd NAK Packet on circuit "n" (client or server)

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� RN0 � RcvNAK0  �1� Circuit "n" not closed�                         � RN1 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Circuit "n" closed    � Discard NAK pkt  (?)    � NR0 �

��������������������������������������������������������������������������Ĵ

� RN1 � RcvNAK1  �1� Reason = Data out of  � Reset seq# to NAK RSN,  � RN2 �

�     �          � � window                � Re-send all un-ack'd    �     �

�     �          � �                       � data pkts w/new seq #s  �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Reason = Pkt data err �                         � RN4 �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� Reason = Other        � Report Reason           � RN3 �

��������������������������������������������������������������������������Ĵ

� RN2 � RcvNAK2  �                         � Mark Any pkts in range  � NR0 �

�     �          �                         � RSN-WIN < # < RSN-1     �     �

�     �          �                         � as sent, release their  �     �

�     �          �                         � ECBs and buffers        �     �

��������������������������������������������������������������������������Ĵ

� RN3 � RcvNAK3  �                         � Mark Any pkts in range  � NR0 �

�     �          �                         � RSN-WIN < # < RSN       �     �

�     �          �                         � as sent, release their  �     �

�     �          �                         � ECBs and buffers        �     �

��������������������������������������������������������������������������Ĵ

� RN4 � RcvNDErr �1� RSN outside send      � Discard NAK pkt         � NR0 �

�     �          � � window                �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� RSN inside send window� Resend RSN packet       � RN2 �

����������������������������������������������������������������������������

Rcvd DATA Packet on circuit "n" (client or server)

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� RD0 � RcvDATA0 �1� Circuit "n" not closed�                         � RD1 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Circuit "n" closed    � Send NAK, circuit not   � NR0 �

�     �          � �                       � active                  �     �

��������������������������������������������������������������������������Ĵ

� RD1 � RcvDATA1 �1� Seq # = next in order � Send ACK, process data  � NR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Seq # > next, but is  � Hold Packet, set T/O    � RD2 �

�     �          � � within window range   � timer                   �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� Seq # > next and NOT  � Send NAK on "next" seq  � NR0 �

�     �          � � within window range,  � number, discard pkt     �     �

�     �          � � or Seq# < next-win-1  �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �4� Seq # < next          � Send ACK, discard pkt   � NR0 �

��������������������������������������������������������������������������Ĵ

� RD2 � RcvDATA2 �1� T/O timer expires     � discard all held, out   � NR0 �

�     �          � �                       � of order pkts, release  �     �

�     �          � �                       � their ECBs and buffers, �     �

�     �          � �                       � Send NAK on "next" seq  �     �

�     �          � �                       � #, "reassembly timeout" �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Rcv Seq # > next, but � Hold Packet             � RD2 �

�     �          � � within window range   �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �3� Seq # > next and NOT  � Send NAK on "next" seq  � RD2 �

�     �          � � within window range,  � number, discard pkt     �     �

�     �          � � or Seq# < next-win-1  �                         �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �4� Seq # < next          � Send ACK, discard pkt   � RD2 �

�     �          ���������������������������������������������������������Ĵ

�     �          �5� Seq # = next in order � Process all pkts which  � RD3 �

�     �          � �                       � have been received in   �     �

�     �          � �                       � order, send ACK on the  �     �

�     �          � �                       � highest pkt # processed �     �

��������������������������������������������������������������������������Ĵ

� RD3 � RcvDATA3 �1� Still have held pkts  � Set T/O timer           � RD2 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� No held pkts remain   � Turn off T/O timer      � NR0 �

����������������������������������������������������������������������������

Send DATA packet on circuit "n" (client or server, n > 0)

��������������������������������������������������������������������������Ŀ

�State� State    � Predicate(s)            � Action(s)               � Next�

�  #  � Name     �                         �                         � St  �

��������������������������������������������������������������������������Ĵ

� SD0 � SndDATA0 �                         � Set retry counter       � SD1 �

��������������������������������������������������������������������������Ĵ

� SD1 � SndDATA1 �                         � Send Packet,            � SD2 �

�     �          �                         � Set T/O Timer           �     �

��������������������������������������������������������������������������Ĵ

� SD2 � SndDATA2 �1� Rcv ACK on packet     � (handled by RAn table)  � NR0 �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� T/O timer expires     � Decrement Retry Counter � SD3 �

��������������������������������������������������������������������������Ĵ

� SD3 � SndDATA3 �1� Retry Counter > 0     � Resend Oldest Packet,   � SD2 �

�     �          � �                       � Set T/O Timer           �     �

�     �          ���������������������������������������������������������Ĵ

�     �          �2� Retry Counter = 0     � Close circuit internally� NR0 �

�     �          � �                       � and report "other guy   �     �

�     �          � �                       � timed out"              �     �

����������������������������������������������������������������������������

############################################