-------------------------------------------------

HOW TO SETUP DESQVIEW AND A BULLETIN BOARD SYSTEM

-------------------------------------------------

Documentation Written By Mike Wood  Jan. 18, 1994

Table of Contents

-----------------

Chapter 1 - General Information

1.1  Introduction

1.2  System Requirements

1.3  Getting Started

Chapter 2 - DESQview Setup

2.1  DV Optimum Setup

2.2  Each Node's PIF Setup

2.21  General

2.22  Advanced

2.3  Caches and RAMdisks

2.4  Batch Files

Chapter 3 - Fossil Driver Setup

3.1  General

3.2  Suggested Setup for Config.sys

3.3  Port Addresses and IRQs

3.4  UART Chips

Chapter 4 - Interrupts

4.1  Sharing Interrupts

4.11  IRQ Hogs

4.12  Sharing Between Two Modems

4.13  Sharing between a Modem and a Non-Modem

4.2  Interrupts to be Avoided

Chapter 5 - Transfer Protocols

5.1  DSZ

5.11  DSZ with COM3, COM4, and non-standard Interrupts

5.12  Line Status Register: 02 Errors

5.2  Other Protocols

Chapter 6 - Forcing a Non-MultiNode BBS Program to run MultiNode

6.1  Potential Problems

6.2  Disadvantages

6.3  How to "FAKE" MultiNode Operation

Chapter 1:  General Information

-------------------------------

1.1  Introduction

This document has been written to aid sysops who are interested in

running a bulletin board system, in a multitasking environment (specifically

DESQview).  Whether you simply want to use your computer as a user is logged

on, a private node for you to log on locally, or to run a MultiNode system

is immaterial, all three are easily accomplished.  If you do NOT need Multi-

Node, simply skip sections as appropriate.  You will encounter a LOT of

problems, and some problems may be solely due to your system's configuration.

This text cannot hope to cover all topics, or all configurations.  However,

I hope to be as informative as possible, to the extent of my experience.  No

information in this document is guaranteed to be correct, use at your own risk.

1.2  System Requirements

386DX or greater

Approximately 1 MB per node

2+ Modems (Hayes AT command set compatible)

Serial card(s) as necessary with 16550 Buffered UART chips

DESQview, DESQview/X, or any other TRUE Multitasking environment.

A lot of patience.

1.3  Getting Started

Install DESQview.  Allow it to set up any program it auto-detects, if you

wish.  Other optimization settings will be covered later, so accept defaults.

I am assuming you will install in C:\DV, if this is not applicable please

substitute your path whenever I mention C:\DV later on in the text.

Chapter 2:  DESQview Setup

--------------------------

2.1  DESQview Optimum Setup

To setup DESQview, load DESQview, press "ALT" to activate the menu if it is

not already active, then "O" to Open Window, then "SE" to Setup Desqview.

Pressing "Space" will allow you to setup a few VERY VERY basic details.  I'll

ignore that, I'm sure you can figure that out on your own.  Press "Enter".

Entries that are NOT listed can be safely ignored, or are trivial.

* Keyboard - Learn [NO]                 ; Saves a bit of CPU Power, Memory

Maintain sep. shift [NO]   ; UNLESS you run a program that uses

; ALT, SHIFT, or CTRL where one of

; these keys ALONE will activate some

; subroutine in an active program.

Change System Keys [NO]

Logical Drives - Ignore               ; Unless you run a Network

Network - Modify as appropriate IF you have a network.

* Performance - Task Processing Time    ; I have found 2/2 to work acceptably.

Foreground:  [2]      ; Setting too low means too much time

Background:  [2]      ; switching between tasks, too high

; slows down the system visibly.

; It may look JERKY to YOU, but likely

; a user will NOT notice this.

- Optimize Communications ; I have this set to YES, contrary to

; X00's documentation for multinode

; use.  I find YES will allow a more

; stable operation, less errors.

; NOTE: You MAY have to change this!

- Allow Swapping [NO]     ; Saying YES will allow programs to be

; swapped to DISK.  This CAN slow down

; the system.  Use if you DO NOT HAVE

; ENOUGH RAM and cannot buy more.

- Manage Printer Cont.    ; I have this set to >>NO<<.  Paragon

; will HANG if loaded in 2 windows if

; this is set to YES.  However YES is

; recommended, so if your bbs program

; WILL LOAD 2 NODES <w/ modems online>

; and both work, set it to YES.  Do not

; worry about this until the system

; works already.

Video Monitor - Do you want gfx and text at the same time [NO]

; This prevents text programs from

; being run in graphics mode.

2.2  Customizing Each Node's PIF

Type ALT to activate DV.  Press "O", followed by "AP" to Add a Program, or

CP to Change Program if you have already added the bbs programs.  Press "O"

to add an "OTHER" program.  You will be prompted for "Path".  Enter "C:\"

even if your board is NOT in the root directory (this is probable).  I am

assuming you will use a .BAT file to load the board, and while batch files

are a little bit slow I find it gives maximum configurability for mail,

optional "resetting the com port", etc.. which are different for each node.

2.21  General Setup

Now you will be shown a screen, which can from now on be activated by using

"CP" or Change Program.  Customize the first page as follows:

Program Name..................:  <YOUR BOARD> Node x (COMx)

Keys to Use on Open Menu......:  P1   ; any 2 keys, I use the last digit

; to show NODE NUMBER and the first

; for me represents "paragon".

Program...:  C:\FRONT-N1.BAT          ; meaning FrontDoor Node 1, you may

; use any batch name you wish.

Parameters:                           ; leave blank unless you do not use

; my batch system and wish to load

; your BBS directly, with parms.

Directory.:  C:\                      ; this should already be here.

* Options

Writes text directly to screen    ; If you have an option in your bbs

; called "Direct Screen Writes" set

; it OFF and this to NO.  If you have

; an option "BIOS Video" set it to YES

; and this to NO.  If you can choose

; either, select BIOS Video, and set

; this to NO.  If you have no option

; in your bbs prog, try NO and if you

; have problems set it to YES.

; NOTE: Direct Screen writes is

; normally faster, but SLOWER in a

; multitasking environment!

Displays Graphics Information [NO]

Virtualize Text/Graphics [NO]     ; If you notice screen "bleeding"

; as one node's characters show up in

; another window, and setting "writes

; text to screen" (see above) does NOT

; solve the problem, try "T".

Uses Serial Ports [NO]            ; Your fossil will manage all serial

; communications.

Requires Floppy [NO]

2.22  Advanced Options

From the Setup menu, press F1.  The top segment can be left as it is.

Customize the bottom as follows:

Close on exit             [ ]          Uses own colours       [Y]

Allow Close Window       *[Y]*         Runs in background    *[Y]*

Uses Math Co-Pro         *[N]*         Keyboard Conflict      [0]

Share CPU when foreground*[Y]*         Share EGA when foregnd [Y]

Can be swapped out       *[N]*         Protection level       [0]

Asterisks (*) denote a setting which is either MANDATORY or there is

absolutely no doubt in my mind that you could need to change it.  Others

are recommended settings, but may be changed if necessary.

2.3  Caches and RAMdisks

2.31 CACHE Programs

There are a LOT of Cache programs on the market, which vary in

effectiveness and effeciency.  There IS a big difference between these

two -- For example, Norton Cache is VERY effective.  But not effecient

at all..  It will cause a tremendous increase in hard disk speed, as long

as you can sacrifice CPU power.  In a Multi-Node environment, this cache

will actually SLOW down the system, where in a 'normal' system, most

people may claim that this is a VERY fast cache.  On the other hand,

HyperDisk has a good combination of the two.  It will increase the speed

of your hard drive, while not noticeably cutting down the processing power

of your computer.  You will have to experiment with various caches out

there -- remember: The only TRUE test is when people are on ALL nodes and

doing something (preferably doing something that requires a lot of HD use).

In addition, you must beware of file protocol errors, namely DSZ's Line

Status Register: 02.  Some caches will cause these errors, some will not.

2.32 RAMdisks

Another alternative to speed up hard drive access is the use of a RAMdisk.

With Revelation, I have found that the best usage of RAM is to put the

EXE and OVR's on a small RAMdrive.  Simply call the program FROM the normal

starting directory (ie: change to that directory) then call the RAMdrive's

drive letter as follows:  x:\bbsprog.exe   to run it.  I have found that

THIS will increase speed *FAR* more than an additional 800k in Cache.

This is equivalent to loading up EVERY node's OVR into EMS/XMS as far as

speed goes, yet only takes ONE loading into EMS/XMS for ALL nodes.  (Thus

eliminating the need for that option, if available (so turn it OFF)).

2.4  Batch Files

You will need to set up batch files to run your BBS.  Set them up in the

C: Root directory, and call them something that indicates both the file's

purpose, and the NODE NUMBER.  Personally I have it set to FRONT-Nx.BAT where

FRONT indicates it runs FrontDoor, and the 'x' is the NODE NUMBER.  Edit these

as follows:

c:\dv\dvansi

cd\bbs_path

bbs_program -Parameters

cd\

exit

If you are using a FrontDoor, simply use the FrontDoor batch file instead.

You also may wish to turn your port ON (you must specify exactly WHICH port

to turn ON!) using BNU (yuk) or X00 (recommended).  You may also wish to load

with a NODE parameter if possible with your bbs.  If you are using a FrontDoor

batch file, remember to change mention of any comm ports!  ** YOU MUST REFER

TO YOUR BBS PROGRAM'S DOCUMENTATION FOR MULTI-NODE PATHS AND OPTIONAL

PARAMETERS **  (If your bbs is >>NOT<< explicitly multi-node compatible, you

can PROBABLY fake it as shown in Chapter 6)

Chapter 3:  Fossil Driver Setup

-------------------------------

3.1  X00 vs. BNU

If you have a choice between these two, I would highly advise the use of X00.

A lot of people sing the praises of BNU, and while these people claim it is

faster than X00, this is simply not the case in a multitasking environment.

It has virtually NO multi-node support, limited shared-file compatibility, and

will generally fuck you over.  Thus I will only document the usage of X00 in

this text.

3.2  Suggested X00 Setup in CONFIG.SYS

DEVICE=C:\X00\X00.SYS E 0=3F8,IRQ4 B,0,38400 (etc) F=15 R=4096 T=2048

^^^   ^^^^^^^^^^ ^^^^^^^^^       ^^^^ ^^^^^^^^^^^^^

|          |        |             |      |  Buffers. Modify

|          |        |             |         if you wish

|          |        |              -- FIFO Activation.

|          |        |                 See Section 3.4

|          |         -- I prefer to lock my ports here but

|          |            it can be done later with XU.EXE

|           -- This locks port 0 (COM1) at address 3F8h with

|              IRQ 4.  This can be modified for each port.

|              **NOTE** if you use this, do NOT specify the

|              "number" of ports to be simultaneously

|              supported.

-- You can simply rename X00.EXE to X00.SYS (I prefer this

because X00 will not initialize if my LAN is already

loaded.  (It's also recommended by the author of X00).

3.3  Table of Common PORT addresses and IRQs

--------------------------------------------

| COMx  |  Address (hex)  |  Suggested IRQ |

|       |                 |                |

|  1    |      3F8        |        4       |

|  2    |      2F8        |        3       |

|  3    |      3E8        |        5* (4)  |  ; If available, use IRQs 5 and 7

|  4    |      2E8        |        7* (3)  |  ; to minimize conflicts.  See

--------------------------------------------  ; Chapter 4

3.4  UART Chips

If you wish to multitask effectively with a high speed movem (ie: above 24oo)

then you absolutely MUST have a 16550 Buffered UART chip.  There is a lot of

information about the different chips available, distinguishing between the

'AN', 'AFN', and a few other extensions for this name.  I have found that all

chips with '16550' on it, will work effectively.  To activate the 16-byte

buffer, simply add a parameter to X00 saying "F=15", which uses a 15-byte

buffer (16-byte is apparently unreliable, according to X00 documentation).

Chapter 4:  Interrupt Requests (IRQs)

-------------------------------------

4.1  Sharing Interrupts

4.11  Interrupt HOGS

Some programs are interrupt "HOGS".  This means that they require absolute

control over the interrupt request at certain times (or at all times).  If

you are attempting to share an interrupt between two programs in which one

(or both) is an interrupt "HOG", there are a few symptoms:  (1)  The programs

will not initialize the modems at the same time.  One program will initialize

the modem, and the other will be forced to wait (or exit with an error) until

the IRQ becomes available.  (2)  While one modem is active (ie: a user is on-

line), the modem sharing that interrupt will not initialize, register a RING,

etc.  ** IF THIS IS THE CASE, YOU >>CANNOT<< SHARE INTERRUPTS **

4.12  Sharing IRQs between Multiple Modems

This section only applies for programs which do >>NOT<< fall under the

previous category.  If your program allows the initialization of two modems

which share IRQs at the SAME TIME, and will allow both modems to operate,

then it is worthwhile to continue looking into IRQ sharing.

4.13  Interrupt Conflicts with NON-Modems

If you attempt to share a modem with another card, you cannot use both cards

simultaneously.  For example, if you have a modem using IRQ7 and a printer

on LPT1, you cannot print while a user is online on that node.  Other

potential problems include LPT2 at IRQ5, sound cards, scanner cards, etc.

4.2  Interrupts to be Avoided

I have noted several problems when using IRQ2, which are seemingly erratic or

unexplicable.  Some DOOR programs balk at the use of IRQ2, even if they are

ignorant of the IRQ.  Some BBS programs do not operate correctly.  If you MUST

use IRQ2, set up the board using (and testing) the other nodes first, then with

the SAME (working) setup, modify all references from the old interrupt to IRQ2

and test thoroughly.  If you notice any problems, you must use a different

interrupt request priority level.

Chapter 5:  Transfer Protocols

------------------------------

5.1  DSZ

DSZ is, to my awareness, the only protocol which is COMPLETELY DESQview aware,

and DESQview polite.  If set up correctly, you will rarely have any problem

with DSZ.

5.11 DSZ With COM3 or COM4, or With NON-Standard Interrupts.

DSZ deactivates X00.  This means that you must explicitly tell DSZ where to

find the ports (if remapped), and what interrupt to use (if not standard).

Most bbs programs do NOT have a MCI code which says the hex address of the

port being used, nor do they have a MCI code for the interrupt (as most bbs

programs assume defaults, or are ignorant of this information).  Thus you

may be forced to setup batch files where the MCI code is included in the

filename.  For example, telegard-based systems (Revelation, Paragon,

Renegade, etc) use the %P (COM port) MCI code.  Instead of using this as a

parameter, modify the protocol so that it loads a program called DSZ%P.BAT

-- this will load DSZ1.BAT for com 1, DSZ2.BAT for com 2, etc.  Simply make

the modifications necessary within the batchfile (information such as

address, IRQ, etc), and after these node-specific modifications, simply add a

%1 %2 %3 %4 %5 (etc).  (These are replaced by the FIRST parameter to the

batch, the second, etc..)

Example:

DSZ.EXE portx 3F8,4 %1 %2 %3 %4 %5 %6 %7

^^^^^ ^^^^^ ^^^^^^^^^^^^^^^^^^^^-- My protocol setup drops as many

|     |                          as seven parameters.  If less are

|     |                          given, the rest will be ignored.

|      -- This is the port ADDRESS followed by IRQ NUMBER.

-- This replaces ALL OTHER port specifications.

5.12  Line Status Register: 02

This error is a recurring problem with multi-node systems using a cache.

If you are using a cache, disable it if you think your system will not be

slowed too much without it.  Alternatives include an on-board caching

controller card for your hard drive, a faster hard drive, etc.  OR, it may

be caused by another problem -- for more information see the DSZ doc file.

5.2  Other Protocol Programs

These are beyond the scope of this document.  There are too many for me to

be experienced with them all.  You must experiment on your own.

Chapter 6:  Forcing a NON-MultiNode BBS Program to run MultiNode

----------------------------------------------------------------

6.1  Potential Problems

The major disadvantage to running a bbs program in this fashion is that you

will be susceptible to file sharing violations, and "unexplicable" dropping

of carrier (caused by a file sharing problem).  This will usually result in

a one-time system error.  For example:  Packing mail while a user is reading

mail, two users editing file descriptions simultaneously, and many more.

HOWEVER, these problems usually just drop the user and re-load.  If the user

calls back, the problem will not happen again.

6.2  Disadvantages

Users will not be able to tell who is online.  Multi-Node chats will not be

convenient -- there will be no way to "Page" another user to load the multi-

node chat door.  And there is (obviously) no internal chat system, or a

"Who's online" system.

6.3  How to "FAKE" MultiNode Operation

In order to minimize the sharing problems, to compensate for the lack of

multiple settings in the modem setup, and to compensate for changes that are

absolutely MANDATORY to be different for each node (ie: online programs, etc),

The bbs must be loaded differently.  Assuming your board is ALREADY SET UP,

modify as indicated.

1.  In your BBS Main directory, add directories named NODE1, NODE2, (etc).

2.  Copy the following files into these directories:

- program.ovl program.dat status.dat (etc) but >>NOT<< the .EXE file

- modem.dat

- online batch files

3.  Replace the command which loads your board to the following:

cd\bbsdir\node1                     ; replace node1 with node2,3,4

x:\bbsdir\bbsprog.exe (parameters)  ; to load the different nodes.

In this manner, your program >>SHOULD<< look in the current directory for

the modem information, online batch files, etc.. and now you can have multiple

modem configurations while sharing the same high message pointers, userlist,

file listings, and all the rest.

------------------------------------------------------------------------------

Infernal Majesty                                       MultiNode Documentation

/ltars o .\\adness                          Second Revision, January 18 1994