System:    Pressurizer

Operating Limits:    Level - 50% to 85%

Function:  Maintains  primary  loop  pressure by  creating  steam

bubble.   Provides  reservoir  of primary  coolant  to

handle sudden changes in temperature, thereby insuring

that  the core remains covered.   Pressurizer  heaters

must  be covered at all times {heaters  located  below

45% level}.

Controls:  Pressurizer heaters operated by F7.  Pressurizer level

controlled  by F2 {Primary Drain Valve} and 'D' {Make-

up Feed Direction Control}.  Use F2 to drain, and 'DP'

to fill pressurizer.  Note:  Make-up Feed Pump must be

operating {'M' control}.

System:    Reactor Vessel

Operating Limits:    Temperature - 75 to 800F

Pressure - 0 to 3000 Psi.

Level - 100%

Rod Height - 0 to 100%

Function:  Provide  control,  structure,  and cooling for reactor

core.   Designed  to maintain core completely  covered

and  adequately cooled.   Core must not be above  212F

without pressurization {See pressurizer}.   Rod height

controls reactivity of core and heat produced.

Controls:  'ESC' command  initiates complete  shutdown  including

starting  emergency pump and make-up  feed  pumps,  as

well  as shutting pressurizer drain valve and dropping

rods to bottom.   'F1' drops rods to bottom and  shuts

pressurizer drain valve.  Both SCRAM commands turn off

automatic  control system.   'R' command controls  rod

height.   Pressurizer  level  maintains  pressure  and

reactor  level.   'P'  command controls  coolant  flow

through core.   'E' command controls emergency cooling

flow to core.

System:    Primary Loop

Operating Limits:    Primary Flow - 0 to 100%

Function:  Primary  loop  provides recirculation  of  pressurized

water  to remove heat from the reactor core.   Flow is

maintained by the primary coolant pump.   The  primary

loop  is  monitored  for any  leakage,  and  excessive

radiation levels {indicating core damage or failure}.

Controls:  'P' command controls coolant flow rate.   See  reactor

vessel,  emergency  cooling system and pressurizer for

other controls.

System:    Emergency Cooling System

Operating Limits:    Level - 0 to 100%

Flow - 0 to 100%

Function:  Provides  rapid  cooling directly to reactor  core  in

case of primary or secondary loop failure.   Emergency

cooling  system  is  designed for one  time  operation

only.   It must then be repaired during a normal main-

tenance shutdown.

Controls:  'ESC' starts 100% flow to reactor.   'E' command  con-

trols  emergency cooling pump flow.   Reactor must  be

completely  shutdown  to  re-fill  emergency   cooling

reservoir.

System:    Heat Exchanger

Operating Limits:    Temperature - 75 to 500F

Level - 65 to 100%

Function:  Provides   heat   transfer  from  primary  system   to

secondary system.  Provides steam generation for oper#

ation of the turbo-generator system.  Isolates potent#

ially contaminated coolant from rest of system.   Pro#

vides  surge  volume  for changes  in  temperature  of

secondary system.  Pressurizes secondary system.

Controls:  'DS' controls heat exchanger level.   If make-up  feed

pump  is off and 'DS' is selected,  the heat exchanger

will be drained.   Otherwise,  heat exchanger will  be

filled.   'P'  controls heat flow into heat exchanger,

and 'S' controls heat flow out of heat exchanger.

System:    Secondary Cooling System

Operating Limits:    Flow  - 0 to 100%

Temperature - 75 to 500F

Level  - 50 to 100%

Pressure - 0 to 1000 Psi

Condenser Inlet Temperature - 75 to 300F

Condenser Pressure - 14 to 0 Psi.

Condenser Outlet Temperature - 75 to 212F

Function:  Provides  recirculation of water to produce steam  for

turbine.   System takes suction on condenser discharge

and provides pressurized water to heat exchanger.

Controls:  'S'  controls coolant flow in secondary system.   'DS'

directs make-up feed water to secondary system.   'F3'

and 'F4' control turbo-generator.

System:    Turbo-Generator

Operating Limits:    Heat exchanger temperature - 212 to 500F

Turbine Speed  - 3,600 RPM

Function:  Produces  electricity  from  steam generated  in  heat

exchanger.   Field control starts electricity  genera#

tion and controls turbine speed.

Controls:  'F3' trips turbine on and off line.   Turbine must not

have  secondary coolant temperature below 212F.   'F4'

trips  generator field on and off.   Field  should  be

selected  at 3,250 RPM and will regulate speed.   'F8'

controls lube oil pump.

System:    Lubricating Oil System

Operating Limits:    Lube Oil Level - 50 to 100%

Lube Oil Pressure - 80 to 90 Psi

Lube Oil temperature - 75 to 200F

Function:  Provides lubrication to the turbine and generator.

Controls:  'F8'  trips lube oil pump on and off.   Lube oil level

can  only be increased during  maintenance  shutdowns.

Auxiliary power 'F10' must be on for pump to operate.

System:    Make-up Feed System

Operating Limits:    Level  - 10 to 100%

Flow - 0 to 100%

Recirculation Flow - 0 to 50%

Function:  Provides  additional coolant to the primary and secon#

dary cooling systems to make-up for losses from  leak#

age or expansion.  Make-up feed pump should be left in

recirculation  mode  when not in  use.   Make-up  feed

reservoir is replenished continuously.

Controls:  'M'  controls  make-up feed flow  rate.   'D'  directs

make-up  feed  to 'P' primary  system,  'S'  secondary

system, 'N' recirculation.

System:    Auxiliary Power

Operating Limits:    None

Function:  Provides  electrical  power during start-up and  shut#

down, and whenever generator is not producing power.

Controls:  'F10' trips auxiliary power on and off.

System:    Alarm and Warning

Operating Limits:    None

Function:  Provides  warnings,  advisory  and  emergency  alarms.

Yellow alarms are advisory or warning in  nature,  and

red  alarms are emergency conditions.   Warning alarms

provide  indication of a condition approaching  damage

levels.  Other warning indications are provided by the

symbols  of  pumps.   An '+' indicates  pump  off,  an

'O'indicates  pump  running and an 'X' indicates  pump

damaged.   Turbine speed is indicated by the vanes  in

the turbine assembly.   When the vanes are horizontal,

the turbine is stopped.   When the vanes are verticle,

the  turbine is at normal operating speed.   The  same

symbols apply to the generator.   Detailed information

on  the  warning  and  alarm  system  is  availble  by

lightpen on the alarm or through the Help key.

Controls:  'Up  Arrow' silences the audible alarm,  and resets it

so that when a new warning occurs,  the alarm  sounds.

'U'  disables the audible alarm,  until 'U' is pressed

again.  'Q' tests all alarms by turning them all on.

System:    Automatic Rod Control System

Operating Limits:    Operating Range - 350 to 505F

Engagement Range - 500 to 750F

Function:  Controls  heat  exchanger temperature to within  limit

set  by  Control Bias value,  1 to  5  degrees.   Heat

exchanger  temperature may be set to the Control  set#

point or may be automatically  calculated.   Automatic

Rod  control  system should be used whenever heat  ex#

changer temperature exceed 494F.

Controls:  'F5' selects automatic control system on or off.   'C'

sets heat exchanger temperature to reference.  'B' set

sensitivity range for control.   This range should  be

set  to  1  for operation near  system  limits.   'F6'

enables automatic temperature calculation depending on

demand.  'ESC' and 'F1' turn off this system  whenever

they are used.

SYSTEM ALARMS

01

RX TEMP - High Reactor Temperature

This alarm indicates high reactor core temperature.   Imme-

diate action should be taken to reduce core temperature  by

using Emergency SCRAM 'ESC', SCRAM 'F1', or Rod Height 'R0'.

Also consider using Emergency Cooling 'E100'.

Warning  (Yellow)  Point:   Reactor core temperature  above

750F.

Danger (Red) Point:  Reactor core temperature above 800F.

02

RX PRESS - Low Reactor Pressurization

This alarm indicates low reactor core pressurization, which

could result in localized boiling and fuel element failure.

Use Emergency SCRAM 'ESC'.

Warning (Yellow) Point:  Reactor pressure below 2 X Reactor

temperature.

Danger (Red) Point:   Reactor pressure below 1.8 X  Reactor

temperature.

03

RX Level - Low Reactor Level

This  alarm  indicates a loss of reactor coolant such  that

the reactor core is in danger of becoming  uncovered.   Use

Emergency Cooling 'E100',  or Make-up Feed to Primary 'M100

DP'.

Warning (Yellow) Point:  None.

Danger (Red) Point:  Reactor coolant level below 100%.

04

Rod Bottom - Rod Bottom Indication

This indication indicates when the control rods are comple-

tely inserted.

Warning (Yellow) Point:  Control Rods on bottom, completely

inserted.

Danger (Red) Point:  None

05

RAD HIGH - High Radiation Level

This  alarm indicates above normal radiation levels in  the

containment building.  This may be due to normal continuous

high  power operation or to fuel element failure.   No  im-

mediate action necessary.

Warning  (Yellow) Point:   Radiation level above 200 Milli-

rem/hour.

Danger  (Red)  Point:   Radiation level  above  500  Milli-

rem/hour.

06

Press Lvl - Pressurizer Level

This  alarm indicates that the pressurizer level is out  of

specification.   A  level  too high will result in a  pres-

surizer failure and primary coolant  leakage.   Pressurizer

level too low will result in pressurizer heater failure and

loss of pressurization to the reactor.   Control level with

Primary Drain Valve 'F2' and Make-up Feed 'DP'.

Warning  (Yellow)  Point:   Pressurizer level above 85%  or

below 50%.

Danger (Red) Point:   Pressurizer level above 90% or  below

45%.

07

Pri Flow - Primary Coolant Flow Rate

This  alarm indicates that the primary coolant flow rate is

out  of range.   If the flow is too low,  the reactor  core

receives inadequate cooling.  If too high, the primary pump

is overstressed and may fail prematurely.

Warning (Yellow) Point:   Primary flow above 90%,  or below

.12 X Reactor temperature.

Danger (Red) Point:  Primary flow 0.

08

Emerg Low - Low Emergency Coolant Level

This alarm indicates that the Emergency Coolant  reservoir

is below safe operating levels.

Warning (Yellow) Point:  Emergency Coolant below 25%.

Danger (Red) Point:  Emergency Coolant level below 10%.

09

Xchg Temp - High Heat Exchanger Temperature

This alarm indicates that the Heat Exchanger temperature is

exceeding design limits.   Action should be taken to reduce

Primary  temperature and to increase Secondary Coolant Flow

'S100'.

Warning (Yellow) Point:   Heat Exchanger temperature  above

495F.

Danger (Red) Point:  Heat Exchanger temperature above 500F.

10

Sec Level - Secondary Coolant Level

This alarm indicates low Secondary Coolant level.   Make-up

Feed 'DS' should be used to bring the level back to 100%.

Warning (Yellow) Point:  Secondary Coolant level below 90%.

Danger (Red) Point:  Secondary Coolant level below 50%

11

Sec Flow - Secondary Coolant Flow

This  alarm indicates that the Secondary Coolant flow  rate

is out of range.  Low Secondary Coolant flow can cause Heat

Exchanger  failure or Secondary Coolant pump damage.   High

flow can result in overstressing the pump.

Warning (Yellow) Point:   Secondary Coolant Flow rate above

90%.

Danger (Red) Point:  Secondary Coolant Flow rate 0.

12

Cnd In Temp - High Condenser Inlet Temperature

This  alarm  indicates high  condenser  inlet  temperature.

This can result in damage to the turbine, Secondary Coolant

pump and to the Condenser circulating pump.

Warning (Yellow) Point:  Condenser inlet temperature  above

275F.

Danger  (Red)  Point:   Condenser inlet  temperature  above

300F.

13

Cnd Hi Press - High Condenser Pressure

This  alarm  indicates high Condenser Pressure due to  high

condensate temperature or low Secondary Coolant flow.  High

Condensate  pressure  damages the Secondary  Coolant  pump.

Increase Secondary Coolant Flow 'S100'.

Warning (Yellow) Point:  Condenser Pressure above 4 Psi.

Danger (Red) Point:  Condenser Pressure above 10 Psi.

14

Cnd Out Tmp - High Condenser Outlet Temperature

This  alarm  indicates high Condenser  outlet  temperature.

High condensate temperature causes damage to the  Secondary

Coolant pump.   Increase Secondary Coolant flow 'S100',  or

decrease Heat Exchanger Temperature.

Warning (Yellow) Point:  Condenser Outlet Temperature above

195F.

Danger  (Red)  Point:   Condenser Outlet Temperature  above

212F.

15

Turb Speed - Turbo-Generator Speed

This alarm indicates that the Turbo-Generator system is not

on  design speed of 3,600 RPM.   Turbo-Generator  overspeed

damages  the turbine and generator.   Generator Trip  valve

'F3'  and Generator Field control 'F4' effect Turbo-Genera-

tor speed.

Warning (Yellow) Point:   Turbo-Generator speed other  than

3,600 RPM.

Danger (Red) Point:  Turbo-Generator speed above 3,636 RPM.

16

LOil Temp - High Lubricating Oil Temperature

This  alarm  indicates High  Lubricating  Oil  temperature.

This  can  damage the Lubricating Oil pump and  the  Turbo-

Generator.

Warning (Yellow) Point:   Lubricating Oil Temperature above

200F.

Danger  (Red)  Point:   Lubricating Oil  Temperature  above

212F.

17

LOil Level - Lubricating Oil Level

This alarm indicates low Lubricating Oil level.   Low level

can damage the Lubricating Oil Pump and Turbo-Generator.

Warning (Yellow) Point:  Lube Oil Level below 35%.

Danger (Red) Point:  Lube Oil Level below 15%.

18

LOil Press - Lubricating Oil Pressure

This alarm indicates unsafe Lubricating Oil Pressure.   Low

Lube Oil pressure can damage the Turbo-Generator.  Lube Oil

Pump control 'F8'.

Warning  (Yellow)  Point:   Lube Oil pressure above 90  and

below 79.

Danger (Red) Point:   Lube Oil pressure above 95 and  below

72.

19

MkFeed Level - Low Make-up Feed Level

This  alarm indicates when Make-up Feed level is below safe

operating levels.   Make-up feed replenishment will  slowly

take place during normal operations; system refilled during

maintenance shutdowns.

Warning (Yellow) Point:  Make-up Feed level below 40%.

Danger (Red) Point:  Make-up Feed level below 5%.

20

Pump Temp - High Pump Temperature

This  alarm indicates that one or more of the six pumps  is

overheating.   The  pumps  are Primary  Coolant,  Secondary

Coolant,  Condenser Circulation,  Lube Oil, Emergency Cool-

ing, and Make-up Feed.

Warning (Yellow) Point:  Pump temperature above 175F.

Danger (Red) Point:  Pump temperature above 212F.

21

Cntrl Warn - Control/Auxiliary Power Warning

This  alarm indicates loss of auxiliary power or  an  error

detected  by  the automatic control system.   Loss of  aux-

iliary  power results in failure of all pumps and  controls

if the generator is not producing power.

Warning (Yellow) Point:   Automatic Control off above  600F

reactor temperature.

Danger  (Red) Point:   Automatic Set-point off and  Control

set point above 494F.  Loss of Auxiliary Power.

22

Turb Trip - Turbo-Generator Tripped

This alarm indicates that the Turbo-Generator trip valve is

open,  and  that  the  secondary coolant  is  being  dumped

directly into the condenser.

Warning (Yellow) Point:  Turbo-Generator Trip valve open.

Danger  (Red) Point:   Turbo-Generator not tripped and Heat

exchanger temperature below 212F.

23

Pri Drain - Primary Drain Valve Open

This  alarm indicates that the Primary Drain Valve is  open

and allowing Primary Coolant to drain from the pressurizer.

Warning (Yellow) Point:  Primary Drain Valve open.

Danger (Red) Point:  None.

24

Pr Htr OFF - Pressurizer Heaters Off

This alarm indicates that the Pressurizer Heaters are  off.

The  Reactor  temperature should not exceed 212F  when  the

Pressurizer Heaters are off.

Warning (Yellow) Point:  None

Danger (Red) Point:  Pressurizer Heaters Off

99

FUNCTION KEY DEFINITIONS (file FUNKEY.DOC)

01

ESC  -  Emergency Shutdown.

This  function key causes the reactor to  execute

an  Emergency  SCRAM.   The rods are sent to  the

bottom,  and  the make-up feed pump speed is  in-

creased  to  100% and the make-up  feed  flow  is

directed to the primary loop.   The primary drain

valve  is shut and the emergency cooling pump  is

started.   This function key will safely shutdown

the  reactor  and minimize damage to the  reactor

core.  It should be used in any emergency.

02

F1  -  SCRAM.

This function key shuts down the reactor by send-

ing  the rods to the bottom and by  shutting  the

primary drain valve.  This function key should be

used  while conducting a normal shut-down of  the

reactor.  Care must be taken to prevent damage to

the  reactor  core due to low  pressurizer  level

when using this function key.

03

F2  -  Primary Drain Valve.

This  function  key  controls the  primary  drain

valve which is used to maintain pressurizer level

within the required range on 50-85%.   This  fun-

ction  key alternately opens and shuts the  drain

valve.

04

F3  -  Turbine Trip Valve.

This  function key controls the position  of  the

turbine  trip  {bypass}  valve which is  used  to

prevent  liquid water from entering the  turbine.

This  valve  should be open  {Turb  Trip  warning

(Yellow)  on} whenever heat exchanger temperature

is  below  100C.   Care should be  taken  not  to

exceed  the condenser pressure/temperature limits

when operating this valve.

05

F4  -  Generator Field Control.

This  function key controls the  generator  field

and  speed control system.   The generator  field

must  be  on for production of power.   When  the

generator field is on, the turbine speed is auto-

matically regulated to 3,600 RPM to maintain  the

electric  current frequency at 60 Hz.   This fun-

ction  key  provides the only direct  control  of

turbine speed.

06

F5  -  Automatic Control.

This  function  key controls  the  automatic  rod

height  control system which is designed to main-

tain  the  heat exchanger  temperature  near  the

control set point temperature.   This system fun-

ctions  only  above 500C reactor temperature  and

will  maintain the temperature within  the  range

specifed  by  the Bias setting below the  control

set point temperature.

07

F6  -  Automatic Set-point.

This  function  key controls  the  automatic  set

point   computer.    This  computer  adjusts  the

control set point and primary and secondary cool-

ant  pumps to maintain the production rate  above

the demand.   This computer should generally only

be used when the demand is above 1700 KW.

08

F7  -  Pressurizer Heaters.

This  function key controls the pressurizer  hea-

ters which are used to maintain a steam bubble in

the pressurizer.   The steam bubble creates pres-

sure  on the primary loop which prevents  boiling

in the reactor core,  which could cause fuel ele-

ment failure or meltdown.    The pressurizer hea-

ters  must be on any time the reactor temperature

exceeds 100C.    Primary coolant pressure must be

at  least 2X reactor core temperature to  prevent

damage to the reactor core.

09

F8  -  Lube Oil Pump.

This  function key controls the operation the the

turbine and generator lubricating oil circulation

pump.   This  pump provides high pressure oil  to

the bearings of the turbine and  generator.   The

Lube  oil pump must be on whenever the turbine is

operating.

10

F9  -  Condenser Circulating Pump

This   function   key  controls   the   condenser

circulating  pump.   This pump  provides  coolant

flow  for the condenser.   The exhaust steam from

the  turbine  is condensed into  water,  and  the

condensate  is pumped to the steam generator  for

recycling.   This pump should be operating  when-

ever the secondary temperature is above 212F.

11

F10  -  Auxiliary Power.

This  function key controls the  auxiliary  power

source  which  is used to provide electricity  to

the  reactor plant whenever the generator is  not

functioning.   Auxiliary power is required during

start up and shutdown of the reactor  plant,  and

should be on at all times.

12

Left Arrow  -  Plant Display.

This  function  key swithes the  display  between  the

graphic  and  numeric displays.   The numeric  display

shows the current value of the specific system values.

This display also shows the last 5 log entrys,  if the

Autolog-Screen  function  is  selected.   The  graphic

display  shows  a schematic of the plant and  a  chart

displaying  the  last  30 minutes  of  the  four  most

important indicators.   Pumps,  valves, and levels are

indicated  on the schematic.   Pumps that are off  are

indicated  by  an  '+',  pumps that  are  running  are

indicated  by an 'O',  and pumps that have failed  are

indicated  by an 'X'.   Shut valves are indicated by a

solid  white circle.   Open valves are indicated by  a

black stripe indicating the direction of flow  through

the valve.  Fluid levels are indicated by white blocks

in the various vessels and storage tanks.  Turbine and

generator  speeds  are indicated by the angle  of  the

throttle  vanes  in the symbols.   A  horizontal  vane

indicates  speed  0,  and  a verticle  vane  indicates

normal operating speed.

13

Up Arrow  -  Alarm Silence.

This  function key switches the audible alarm  on  and

off.   The alarm sounds whenever there is a warning or

danger indication on the alarm panel.

COMMAND KEYS (file COMKEY.DOC)

01

A     -  Select Auto Log Feature.

The  'A'  command selects the Auto Log  feature  which

provides  a  means  to make a hard copy  record  of  a

simulation run.   The 'AE' commands enables the  auto-

log feature and allows selection of the destination of

the log output.  'S' selects the display screen on the

numeric display screen,  'D' selects a disk file named

'HISTORY.DAT',  and  'P' selects the printer connected

to  'LPT1:'.   Auto-Log reocrds every command  isssued

and any danger warnings issued by the  reactor.   This

feature  can  be  disabled by the 'AD'  option  or  by

pressing 'A' and 'Return'.

02

B     -  Set Bias value.

The  'B' command allows the setting of the bias  value

used  by the automatic control system to maintain  the

heat exchanger temperature near the set point tempera-

ture.   This  value may range from 1 to 5.   1 is  the

most sensitive setting and should be used whenever the

reactor is operating close to its' limits.  This value

may only be changed when automatic control is on.  The

set  point  temperature is controlled by the 'C'  com-

mand.

03

C     -  Set Automatic Control Temperature.

This command sets then heat exchanger temperature that

will  be  maintained by the automatic control  control

system {F5}.   The value can be set anywhere from 350F

to  505F.   The automatic control program will  adjust

rod height to maintain the heat exchanger  temperature

within  the  Bias setting of the control  temperature.

This  temperature  will be automatically  adjusted  to

meet power demand when Automatic Set-point {F6} is on.

This value can only be changed when automatic  control

is on.

04

D     -  Make-up Feed Destination.

This  key allows selection of the destination for  the

make-up feed flow.  'P' causes make-up feed to be sent

to the primary loop, 'S' causes feed to be sent to the

secondary  loop,  and 'N' causes feed to be  recircul-

ated.   The  make-up feed pump should be operating for

flow  to go to the primary or  secondary  loops.   The

secondary  loop will drain if 'S' is selected and  the

make-up feed pump is off.

05

E     -  Emergency Coolant Pump Speed.

This command selects the emergency cooling pump speed.

The pump can either be off, speed 0, or on, speed 1 to

100%.   This  pump is used to provide rapid cooling to

the  reactor core.   The emergency cooling  supply  is

limited, but is adequate to lower the core temperature

to safe levels.

06

F     -  Freeze and Save Simulation Run.

This  command  stops the simulation run and saves  the

pertinent variables to a file named 'RXFRZ.DAT'.  When

the simulation is started at a later time,  the opera-

tor  is asked whether the saved simulation  should  be

executed.   A 'Y'es answer will load the variables and

restart  the  simulation at the point it  was  frozen.

Only one simulation run can be frozen at any time.

07

G     -  Current Production Graph

This command graphs the electrical output against  the

demand.   The  graph  shows  the most recent  10  hour

period.

08

09

M     -  Make-up Feed Pump Speed.

This  command  selects the speed of the  make-up  feed

pump.  The pump can be off, speed 0, or on, speed 1 to

100%.  This pump is used to provide additional coolant

to the primary and secondary cooling systems.   As the

reactor  heats up,  primary coolant must be drained to

maintain  the primary volume within  limits.   As  the

reactor  cools down,  coolant must be added to make up

for the drained coolant.   The secondary system  loses

coolant through gland sealing steam and minor leakage.

This pump should be on at all times, running at 50% or

less.   In  this recirculation mode,  make-up feed  is

instantly available.

10

P     -  Primary Coolant Pump Speed.

This  command  selects the pump speed of  the  primary

coolant  pump.   This  pump circulates  high  pressure

water through the reactor core and transfers heat from

the reactor to the heat exchanger.  This pump normally

operates at 90% or above.  Both the primary and secon-

dary coolant pumps must both be off to completely shut

down the reactor.

11

Q     -  Alarm and Warning Test

This command tests the alarm and warning indicators on

the  console.   Pressing this key a second time resets

the  alarm and warning indications and shows only  the

active alarms.

12

R     -  Rod Height.

This  command  selects the control  rod  height.   The

control   rods   provide  direct  control   over   the

reactivity of the core, and thereby control the core's

temperature.   The  higher the rods,  then the core is

more reactive,  and more heat is produced.  The opera-

ting  range of the rods is between 0,  on the  bottom,

and 100%,  completely withdrawn.   The two SCRAM  com-

mands {ESC  and  F1} cause the rods  to  be  instantly

dropped to the bottom.

13

S     -  Secondary Coolant Pump Speed.

This  command selects the speed of the secondary cool-

ant  pump.   This pump circulates the condensed  steam

from the cooling tower/condenser to the heat exchanger

where  it again produces steam.   This  pump  normally

operates  at  90% or above.   Both this pump  and  the

primary  coolant  pump must be off to completely  shut

the reactor down.

14

T     -  Terminate Simulation

This command terminates the simulation and  calculates

a  score based on the production rate,  damages incur-

red,  and production deficit.   The score is  adjusted

for  the length of the simulation run.   A  cumulative

score   is  maintained  in  the  file  'RXSTATUS.DAT'.

Erasing this will clear the cumulative score.

15

U     -  Disable Alarm

This  command switched the audible alarm system on and

off.

16

V     -  Current Version

This  command displays the current version  number  of

the  Reactor-100 main program.   Please refer to  this

number when reporting any problems with the program.

17

Z     -  Change Timer Setting

This command allows changes to the time interval  bet-

ween  screen updates.   The allowable intervals are  0

(no  delay),  or 3 to 10 seconds.   The normal  update

interval is once every 2.5 seconds.

18

'H'   -  Provides information.

Help  is available by using the help key or using  the

lightpen.   The  help menu can be used by entering the

number next to the subject of interest.   The lightpen

can be used to select Help on any item of interest  by

pointing  the lightpen at the subject and pressing the

switch.   The  lightpen can also be used on  the  Help

menus.

19

Tab   -  Bypass Sign-on Message

This command can be used during the program sign-on to

bypass  the delay.   This will also bypass the initial

Help screen.