VESA SuperVGA BIOS Extensions

--------------------------------------------------------------------------

- VESA Standard Modes -

15-bit mode	7-bit mode

number		number		Resolution	Colors

------------------------------------------------------

100h		-		640x400		256

101h		-		640x480		256

102h		6Ah		800x600		16

103h		-		800x600		256

104h		-		1024x768	16

105h		-		1024x768	256

106h		-		1280x1024	16

107h		-		1280x1024	256

108h		-		80x30		color text

109h		-		132x25		color text

10Ah		-		132x43		color text

10Bh		-		132x50		color text

10Ch		-		132x60		color text

--------------------------------------------------------------------------

- Return Status from a VESA BIOS Extention Call	-

AL == 4Fh; Function is supported

AH == 00h; Function call successful

NOTE: All non-zero returns in AH should be treated as failures.

--------------------------------------------------------------------------

* Return Super VGA Information *

Entry:	AH = 4Fh	; Super VGA support

AL = 00h	; Return Super VGA Information

ES:DI   	; Pointer to 256 byte buffer

Exit:	AX		; Status

--------------------------------------------------------------------------

- Structure of the Super VGA Information buffer -

VgaInfoBlock	struc

VESASignature	db 'VESA'	;4 signature bytes

VESAVersion	dw ?		;VESA version number

OEMStringPtr	dd ?		;Pointer to OEM string

Capabilities	db 4 dup (?)	;capabilities of video environment

VideoModePtr	dd ?		;pointer to supported SuperVGA modes

TotalMemory	dw ?		;(VBE v1.1) :Num of 64K blocks on board

db (236) DUP 00h

VgaInfoBlock	ends

--------------------------------------------------------------------------

* Return Super VGA Mode Information *

Entry:	AH = 4Fh	; Super VGA support

AL = 01h	; Return Super VGA Mode Information

CX = Super VGA video mode

ES:DI   	; Pointer to 256 byte buffer

Exit:	AX		; Status

--------------------------------------------------------------------------

- Structure of the Super VGA Information buffer -

ModeInfoBlock	struc

;mandatory information

ModeAttributes		dw ?	;mode attributes

WinAAttributes		db ?	;Window A attributes

WinBAttributes		db ?	;Window B attributes

WinGranularity		dw ?	;window granularity

WinSize		dw ?	;window size

WinASegment		dw ?	;Window A start segment

WinBSegment		dw ?	;Window B start segment

WinFuncPtr		dd ?	;pointer to window function

BytesPerScanLine	dw ?	;bytes per scan line

;extended information

;optional information

XResolution		dw ?	;horizontal resolution

YResolution		dw ?	;vertical resolution

XCharSize		db ?	;character cell width

YCharSize		db ?	;character cell height

NumberOfPlanes		db ?	;number of memory planes

BitsPerPixel		db ?	;bits per pixel

NumberOfBanks		db ?	;number of banks

MemoryModel		db ?	;memory model type

BankSize		db ?	;bank size in K

NumberOfImagePages	db ?	;(VBE v1.1) Number of pages

Reserved		db 1	;(VBE v1.1) Signature ID

db (225) DUP 00h

ModeInfoBlock	ends

--------------------------------------------------------------------------

- Format of ModeAttributes in the Super VGA Information Structure -

D0 = Mode supported in hardware

0=Mode not supported in hardware

1=Mode supported in hardware

D1 = Extended information available

0=Extended mode information not available

1=Extended mode information available

D2 = Output functions supported by BIOS

0=Output functions not supported by BIOS

1=Output functions supported by BIOS

D3 = Monochrome/color mode

0=Monochrome mode

1=Color mode

D4 = Mode type

0=Text mode

1=Graphics mode

D5-D15 = Reserved (=0)

--------------------------------------------------------------------------

- Format of WinAAttributes and WinBAttributes in the Super VGA

Information Structure -

D0 = Window supported

0=Window is not supported

1=Window is supported

D1 = Window readable

0=Window is not readable

1=Window is readable

D2 = Window writable

0=Window is not writable

1=Window is writable

D3-D7 = Reserved (=0)

--------------------------------------------------------------------------

- Format of MemoryModel in the Super VGA Information Structure -

00h = Text mode

01h = CGA graphics

02h = Hercules graphics

03h = 4-plane planar

04h = Packed pixel

05h = Non-chain 4, 256 color

06h-0Fh = Reserved, to be defined by VESA

10h-FFh = To be defined by OEM

--------------------------------------------------------------------------

- Set Super VGA Video Mode -

Entry:  AH = 4Fh 	; Super VGA support

AL = 02h 	; Set Super VGA Video Mode

BX = Video mode

D0-D14 = Video mode number

D15 = Clear memory flag

0=Clear video memory

1=Don't clear video memory

Exit:   AX 		; Status

-------------------------------------------------------------------------

- Return current video mode -

Entry:  AH = 4Fh	; Super VGA support

AL = 03h	; Return current video mode

Exit:   AX		; Status

BX 		; Current video mode

-------------------------------------------------------------------------

- Save/Restore Super VGA Video State -

Entry:  AH = 4Fh	; Super VGA support

AL = 04h	; Save/Restore Super VGA Video State

CX = Requested states

D0 = Save/Restore video hardware state

D1 = Save/Restore video BIOS data state

D2 = Save/Restore video DAC state

D3 = Save/Restore video Super VGA state

DL = Sub-function

00 = Return save/restore state buffer size

01 = Save Super VGA video state

02 = Restore Super VGA video state

ES:BX = pointer to buffer (DL = 01, or DL = 02)

Exit:   AX		;Status

BX =  Number of 64-byte blocks to hold the state buffer (DL = 00)

-------------------------------------------------------------------------

- Super VGA Video Memory Window Control -

Entry:  AH = 4Fh	; Super VGA support

AL = 05h	; Super VGA Video Memory Window Control

BH = Sub-function

00=Select Super VGA Video Memory Window

01=Return Super VGA Video Memory Window

BL = Window A/B (0/1)

DX = Window position in video memory (in window granularity units)

Exit:   AX		; Status

DX = Window position in video memory (in window granularity units)

-------------------------------------------------------------------------

- Super VGA Set Logical Scan Line Length - (VBE v1.1)

Entry:  AH = 4Fh	; Super VGA support

AL = 06h	; Super VGA Set/Get Logical Scan Line Length

BL = 00h	; Set Scan Line Length

CX = Desired Width in Pixels

Exit:   AX		; Status

BX = new Bytes Per Scan Line

CX = actual Pixels Per Scan Line

DX = total Number of Scan Lines

-------------------------------------------------------------------------

- Super VGA Get Logical Scan Line Length - (VBE v1.1)

Entry:  AH = 4Fh	; Super VGA support

AL = 06h	; Super VGA Set/Get Logical Scan Line Length

BL = 01h	; Get Scan Line Length

Exit:   AX		; Status

BX = Bytes Per Scan Line

CX = Pixels Per Scan Line

DX = total Number of Scan Lines

-------------------------------------------------------------------------

- Super VGA Set Display Start -	(VBE v1.1)

Entry:  AH = 4Fh	; Super VGA support

AL = 07h	; Super VGA Set/Get Display Start

BH = 00h	; reserved (must be 00h)

BL = 00h	; Set Display Start

CX = Virtual X coordinate

DX = Virtual Y coordinate

Exit:   AX		; Status

-------------------------------------------------------------------------

- Super VGA Get Display Start - (VBE v1.1)

Entry:  AH = 4Fh	; Super VGA support

AL = 07h	; Super VGA Set/Get Display Start

BL = 01h	; Get Display Start

Exit:   AX		; Status

BH = 00h	; reserved (will be 00h)

CX = Virtual X coordinate

DX = Virtual Y coordinate

-------------------------------------------------------------------------

###############################################################################