PARSETUP cable specifications.

The following is a connection list for the cable required by PARSETUP. This

is the exact same cable used by MS-DOS 6.2's INTERLNK, and LapLink's parallel

cable is also compatible. You can build your own quite easily with a

small DB25 jumper box, available at Radio Shaft, or you can purchase

ready-made LapLink cables from a number of sources.

The following eleven connections are required:

25 pin          25 pin

======          ======

pin 02  <    >  pin 15

pin 03  <    >  pin 13

pin 04  <    >  pin 12

pin 05  <    >  pin 10

pin 06  <    >  pin 11

pin 10  <    >  pin 05

pin 11  <    >  pin 06

pin 12  <    >  pin 04

pin 13  <    >  pin 03

pin 15  <    >  pin 02

pin 25  <    >  pin 25  (Ground)