Preliminary Documentation for PARSETUP

PARSETUP is a Parallel Printer Port device driver for id

Software's DOOM (tm) which will allow two computers to play

across a LapLink or InterLnk parallel cable. This file is a quick

and dirty substitute for real documentation.

Connect your computers together using a LapLink or InterLnk

cable (see CABLE.DOC for wiring diagram).

Use PARSETUP just as you would use IPXSETUP or SERSETUP. In

addition to the standard DOOM command line switches, PARSETUP

understands the following:

-player1

Forces console to be player 1 (Indigo) to override the

automatic player choice. Player 0's machine is the "master" and

sends the network start information at the start of the game. This

option can sometimes be helpful when systems of vastly different speeds

are linked together for DOOM, or if you just want to manually decide

which player will be Green and which will be Indigo.

-port

Specify the base I/O port for the printer in C-style hex

notation. Common values are 0x378, 0x278, and 0x3bc.

-irq

Specify the IRQ setting used by your printer port. Can be any

value in the range [2..7], although printer ports are usually 5 or 7.

PARSETUP defaults to I/O port 0x378, IRQ 7 which corresponds

to LPT1: on most systems.

-debug

Causes PARSETUP to perform some rudimentary testing on your

parallel link cable. See the troubleshooting section of this

document for morte details.

Troubleshooting

Q: I connected my computers together with a cable, but all I

see is "wrote: PLAY0_0" scrolling endlessly up the screen. What's

wrong?

A: There are a couple of problems which can cause this. You

might have the wrong kind of cable; the wiring of the cable used

for PARSETUP must have AT LEAST the connections listed in

CABLE.DOC. Extra wires probably won't hurt, but too few wires or

wires which connect the wrong pins will prevent PARSETUP from

communicating across that cable. Once you have verified that your

cable is wired correctly, you can test the cable using the

InterLnk program which comes with recent versions of MS-DOS.

Alternatively, you can invoke PARSETUP with the -debug

switch, which will cause PARSETUP to perform a short test

procedure before starting play. Make sure that BOTH computers

start PARSETUP with the -debug switch as well as the -port and -

irq settings appropriate for each machine. Once started, PARSETUP

will attempt to send a sequence of numbers across the link to

test all possible bit combinations which may occur during a game.

If the cable is working correctly, the numbers should scroll

rapidly up the screen and the test should successfully conclude

within a few seconds. If the computers hang on "Waiting for 0..."

it's possible that there is an incorrect -port setting on one of

the machines. Depending upon the type of printer port installed

in the machine, the base port address can be 0x378, 0x278, or

0x3bc. My suggestion is to try all three values and see if one of

them works. On the other hand, if the computers hang on a number

> 0, the port setting is correct but it's possible that one or

more of the connections needed by PARSETUP is open (i.e. not

connected). Double check your cable and try again.

Q: OK, I did all of the above, and both computers pass the -

debug test sequence, but when I try to play I *still* get an

infinite list of "wrote: PLAY0_0" strings on my screen. What

gives?

A: Since you verified that the port address and cable

connection are both OK in the earlier steps, the next problem

area involves interrupt request (IRQ) settings. A printer port

can be configured to generate an Interrupt ReQuest on one of the

available IRQ lines, usually IRQ7. In most systems, the interrupt

capability of the printer port is unused. For this reason,

systems are often configured such that network cards, sound

cards, mouse cards, or some other peripheral makes use of this

"unused" printer port IRQ line. This situation is bad news for

us, since two separate cards on a PC expansion bus cannot share

the same IRQ level. Thus, if your network card or your

SoundBlaster is set to use IRQ7, and your printer port card tries

to use IRQ7 also, the result is a conflict and PARSETUP probably

won't work.

To correct this, make a note of the IRQ settings on each

card plugged into your PC's expansion bus, as well as the actual

IRQ setting used by your printer port. If you do find a conflict,

you must change one of the cards to use a different IRQ. NOTE:

Some motherboards with built-in printer ports allow you to

enable, disable, and change the printer port IRQ settings using

jumpers on the motherboard or in the BIOS SETUP. Check your

computer documentation for more information.

Once you have reconfigured your expansion cards to avoid IRQ

conflicts, retry PARSETUP.

TO REPORT BUGS:

To help me track down any bugs you might encounter, I'll need as

much information as possible relating to your system

configuration. In particular, the following info for BOTH

computers used in your game session would be helpful:

Processor (386, 486, Pentium, Intel/AMD)

Clock Speed (33Mhz, 66MHz, 90Mhz, etc.)

Printer port BIOS designation (LPT1:, LPT2: or LPT3:)

Printer port base I/O address you used on the PARSETUP command

line Printer port IRQ you used on the PARSETUP command line

Other command line switches passed to PARSETUP (e.g. -altdeath, -file)

Results of running PARSETUP with the -debug switch

Email questions, comments, and bug reports to asre@uiuc.edu