5

AMS Models

The AMS simulates manned lunar exploration.  Included are one pilot (the

player), eight astronauts, an Orbiting Laboratory (OL), a reusable Landing

Vehicle (LV), a Manned Maneuvering Unit (UN), a Permanent Lunar Habitat (HB), a

Mobile Base (MB), a small Roving Vehicle (RV), and a Portable Life Support

System (PL or PLSS).  Many AMS models and development notes are presented in

this document.  Topics include:

The Moon

Pilot and Astronauts

Power Subsystems

Environmental Subsystems

Damage and Repair

Orbiting Laboratory

Landing Vehicle

Manned Maneuvering Unit

Permanent Lunar Habitat

Mobile Base

Roving Vehicle

Portable Life Support System

The Moon

The moon is modeled as a perfect sphere with a	90� NORTH

radius of 1738.3km.  The gravitational field    ...............		����Ŀ

is a simple inverse square law with a	  ...:	s |	  :...	 �MOON�

surface acceleration of 1.624m/s�.	   .:	    u |		 :.    ������

Global reference points are the equator .:		 b |		   :.

(0� latitude) and the subearth	    :		   e |			:

meridian (0� longitude).  The lunar   :		    a |			 :

solar rotation period is 29.53 days. :			r |			  :

:			 t |			   :

The lunar surface has constant	 :			 h |	   equator   :

altitude but is covered with    -90�:------------------+- 0� -------------: 90�

rocks, craters, hills,		WEST:			 m |			   :EAST

mountains, valleys, and rilles.	:			 e 0�			  :

Actual lunar features larger than    :			r |			  :

about 1� (30km) are included in the   :		    i |			 :

AMS map between �32� latitude and �80� :		   d |			:

longitude.  Smaller features are	   :.		 i |		   .:    view

derived using a two-step pseudorandom	:.	    a |		 .:	 from

process that generates features as small as :...	n |	  ...:	  EARTH

7.5x15m.  Mountains cover both poles and the    :.............:

antimeridian (�180� longitude).			    -90� SOUTH

Moon (continued)

Lunar surface features are listed below in increasing order of ruggedness.

Manmade objects, visible only on 4096:1 maps, are assigned other symbols listed

here and in LUNARMAP.DOC.  The dark side of the moon is indicated by a field of

"�" symbols.

SYMBOL  RUGGEDNESS  MEANING				 SYMBOL  MEANING

------  ----------  ------------------------    ------  -----------------------

" "	  1.0	featureless			    �	Landing Vehicle

�	   1.1	relatively smooth, rocky	 �	Mobile Base

�	   1.3	small, subdued crater	    �	Roving Vehicle

+	   1.4	moderate, rocky			#	pilot

�	   1.7	rolling hills			  �	Apollo descent stage

o	   2.0	medium crater			  �	Surveyor

*	   2.5	rugged, rocky			  �	Luna

O	   3.3	large crater			   �	Permanent Lunar Habitat

�	   4.0	mountain				  �	solar array

U	   5.0	valley				    �	warning light

\	   6.7	rille

/	  10.0	rille

#	  20.0	major rille

Pilot and Astronauts

The pilot and eight astronauts each have a mass of 80kg.  Three astronauts are

assigned to the OL, three to the HB, and two travel with the pilot.  While the

astronauts are not explicitly included in the simulation, the physiology of the

pilot is modeled.

Injury:  Pilot injury may occur during landing, docking, or EVA, or due to

mechanical failure or lack of oxygen.  Injuries heal slowly with time and

affect pilot efficiency.  The pilot is dead when his injuries exceed 100%.

Sleep:  The pilot requires about 8hr of sleep per day, normally between about

UT 22:00 and UT 06:00.  Lack of sleep affects pilot efficiency.

Rest:  The pilot requires two or three 2hr rest periods per day, during which

he also eats his meals.  Pilot efficiency is substantially restored during

these rest/meal periods.  One rest should be taken after waking up and another

before sleeping.

Consumables:  Each astronaut, including the pilot, requires about 0.8kg of

oxygen and 1.8kg of water per day.  A lack of oxygen very quickly leads to

injury.  A lack of water decreases efficiency.

Pilot and Astronauts (continued)

Metabolic Rate:  The pilot's metabolic rate is a measure of his current level

of activity.  Some activities are very strenuous, e.g., sample collection, and

result in higher metabolic rates than those activities that are less strenuous,

such as resting.  High metabolic rates reduce pilot efficiency.  Avoid

metabolic rates greater than the pilot efficiency.

Efficiency:  Pilot efficiency is a measure of his overall condition.  It

determines the maximum effort (metabolic rate) that he can sustain without

problems.  Efficiency depends on injury, sleep, rest, consumables, and time-of-

day.  Generally, efficiency increases in the morning and early afternoon (UT

03:00 to UT 15:00) and decreases from late afternoon to late night (UT 15:00 to

UT 03:00).  The pilot is also somewhat less efficient while he is wearing a

PLSS.

Power Subsystems

The power subsystems of the OL, LV, HB,	���������������������������������Ŀ

and MB contain solar arrays, batteries,	#						   �

fuel cells and hydrogen, oxygen, and  ����������ͻ   ���������ͻ		    �

water supplies.  The solar arrays	� Hydrogen ���#�	    �  ���������ͻ �

provide electrical power when	    ����������ͼ   �  Fuel   �  �	    ���

illuminated.  (400W/m� at 30%				    �	    ��#�  Water  �

efficiency).  Surplus power is used   ����������ͻ   �  Cells  �  �	    �Ŀ

to recharge the battery (at 50%	  �  Oxygen  ���#�	    �  ���������ͼ �

efficiency) or to disassociate	   ����������ͼ   ���������ͼ	  #	 �

excess water into hydrogen and		    #		   #		  �	 �

oxygen (1g water/29kJ at 45% efficiency).   �	   ���������ͻ  ���������ͻ �

Fuel cells supply power and water		 �	   �	    �  �  Solar  � �

(3.6kJ/1g water at 25% efficiency) when	�	   � Battery �#Ķ	    � �

necessary to maintain adequate battery	 �	   �	    �  �  Cells  � �

charge.							  �	   ���������ͼ  ���������ͼ �

�����������������������������������

The power subsystems of the UN, RV, and PLSS	 �����������ͻ  ������������ͻ

contain batteries that are recharged along with   �		 �  � Recharging �

other consumables (UN nitrogen and PLSS oxygen    � Batteries �#Ķ		  �

and water) when the device is stored onboard the  �		 �  �   System   �

OL, LV, HB, or MB.						  �����������ͼ  ������������ͼ

Environmental Subsystems

Atmospheric Composition and Density:  The OL, LV, HB, and MB atmospheres are

4:1 mixtures of nitrogen and oxygen at a pressure of one atmosphere or 100kPa.

Atmospheric density is about 1.2kg/m^3.  The PLSS uses a 2:1 nitrogen:oxygen

mixture at 50kPa so that oxygen prebreathing prior to EVA is unnecessary.

Leak Rates:  The leak rate for the OL and HB is normally 0.24l/min (5mg/s).

The normal leak rate for the LV and MB is 0.06l/min (1.25mg/s).  For the PLSS,

it is 0.05l/min (1mg/s).

Depressurization Losses:  Airlocks are used in the OL and HB but not in the LV

or MB.  The residual pressure after depressurization at the start of an EVA is

1.5kPa, leaving a nonrecoverable atmospheric density of 0.018kg/m^3.

Depressurization takes about 3min.

Reserve Supplies:  Atmospheric reserves are maintained in separate nitrogen and

oxygen tanks to replace losses due to normal leaks, cabin or airlock

evacuation, and astronaut usage.  An emergency supply, enough to completely

replace the standard atmosphere in the cabin, is also provided.

Damage and Repair

Damage:  Damage may occur during landing, docking, or movement on the lunar

surface.  Random breakdowns may also occur.  Battery damage increases the

discharge rate.  Damage to fuel cells or solar arrays decreases the amount of

power delivered.  Hydrogen, nitrogen, oxygen, RCS, vacuum, or water subsystem

damage increases the leak rate.  Mechanical structure damage increases the

atmospheric leak rate for pressurized systems and the likelihood of further

breakdowns.  Damage to navigation subsystems increases errors.  Drive subsystem

damage decreases the maximum speed.  Damage to the LV RCS or main engine

increases the likelihood of catastrophic failure.  Docking mechanism or landing

gear damage decreases the chance of successful docking or landing.

Repair:  The pilot may repair subsystems that are accessible to him.  Repairs

may take from less than 5min up to 2hr or more depending on the amount of

damage.  OL and HB damage may be repaired by onboard astronauts.

Orbiting Laboratory

The OL serves as the main transfer point for astronauts and equipment traveling

to and from the lunar surface.  It orbits the lunar			 LV DOCKING

equator at an altitude of 150km and consists of   ����ѻ #SOUTH�Ŀ PORT  ����ѻ

two cylindrical habitat modules (each 5m in	  ����Ŷ � ���������Ŀ   ����Ŷ

diameter and 10m long), two 5x20m solar power	����Ŷ   �:::::::::�   ����Ŷ

panels, and supporting structure.  A docking port ����Ŷ   �����������   ����Ŷ

for the LV is located on the south side.  OL	 ����������������:�����������

subsystems include power, environmental,		����Ŷ   ����������   ����Ŷ

consumables, and docking radar.			    ����Ŷ   �:::::::::�   ����Ŷ

����Ŷ   �����������   ����Ŷ

Power:  Normal power consumption is 34.8kW	   ����ϼ	  ���	  ����ϼ

during work hours (UT 06:00 to 22:00) and 1kW during sleep	   view from SUN

periods.  The docking radar uses 1kW.  Average power usage is

normally 23.6kW but may be reduced to 1.7kW in emergencies.  The solar array

has an area of 200m� and provides up to 80kW of electrical power.  Fuel cells

deliver 27kW when required.  Battery capacity is 445MJ and can supply emergency

power for 72hr.  The water tank can hold 2920kg, fuel cell production for 72hr.

Hydrogen (324kg) and oxygen (2592kg) are available for fuel cell use.

Orbiting Laboratory (continued)

Environmental:  The inhabitable volume is 196m^3 and requires 235kg of air.

The airlock has a volume of 27m^3 and a depressurization loss of 0.5kg per use.

Reserves are maintained for 360 day leak replenishment (160kg), 100 airlock

uses (50kg), and emergencies (235kg) for a total reserve of 445kg (356kg

nitrogen and 89kg oxygen).

Consumables:  In addition to other supplies, the OL has the capacity to supply

one UN for 30 EVAs (750kg of nitrogen propellant) and the LV for six complete

lunar exploration missions.  Total capacities are hydrogen (11,298kg), oxygen

(69,211kg), nitrogen (1652kg), and RCS propellant (1920kg).

Docking Radar:  The docking radar provides an accurate measurement of the

relative position of the LV with respect to the OL and has a maximum useful

range of 30km.

Landing Vehicle

The LV is an advanced version of those    DOCKING�������ͻ    #		  CREW

used during the early Apollo lunar		 PORT�����    �  UP	 MODULE

exploration missions and is totally # �����������������������������ͻ #

reusable.  Major components	  ���Ķ������ͻ � �� � � � �����ͻ����Ŀ TRIAD

include the Crew Module and the  #� O ��  #  � � �� � � � �  #  �� O �#

Propulsion Module.  Subsystems    ���Ķ������ͼ � �� � � � �����ͼ������  LEFT

include power, environmental,	    �����������������������������ĺ	 ���#

consumables, reaction control,    ���Ķ::::::�� � �����Ŀ EVA�::::::���Ŀ

and navigation.			   #� O �::::::� � � ��� �EXIT�::::::� O �#

���Ķ::::::�� � ������� � �::::::�����

Crew Module:  The Crew Module	  # �����������������������������͹ #

ferries the pilot and astronauts to   � � � � � � ����Ĵ� � � � � � �

the lunar surface and back to the OL, � � � � � � ����Ĵ� � � � � � �

and serves as living quarters during /� � � � � � ����Ĵ� � � � � � �\

missions that do not utilize the    /ź � � � � � ����Ĵ� � � � � � ��\

MB.  It stands 3.2m tall, has a   /��ź � � � � � ����Ĵ� � � � � � ����\

dry mass of 1611kg, and can	 /���Ŷ � � � � � ����Ĵ� � � � � � �����\

carry up to 1200kg of lunar	/����Ŷ � � � � � ����Ĵ� � � � � � ������\

samples.  A port is provided   /���/  �����������������������������ͼ  \���\

that allow the LV to dock	/���/  LANDING	  ���Ĵ    PROPULSION  \���\

with the OL.			 �������� GEAR	   ���������	 MODULE ��������

Landing Vehicle (continued)

Propulsion Module:  The Propulsion Module carries the bulk of supplies required

to perform a lunar exploration mission.  It is 3.8m tall, has a dry mass of

1968kg, and has four legs to cushion landing and to provide support on the

lunar surface.  The main engine burns a 6:1 mixture of liquid oxygen and liquid

hydrogen and can be throttled between 10% and 100% of its rated thrust of

82,240nt.  At full throttle, it uses propellant at 18.9kg/s (16.2kg/s oxygen

and 2.7kg/s hydrogen).  Total propellant capacity is 1809kg hydrogen and

10,854kg oxygen.

Power:  Normal LV power consumption is 1.3kW with three astronauts present and

130W when there are none.  The OL and DSN transponders require 130W each.  The

main engine and landing radar each consume 1kW.  The solar array has an area of

4m� and provides up to 800W of electric power.  Fuel cells deliver 2.7kW when

required.  Battery capacity is 78MJ and can supply emergency power for 168hr.

The water tank can hold 180kg, fuel cell output for 15 days.  Hydrogen (20kg)

and oxygen (160kg) are available for fuel cell use.

Landing Vehicle (continued)

Environmental:  The LV inhabitable volume is 10m^3 and requires 12kg of air.

The depressurization loss is 0.18kg.  Reserves are maintained for 30 day leak

replenishment (4kg), 20 cabin evacuations (4kg), and emergencies (12kg) for a

total reserve of 20kg (16kg nitrogen and 4kg oxygen).  An additional 30 day

supply of oxygen for three astronauts (72kg) is also provided.

Consumables:  In addition to other supplies, the LV carries enough nitrogen to

charge the UN for three EVAs (75kg).  Total consumable capacities are hydrogen

(1829kg), oxygen (11,090kg), and nitrogen (91kg).  The LV can carry up to

1200kg of lunar samples.

Reaction Control:  The RCS consists of eight modules of three thrusters each

(triads) and is used to orient the LV and provide small changes in velocity.

PITCH, ROLL, and YAW commands cause short bursts that change the spin by 1�/s.

FORWARD/BACK, LEFT/RIGHT, and UP/DOWN commands turn on sets of thrusters to

make small velocity changes.  The translation thrusters have three throttle

settings -- LOW (1%), MEDIUM (10%), and HIGH (100%).  The RCS thrust at full

throttle is 1970nt and uses 0.5kg/s.  A 1�/s spin change requires 0.07kg

propellant.  The maximum propellant load is 320kg.

Landing Vehicle (continued)

Navigation:  The LV has an INS that provides estimates of position, velocity,

and acceleration.  Orbital parameters may also be displayed.  Major components

of the guidance system are the gyros, accelerometers, DSN and OL transponders,

and the landing radar.

Gyros provide information on the attitude of the LV with respect to local

north, east, and vertical axes.  Accelerometers are used to measure the

acceleration of the LV during main engine and RCS burns.

The DSN transponder provides the INS with a measurement of the position (�15m)

and velocity (�0.25m/s) of the LV once each minute.  The OL transponder (or

docking radar) provides the INS with more accurate measurements (�0.5m position

and �0.05m/s velocity) every 10s.  It has a maximum useful range of 30km.  The

landing radar provides the INS with accurate measurements of LV position (�1m)

and velocity (�0.25m/s) relative to the lunar surface every 2s.  It has a

maximum useful altitude of 30km.

Manned Mobility Unit

The UN is generally used only during an emergency orbital EVA to transfer

between the OL and LV when docking is impossible.  The UN has a dry mass of

175kg.  The pilot and PLSS add 150 kg.  UN subsystems include power and

nitrogen propulsion.

Power System:  The maximum battery charge is 5.8MJ.  At a power consumption of

130W, the maximum EVA duration is about 12hr.  Recharging the battery requires

about 3.5hr.

Propulsion: The UN carries a maximum charge of 25kg nitrogen at 22,700kPa for

use as a propellant.  Small thrusters provide a force of 7nt and use 25g/s.

Acceleration is about 0.02m/s� for a maximum velocity change of about 20m/s.

Attitude control uses 13g for a 1�/s spin change.

Permanent Lunar Habitat

The HB is the focal point for many lunar surface activities such as materials

processing and equipment maintenance.  It is located just northeast of the

subearth point at 0�	 ����ѻ����ѻ����ѻ	  �Ŀ	  ����ѻ����ѻ����ѻ

latitude and 0�		 ����Ŷ����Ŷ����Ŷ   ���������Ŀ   ����Ŷ����Ŷ����Ŷ

longitude.  Like the OL,  ����Ŷ����Ŷ����Ŷ   �:::::::::�   ����Ŷ����Ŷ����Ŷ

the HB consists of two    ����Ŷ����Ŷ����Ŷ   �����������   ����Ŷ����Ŷ����Ŷ

cylindrical habitat	  ����������������������:����:�����������������������

modules (each 5m in	  ����Ŷ����Ŷ����Ŷ   ����������   ����Ŷ����Ŷ����Ŷ

diameter and 10m long)    ����Ŷ����Ŷ����Ŷ   �:::::::::�   ����Ŷ����Ŷ����Ŷ

and six 5x20m solar power ����Ŷ����Ŷ����Ŷ � �����������   ����Ŷ����Ŷ����Ŷ

panels.  HB subsystems    ����ϼ����ϼ����ϼ # EAST���AIRLOCK����ϼ����ϼ����ϼ

include power, environmental, and consumbles.			    view from ABOVE

Power:  Normal power consumption is 67.6kW during work periods (UT 06:00 to

22:00) and 1kW during sleep periods.  Average power usage is normally 45.4kW

but may be reduced to 1.7kW in emergencies.  The solar array has an area of

600m� and provides up to 240kW of electrical power.  Fuel cells provide 81kW

when required.  The battery capacity is 445MJ and can supply emergency power

for 72hr.  Water capacity is 24,750kg, required to hold fuel cell production

for about 15 days.  Hydrogen (2750kg) and oxygen (22,000kg) are available for

fuel cell use.

Permanent Lunar Habitat (continued)

Environmental:  The inhabitable volume is 196m^3 and requires 235kg of air.

The airlock has a volume of 27m^3, for a depressurization loss of 0.5kg per

use.  Sufficient reserves are maintained for one complete atmospheric purge

(235kg), 360 day leak replenishment (160kg), and 300 airlock uses (150kg) for

a total air mass of 545kg (436kg nitrogen and 109kg oxygen).

Consumables:  In addition to other supplies, the HB has the capacity to supply

the MB for six missions and the LV for one mission.  Total capacities are

hydrogen (4700kg), oxygen (33,200kg), nitrogen (1060kg), and RCS propellant

(320kg).  During the lunar day, oxygen can be manufactured at the rate of

20kg/day.

Mobile Base

The MB can carry three astronauts and up to 1200kg of lunar samples and is

used as the base of operations  �����������¿				�����������¿

for extended lunar missions of  �		 �  MB SAMPLE BOX	�		 �

up to 45 days.  MB subsystems   ������������� �����������Ŀ	 �������������

include power, environmental,	   ����������������������������������ϻ

consumables, and drive.		    �:����Ŀ:�: : : : : : :����ѻ����ѻ���Ŀ

�:�    �:�: : : : : : :����Ŷ����Ŷ�   �

Power:  Normal power consumption is  �:�    �:�: : : :FACE :����Ŷ����Ŷ� R �

310W with three astronauts present   �:����͵:�: : : :#��� :����Ŷ����Ŷ�   �

and 130W with none.  The drive	  �:�    �:�: : : : : : :����Ŷ����Ŷ� V �

requires 6.4kW at full throttle.	�:�    �:�: : : : : : :����Ŷ����Ŷ�   �

When the battery is fully charged    �:������:�: : : : : : :����ϼ����ϼ�����

(94MJ), the MB has a range of	   ����������������������������������Ѽ

100km (at 400J/m) and a 48 hr   �����������¿ �  AIRLOCK  � �    �����������¿

emergency supply.  The solar    �		 � ������������� #    �		 �

array has an area of 6m� and    �������������		   LEFT   �������������

provides up to 2.4kW of electrical power.  Recharging the battery takes about

11hr.  Fuel cells can supply 2.7kW when required.  The water tank has a

capacity of 180kg, fuel cell output for 15 days.  Hydrogen (20kg) and oxygen

(160kg) are available for fuel cell use.

Mobile Base (continued)

Environmental:  The MB inhabitable volume is 20m^3 and requires 24kg of air.

The depressurization loss is 0.36kg.  Reserves are maintained for 60 day leak

replenishment (8kg), 35 cabin evacuations (13kg), and emergencies (24kg) for a

total reserve of 45kg (36kg nitrogen and 9kg oxygen).  An additional 60 day

supply of oxygen for three astronauts (144kg) is also provided.

Consumables:  In addition to other supplies, the MB has enough capacity to

supply 20% of one LV mission.  Total consumable capacities are hydrogen

(386kg), oxygen (2540kg), nitrogen (55kg), and RCS (64kg).

Drive:  The MB has a maximum speed of 16m/s.  To improve efficiency, avoid

very rugged areas.

Roving Vehicle

The RV has a mass of 210kg, can carry two astronauts and up to 60kg of

lunar samples, and is used as   �����������¿				�����������¿

the primary conveyance during   �		 �   ASTRONAUT	   �		 �

most EVAs.  RV subsystems	  �������������	  SEATS	   �������������

include power and drive.		   ����������������������������������ϻ

�		 �����������Ŀ   �����Ŀ�

Power:  When the battery is fully    �		 �����������Ĵ   �����Ĵ�

charged (14.8MJ), the RV has a	  �		 �����������Ĵ   �����Ĵ�

range of 100km (at 100J/m) and	  �  FACE	�������������   �����Ĵ�    RV

can maintain non-drive systems,	 �  #���	�����������Ŀ   �����Ĵ�SAMPLE

consuming 130W, for a 10hr EVA.	 �		 �����������Ĵ   �����Ĵ�   BOX

Recharging the battery requires	 �		 �����������Ĵ   �����Ĵ�

about 8.2hr.					�		 �������������   ������ٺ

����������������������������������Ѽ

Drive:  The RV drive has a	 �����������¿	    �		�����������¿

maximum speed of 8m/s.  To	 �		 �	    # LEFT	�		 �

improve efficiency, avoid very  �������������				�������������

rugged areas.

Portable Life Support System

One PLSS, with a mass of 70kg, is provided for the pilot and each astronaut to

allow EVAs.  Donning or removing the PLSS requires about 5min.  PLSS subsystems

include power, environmental, and consumables.

Power:  PLSS power consumption is 16W plus a maximum of 64W for pilot cooling.

The battery capacity is 2.9MJ, enough to sustain a 10hr EVA at maximum pilot

effort.  Recharging takes about 1.6hr.

Environmental:  The PLSS carries enough oxygen to sustain a 10hr EVA at maximum

effort (1kg) plus an emergency supply (1kg) and leak replenishment (0.1kg) for

a total of 2.1kg.

Consumables:  The PLSS carries 4kg of water, enough for a 10hr EVA with 8hr

reserve at maximum effort.