TURTLE BEACH SYSTEMS

WAVE LITE FOR WINDOWS 3.1

Shipped with the COVOX VOICE BLASTER

Be sure to look for other fine COVOX, Inc. products .

PROUD TO SAY                                MADE IN THE USA

TABLE OF CONTENTS

Chapter 1: INTRODUCTION

What Is Wave Lite?

Registration

System Requirements

Using the Manual

Update Policy

Upgrading to Wave

Copyrighted Material

Chapter 2: INSTALLATION

Chapter 3: OVERVIEW

Program Concepts

Menus

Toolbar & Overview

Soundfile Windows and Icons

What Is Sampling?

Chapter 4: ABOUT SAMPLING

The Basics of Sound

Sampling Sounds

Playback

Chapter 5: PLAYBACK AND RECORDING

Recording

Recording Errors and

Disk Fragmentation

Chapter 6: VISUAL EDITING

The Soundfile Window

The Overview

The Time Displays

The Time and Amplitude

Resolution Sliders

Right Mouse Button Zooming

The Selected Area and the

Insertion Cursor

The Edit Icons

The Previous View Icon

Cut and Paste Editing

Muting

Undoing Edits

Chapter 7: The File Menu

The File Menu

The Soundfile Information Dialog

The Windows Menu

Chapter 8: The Windows/Options Menus

The Options Menu

Preferences

Display Setup

System Check Options

Chapter 9: Sampling Techniques

WAVE LITE

Chapter 1

INTRODUCTION

WHAT IS WAVE LITE?

Wave Lite is a program for recording, editing, and otherwise

dealing with waveform audio in the Windows environment.

Wave Lite is the result of our many years of experience in the

professional audio field, where we have pioneered the use of PCs

as full-fledged recording systems. As such, it will finally allow

you to fully harness the digital audio capabilities of Windows

3.1 and/or the Multimedia PC. Before You Do Anything Else...

Check your disks for a file named README.DOC and, if you find

one, view or print it. This file lists any changes that we may

have made to Wave Lite since we printed this manual.

REGISTRATION

We hope you'll take the time to fill out and send back your

registration card right now. Here are some reasons why you

should:  We only update registered users. You'll be notified when

any program revisions, bug fixes, or upgrades are made.

We only offer low-cost upgrades to registered users.

The information you give us on the registration card lets us

know your uses and needs for Wave Lite and the future products in

which you're interested.

Send in the Card!

SYSTEM REQUIREMENTS

In order to use Wave Lite, you will need the following hardware

and software:

Windows 3.1 or greater, or Microsoft Windows 3.0 and the

Windows Multimedia Extensions.  A computer (386SX or better) with

the requisite equipment to run Microsoft Windows.

A Windows-compatible sound device.  Wave Lite will work with any

Windows 3.1 or MPC compatible sound card, including Turtle

Beach's own MultiSound, Covox's Sound Master II, the Creative Labs'

Sound Blaster and Sound Blaster Pro, or the MediaVision Pro

AudioSpectrum series.

USING THE MANUAL

This manual is designed primarily to be a reference document; we

don't expect you to read it from cover to cover. Nevertheless,

you will need to read at least a few chapters before you dive

into Wave Lite itself. Chapter 1 covers all the basic stuff that

any software manual has to have--the friendly intro to the

program, support and upgrade information, etc. Chapter 2

covers installation. In Chapter 3 we finally cut to the chase,

with an overview of Wave Lite's operation. If you've never worked

with waveform audio or sampled sounds before, you'll also want to

read Chapter 4, which provides an introduction to these subjects.

From there on, we suggest that you only open the manual as

needed. As a general rule, if you need help on any subject, you

should turn to  the index first. This will tell you exactly where

to find the information you need. And don't forget to make use of

Windows' on-line help facilities!

UPDATE POLICY

Once registered, you are entitled to maintenance updates (bug

fixes, minor enhancements) for one year from date of purchase.

There is a nominal update charge to cover disks and shipping.

A major update (one that adds significant new features) is

available free to anyone who purchased Wave Lite in the three

months preceding the update. Older users will be charged a fee

commensurate with the amount of added function.

PROGRADING TO WAVE

Wave Lite is a slimmed-down version of Wave, our professional

sound editing system for Windows. While the tools included in

Wave Lite will probably be adequate for most of the things you'll

want to do with sound in the Windows environment, Wave provides

the extra functions needed to do "studio-quality" digital

recordings. These features include:

Four-band parametric equalization.

Three-dimensional Fast Fourier Transform (FFT) analysis.

Mixing of part or all of up to three soundfiles.

Crossfading, gain adjustment, and muting.

Full digital effects (reverb, chorus, echo)

Time compression and expansion.

Easy-to-use effects presets.

Import and export to a wide variety of file formats.

As an owner of Wave Lite, you're entitled to a special low price

on  the full Wave package. For full details, see the Wave Upgrade

card enclosed with the Wave Lite package.

COPYRIGHTED MATERIAL

This document and the accompanying software are copyrighted and

all rights are reserved by Turtle Beach Systems.

Copying of this documentation or software for other than backup

uses is illegal.

This document and accompanying machine readable disks may not, in

whole or in part, be copied, photocopied, reproduced, translated

or reduced to any electronic medium without prior written consent

of Turtle Beach Systems, Inc.

TRADEMARKS USED

COVOX & Sound Master II, are Registered Trademarks of Covox, Inc.

Voice Blaster is a trademark of Covox, Inc.

Wave, Wave Lite, and MultiSound are trademarks of Turtle Beach

Systems, Inc. Sound Blaster and Sound Blaster Pro are trademarks

of Creative Labs Ltd. Windows is a trademark of Microsoft Corp.

Pro AudioSpectrum is a trademark of Media Vision Inc.

IBM is a trademark of International Business Machines Corp.

MPC is a trademark of the Multimedia PC Marketing Council

MANUAL REVISIONS 1.1

Copyright 1993

Turtle Beach Systems, Inc.

All Rights Reserved

Printed in the United States of America

CHAPTER 2

INSTALLATION

INSTALLING WAVE LITE

Installing Wave Lite in your system is about as easy as could be.

The Wave Lite installation program runs under Windows (just like

Wave Lite itself), so there are no confusing intro screens or

bizarre key commands ("Press Ctrl-Alt-F7 to continue") to scratch

your head over. To install Wave Lite, insert the Wave Lite

diskette in drive A (or B, if you prefer), then select Run...

from the Program Manager's File menu. A dialog

like that shown below will appear:

In the text entry field, type A:\SETUP (or B:\SETUP, if you put

the Wave Lite diskette in the B drive), then click OK. The Wave

Lite installation program will take over from there, pausing only

to ask you in which directory you'd like to install the program.

(We suggest that you simply use the default.) When the Wave Lite

program group and icon appear, you'll be ready

To begin

recording and editing waveform audio. Double-click on the Wave

Lite icon, then prepare to be amazed...

CHAPTER 3

WAVE LITE OVERVIEW

Despite Wave Lite's internal complexity, outwardly it is a very

straightforward program. If you've ever used a tape deck, you'll

already be familiar with many of the program's functions. Of

course, Wave Lite does have many other capabilities that you'd

never find in the most well-equipped conventional studios, so a

complete description of its functions is certainly in order. In

this chapter, we'll do just that, starting with the basic

concepts of Wave Lite. Wave Lite

PROGRAM CONCEPTS

Wave Lite makes use of the traditional Windows Multiple Document

Interface (MDI), which is similar to that used in Word for

Windows, File Manager, and many other applications. Up to four

document windows may be open at any time. These are used for

editing four separate soundfiles, which is our name for

digital recordings. Wave Lite's editing tools take many forms. At

the simplest level, you may cut and paste sections of sound,

either within a single soundfile or across files, or draw sound

freehand using the waveform pencil. (And please remember that

"simple" is a very deceptive word; after all, the razor blade and

splicing block are simple tools, but they are immensely powerful

as well.)

An Important Note  Unlike most other software, Wave

Lite does not load a copy of its data into memory when you open a

sound file. All editing is done directly on the data stored on

the hard disk. If you're used to working with other types of

software, such as graphics programs, word processors, or even

MIDI programs, this may seem... strange, to say the least.

However, this is typical of the way digital audio programs work,

and for a very good reason. When audio is sampled and stored at

high resolutions, it can eat up a tremendous amount of memory.

CD-quality sound (two channels of 16-bit audio, sampled at 44.1

kHz) uses a stunning ten megabytes per minute of sound recorded.

Even with the low prices of RAM today, it is impractical to

expect a desktop computer to hold this amount of data in RAM.

When using Wave Lite to edit an existing sound (such as those

included with Multimedia Windows), it is good practice to make a

backup copy before you start your editing session.

Menus The menu bar at the top of Wave Lite's main window contains

several menus which provide access to virtually all of the

program's functions.

FILE MENU

The File menu contains items for managing disk files,

and for exiting the program. This menu is discussed in Chapter 7.

Edit menu;The Edit menu is used to access several editing tools,

including cut and paste functions and undo. The Edit menu options

are discussed in Chapter 6. The Windows menu is used to change

the arrangement of the four soundfile windows within Wave Lite's

main window. The items in this menu are discussed in Chapter 8.

The Options menu contains several items for setting parameters

for other parts of the program, as well as system check functions

and other items. The Options menu is discussed in Chapter 8.

THE TOOLBAR AND OVERVIEW

The area immediately beneath the menu bar is called the toolbar.

This area contains several numeric displays and buttons which

provide quick access to many of the most commonly used functions.

THE TOOLBAR

The upper left corner of the toolbar contains several displays of

times that are related to the active soundfile window, as well as

sliders for changing the X (time) and Y (amplitude) resolution of

the display. The right side of the toolbar contains icons for

playing the recording, for selecting the mouse editing modes, and

for retrieving previous views of the window. Immediately below

the toolbar is a display called the overview, so named because it

displays a timeline overview of the entire recording. The

overview is used primarily to select the portion of the recording

that is displayed in the active soundfile window.

SOUNDFILE WINDOWS AND ICONS

Wave Lite's main window is home to up to four child windows.

These windows may be minimized within the main window, in which

case they will appear as icons. Double-clicking on any icon will

maximize the window for that icon. When you start the program,

the first soundfile window will already be open, and the

remaining windows will be minimized. Each of the four soundfile

windows displays both channels of the recording.

The components of a soundfile window are shown below.

SOUNDFILE WINDOWS

The top most soundfile window is referred to as the active

soundfile window.

The active window is connected to Wave Lite's menus and toolbar;

all recording, editing, and playback operations deal with the

sound data shown in the active window. The soundfile windows are

discussed in excruciating detail in Chapter 6.

CHAPTER 4

ABOUT SAMPLING

WHAT IS SAMPLING?

Sampling, in musician's terms, is the process of converting a

sound to a series of numbers which can be stored and manipulated

by a computer. The term sampling comes from a description of the

physical process used by the hardware that converts sound waves

to digital data, which periodically takes a sample of the input

wave and converts it to digital data. Sampling is also sometimes

referred to (especially in the computer games arena) as

digitizing. In order to master the Multimedia PC's digital audio

capabilities and the operation of Wave Lite, it is important to

understand at least a little bit of the mechanics of

sampling--how it works, and what its limitations

are. This in turn requires some knowledge of the physics of

sound.  To that end, we have provided some background information

on sound and sampling in this chapter. It's not our intention at

this point to bring you up the level of a graduate acoustics

engineer, but this should at least be enough to get you

started.

THE BASICS OF SOUND

Simply put, a sound is a fluctuation in air pressure that can be

sensed by our ears. When something happens that creates a

sound--for instance, a tree falling in the woods--, the air

vibrates in a certain way. These vibrations are called sound

waves. Our ears sense these waves, and send signals to our

brains, which then interpret the sound and tell us what it means.

The part of the brain that deals with sound is an especially

complex and wonderful piece of software (or wetware, as

programmers like to call it). It takes the chaotic fluctuations

of the sound waves and transforms them into a continuum of

frequencies--that is, independent rates of vibration. This

transformation is what allows us to enjoy music, to recognize

words, to tell whether a recording was made in a bathtub or a

cathedral--in short, it is the key to the way we extract

information from sound.

Because the concept of frequencies is so important to the way we

interpret sounds, it should be no surprise that audio engineers,

musicians, and others who deal with sound use frequencies to

describe the contents of a sound. Frequencies are measured in

units of hertz (abbreviated Hz). A hertz is defined as one cycle

per second. Human beings can hear frequencies in the range of 20

Hz to 20,000 Hz--that is, vibrations that repeat between 20 and

20,000 times per second.

The most simple sound is something called a sine wave. A sine

wave consists of a single frequency. As you may have guessed, the

sine wave does not exist in nature--it is a theoretical

abstraction which is used primarily in analyzing , testing, and

simulating audio, electrical, and mechanical systems. A ll sounds

that occur in the real world, including natural and synthesized

instruments, sound effects, narration, and anything else you can

think of, contain many  different frequencies. The exact

frequency values of these components of the sound, as well as

their relative levels and the way they change over time, are what

let us distinguish one sound from another.

The reason that it is so important to understand the frequency

content of a sound is that most audio systems--whether you're

talking about a telephone, an ultra-high quality stereo system,

or the sound board in your PC--can only handle a limited range of

frequencies. For example, the frequency response (or bandwidth)

of a telephone is somewhere in the range of 400 Hz to 8,000 Hz.

This is substantially less than what we can hear, but it's all

that is necessary to convey the important information in a

person's voice. (It also explains why the telephone sounds so

lousy in comparison to, say, an FM radio.) Many of the

requirements of sampling limit the frequency response of sampling

systems like that found in your PC, as we'll learn in the next

section.

SAMPLING SOUNDS

A microphone works in a manner similar to an ear. When a sound

strikes a microphone, the mike produces an electrical wave which

is (more or less) an exact replica of the pressure wave that

produces the sound. Because there is a one-to-one correspondence

between the air pressure and the voltage coming out of the mike,

this wave is called an analog signal. You've probably heard the

term "analog" used to describe certain types of audio equipment,

such as analog tape recorders, etc. When someone describes an

audio device as an "analog" device, it means that the machine's

circuitry operates only on the analog waveform that represents

the sound. Computers, on the other hand, do not make use of

analog signals. Computers operate in the digital realm, where

everything is represented as a series of on/off switches, or

bits. Therefore, in order to store a sound in a computer's

memory, it is necessary to convert the continuously changing

analog signal that represents the sound, to digital data.

Not surprisingly, the device used to convert the analog signal to

digital data is called a digital to analog convertor, or DAC.

This outputs a number, in binary format, that represents the

input signal at any given instant in time. Now all we have to do

is store those numbers in memory, and we'll be able to say we

have the sound stored, right? Well, yes and no. The numbers put

out by the DAC do represent the sound--but for a variety of

reasons, it's impossible to store every one of them in memory.

The way this problem is handled is to sample the output from the

DAC at a regular rate. This rate, called the sample rate, is one

of the most important factors in determining the quality of a

sampled sound.

SELECTING A SAMPLE RATE

The sample rate is also sometimes called the sampling frequency,

because it too can be measured in hertz. From this, you can

probably deduce that there is an important relationship between

the sample rate and the frequency content. Take a look at the

following diagram. This shows a signal that contains a single

frequency (a sine wave) which we'd like to sample.

A SINE WAVE

There are a lot of things you can say about the sine wave, but

for now, take note of two facts: it is very regular, and it has a

single high point and a single low point. Because of these two

facts, and thanks to the physics of electronic filtering (which

we will not be discussing here!), it turns out that it is only

necessary to sample a sine wave twice during each cycle in order

to get a "complete picture" of it. For more complex waves, such

as those associated with sound, the same thing is true. Even

though real-world sounds are made up of many different

frequencies (as opposed to the single frequency found in the sine

wave), they can be sampled adequately by using a sample rate that

is twice as high as the highest frequency in the signal. This is

so important, it bears repeating: In order to sample a signal

properly, the sampling frequency must be at least twice as high

as the highest frequency component in the signal. This fact,

which can be proven by rigorous mathematical analysis, is the

most important single notion in sampling. If you sample at a rate

that is at least twice the highest frequency in your input

signal, everything will be OK. If the sample rate is less than

twice the highest frequency, however, you'll get an inaccurate

representation of the input. But what does "an inaccurate

representation" mean?

In this case, it means a lot. When a signal is sampled at a rate

less than twice its highest frequency, very strange things

happen. The math behind this is quite complex and not at all

obvious, (so we won't go into it), but what happens is that new

frequencies are generated and added to the sampled sound. These

frequencies are related to both the input frequencies and the

sample rate in such a way that they are virtually guaranteed to

sound unpleasant. This phenomenon is known as aliasing, and is

one of the nastiest bugaboos in the world of sampling.

Fortunately, aliasing is not a serious problem with most modern

sampling systems. These devices include special circuits, called

anti-aliasing filters, which automatically restrict the bandwidth

(or frequency content) of the input signal based on the sampling

rate.  For example, if you tell your PC to sample a sound at 12

kHz, it sets its anti-aliasing filters to reject frequencies over

6 kHz. A similar filter is used on the output when the sound is

played back, to smooth off the "rough edges" created when the

analog data is digitized. The sample rate used by a particular

sampling system can usually be set in its software, though the

upper value is limited by the hardware. Compact disks use

a fixed sample rate of 44.1 kHz, while digital audio tape (DAT)

players work at either 44.1 kHz or 48 kHz.

The Multimedia PC document issued by Microsoft specifies that the

sound hardware be capable of sampling at 11.025 kHz and 22.050

kHz, at a minimum. All MPC-compatible sound hardware can perform

at this level, though some boards are capable of delivering

CD-quality performance.

SAMPLING RESOLUTION

Another important factor in determining the quality of a sampled

sound is the sampling resolution. This term refers to the number

of discrete levels that are used in the analog to digital and

digital to analog conversion processes. Sampling resolution is

measured in bits, which refer to the amount of memory required to

store each  individual sample. The number of different values

that can be stored in a collection of bits is equal to 2 raised

to the "bit'th" power. For example, an eight bit sample is

digitized to 28, or 256, different levels. A sixteen bit sampling

system, on the other hand, senses 216, or 65536, different

levels.

Obviously, the more bits of resolution that are used, the more

closely the sampled signal will represent the analog signal,

which has an essentially infinite resolution. However, higher

resolutions require more memory, so some tradeoffs must be made.

Eight bits is generally accepted as the lowest sampling

resolution that can be used to obtain reasonable results, while

16 bit resolution is preferred for professional applications.

Unlike the sample rate, the resolution of a sampling system is

almost always determined by the hardware. The minimum resolution

that is permitted for MPC hardware is eight bits. Compact disks

and digital audio tapes use 16-bit samples, though many players

only use 14 bits in their output circuitry. Other musical

instruments may use 12, 18, or even more bits. Wave Lite uses 16

bits of resolution for its internal sample storage and editing.

Looping Back in the early days of sampling, when computer memory

was rationed out in 16K memory chips, musicians were faced with a

serious problem.

How could sustained musical sounds, like a trumpet or violin

note, be stored in RAM, without using a small fortune in memory?

Part of the solution, of course, was to use low sample rates and

resolution, but an equally important factor was the development

of sample looping. Consider the sound of the trumpet. After an

initial attack transient, in which the nature of the sound is

changing rapidly, it reaches a "steady state", in which the sound

changes very little. At this point, each cycle of the trumpet

waveform is pretty much identical to the one before it. Because

of this, it is possible to simply delete most of the trumpet

sample from memory, and then instruct the sampler to repeat a

single cycle of the waveform. Musicians call this process,

logically enough, looping. While looping is simple in theory, it

is a bit more complicated in practice. For example, most musical

instruments have subtle periodic changes in pitch and timbre

during their steady state period. Looping a single cycle of the

waveform therefore results in a sound that is statit and

lifeless--more like a cheesy organ than a living, breathing

instrument.

As sampling moved out of the laboratory and into the keyboard

rack, other complications were encountered and dealt with along

the way. Thanks in no small part to the difficulty of creating a

good loop, programs like Turtle Beach's SampleVision have become

very popular with musicians.  However, for the sound effects and

narration gigs that are the primary applications for the

Multimedia PC, looping is a lesser concern. Even so, there are a

variety of uses for looping in this arena, ranging from simple

repetitions of sounds to "Max Headroom" stuttering effects.

References De Furia, Steve, and Scacciaferro, Joe, The Sampling

Book, Hal Leonard Books Pohlman, Ken, Principles of Digital Audio

CHAPTER 5

PLAYBACK AND RECORDING

PLAYBACK

Wave Lite has many different options for playing back a

soundfile. All or part of a soundfile may be played back, or a

segment of a soundfile can be made to play repeatedly. The Play

Icon Most of the playback options are centered around the Play

icon, which is shown to the left.

If a section of the recording is selected, then a single click

with the left mouse button on the Play icon will play the

selected area. If a selected area does not exist, then

left-clicking on the Play icon will tell Wave Lite to

play the material displayed in the active soundfile window.

The entire soundfile may be played by double-clicking on the Play

icon. Double-clicking at any location in the overview display

will play the soundfile, beginning at that point. The <Space> bar

performs the same function as a single click on the Play icon:

Either the selected area plays if there is one, or the sound in

the active soundfile window plays if not. <Shift-Space> plays the

entire soundfile. The <Space> bar also stops playback.

RECORDING

Wave Lite's recording functions are accessed by clicking on the

microphone-like Record icon located in the toolbar. If you start

recording in an empty soundfile window, a file selection dialog

will allow you to select an existing soundfile in which to

record, or to enter the name of a new soundfile. After you

specify a file name, or if you are recording into an existing

soundfile, the Record dialog will appear. Record Dialog

The Record Dialog The Record dialog contains a wide variety of

controls and displays that affect the recording process. Many of

its functions are identical to the analogous functions in an

analog tape deck, while others are unique to the digital

recording medium. The Review, Record, Play, Stop, and Cue buttons

are all similar (but not identical) to the equivalent buttons on

a tape deck. Recording starts as soon as the Record button is

pressed; there is no need to press Play. Recording stops when the

mouse button is clicked, or the <Space> bar is pressed. The

<Space> bar can also be used to start playback.  The Cue and

Review buttons (These buttons are called "Fast orward" and

"Rewind" on many tape decks.) change the value in the time

counter below the "transport" buttons.  Playback or recording

starts at the time shown in this counter when the Play or Record

buttons are clicked. You can also position the starting point for

recording or playback with the three buttons to the right of the

counter. These buttons allow you to instantly jump to the end of

the recording (GTE, or Go To End), the beginning (RTZ, for Return

To Zero), or the beginning of the selected area (SEL). The

Sampling Rate buttons are used to set the sound hardware's

sampling frequency. The Resolution buttons determine whether

individual samples will be stored in a single byte (8 bits) or an

entire word (16 bits) of memory. This dialog will only allow you

to select options that your sound card can handle, so for

example, Sound Blaster users will not be able to choose 44.1 Khz

recording. Changing the sample rate and/or the resolution allows

you to balance fidelity and memory usage, depending on your

needs. For more on sample rate and resolution, see page 15.

The Channels button allows you to record in mono or stereo, if

your sound hardware has that capability. Note that these settings

cannot be changed once you begin recording into a new soundfile,

or if you are recording new material in an existing soundfile.

The level meters on the right side of this dialog provide an

animated display of the audio input. While the level meter is

running, you can set your sound card record level either

by manually adjusting it or by using manufacturer-supplied

Windows tools to do so. These bar graphs show the instantaneous

and peak input levels for each channel. This is an essential tool

when you are recording from an analog source. The level meters

are switched on by clicking on the Pause button. You can turn off

the meters by clicking Stop or if you are ready to record, click

Record.

HOW TO CREATE A NEW RECORDING

1) Open a new window by double-clicking on an Untitled icon.

If all four windows are in use, use the New option in the File

menu to clear a window.

2) Click on the Record icon.

3) In the dialog that appears, type the name of the new

recording, then click on OK.

4) Click Pause to start the level meters

5) Start the source tape or CD and adjust the output level

while monitoring the level display. When the

levels are set properly, and re-cue your source tape or CD.

6) Click on the Record button, then start the source tape.

7) When the recording is complete, press <Space> to stop

recording, then click on Done to return to the

editing window.

HOW TO ADD MATERIAL TO THE END OF AN EXISTING RECORDING

1) Select the window containing the soundfile in which you

wish to record.

2) Click on the Record icon.

3) When the Record dialog appears, click on the GTE button to

go to the end of the recording.

4) Now follow steps 4 to 7 of How to Create a New Recording,

above,  to complete the recording.

RECORDING ERRORS

On occasion, if you are recording using Wave Lite's highest

sample rate and resolution--that is, 16 bit stereo at 44.1 kHz,

DOS may not be able to write data to the hard disk as fast as the

sound hardware is sending it. When this happens, a Record Error

message will appear on the screen. Wave Lite will keep a running

count of the errors as they occur. Because of the nature of these

errors, they will usually result in very nasty glitches in the

sound--not the kind of thing you'd like to have in your high

quality digital recordings, for sure. Fortunately, this situation

is not as deadly as it seems, as it can be dealt with quite

easily. Understanding why it occurs, and why the solution works,

can be a bit trickier. For this reason, we'll first present the

solution, with no explanation, and then provide more detailed

information for those hardy souls who crave full enlightenment on

the minutiae of DOS.

THE SOLUTION

When recording errors occur, they can almost always be corrected

by simply rerecording the same material--as long as the second

recording is no longer than the first. To prevent recording

errors from happening you will need to defragment your hard disk,

using a disk defragmenter (or "optimizer") such as Vopt (our

favorite) or those found in PC Tools, Norton Utilities, or Mace

Utilities.

THE EXPLANATION

When DOS writes a file to disk, the file is not written to disk

in one big chunk, but is actually broken into smaller chunks and

stored in small areas on the disk called clusters. When the disk

is new, its empty clusters are all grouped together, but as time

goes by and more files are written to and deleted from the disk,

the group of empty clusters becomes broken up, or fragmented.

When DOS writes a file, or adds data to an existing file, it must

first search through the disk and find enough empty clusters to

store the data. If the clusters are all bunched together, this

doesn't take very long, but if all of the free clusters are

scattered around the disk, DOS may take a bit longer to do this.

At some point, it can no longer keep up with the sound hardware,

and recording errors occur. Happily, these recording errors do

not interfere with DOS at all, and it will continue to

successfully search for free clusters as long as recording

continues. Even better, this list of free clusters is retained by

DOS, so that the next time a recording is made, it already knows

where to store the data, and recording errors won't occur. A disk

defragmenter prevents further recording errors by bringing all of

the empty clusters together. Another good way to create a large

block of empty clusters is to erase any unnecessary soundfiles or

programs from your disk, preferably as soon as you are done with

them.

CHAPTER 6

VISUAL EDITING

THE MIRACLE OF VISUAL EDITING

One of the most important benefits of the digital audio

revolution is the long-awaited ability to look at sound rather

than to simply listen to it. The wave nature of sound has been

well understood for centuries, but being able to look at a sound

and thereby get a new perspective on such things as clicks,

envelopes, noise, and other audio phenomena has been a great help

to musicians and audio engineers. Early practitioners of the

audio arts had to be content with oscilloscopes, sonograms, and

other laboratory equipment that was not particularly well

suited to musical purposes, but since the advent of the personal

computer the tools available to us have evolved considerably,

culminating in advanced sample-editing programs like Wave Lite.

Like all good digital audio programs, Wave Lite not only lets you

see your music, it also allows you to visually edit your

recordings as well. In this chapter, we'll cover all aspects of

visual editing in Wave Lite.

THE SOUNDFILE WINDOW

The soundfile window, which was discussed briefly in Chapter 4,

is the heart of Wave Lite's visual editing system. It contains

two views of the recording, several time displays, resolution

sliders, and a variety of icons for editing and other functions.

These items are all described below.

THE OVERVIEW

The overview is a miniature representation of the soundfile in

the active (top) soundfile window. The overview is used to select

the portion of the soundfile that appears in this window. The

overview appears just below the toolbar.

SOUNDFILE OVERVIEW

The highlighted portion of the overview corresponds to the data

in the active soundfile window. Dragging the mouse across the

overview will highlight a new area and, consequently put a new

section of sound in the window.

If you drag with the left mouse button down, the amount of sound

displayed in the top window may change, but if you drag with the

right mouse button, you can slide the view around without

changing its size. Note that if the length of the highlighted

portion of the overview is very long, Wave Lite may take an

excessive amount of time to redraw the window. If this happens,

you can press the <Space> bar to cancel the calculation, and

Wave Lite will only plot that portion of the active soundfile

window that it has calculated. How to View a New Section of Sound

with the Overview

1) Position the mouse cursor in the overview, at the beginning

of the section of sound you wish to view.

2) Drag the mouse across the area you wish to view. The area

you specify will be highlighted.

3) Release the mouse button, and the highlighted area will

appear in the active soundfile window.

THE SOUNDFILE WINDOW

The topmost, or active, soundfile window is where most of the

action in Wave Lite takes place. This window is used to view a

chunk of your recording to any level of resolution (in both time

and amplitude) that you desire. It is also used for selecting

areas to edit, placing insertion points, and waveform drawing.

SOUNDFILE WINDOW

The calibration units along the left and bottom edges of the

view, as well as the type of grid used in the soundfile windows,

are all set in the Display Setup dialog, described on page 49.

The highlighted section of sound in this screen shot is called

the selected area, and is discussed later in this chapter.

The maximum amount of soundfile that can be "grabbed" from the

overview is set by the Maximum Zoom parameter in the Display

Setup dialog, which is discussed on page 50.

The reason we allow you to limit the size of the soundfile window

display has to do with the mechanics of hard disk recording. When

Wave Lite draws the soundfile plot, it takes almost as long to

get the data from disk as it would to play the data. This means

that, if you want to grab a ten minute recording from the

overview, redrawing the soundfile window takes about ten minutes.

This is not practical, so we've added this feature. If you

attempt to make the soundfile window's display longer than the

Maximum Zoom time by dragging the mouse in the overview, it will

be limited to this value. Note that you can still view larger

sections using the time resolution slider, since its operation is

not limited by Maximum Zoom.

The portion of the recording displayed in the active soundfile

window can be positioned by ear by setting the Play Mode switch

to Tape, as described on page 48. In a stereo soundfile, the

upper part of the display in each soundfile window corresponds to

the left channel of the recording, and the lower part represents

the right channel.

THE TIME DISPLAYS

The five displays in the upper left part of the toolbar show time

values that relate to the recording. These displays are shown

below.

TIME DISPLAYS

The display in the upper left corner shows the length of the

soundfile. The display immediately below this shows the start

time of the sound displayed in the active soundfile window.

The two fields in the lower right part of this display show the

start time and duration of the selected area, if any. If there is

no selected area, then the upper display shows the location of

the insertion cursor, and the lower display shows the sample

value at the cursor position. .

The units used for the four displays discussed above are set in

the Display Setup dialog, which is described on page 49. This

dialog is available through the Options menu, or it may be

brought up by clicking on the units portion of any of these

displays. The classy-looking display in the upper right is

slightly different that the other four. It always displays time

in hours, minutes, seconds, and frames, regardless of the units

used by the other displays. If Wave Lite is playing, it shows a

running count of the play time. If Wave Lite is stopped, then it

shows the same value as the Start display, that is, the location

of the left edge of the active soundfile window.

THE TIME AND AMPLITUDE RESOLUTION SLIDERS

The two sliders located between and beneath the time displays are

used to change the resolution of the active soundfile window. The

horizontal slider changes the time, or X, resolution of the view,

and the vertical slider controls the amplitude, or Y, resolution.

These controls are used to zoom in and out on the soundfile.

When the time resolution slider is all the way to the left, up to

two minutes of the soundfile will be displayed, and when it is

all the way to the right, approximately 64 sample frames will be

displayed, depending on the type of video card you're using.

Clicking to the right or left of the slider handle will move the

handle by one "notch", which will change the resolution by a

factor of four. Similarly, the entire height of the sound data in

the active soundfile window will be displayed when the amplitude

resolution slider is at the top; when it is at the bottom, only a

very small portion of the full range is displayed. Clicking above

or below the slider handle changes the resolution by a factor of

two.

SCROLLING THE SOUNDFILE

When either the time or amplitude resolution is set such that

only part of the full range is visible, the soundfile window's

scroll bars become active. These can be used to view different

areas of the soundfile. The size of each scroll bar's handle

shows the size of the viewed area in relation to the full range

(either  time or amplitude). The screen shot on the following

page shows a soundfile window in which the resolution sliders and

scroll bars have been used to zoom in on a small section of

sound.

SOUND DATA MAGNIFIED WITH RESOLUTION SLIDERS

Typically, you would use the time resolution slider only when you

want to look at a very large section of the soundfile, since this

slider does not have the automatic limiting that occurs when you

drag the mouse in the overview, as discussed on page 30. The

amplitude resolution slider, on the other hand, is a very

convenient way to zoom in on part of a waveform.

CENTERING THE AMPLITUDE DISPLAY

Because it is often necessary to center the zero amplitude line

in the active soundfile window, and because this can be difficult

to do using the vertical scroll bar, the Windows menu contains a

special command for this, called Center Y Resolution. Clicking on

this menu item, or pressing <Alt-C>, will center the view around

the zero line.

CANCELING LONG REDRAWS

If you accidentally move the time resolution slider too far to

the left, you may find that Wave Lite takes a long time to redraw

the soundfile window. If you tire of waiting for this to happen,

you can Click the left mouse button or press the <Space> bar at

any time to cancel the redraw.

The right mouse button, described in the next section, provides

an alternate method for changing Wave  Lite's display resolution.

HOW TO MAGNIFY A SMALL TIME SLICE

1) Drag the mouse across the section of the overview that

corresponds to the section you wish to view. The left edge

of the highlight should be at the left edge of the desired

segment.

2) Slide the time resolution slider all the way to the right.

3) If necessary, use the scroll arrows along the bottom edge

of the window to zero in on the desired segment.

Right Mouse Button Zooming

The right mouse button may be used at any time to zoom in on a

particular section of the active soundfile window. This is

equivalent to adjusting the time and amplitude resolution

sliders, as well as the horizontal and vertical scroll bars, in

one fell swoop. When the mouse cursor is inside the active

soundfile window and right mouse button is pressed, the cursor

changes to a pointing arrow. Dragging the mouse down and to the

right across the window will cause a rubber box to appear.

When you release the mouse button, the data enclosed in the box

will expand to fill the window. Because it is often desirable to

be able to zoom in on a particular time segment in a soundfile

without changing the amplitude resolution, we have provided an

option to prohibit changes in Y resolution during zooms. If a

check mark appears beside Lock Y Res in Zooms in the Windows

menu, then the amplitude resolution will not change when using

the zoom mode.  This switch can also be toggled by the <L> key.

HOW TO MAGNIFY THE PEAK OF A WAVEFORM

1) Pull down the Windows menu and make sure that Lock Y Res in

Zooms is turned off.

2) Position the mouse cursor above and to the left of the

section of sound you wish to view, then drag the mouse down

and across the desired section.

3) Release the mouse button.

The Selected Area and the Insertion Cursor

Most of Wave Lite's editing operations are designed to be used on

only part of the soundfile. Such operations are applied to what

we call, logically enough, the selected area. The selected area

is the highlighted portion of the active soundfile window.

The mouse may be used to select an area any time the Select icon

is active, as described on page 37. To select a section of a

soundfile, drag the mouse across the area you wish to select. The

boundaries of the selected area may be changed by holding down a

<Shift> key and either clicking at the place you want the

selected area to begin or end, or dragging the boundary. This

technique is especially useful if you wish to fine tune the edges

of the selected area. If a selected area is not defined, then an

insertion cursor will appear somewhere in the soundfile. The

insertion cursor is a vertical blinking line, which is used to

tell Wave Lite where to insert new  material when pasting.

The insertion cursor is placed simply by clicking once in the

active soundfile window.

Locating the Selected Area or Insertion Cursor

If a selected area or insertion cursor is present but is not

visible in the soundfile window (probably due to scrolling), then

the selected area or insertion cursor still exists, even though

it can't be seen. In this case, the location of the selected area

or insertion cursor is shown in the time displays.

The display that shows the length of the selected area will be

blank if an insertion cursor is in use. To display the selected

area or insertion cursor in the window, press the <=> key. You

can also use the F9 and F10 keys to jump to the beginning or end

of the selected area; this is especially handy if the size of the

active soundfile window is mu ch smaller than the selected area.

HOW TO PRECISELY DEFINE THE SELECTED AREA

1) Zoom in on the location that you want to be the beginning

of the selected area, using the time resolution slider and

the horizontal scroll bar.

2) Click on the Select icon.

3) Place the mouse cursor at the desired location and drag the

mouse a short distance to the left.

4) Use the horizontal scroll bar to locate the other desired

endpoint for the selected area.

5) Hold down either <Shift> key, and drag to the desired

endpoint.

THE EDIT ICONS

The two icons shown at the left are called the Edit icons,

because they determine what editing function the mouse will

perform in the active soundfile window. These two buttons are

radio buttons, of which only one may be active at any time. The

Edit icons are the Draw icon and the Select icon.

THE SELECT ICON

When the Select icon is active, the mouse may be used either to

define a selected area or to place the insertion cursor. The

mouse cursor changes to an "I" beam shape when it is inside the

active soundfile window in this mode. Dragging the mouse selects

an area, or clicking the mouse places an insert cursor. These

operations are discussed in detail on page 35.

THE DRAW ICON

When this icon is active, the mouse can be used as a pencil, to

draw new sound data. This is primarily useful for removing

clicks, pops, and similar glitches in a recording. When waveform

drawing is enabled, the mouse cursor takes the form of a pencil

when it is inside the active soundfile window. Dragging the mouse

across the window draws in new sound data. When the mouse button

is released, Wave Lite will clean up the display, so don't be

dismayed if your drawing seems a bit sloppy at first. The Draw

icon is only available when the time resolution is sufficiently

high to make drawing practical. (The sound data is displayed as a

set of  lines, rather than as a solid curve, when this is so.)

HOW TO REMOVE A CLICK IN A SOUNDFILE

1) Drag the mouse across the section of the overview that

contains the click to view that part of the soundfile.

2) Use the right mouse button to zoom in on the click, as

described on page 34. If necessary, click once somewhere on

the screen to place the insertion cursor, then press the

<Space> bar to play the contents of the active soundfile

window in order to confirm your selection.

3) Click on the Draw icon.

4) Place the mouse cursor at the left edge of what seems to be

the base of the click, and then draw a line from that point

to what seems to be the trailing edge of the click.

5) Note that some clicks may be too wide to be deleted with

this method. If the click does not sound better when the

soundfile is played, select Undo Last Edit from the Edit

menu to replace the click.

THE PREVIOUS VIEW ICON

The Previous View icon is used to step through the last eight

views of the soundfile. Clicking on this icon will bring up the

last view of the current soundfile window. In this sense, a

"view" includes the settings of both resolution sliders and the

scroll bars. Selecting a previous view does not change the status

of the selected area or insertion cursor. If the stack of

previous views is empty when you click on this icon, you will

hear a beep. The <Backspace> key performs the same function as

this icon.

CUT AND PASTE EDITING

An important part of visual editing  in any digital audio program

is cut and paste editing. These functions are handled in Wave

Lite through the Cut, Copy, Delete, and various Paste items in

the Edit menu. Most of these functions make use of a temporary

storage area called the cut buffer. This buffer is actually a

file on the hard disk. If there is not enough room on the hard

disk to create a cut buffer when the need arises, Wave Lite will

issue an error message. When you quit Wave Lite, the contents of

the cut buffer are lost.

CUT

This item removes the material in the selected area and places it

in the cut buffer. The Cut function is duplicated by the

<Shift-Del> key combination.

COPY

This item is similar to Cut except that the selected area is not

deleted from the soundfile; it is simply copied to the cut

buffer. Pressing<Ctrl-Ins> performs the same function.

DELETE

Delete is like Cut without Copy: It deletes the selected area

from the soundfile without saving it in the cut buffer. This is

handy if you wish to delete something without disturbing the

contents of the cut buffer. Delete is duplicated by the

<Ctrl-Del> key combination.

PASTE INSERT

Paste Insert is used to place the contents of the cut buffer back

in the soundfile. It is only available if something has been

placed in the cut buffer; otherwise, this item is disabled.

The exact function of the Paste Insert command depends on whether

a selected area or insertion cursor is present. If a selected

area exists,  Wave Lite first deletes the selected area, then

inserts the contents of the paste buffer; in other words, the

contents of the cut buffer replace the selected area. If an

insertion cursor exists, the new material is inserted at that

point, and nothing is deleted from the soundfile. Paste Insert

can also be used to create a new soundfile. If you open an

untitled soundfile window and then click on Paste Insert, Wave

Lite will present a file selector where you can enter the name of

the new soundfile. The data in the cut buffer will then be pasted

into the new soundfile. Paste Insert is duplicated by

<Shift-Ins>.

PASTE OVER

This option is similar to Paste Insert, except that it always

replaces data in the soundfile, regardless of the existence of a

selected area. The length of the replaced data is identical to

the length of the cut buffer. <Shift-O> duplicates this menu

item.

PASTE FILL

This function will repeatedly copy data from the cut buffer to

the selected area, replacing any existing data. This is primarily

useful for dialog editing, when you'd like to replace an

extraneous sound with some ambient background noise. Simply copy

a few seconds of background noise (or "room tone") to the cut

buffer, then select the area you want to replace and select

Paste Fill. The <Shift-F>  key combination duplicates Paste Fill.

AUDITION CUT BUFFER

Audition Cut Buffer plays back the sound stored in the cut

buffer. This is useful if you forget exactly what is stored

there. Playback can be stopped by pressing the <Space> bar or

clicking the mouse. The <A> key duplicates this function.

RESTRICTIONS

Cut and paste editing is fairly unrestricted in Wave Lite. You

may paste material at any point in a soundfile, or from one

soundfile to another. The only limitation is that the format

(mono or stereo) of the destination soundfile must match that

from which the material in the cut buffer was taken.

If you try to paste the cut buffer into a soundfile with a

different format, Wave Lite will issue an error message.

You can paste data from a soundfile with a different sample rate

into a soundfile. When this happens, the pasted data will be

played back at the sample rate of the destination soundfile.

How to Remove Silence from the Beginning of a Soundfile

1) Position the window at the beginning of the soundfile.

2) Make sure the Select icon is active, then drag the mouse

across the silent area.

3) Zoom in on the beginning of the sound by pressing the right

mouse button and dragging the mouse diagonally across that

part of the soundfile.

4) Click on the Select icon, then fine tune the end of the

selected area by holding down the <Shift> key and clicking

just before the beginning of the sound.

5) Pull down the Edit menu and click on Delete.

HOW TO COPY MATERIAL BETWEEN SOUNDFILES

1) Make sure that the two soundfiles you wish to use as the

source and destination are each open in their own windows.

2) Top the window containing the source soundfile.

3) Select the area you wish to copy, then pull down the Edit

menu and click on Copy.

4) Top the window containing the destination soundfile.

5) Place the insertion cursor in the destination soundfile by

clicking at the location where the copied material is to be

inserted.

6) Pull down the Edit menu and click on Paste Insert.

MUTING

The Mute option, in the Edit menu, can be used to erase a portion

of either or both channels. Clicking here, or pressing <Ctrl-M>,

will set the data in the selected area to zero, which is the

computer's way of erasing audio.

UNDOING EDITS

The Undo Last Edit option can be used to undo the effects of any

destructive editing operations, such as Cut, Delete, or Paste.

Undo Last Edit is only available when a check mark appears to the

left of the undo Enabled switch. Clicking on Undo Enabled will

toggle the undo feature. If undo is disabled, the words No Undo

will also appear in Wave Lite's menu bar. When Wave Lite performs

an editing operation, it first makes a copy of the section of the

soundfile that will be edited. This copy is placed in a special

file called the undo buffer. If you decide to undo an edit, Wave

Lite simply swaps the edited section of the soundfile with the

undo buffer. Since creating the undo buffer can be quite

time-consuming, and may also involve a lot of hard disk space, we

have given you the option of disabling the Undo Last Edit option.

We suggest that when you are preparing to make massive changes to

an entire soundfile, you first make the change to a small section

of the file with undo enabled. Once you are confident of the

results you'll be getting, turn the undo feature off, and then

edit the entire file. The status of the Undo Enabled switch is

saved in Wave Lite's configuration file. The <Alt-Backspace> key

combination duplicates the Undo Last Edit function. The <U> key can

be used to toggle the Undo Enabled switch.

CHAPTER 7

THE FILE MENU

Wave Lite's File menu contains several options for managing

soundfiles. In this chapter, we will discuss the contents of the

File menu, as well as some considerations for hard disk

management.

THE FILE MENU

This menu contains several options for managing soundfiles. Its

contents are described below.

NEW

This option removes the soundfile from the top window and sets

the window's name to "Untitled". Because the soundfile is

contained entirely in disk files, no data is lost when New is

selected.

OPEN

This opens a soundfile and places it in the top window. A file

select dialog will present you with a list of the available

soundfiles.

SAVE AS

This option is used to make a copy of the soundfile in the active

window. A file select dialog will allow you to enter the name to

save the soundfile under.

DELATE FILE

This item can be used to delete any file from the disk. The

extension defaults to .WAV, but you can change this to any

extension you wish, and delete any file type.

EXIT

This terminates Wave Lite. Because audio data is stored on the

hard disk at all times, there is no need to save your soundfiles

before quitting the program.  You may also quit Wave Lite by

pressing <Alt-F4>.

THE SOUDNFILE INFORMATION DIALOG

The Soundfile Info option in the Edit menu brings up a dialog

which allows you to view important information about and to add

comments to the soundfile.

SOUNDFILE INFORMATION DIALOG

The Sample Rate, Resolution, and Channels fields show the obvious

information.  These values are determined when the soundfile is

recorded, and cannot be changed from this dialog. Block Align and

Format Tag are two data fields found in every Windows-compatible

.WAV file, and are provided here solely for the interest of the

tech-heads out there. (You know who you are.) The SMPTE Offset

field allows you to enter an offset time, in SMPTE units,

that will be added to the time display.

CHAPTER 8

THE WINDOWS AND OPTIONS MENUS

THE WINDOWS MENU

The Windows menu contains items that affect the arrangement of

the windows on the screen, as well as some switches for altering

the operation of zooming in the active soundfile window.

Lock Y Res in Zooms disables zooming in the Y direction. If this

option is checked, then the amplitude resolution will not change

when using the zoom mode. Center Y Resolution is used to center

the soundfile's zero line in the middle of the window. This is

especially useful when you wish to zoom in on a zero crossing

point, as  it can be difficult to center the display vertically

with the scroll bar. The next four items in the Windows menu

change the arrangement of the windows on the screen. Cascade

Windows places the open soundfile windows in an overlapping

diagonal arrangement. Tile Windows places each open window in a

corner of the screen. Strip Windows puts the windows in

horizontal strips, one above the other. Arrange Icons simply

places the icons for any minimized soundfile windows in a

convenient arrangement at the bottom of the main window. .

The names of the soundfiles in each window appear at the end of

the Windows  menu. Selecting any of these will bring the

corresponding window to the top.

THE OPTIONS MENU

The Options menu contains several different items for adjusting

Wave Lite's display and operational parameters, for performing

system checkout functions, and other miscellaneous functions.

These items are described in this chapter.

PREFERENCES

This menu item brings up the following dialog, which is used to

set many of Wave Lite's global operating parameters; that is,

settings that affect the operation of the entire program. You

shouldn't need to spend much time here; the usual procedure is to

set these parameters once, then forget that this dialog even

exists.

PREFERENCES DIALOG

INSERTION CURSOR BLINK SPEED

Insertion Cursor Blink Speed sets the rate at which the insertion

cursor blinks.

SCREEN COLORS

The combo box labeled Set Colors of and the buttons labeled

Colors are used to change the colors used in the soundfile

windows. The Set Color of list determines what part of the

display will be changed by the Colors buttons. You may set a

different waveform plot and highlight color for each soundfile

window; all of the other color settings affect every soundfile

window.

SOUNDFILE PATH

This setting tells Wave Lite where to look for and store

soundfiles. Remember that you can always override this in the

file selection dialogs.

WAVE LOGO

This switch displays or hides the  Wave Lite logo that appears in

the toolbar.

PLAY MODE

RECORD MODE

These buttons tell Wave Lite where it should position the

beginning of the active soundfile window when playback or

recording is finished. If Hard Disk is selected, the position of

the window will not change when playback or recording is

finished. This is the mode you'll want to use most often. If Tape

is specified, the window will be positioned at the point where

playback or recording stopped. This is handy if you want to

locate a section of sound by ear.

DISPLAY SETUP

The Display Setup dialog is used to set several options

pertaining to the soundfile windows. This dialog can be brought

up either by selecting Display Setup, or by clicking on the units

in any of the time displays in a soundfile window.

DISPLAY SETUP DIALOG

X AXIS CALIBRATION UNITS

The X Axis Calibration Units setting determines how time is

displayed in the overview, soundfile windows, and time displays.

If set to SF Time, time is displayed in hours, minutes, seconds,

and hundredths of seconds. Time Code is similar, except that the

hundredths of seconds display is replaced by SMPTE frames.

Decimal and Hex tell Wave Lite to calibrate time in terms

of sample frames, using either decimal or hexadecimal notation.

Film displays times in terms of film feet and frames, at rates of

1.5 feet per second and 24 frames per second, and Beat causes

times to be displayed in terms of beats.

Y AXIS CALIBRATION UNITS

Y Axis Calibration Units sets the calibration used for amplitude

in the soundfile windows. If set to % Amplitude, the units on the

Y axis will range from -100 to 100. If set to Decimal Samples,

the Y axis is calibrated in individual sample levels, from -32767

to 32768. Hex Samples is similar to Decimal Samples, except that

a two's complement hexadecimal numbering system used.

CALIBRATION LINES

X Calibration Lines and Y Calibration  Lines affect the grid

lines that are drawn in the soundfile windows. The X axis

(horizontal) lines can be turned on or off. For the vertical (Y

axis) calibration lines, you have three choices:

No lines at all (Off),

Short lines at the bottom of the display (Baseline),

Full-height lines (Full Height).

MAXIMUM ZOOM

The Maximum Zoom field determines the maximum size of the

soundfile window when grabbing it from the overview. The

rationale for this setting is explained on page 29. Note that you

can still view longer sections of the soundfile by using the time

resolution slider.

SYSTEM CHECK OPTIONS

The two system check functions described below are used to

provide information about things you normally should not need to

worry about. If, however, you ever experience problems using Wave

Lite, this is the place to start. If you ever need to call us for

help, we request that you first run through the following items

and write down the information that appears in each check, before

calling.

I/O DEVICE SELECT

This option allows you to select from different wave audio

devices that may be loaded in your system. This is useful for

authoring sounds that must be played back on a variety of sound

cards.

PC EQUIPMENT CHECK

This item displays important information about your computer. The

dialog that appears after you select this option shows the amount

of free memory available to Windows, the resolution of your video

system, and the capabilities of your sound card.

HARD DISK CHECK

This menu option looks at the drive specified in the Soundfile

Path field in the Preferences dialog and displays the amount of

free space on that drive, along with the drive's total capacity.

CHAPTER 9

SAMPLING TECHNIQUES

This chapter will take you through several procedures that you

will commonly use in order to acquaint you with the program and its

interface.

SAMPLING FROM A MICROPHONE

The input circuitry on many sound boards is designed for sampling

directly from a microphone. This is convenient if a microphone is

the only piece of audio gear you own, but for a variety of reasons,

it is much easier to get a good sample if you first record your

voice on a tape deck, and then sample the sound from the tape, as

described in the next section. When recording from a microphone, it

is important to obtain the highest possible level without

overdriving the sound hardware's analog to digital converter.

Monitoring the level meters in the Record dialog is

the best way to ensure this. As you speak into the mike, the

indicator in the meter should just barely touch the top of the

meter at its highest point.

Don't let it slam against the top of the meter for any length of

time, as this will result in some truly nasty digital distortion.

It may require a few "takes" to get this right, but with practice

it will get easier.

SAMPLING FROM A CD OR TAPE

Sampling from a CD or tape is much easier than sampling from a

microphone, because the signal is consistent: It will always be at

the same level each time you play it, which makes it much easier to

get a good recording. It is also easier to  get a good vocal "take"

when recording on tape than it is when sampling from a mike,

because you don't have to deal with the computer and the

talent simultaneously. Depending on which sound board you use,

there may be some signal level matching problems when recording

from such a source. CD players and tape decks output something

called a line level signal, which is much "louder", electrically

speaking, than the signal from a microphone. While many sound

boards can accept either line level or mike signals, some

(including the original Sound Blaster) are designed to accept only

microphone level signals. Sampling directly from a CD with such a

board will result in serious distortion. The solution is to use a

special "attenuator plug", which reduces the output

level of the CD player to match that of a microphone. This might

sound like an esoteric, hard-to-find beast, but it's not: Radio

Shack carries attenuator plugs (part number 274-300). Connect this

between one channel of your CD player or tape deck and the sound

board's microphone input. If you're sampling from a stereo sound

source like a CD or taped music, there's another problem: many

sound boards can only handle mono signals. In this case, you'll

need to put an audio mixer between the source and the mono

line input in order to mix the two stereo channels to one.

The following diagram illustrates this arrangement.

SAMPLING FROM A TAPE DECK

APPENDIX A

LICENSE AND WARRENTY

LICENSE AGREEMENT

This is a legal agreement between you the end user, and Turtle

Beach Systems, Inc. BY FILLING OUT AND RETURNING THE ENCLOSED

WARRANTY CARD, YOU ARE AGREEING TO BE BOUND BY THE TERMS OF THIS

AGREEMENT. IF YOU DO NOT AGREE TO THE TERMS OF THIS AGREEMENT,

PROMPTLY RETURN THE PROGRAM MATERIALS AND REFERENCE MANUALS TO

PLACE OF PURCHASE FOR A FULL REFUND.

1. GRANT OF LICENSE. Turtle Beach Systems grants you the right to

use one copy of the enclosed Turtle Beach Systems Inc. software

product (SOFTWARE) on a single terminal connected to a single

computer (i.e. with a single CPU). You may not network the SOFTWARE

or otherwise use it on more than one computer or computer terminal

at a time.

2. COPYRIGHT. The SOFTWARE is owned by Turtle Beach Systems and

is protected by United States copyright law and international

treaty provisions. Therefore, you must treat the SOFTWARE like any

other copyrighted material (e.g. a book or musical recording)

except that you may either (a) make one copy of the SOFTWARE

solely for backup or archival purposes, or (b) transfer the

SOFTWARE to a single hard disk provided you keep the original

solely for backup or archival purposes.

3. OTHER RESTRICTIONS. You may not rent or lease the SOFTWARE,

but you may transfer the SOFTWARE and accompanying documentation on

a  permanent basis provided you retain no copies and the recipient

agrees to the terms of this agreement. You may not reverse

engineer, decompile or disassemble the SOFTWARE.

LIMITED WARRANTY

Although every effort has been made to ensure the accuracy of the

text and illustrations in this manual, no guarantee is made nor

implied in this regard.

1. WHAT IS COVERED

This warranty covers defects in material and

workmanship for a period of thirty days from

date of purchase from a Turtle Beach dealer.

2. HOW TO OBTAIN WARRANTY PERFORMANCE.

Return your disk(s) to place of purchase along with proof of

purchase. No claim for warranty will be honored without proof of

purchase.

3. LIMITATIONS

Any implied warranties, including warranties of merchantability and

fitness for a particular purpose, are limited to the length of this

warranty. Turtle Beach's liability for any defective product is

limited to the replacement of the product. Turtle Beach Systems

will not be liable for damages based on inconvenience, loss of use

of disks, loss of time, interrupted operation or commercial loss,

any other damages, whether incidental, consequential, or otherwise

except damages which may not be excluded under applicable law. Some

states do not allow limitations on how long an implied warranty

lasts and/or do not allow the exclusion or limitation of incidental

or  consequential damages, so the above limitations and exclusions

may not apply

to you. This warranty gives you specific legal rights and you may

also have other rights which vary from state to state.