286|DOS-Extender Options

------------------------

Copyright 1991 by Phar Lap Software, Inc.

The copy of Phar Lap's 286|DOS-Extender bound into this program

can be configured with a number of command switches. Because

286|DOS-Extender is self-configuring, and because it is compatible

with all major industry standards (such as VCPI, DPMI, and XMS), it

should almost never be necessary to use these switches. However,

should the need arise to configure the DOS extender, this following

provides information on the various switches and on the different

ways to set them.

The following is a summary of 286|DOS-Extender switches:

-A20 n              Set mode switch A20 address line method

-CONTROL n          Set mode switch 80286 regain-control method

-EXTBLOCK n         Set extended-memory block size

-EXTFIRST           Allocate extended memory before conv. (Default)

-EXTLIMIT n         Set max. bytes of extended memory used

-EXTLOW addr        Set lowest extended memory address used

-EXTHIGH addr       Set highest extended memory address used

-ISTKSIZE n         Set size of interrupt stacks (1K blocks)

-LDTSIZE n          Set number of LDT entries

-LOADDLL dlls       Load one or more dynamic-link libraries

-NISTACK n          Set number of interrupt stacks

-REALBLOCK n        Set conventional-memory block size

-REALFIRST          Allocate conventional memory before extended

-REALLIMIT n        Set max. bytes of real memory used

-RESET n            Set mode switch 80286 reset method

-SWITCH name        Select mode switch method (AUTO, AT, 386,

SURESWITCH, PS2, INBOARD, 6300, ACER,

VECTRA, VCPI, or FAST)

-SWITCHFILE name    Select external switching .BIN file

-VDISK              Compatibility with programs that incorrectly

implement the VDISK allocation standard

-XFERSIZE n         Set size of transfer buffer (1K blocks)

-XMS option         Controls handling of XMS extended memory

There are additional 286|DOS-Extender switches for use by

application developers. If you need information that doesn't seem to

be provided in this document, please contact the vendor of your

program.

RUN286 environment variable

---------------------------

One way to set DOS extender switches is with a DOS environment

variable named RUN286. For example:

C:\>set run286=-realfirst

(Note that there is no space between the equals sign = and the

dash -REALFIRST.)

Now when you run the DOS-extended program, the built-in copy

of 286|DOS-Extender will use the -REALFIRST switch (which is

explained below).

Placing an invalid switch in the RUN286 environment variable will

produce a help screen of the most important switches the next time

you run the program. For example:

C:\>set run286=-help

To remove this (or any other) invalid switch so that you can

continue to run the program in the normal way, set RUN286 equal to

nothing:

C:\>set run286=

DOS environment variables such as RUN286 are not permanent. If

you want to ensure that the RUN286 variable gets set each time you

start your computer, place the SET command in your AUTOEXEC.BAT file.

Using CFIG286

-------------

Options can also be placed directly inside the DOS-extended

program, using the CFIG286 utility. For example:

C:\APROGRAM>cfig286 aprogram.exe -realfirst

The CFIG286 utility can also be used to tell 286|DOS-Extender to

use a command file, or an environment variable with a different name

from RUN286 (this is particularly useful if you have more than one

program which uses 286|DOS-Extender). For example:

C:\APROGRAM>cfig286 aprogram.exe %aprogram

C:\>set aprogram=-realfirst

C:\>aprogram

See the file CFIG286.TXT for a complete description of the

CFIG286 utility.

Switches

--------

The 286|DOS-Extender switches can be divided into several

categories:

Memory Allocation:

-EXTBLOCK n       Set extended-memory block size

-EXTFIRST         Allocate extended memory before conv. (Default)

-EXTLIMIT n       Set max. bytes of extended memory used

-EXTLOW addr      Set lowest extended memory address used

-EXTHIGH addr     Set highest extended memory address used

-REALBLOCK n      Set conventional-memory block size

-REALFIRST        Allocate conventional memory before extended

-REALLIMIT n      Set maximum bytes of real memory used

-XMS option       Controls handling of XMS extended memory

Mode Switching:

-A20 n            Set mode switch A20 address line method

-CONTROL n        Set mode switch 80286 regain-control method

-RESET n          Set mode switch 80286 reset method

-SWITCH name      Select mode switch method (AUTO, AT, 386,

SURESWITCH, PS2, INBOARD, 6300, ACER,

VECTRA, VCPI, or FAST)

-SWITCHFILE name  Select external switching .BIN file

Interrupt Stacks:

-ISTKSIZE n       Set size of interrupt stacks (1K blocks)

-NISTACK n        Set number of interrupt stacks

Miscellaneous:

-LDTSIZE n        Set number of LDT entries

-LOADDLL dlls     Load one or more dynamic-link libraries

-VDISK            Compatibility with programs that incorrectly

implement the VDISK allocation standard

-XFERSIZE n       Set size of transfer buffer (1K blocks)

In the remainder of this section, these switches are described in

alphabetical order.

In most cases, the full name of the switch does not need to be

provided. In the following descriptions, square brackets are used to

indicate optional charatcers. For example, -l[dt]s[size] means that

this switch can be specified as -ldtsize, -ls, -LDTSIZE, or -LS.

Where two or more switches are mutually exclusive, the default

switch is designated as "(Default)." For example, in the entries for

-REALFIRST and -EXTFIRST, the entry for -EXTFIRST carries the

designation (Default), indicating that it is generally not necessary

to specify this switch, since it is already 286|DOS-Extender's

default operation.

Note that any switches which take numeric arguments can take

either decimal numbers or hexadecimal numbers (for example, 7Bh).

Hexadecimal numbers may contain an underbar separator for readability

(for example, 40_000h).

Examples provided below are always shown using the SET RUN286=

syntax. However, all these switches may also be set using the CFIG286

utility, a command file, or an application-specific environment

variable. See CFIG286.TXT for details.

-A20[METHOD] method#

--------------------

One of 286|DOS-Extender's jobs is to figure out the best way to

switch the machine between real and protected mode. It does this

automatically, without user intervention. However, sometimes greater

control might be needed. The -SWITCH option provides high-level

control over mode-switching techniques; the -A20, -CONTROL, and

-RESET switches provide low-level control over the technique

286|DOS-Extender uses to switch between real and protected modes.

This is almost never necessary, since 286|DOS-Extender almost always

does the right thing. However, these options might be necessary on

strange hardware.

The -A20 switch selects one of the following techniques for

enabling and disabling of the processor's A20 address line:

-A20 method#

0 = default

1 = IBM AT

2 = IBM PS/2

3 = HP Vectra

4 = AT&T 6300

5 = use XMS (eXtended Memory Specification)

For example:

C:\>set run286=-a20 1 -reset 2 -control 2

-CONTROL[METHOD] method#

------------------------

The -CONTROL switch selects one of the following techniques for

regaining control after the reset of an 80286 processor:

-CONTROL method#

0 = default

1 = old IBM ATs

2 = standard IBM ATs

3 = slow IBM AT technique

4 = IBM PS/2

5 = 80386

6 = VCPI

7 = HP Vectra

8 = AT&T 6300

(See -A20 for an example.)

-EXTB[LOCK] n

-------------

-EXTBLOCK sets the extended-memory block size. The default block

size is 512k bytes. When -EXTBLOCK is a large number, 286|DOS-Extender

does not have to allocate extended memory as frequently, but it may

waste memory. Using -EXTBLOCK, you can control this time vs. space

tradeoff. For example, the following changes the extended-memory block

size to 256k (40000 hex) bytes:

C:\>set run286=-extblock 40_000h

-EXTF[IRST] (Default)

-----------

The -EXTFIRST option corresponds to the default 286|DOS-Extender

memory allocation strategy: allocate extended memory before

conventional memory. This can be changed with the -REALFIRST option.

-EXTLI[MIT] n

-------------

The -EXTLIMIT option sets the maximum bytes of extended memory to

be used by a protected-mode program. For example, the following both

limit MEM to one megabyte of extended memory:

C:\>set run286=-extlimit 1048576

C:\>set run286=-extli 100000h

To prevent the program from using *any* extended memory:

C:\>set run286=-extli 0

-EXTL[OW] addr

-EXTH[IGH] addr

---------------

The -EXTLOW and -EXTHIGH options set the lowest and highest

extended memory address to be used by the program. By default, all

extended memory (memory above one megabyte) that is not already

allocated to other programs is available for use by applications. Use

the -EXTLOW and -EXTHIGH switches to keep 286|DOS-Extender from using

extended memory owned by another program that is incompatible with

any of the industry standards for extended memory use.

For example, you have installed a 1/2 megabyte disk cache

starting at 100000h (one megabyte) which does not properly follow the

INT 15h rules for allocating memory, you can use -EXTLOW to keep

286|DOS-Extender from trying to allocate this same block of memory.

It's also a good idea to specify an additional 64k bytes slopover,

just to be safe. In the following, 286|DOS-Extender will not use INT

15h memory allocation for any address below 100_000h (one megabyte) +

80_000h (1/2 megabyte) + 10_000h (64k) = 190_000h:

C:\>set run286=-extlow 190_000h

-IS[TKSIZE] n

-------------

The -ISTKSIZE and -NISTACK switches are used to control how much

memory is allocated to the buffers used to provide stack space when

switching from protected to real mode. For the vast majority of

application programs, the default settings of these parameters are

adequate.

The -ISTKSIZE option sets the size of 286|DOS-Extender interrupt

stacks. The argument n is in 1k blocks. The default interrupt stack

size is 1k. For example:

C:\>set run286=-ni 6 -istk 2

-L[DT]S[IZE] n

--------------

The -LDTSIZE option determines the size of a protected mode

program's Local Descriptor Table (LDT), where n is the number of

selectors. The default LDT size is 512 selectors. Select a larger

number if the program runs out of selectors; select a smaller number

to conserve memory if the program doesn't require 512 selectors (each

selector requires 8 bytes). For example, the following increases

the size of the LDT:

C:\>set run286=-ldtsize 1024

-LOADDLL dlls

-------------

The -LOADDLL switch is used to force-load one or more dynamic

link libraries (DLLs) before a DOS-extended program starts. The

switch takes as an argument a list of one or more DLLs to be loaded.

Names in the list are separated by commas. For example:

C:\>set run286=-loaddll moe,curly,larry

286|DOS-Extender loads the DLLs specified with the -LOADDLL

switch after it completes initialization and before an application is

loaded and run. If DLLs are not located in the same directory as

the DOS-extended application, specify the full pathname:

C:\>set run286 =-loaddll \addins\phoobar.dll

An application may use the -LOADDLL switch as part of an add-in

facility.

-NI[STACK] n

------------

The -ISTKSIZE and -NISTACK switches are used to control how much

memory is allocated to the buffers used to provide stack space when

switching from protected to real mode. For the vast majority of

application programs, the default settings of these parameters are

adequate.

The -NISTACK option sets the number of 286|DOS-Extender interrupt

stacks. By default, there are 6 interrupt stacks.

For example, the following program allocates an very large number

of big interrupt stacks (ten 6k stacks):

C:\>set run286=-istksize 6 -nistack 10

-REALB[LOCK] n

--------------

-REALBLOCK sets the conventional-memory block size. The default

block size is 64k bytes. When -REALBLOCK is a large number,

286|DOS-Extender does not have to allocate conventional memory as

frequently, but it may waste conventional memory. Using -REALBLOCK,

you can control this time vs. space tradeoff. For example, the

following lowers the conventional-memory blocksize to 32k bytes:

C:\>set run286=-realblock 8000h

-REALF[IRST]

------------

By default, 286|DOS-Extender uses extended memory before

conventional (real) memory. This default is the -EXTFIRST strategy;

it is generally what one wants, because it leaves the precious memory

below the one-megabyte line free for as long as possible, while still

allowing a protected-mode program access to conventional memory once

extended memory is exhausted.

However, you might want to leave *extended* memory free for as

long as possible, perhaps so that another program built with

286|DOS-Extender or 386|DOS-Extender had more extended memory. In

this case, you would use the -REALFIRST option:

C:\>set run286=-realfirst

-REALLI[MIT] n

--------------

The -REALLIMIT option selects the maximum number of bytes of real

(conventional) memory your protected mode program can access. Use

this switch if you need to reserve memory for real mode TSRs, network

drivers, etc. The default -REALLIMIT is one megabyte (that is, no

limit). For example, the following both prevent a DOS-extended

program from using more than 256k bytes of conventional memory:

C:\>set run286=-reallimit 262144

C:\>set run286=-realli 40000h

To prevent it from using *any* conventional memory:

C:\>set run286=-realli 0

-RESET[METHOD] n

----------------

The -RESET switch selects one of the following techniques for

resetting 80286 processors:

-RESET method#

0 = default

1 = triple fault

2 = OUT to 8042 keyboard controller

(See -A20 for an example.)

-SW[ITCH] name

--------------

In contrast to the micro control provided by the -A20, -RESET,

and -CONTROL options, the -SWITCH switch selects one of the following

macro switching techniques:

AUTO        default

AT          force use of old IBM AT technique

FAST        force use of standard IBM AT technique

386         force use of 80386 instruction set technique

SURESWITCH  force use of Phar Lap SureSwitch (TM) technique

PS2         force use of PS/2 technique

INBOARD     force use of Intel InBoard technique for PC/XT

with 286/386 accelerator

6300        force use of AT&T 6300 technique

ACER        force use of Acer technique

VECTRA      force use of HP Vectra technique

VCPI        force use of VCPI technique

For example:

C:\>set run286=-switch inboard

Obviously, you should not specify -SWITCH 386 on an IBM AT, or

-SWITCH VECTRA on a Compaq 386! Furthermore, you should almost never

have to use this switch in the first place, because 286|DOS-Extender

will take care of such details automatically.

-SW[ITCH]F[ILE] name

--------------------

The -SWITCHFILE option uses switching code in an external

.BIN file. For example:

C:\>set run286=-switchfile wow586.bin

-VDISK

------

The -VDISK switch is a workaround for compatibility problems with

other programs which do not correctly follow the VDISK standard for

allocating extended memory. If 286|DOS-Extender refuses to run a

program because of inconsistent VDISK allocation signatures, this

switch can be used to force 286|DOS-Extender to run the program

anyway. The larger of the two VDISK allocation marks present will be

used. Before using this switch, you should check the allocation sizes

printed out with the error messages when 286|DOS-Extender refuses to

run the program. If the larger of the two numbers printed out does not

seem reasonable, it will be necessary to calculate how much extended

memory is in use by other programs to use the -EXTLIMIT switch to

inform 286|DOS-Extender of the correct value.

-XFER[SIZE] n

-------------

The -XFERSIZE option sets the size of the conventional-memory

transfer buffer used for DOS and BIOS calls. Whenever a

protected-mode program makes an INT 21h or BIOS request that uses a

pointer to a buffer, behind the scenes 286|DOS-Extender copies the

data from your protected mode buffer to the conventional memory

transfer buffer, or vice versa. Requests involving buffers larger

than the conventional memory transfer buffer are split into smaller

requests (invisibly to your program, of course). Protected mode

programs that make many DOS file I/O requests with large blocks might

benefit from using a larger transfer buffer. The size is specified in

1k increments. For example, to make the transfer buffer 8k:

C:\>set run286=-xfersize 8

-XMS option

-----------

The -XMS controls how 286|DOS-Extender handles XMS extended

memory. The following options are provided:

OFF         Do not use XMS memory.

AUTO        [Default] If running under Qualitas 386MAX, do not use

XMS memory. Otherwise, use XMS memory and assume that the

XMS driver uses physical addresses.

LIN[EAR]    Not supported at present. Implemented as OFF in this

release.

PHYS[ICAL]  Use XMS memory if present and assume the XMS driver uses

physical addresses.

This switch is necessary because some XMS memory managers (such as

Qualitas 386MAX) return linear addresses, whereas others (such as

Microsoft HIMEM.SYS and Quarterdeck QEMM) return physical addresses.

- END -