Using TELLME

------------

Copyright 1991 by Phar Lap Software, Inc.

TELLME is a utility that produces a report describing the system

on which it is run. TELLME provides information on the CPU, BIOS,

DOS, environment, extended memory, XMS, EMS, VCPI, DPMI, and so on.

The utility provides a complete picture of a system so that

compatibility problems can be resolved. TELLME provides the following

information about a system:

o   Type of CPU and numeric coprocessor

o   Information about the DOS version being run

o   Information about the BIOS ROM being used

o   A print out of the CONFIG.SYS and AUTOEXEC.BAT files

o   A list of important environment variables for 286|DOS-Extender

(PATH, LIBPATH, RUN286, LIB, INCLUDE etc.)

o   Information about various extended memory allocators (INT 15H, XMS,

EMS/VCPI, VDS, and DPMI)

o   Presence of Windows, DESQview, SHARE, and DOS extenders.

o   Statistics about the RUN286.EXE file and its DLLs.

o   Conventional memory usage

o   Speed of mode switches under 286|DOS-Extender

Most of TELLME runs in real mode. The last portion of the program

uses a separate program, TELLPROT.EXE, to launch 286|DOS-Extender and

run in protected mode.

For example, if you have two megabytes of memory, yet receive

"insufficient memory" messages from a DOS-Extended application built

with 286|DOS-Extender, you can run TELLME to see what the problem

is:

C:\>tellme > tellme.log

TELLME.LOG might, for instance, show that an XMS extended-memory

manager is present, but that no XMS memory is available:

-----XMS-------------------------------------------------------

XMS version.....................................2.0

Driver version..................................2.96

HMA.............................................Present

A20.............................................Off

Size of largest XMS block.......................0K bytes

Total free XMS memory...........................0K bytes

One situation in which this might happen is you are running under

Windows 3 Standard or Real mode, but have not set the "KB Limit" in

the XMS Memory section of an appropriate Windows .PIF (Program

Information) file. The result is that DOS-extended applications can't

get any extended memory. In any case, TELLME can tell you what's

going on.

If you have isolated a problem to a particular area, you can

tell TELLME to only run certain tests. For example, the following

runs only the XMS test:

C:\>tellme -xms > tellme.log

The final test that TELLME performs uses TELLPROT.EXE to run

286|DOS-Extender.

A complete list of TELLME switches is provided by running TELLME

-HELP. Rather than run the entire TELLME test suite, selected

portions can be turned on or off. A complete list of TELLME switches

is provided by running TELLME -HELP or TELLME -?:

C:\>tellme -?

TELLME [switches]

-CPU                 Perform CPU test

-BIOS                Perform BIOS test

-DOS                 Perform DOS test

-MEM                 Perform conventional memory test

-CONFIG              Print out CONFIG.SYS

-AUTOEXEC            Print out AUTOEXEC.BAT

-ENV                 Print out important environment variables

-INT15               Perform INT 15 extended memory test

-XMS                 Perform XMS test

-EMS or -VCPI        Perform EMS test

-VDS                 Perform VDS test

-DPMI                Perform DPMI test

-FILE                Print out DOS-Extender version information

-MOUSE               Perform mouse test

-RUN                 Run the DOS-Extender

Placing a "NO" in front a of switch disables a test.  Example: -NOEMS

These switches can also be used to avoid certain tests. For

instance, in the unlikely event that 286|DOS-Extender does not work

on a machine (for example because of an buggy BIOS), then the TELLME

test that runs 286|DOS-Extender won't work either and should be

avoided, at least until the incompatibility has been resolved. To do

this, run TELLME -NORUN. For example:

C:\>tellme -norun > tellme.log

If all the TELLME tests run properly, then an application built

with 286|DOS-Extender should also run properly in the same

configuration. This makes TELLME a useful test of 286|DOS-Extender

compatibility.

In addition to TELLME, compatibility problems can also be

diagnosed with the SWTEST and BIOSDUMP utilities. These are used for

testing the protected/real mode and A20 switching methods used by

286|DOS-Extender, and to generate ROM BIOS images. See the document

SWTEST.TXT for more details.

- END -