Windows 3 and DOS-Extended Applications

---------------------------------------

Copyright 1991 by Phar Lap Software, Inc.

Applications produced with 286|DOS-Extender can run in the DOS

box of all three modes of Microsoft Windows 3.0 and higher. This

includes 386 Enhanced mode.

You can tell which the mode you are in by selecting "About

Program Manager..." from the Program Manager Help menu.

Windows Standard and Real Modes

-------------------------------

In Windows 3 Real and Standard mode, 286|DOS-Extender can get

extended (XMS) memory from the HIMEM.SYS device driver that comes

with Windows. You will need to set the XMS memory "KB Limit" in a

Program Information File (PIF).

You can use the Phar Lap TELLME utility to make sure that your

application is getting the extended memory it needs when running

under Windows Real and Standard modes. For example, the following

indicates that no XMS extended memory is available:

C:\>tellme -xms

-----XMS-------------------------------------------------------

XMS version.....................................2.0

Driver version..................................2.96

HMA.............................................Present

A20.............................................Off

Size of largest XMS block.......................0K bytes

Total free XMS memory...........................0K bytes

The easiest way to make sure that your DOS-extended programs get

sufficient XMS memory in Windows Standard and Real mode is to use the

Windows PIF Editor to create a _DEFAULT.PIF file with an XMS Memory

KB Limit setting of several megabytes. Windows uses _DEFAULT.PIF if

no application-specific .PIF file is available; _DEFAULT.PIF

corresponds to a mythical program called _DEFAULT.EXE. Alteratively,

you can create a COMMAND.PIF file for the DOS box, or a .PIF file

that bears the name of your program.

Using PHARLAP.386 in Enhanced Mode

----------------------------------

In Enhanced mode, 286|DOS-Extender uses the DOS Protected-Mode

Interface (DPMI) services provided by Windows. You don't need to

change any .PIF file settings.

DOS-extended applications running under Windows 3 enhanced mode

automatically get virtual memory.

If you will be running 286|DOS-Extender under Windows 3.0

Enhanced mode, and do not have an 80287 or 80387 numerics coprocessor

or an 80486 processor (with built-in numerics), you will need to

install PHARLAP.386. This 32-bit virtual device driver allows

floating-point emulation to work properly under Windows 3.0 enhanced

mode.

PHARLAP.386 should be copied to your Windows subdirectory, and

the following line should be added to the [386enh] section of your

Windows SYSTEM.INI file:

DEVICE=PHARLAP.386

The SYSTEM.INI file is a standard text file located in your

Windows 3.0 subdirectory.

Nested DOS Extenders

--------------------

The DPMI server in Windows 3.0 only allows one DOS extender to be

running at a time in a given DOS "box" (virtual machine). That is,

DOS extenders cannot be "nested" under Windows 3.0 Enhanced mode.

For example, you cannot run 286|DOS-Extender applications from

inside the DOS shell of a DOS-extended application like Lotus 1-2-3

Release 3.1, nor can you run DOS-extended applications like 1-2-3

from within the DOS shell of a 286|DOS-Extender application. (A DOS

shell would generally be created with the C system() function.)

If 286|DOS-Extender is started under the DOS shell of a

DOS-extended application under Windows 3.0 Enhanced mode, it will

issue an error message:

Fatal Error 286.1090: Nested DOS Extenders cannot be run in Windows 3.0

enhanced mode.

- END -