EPOST.DOC                                                             6/22/92

_____________________________________________________________________________

The way that you inform the post that you want a 4 axis wire format is by

adding extra characters behind the HCode, VCode, CtrIncremental? and CtrCodes

keywords as in these sample lines taken from the POSTFORM.E file:

HCode X U               { Indicates U and V for auxilliary

VCode Y V                 plane movement. }

...

CtrCodes I J K L        { Indicates K and L for Aux. plane Circle

center codes. }

CtrIncremental? Y N     { "Y Y" causes K,L to be inc. from I,J.

"Y N"  makes K,L inc'l from arc start on

Aux. plane. }

Of course, the formats for the letters U, V, K, and L must be set in the

letter format section of the PostForm file as well (this is usually found

right after the NAME line at the top of each machine format in the

PostForm file).

- Programming mode must be absolute: no "INCREMENTAL? Y" line is allowed

in the PostForm file.

- The INFEED and OUTFEED sequences can only be 1 block long each.

NEW! Ver 3.5 EPOST Information

_____________________________________________________________________________

EPOST is a generic user configurable 4 Axis wire EDM post processor that

works almost identically like MPOST. EPOST requires an appropriate

POSTFORM.E or POSTFORM.EUC file and operates in the same fashion as MPOST.

EPOST requires 2 incoming INC files the first for the 'program' (XY)

plane, and the second for the 'Auxiliary' (UV) plane. The two INC files

will be created with NC 2 Axis Contour under the following guidlines.

____________________________________________________________________________

NEW! Ver 3.5 EPOST General 4 Axis Wire EDM Instructions

____________________________________________________________________________

1.) Create geometry for both 'program' plane and 'auxiliary' plane. The

contours should be oriented parallel to the XY plane. However, the Z

position of each contour is not important, since the post processor

ignores any Z information.

2.) Create a cross section surface with synchronized 'yes' to verify the

synchronization of the XY and UV contours. If SURFCAM does not prompt

for synchonized before creating a surface than break, trim, or

increment the geometry until the XY and UV contours have the same

number of elements.

3.) NC 2 Axis Contour the XY and UV contours with the infeed type set to

'With C-Comp' and the tool number must be greater than zero, even

though tool numbers are not required in the actual NC code. The INC

filename for the UV contour must be the same as the XY INC file with

an X appended to the filename.

EXAMPLE:  EDM.INC for the XY contour

EDMX.INC for the UV contour

NOTE: Both INC files must be the same size and the tool motion must

be in the exact same order. This may preclude the use of splines

within the contours because you may not be able to control the

number of linear moves required to cut the spline.

4.) To run EPOST from the DOS prompt type EPOST and the XY INC file.

EXAMPLE:  EPOST INC\EDM.INC

NOTE: EPOST will automatically merge the XY and UV INC files without

having to specify the UV filename. EPOST can be used inside

SURFCAM by updating the SURFCAM.PST file.

____________________________________________________________________________

NEW! Ver 3.5 POSTFORM.EUC Configuration

____________________________________________________________________________

Configuring EPOST is almost to MPOST with the following differences.

To indicate 4 Axis wire output specify the necessary UV word addresses

after the XY word addresses for HCode, VCode, CtrIncremental?

and CtrCodes parameters.

EXAMPLE:    HCode X U              ' Indicates U and V for auxilliary

VCode Y V              ' plane movement

CtrCodes I J K L       ' Indicates K and L for UV plane

' Circle center codes

CtrIncremental? Y N    ' Y Y causes K,L to be inc. from I,J

' Y N makes K,L inc. from arc start

on UV plane

The formats for the letters U, V, K, and L must be set in the letter

format section of the POSTFORM file.

NOTE: Programming mode must be absolute: no 'INCREMENTAL? Y' line is

allowed.

INFEED and OUTFEED sequences can only be 1 block long each.

NOTE: UV plane axis positioning values are now assumed to be absolute

values unless otherwise indicated in the letter format section

of the POSTFORM.EUC file. If you want the U and V values to be

incremental from the X and Y values on the same line, then format

the U and V letters as follows:

EXAMPLE:   U ->3.>4 IncFrom X

V ->3.>4 IncFrom Y

Arc center codes for UV, typically K and L, are assumed to be

incremental from the start of the UV plane arc. If need them to

be measured incrementally from the I, J values of XY plane then

add another 'Y' behind the 'CtrIncremental?' parameter.

EXAMPLE:   CtrIncremental? Y Y

____________________________________________________________________________

Ver 3.5 END of EPOST

____________________________________________________________________________