DSNFIX  :  utility to fix certain defects in DSN files

USAGE   :  dsnfix [-w] [-l#] file[.dsn] [file2[.dsn]] ...

-w   : Do not create new fixed "file.tmp"

-l#  : Mark data on layers specified as deleted

Wild cards are supported for file names.

DESCRIPTION :

DSNFIX attempts to fix certain problems with DSN files.  It does not

modify the original file but it creates a file with the same name as

the input file but with an extension '.tmp' that contains the fixed

data (unless the "-w" flag is used).  Although the file has an extension

'.tmp', it actually is a dsn file.  Usually after running DSNFIX the fixed

file is renamed and the extension changed to '.dsn'.

DSNFIX does not actually delete data in the '.tmp' file, it marks

the items as deleted. To delete the items from the file, rename the

'.tmp' file, read the renamed file in SURFCAM and save it back.  The

deleted items will then be removed from the file.

Following are the problems that DSNFIX tries to correct :

a)  An attempt is made to identify all data that could cause "Floating

point errors" while loading into SURFCAM.  These items are usually

characterized by very large numbers.  These items are marked

deleted and it will describe the type that it deleted.

b)  If for some reason there is incomplete data at the end of the file

DSNFIX will try to truncate it and mark the last item as illegal.

When the fixed file is loaded into SURFCAM, you may get a message

"Illegal type in file, file truncated" corresponding to this item.

The remainder of the file should not be affected by this.

c)  If the version number of the DSN file has been messed up (A message

"DSN file is not in correct format, run DSN2NEW" comes up when

you try to read the file in SURFCAM and you know it is not an old

SURFCAM file),  DSNFIX will prompt you if the version number be set

to the current version.  If you choose No, the dsn file will be

skipped.