Hewlett-Packard 27247B/27252A Ethertwist Packet Driver

The file HPPCLANP.COM Crynwr Packet Driver for the Hewlett Packard Ethertwist

27247B/27247A network adapter cards..

The following files are included in the DRIVERS9 directory:

HPPCLANP.COM  (the HP 27247B/27252A Packet Driver)

HP.DOC        (Notes on using the driver)

Configuration notes:

HPPCLANP.COM is version 10.2.2 of the HP 27247B/27247A Packet Driver

The software interrupt (0x61), and the I/O address will need to be in

the command line when running the packet driver.

usage: hppclanp [-n] [-d] [-w] <sw-int> <io_addr>

-d  Delay initialization for boot roms

-n for Novell pseudo 802.3 framing

-w for use with Microsoft windows 3.x

<sw_int>  Software interrupt 60-80 hex, example 0x61

<io_addr> I/O address

Example:     HPPCLANP -n -w 0x61 0x300

Where:

HPPCLANP:  is name of the packet driver

0x61:  is the software interrupt 61

0x300 is the I/O port