STANDARD MICROSYSTEMS CORPORATION

NDIS driver

For TokenCard EliteSeries LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1992 Standard Microsystems Corporation.

All rights reserved.

This installation guide provides the following instructions:

1.  Overview

2.  PROTOCOL.INI Parameters

1.  OVERVIEW

------------

The NDIS driver is designed to work with TokenCard EliteSeries LAN

Adapters. It conforms to the Network Driver Interface Specification(NDIS)

defined by Microsoft/3Com.

The contents of the \NDIS directory are described below:

\NDIS

INSTALL.DOC        Installation Guide (this File)

RELEASE.DOC        Version history and release information

SMC8100.OS2        OS/2 NDIS MAC driver

SMC8100.DOS        DOS NDIS MAC driver

PROTOCOL.INI       PROTOCOL.INI file sample

SMCDOSMC.NIF       NIF file for Micro Channel adapters

using DOS and IBM LAN Server

SMCOS2MC.NIF       NIF file for Micro Channel adapters

using OS/2 and IBM LAN Server

SMCDOSAT.NIF       NIF file for AT adapters using DOS

and IBM LAN Server

SMCOS2AT.NIF       NIF file for AT adapters using OS/2

and IBM LAN Server

\10NET

AUTOEXEC.BAT       Example AUTOEXEC.BAT for 10NET.

CONFIG.SYS         Example CONFIG.SYS for 10NET.

PROTOCOL.INI       Example PROTOCOL.INI for 10NET.

\DEC_PATH

INSTALL.DOC        Installation instructions for DEC PathWorks.

\FTP

CONFIG.SYS         Example CONFIG.SYS for FTP.

PROTOCOL.INI       Example PROTOCOL.INI for FTP.

\PCNFS

INSTALL.DOC        Installation instructions for PCNFS.

\NET_MAN

CONFIG.SYS         Example CONFIG.SYS for NET Manager.

PROTOCOL.INI       Example PROTOCOL.INI for NET Manager.

\LANSERVE

INSTALL.DOC        Installation instructions for IBM LAN Server.

\PCLAN

INSTALL.DOC        Installation instructions for IBM PC LAN

Program.

Install the adapter card in your PC using the instructions in the

Installation Guide included with the adapter.  Proper installation

can be verified by running the Diagnostic Program under DOS.

NOTE: The Diagnostic Program will not run properly from the OS/2

compatibility box.

2.  PROTOCOL.INI PARAMETERS

---------------------------

Parameter	   Description and Values

---------     ----------------------------------------------------

IRQ     Specifies the interrupt used by the adapter.  The

default is 3.  IRQ3 is used by COM2. If COM2 is in

use, choose another IRQ value.  The interrupt is

selected via a jumper on some adapters, or by way of

the EZSETUP program supplied with other adapters.

Consult the installation guide for information on how

to determine or modify the interrupt. This parameter

is always a decimal number in the range of 2 - 15.

Not all values are supported by all adapters.

IOBase     Specifies the base I/O address used by the adapter.

The default is 0x280.  The address is selected via a

jumper on some adapters, or by way of the EZSETUP

program supplied with other adapters. Consult the

Installation Guide for information on how to determine

or modify the address.  This parameter is a hexadecimal

number in the range of 0x200 - 0xFFFF. Hexadecimal

numbers must be preceded by '0x' to distinguish them

from decimal numbers.  Remember that not all values

are supported by all adapters.

RAMAddress     Specifies the Shared RAM segment address used by the

adapter.  This value is established under software

control by the driver.  The default Shared RAM segment

address is 0xD000.  This address is recommended unless

it conflicts with some other device in the PC.  Consult

the adapter Installation Guide or print the file

SYSMAP.DOC on the SuperDisk for more information on how

to select a shared RAM address.  The driver uses

hexadecimal values in the range of 0xA000 - 0xFFFF.

SlotNumber     Specifies the number of the slot containing the adapter

card in a Micro Channel machine.  This parameter is a

decimal number in the range of 1 - 10.

NetAddress	   Specifies a local LAN node address. Enter as a string of

characters enclosed in double quotation marks, for example:

"0000C0010203"

Do not use spaces. Any hexadecimal digit is acceptable.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.