Appendix A

VCR MCI Command-String Syntax

VCR MCI Command-String Syntax

VCR devices support the following set of commands. VCR devices also support notify, test, and wait as optional flags. You can add one, two, or all of these flags to any VCR MCI command.

The following conventions are used in this appendix:

| Type style | Used for |
| --- | --- |
| Bold | A specific term intended to be used literally. When used in the command column, it represents the MCI string command. When used in the arguments column, it represents a flag. For example, the capability command and the can play flag must be typed as show. |
| Italic | Place holders for information you must provide. The MCI command or flag associated with the information must precede it. For example, to use the alias words for the device_name with the capability command, type “capability words can play.” |
| ¦ | Divider for mutually exclusive arguments. When multiple arguments are separated by this symbol, only one of them can be used for each command. For example, the items in the list are mutually exclusive and you must select only one to use with the capability command. (Do not type the ¦ with the argument.) |
| { } | Required argument. You must include an argument enclosed by braces. For example, you must use one of the arguments can eject, can play, can record, can save, compound device, device type, has audio, has video, uses files with the capability command. (Do not type the braces with the argument.) |
| [ ] | Optional argument. For example, the alias, type, and shareable flags are optional for the open command. You can use any combination of these flags. You can also use optional arguments with a required argument. (Do not type the brackets with the argument.) |

| Command | Arguments |
| --- | --- |
| capability item | {can detect length |
|  | ¦ can eject |
|  | ¦ can freeze |
|  | ¦ can monitor sources |
|  | ¦ can play |
|  | ¦ can preroll |
|  | ¦ can preview |
|  | ¦ can record |
|  | ¦ can reverse |
|  | ¦ can save |
|  | ¦ can test |
|  | ¦ compound device |
|  | ¦ device type |
|  | ¦ has audio |
|  | ¦ has clock |
|  | ¦ has timecode |
|  | ¦ has video |
|  | ¦ number of marks |
|  | ¦ seek accuracy |
|  | ¦ uses files} |
| close |  |
| cue item | {input [from postion ] [to position] [preroll] ¦ output} [from postion ] [to position] [preroll] [reverse] |
| escape items |  |
| freeze item | [input ¦ output] [field ¦ frame] |
| index item | [on ¦ off] |
| info item | {product ¦ version} |
| list item | {audio source {count ¦ number index} ¦ video source {count ¦ number index}} |
| mark item | {write ¦ erase} |
| open items | [alias alias] |
|  | [shareable] |
|  | [type device_type] |
| pause |  |
| play items | [from pos] [to pos ¦ reverse] [at time] [scan] |
| record items | [initialize ¦ preview ¦ [from pos] [to pos]] [at time] |
| resume |  |
| seek items | {to pos [at time] ¦ { to end ¦ to start ¦ mark mark_num} [reverse] [at time]} |
| set items | {time mode {detect ¦ timecode ¦ counter}} |
|  | ¦ counter {format ¦ value} |
|  | ¦ [time format {milliseconds ¦ ms ¦ frames ¦ smpte 24 ¦ smpte 25 ¦ smpte 30 ¦ smpte 30 drop ¦ hms ¦ tmsf ¦ msf} |
|  | ¦ record format {SP ¦ LP ¦ EP} |
|  | ¦ speed factor |
|  | ¦ length duration |
|  | ¦ index {timecode ¦ counter ¦ date ¦ time} |
|  | ¦ power {on ¦ off} |
|  | ¦ assemble record {on ¦ off} |
|  | ¦ preroll duration duration |
|  | ¦ postroll duration duration |
|  | ¦ pause timeout timeout |
|  | ¦ tracking {plus ¦ minus ¦ reset}} |
| setaudio items | {source to {tuner ¦ line ¦ svideo ¦ aux ¦ generic ¦ mute} number number |
|  | ¦ record [track track_number] {on ¦ off} |
|  | ¦ monitor to {tuner ¦ line ¦ svideo ¦ aux ¦ generic ¦ mute ¦ output} number number} |
| settimecode items | record {on ¦ off}} |
| settuner items | {number number} |
|  | ¦ channel {channel ¦ up ¦ down ¦ seek up ¦ seek down}} |
| setvideo items | {source to {tuner ¦ line ¦ svideo ¦ aux ¦ generic ¦ mute ¦ rgb} number number |
|  | ¦ record [track track_number] {on ¦ off} |
|  | ¦ monitor to {tuner ¦ line ¦ svideo ¦ aux ¦ generic ¦ mute ¦ output ¦ rgb} number number |
| signal items | {{{[at position] [every interval]} [return position] } ¦ cancel [id] [uservalue id]} |
| status items | { assemble record |
|  | ¦ audio {record [track track_number] ¦ source [number] ¦ monitor [number]} |
|  | ¦ channel [tuner number number] |
|  | ¦ clock {[id] ¦ [increment rate]} |
|  | ¦ counter |
|  | ¦ counter {format ¦ resolution} |
|  | ¦ frame rate |
|  | ¦ index ¦ index on |
|  | ¦ length [track trackno] |
|  | ¦ media type |
|  | ¦ mode |
|  | ¦ number of {audio ¦ video} tracks |
|  | ¦ pause timeout |
|  | ¦ play format |
|  | ¦ position {[track trackno] ¦ start} |
|  | ¦ postroll duration |
|  | ¦ power on |
|  | ¦ preroll duration |
|  | ¦ ready |
|  | ¦ record format |
|  | ¦ speed |
|  | ¦ time {format ¦ mode ¦ type} |
|  | ¦ timecode {present ¦ record} |
|  | ¦ video {record [track track_number] ¦ source [number] ¦ monitor [number]} |
|  | ¦ write protected |
| step items | [by frames] [reverse] |
| stop item |  |
| unfreeze item | [input ¦ output] |