Image Color Matching

in

Win32

A

Proposal

ver 0.23

July 30, 2026

MICROSOFT CONFIDENTIAL

µ

Prolog	4

Goals	4

We want color WYSIWYG.	4

Windows should be the best platform for DTP and presentations.	4

Requirements to meet the goals.	5

Windows 3.1 Color Architecture	5

Logical Color Space	5

Calibrated RGB	6

Device Color Coordinates	6

CMYK space	6

Image Color Matching Support	6

Color Communication	6

Color Communication Model	6

Color Communication Architecture	8

Usability	9

Implementation details	10

Application APIs	10

ICM control	11

Device Characterization Utilities	11

Writing an Image Color Matcher	11

Color Profiles	12

API, flags, #defines, and structures	12

Flags	12

#define's	12

Structures	13

Application APIs	14

EnableICM	16

CreateColorSpace	16

SetColorSpace	17

GetColorSpace	17

GetLogColorSpace	18

DeleteLogColorSpace	18

GetColorProfile	19

SetColorProfile	19

EnumColorProfiles	20

EnumColorProfilesProc	20

CheckColorsInGamut	21

ColorMatchToTarget	22

LoadImageColorMatcher	23

FreeImageColorMatcher	23

GetDeviceGammaRamp	24

SetDeviceGammaRamp	24

API for Color Matching DLL	25

CMGetInfo	25

CMCreateTransform	26

CMDeleteTransform	27

CMTranslateRGB	28

CMTranslateRGBs	29

CMCheckColorsInGamut	30

CMEnumColorProfiles	31

EnumColorProfilesProc	31

Entry points, flags, and structures added to the DDI for device independent color.	32

Structures	32

DDIGammaRamp	34

Metafiles and device independent color	35

DIBS and device independent color !! not quite right yet !!	35

# Prolog

This memo attempts to answer the question, "What should we do about device independent color in Chicago?" It starts off with a quick discussion of the problem, goes through a suggested color architecture and winds up with a proposed color API. This API covers different sets of functionality: image color matching API's for applications, API's for installable color matchers, and API's for color utility programs. This memo assumes familiarity with the GDI model of DC's and objects. The set of functionality proposed here may be expanded or contracted, depending on feedback and resources.

# Goals

## We want color WYSIWYG.

WYSIWYG is an important goal of the graphics layer in Windows. Traditionally WYSIWYG is achieved when objects on the screen end up 'in the same place' on the printer and spatial relationships between objects are preserved. In order to format a printed document it is important that the display accurately represent the printed page; otherwise a tedious and error prone process is required of tweaking the display image and printing out test pages. The more exact the match the happier our customers are. However in the case of color Windows is not WYSIWYG. When users scan in images the colors on the screen do not match the colors on the printer, and neither is necessary a good match to the original.

## Windows should be the best platform for DTP and presentations.

But color WYSIWIG is not sufficient for Windows users in the same way that spatial WYSIWIG is not sufficient. WYSIWIG means that the image on the screen matches the image on the printer, but it does not mean that the same image will be produced on different printers, say with different pixels per inch. A document is not allowed to reflow just because the user changed monitors! Objects are not allowed relative motion and colors are not allowed to change. For example, if multimedia shopping catalogs are to be a success then advertisers will need assurance that the ads they design will be accurately reproduced on the screen. We can also expect graphic designers to work cooperatively on a project, this implies that the displayed colors must be the same across all their machines. In order for colors to be constant across all devices means that there must be some device independent way of specifying color. In the Windows nomenclature what are needed are 'logical colors'. A logical color has an objective meaning independent of any output device. There also must be plumbing in the system to get logical colors communicated between devices; input devices, such as scanners, included.  These output devices must then be able to accurately realize the logical color asked for, perhaps with system level support. These three items, logical colors, plumbing, image color matching support, must be satisfied to get device independent color.

Now an important point must be made. The perception of color involves a viewer with a very complicated neuron based information filter. For example the white on the screen is generally bluish, but the wonderful filtering going on in our brain interprets it as white. Having the color on the printed page match spectrally would be the wrong thing to do, compared to the white of the blank paper the printed screen white would appear blue. Color matching is not just mathematical transformations blindly applied. A better term than color matching is 'image color matching' (ICM) which makes explicit that it is the image as a whole, and not pixels, that need matching. Defining and passing around a standard color space, and reproducing those colors as accurately as possible is not sufficient for device independent color. The human perception of color is weird, not intuitive, and hard. And it only gets harder, different devices have areas of non-overlapping color. For example the screen can display cyans that no printer will be able to produce, likewise there are reds on a page you will never get on a display. The range of colors a specific device can produce is called its gamut. What do you do when you can't even reproduce the color asked for? What does the system do when the image you designed on the screen contains many colors that the printer cannot accurately reproduce? You do gamut matching. This can range from the simple (such as truncating to the surface of the printer gamut), to the sophisticated (such as moving all colors a bit to maintain contrasts). Accurate color reproduction is an active research topic, and there are many different, sophisticated, evolving methods. Furthermore the techniques used are usually closely tied to specific printer technology. There are companies that exist solely to develop and sell 'color management systems'. (It is interesting to note that these systems are as much about specific device dependencies as they are about device independent color. All of them are designed around the model that the user is composing for a specific output device, not an abstract output device. For example, for screen representations of the printed image they allow visualization of the colors that the printer can't do, generally by displaying them as black.)

## Requirements to meet the goals.

To summarize and repeat, there are three minimum requirements. 1) Definition of a logical color space. 2) Image color matching support, by device drivers or the system. 3) Application control of color communication to the output device.

# Windows 3.1 Color Architecture

Windows 3.1 doesn't have one, or rather it has a minimalist one. Applications specify the color of objects (pens, brushes, text, and pixels), through a 24 bit triplet RGB. This triplet is passed as the lower 24 bits of a 32 bit quantity. This 32 bit quantity is called a ColorRef, the high byte can be flags or other stuff. GDI does not look at or process RGB triplets in any way. It merely passes them on through, from the application to the device. See diagram below. There are 8 bits per channel, where the 'R' represents red, the 'G' green, and the 'B' blue. There are no exact definitions for red, green, and blue. The color that gets produced is totally under the whim of the device. The red of the display does not necessarily (and almost never does) look like the red of an inkjet printer, the red of a thermal wax printer, or the red of a film recorder; of course, these devices do not match among themselves. Not only are the endpoints of the RGB channels not defined, the scale of 0 to 255 is not defined either. A value of 127 does not mean that color has half the intensity of the 255 color. Some devices interpret it that way, some don't. The interpretation of a linear ramp in perceived intensity is known as gamma correction. Some Windows display drivers do gamma correction, some don't; this means that just by changing drivers the images on a user's display can change.

µ §

# Logical Color Space

All standard color spaces are based on the 1931 CIEXYZ standard. Various permutations are used for various reasons. Calibrated RGB is popular because displays work that way. L*a*b* is technically cool because it models the human visual system better and fits into 8 bits without producing artifacts. HSV is used in UI because it is somewhat intuitive. But again, all of these spaces are mathematically equivalent. For simplicity Windows will allow only device color coordinates and calibrated RGB across the API. Applications will be able to set their color space on a per DC basis.

## Calibrated RGB

The logical color space will be calibrated RGB. The RGB endpoints will be defined by CIEXYZ triplets. (Or equivalently by chromaticity doublets and the white point.) The triplets will be 3 32 bit words. A gamma will set the response curves of each of the primaries. Obviously if an application just wanted to work in CIEXYZ space it could set the endpoints of its RGB to match. There will be a stock color space object that calibration utilities will be able to set. If this file is not present then GDI will default to some popular monitor, like the NEC 4G.

## Device Color Coordinates

At various times applications may wish to work in the device color space of an arbitrary output device. In this case applications can set the characterization of the device as their color space.

## CMYK space

Many ISV's have asked for the capability of using CMYK instead of RGB as the color indicator. These applications do their own color separation and wish to pass device color coordinates to CMYK printers. We will define a color space and new ColorRef for device CMYK. These values will always be sent to the device driver without color transformations.

## Image Color Matching Support

If we wish to claim to device independent color in Chicago we must have some basic level of support for color and gamut matching. Because image color matching is weird and evolving, and there are many ISV's with expertise, we should simply license code from a third party. We will attempt to ship characterization information for the best selling printers. For less popular printers we'll see what happens. It is also expected that IHV's that presently have color expertise, such as HP and Apple, will put image color matching support into their drivers. Note though, that drivers should not have to be updated in order to get ICM. As long as there is a calibration file for the device the system will do the right thing.

# Color Communication

## Color Communication Model

Before diving into implementation details, and constructing API's, we build a conceptual model of color communication flow that builds a simple framework in which to place these different topics.

So far we have concentrated on physical output devices and the colors that the physical phosphors and inks limit them to. But applications can also write to bitmaps and DIBs. Anything that an application can perform graphics operations on we define to be a surface. This includes metafiles, bitmaps, and DIBS in addition to printers, plotters, displays, film recorders, and other output devices. A surface has immutable properties. Some of the currently defined properties are size, color capabilities, resolution, dot size, and palette capabilities. To this list we add color characterization. The color characterization of a surface is the mapping from CIEXYZ space to the color coordinates of the surface. For physical output devices the meaning is clear, the color coordinates are the amounts of R or B or G, or C or Y or M or K. But what about bitmaps, DIBs, metafiles, and somesuch?

Note that I have distinguished bitmaps from DIBs, and the distinguishing factor is that bitmaps are device specific and DIBs are, well, Device Independent Bitmaps. To make the distinction clearer I will call bitmaps by their proper name, compatible bitmaps; they are created by CreateCompatibleBitmap(). CreateCompatibleBitmap() is a call to the driver of a physical device to create an offscreen bitmap. This bitmap can be in offscreen device memory or system memory. It is inaccurate to call a compatible bitmap a 'memory bitmap' even though it may reside in memory; the fact that it is in memory is irrelevant and implies things it shouldn't. A compatible bitmap is an extension of a physical device - except for its size it has the same properties as the device. The architecture of the palette manager in Windows follows this model. There is no color table tied to the bits in a compatible bitmap because the bits can only be interpreted against the device hardware palette. Specifically it has the same immutable color characterization. This is why in the diagram below I have drawn compatible bitmaps in the same box as the originating device.

DIBS are a surface. Currently they have no color characterization associated with them. Device independence means that on all devices the output must be the same, including the colors. To meet this goal we will add to the DIB header color characterization. Unlike other surfaces however the characterization can be under application control.

Metafiles are similar to DIBs in terms of device independence. DIBs are composed of bits, metafiles are composed of drawing commands. We need to add a record to metafiles that state what the wanted color characterization is.

On the application side is the definition of a color space. This is on a per DC basis, every DC will have associated with it an application definable color space. The definition of a color space communicates two items. The first is the form of the color information passed in the DWORD that traditionally has held an RGB triplet. We will allow for device CMYK to be used instead of RGB's. The second item is how they wish the RGB triplets to be interpreted in terms of CIEXYZ. Or to be redundant, the color space is a mapping from RGB to CIEXYZ.

The diagram on the next page shows the framework. The color converters in the middle do not necessarily represent separate components. One or both could be in GDI or in the device driver. The next section discusses this.

µ §

## Color Communication Architecture

The model above stressed the difference between the color space applications work in and the color characterization of the device. But now we are forced to implementation details and the question becomes, 'Where do we put the DDI line in the color model? Do we pass across the DDI 1) device color coordinates, 2) CIEXYZ, 3) the application defined color space, or 4) some combination of the above?' For compatibility, application control, and having GDI perform image color matching we must pass across, at a minimum, device color coordinates. Since CIEXYZ and the application's color space are related, it seems redundant to have the choice to pass either. In order to keep mathematical operations, which could lead to rounding error accumulations, to a minimum CIEXYZ will not be passed through. The applications color space coordinates will be passed through. Please see the diagram below.

µ §

Basically the change from the model diagram is a DLL that transforms from the logical color space to device color coordinates in place of the two operations with CIEXYZ as an intermediary.

In order for Windows to do image color matching output devices must ship with characterization files that map between device color coordinates and a logical color space. If the device driver wishes to do its own matching that is allowed. Windows will check to see if the driver has implemented the Image Color Matcher API, if it does then GDI will pass through the ColorRef unaltered.

Some 24 (and higher) bit display adaptors support downloadable gamma correction tables. It is assumed that any utilities that set these tables will also update the profile for the screen.

# Usability

Let us focus on the user for a moment. What does the user want? How does the user use and interact with ICM?  Should the screen's colors be manipulated to match the printer's, or should the printer's colors be manipulated to match the screen, or should both be manipulated to match the logical color space? Should this be under user control? I claim that different users will want different approaches. Transforming from the logical color space to the device color space is not necessary fast, taking the speed hit when going to the screen may not be acceptable. In addition, current DIBs are not defined in a logical color space, they are generally defined against the current display. So it is reasonable to expect that some users will want the device colors to match the logical colors and some won't.

The default setting of the matching choices for the screen and printer will be off? There will be a property sheet in the printer setup dialog to enable ICM if a profile is present for the given configuration. This will allow ICM to work without requiring updated drivers. However, it's still going to take time, education, and experience for users and ISV's to figure out where ICM makes sense for them, and when it should and shouldn't be turned on.

Windows applications historically compose for the specific target printer; basically because fonts were device specific, or to take advantage of other specific features, or to get the system out of their way so they can control everything. ICM can get in the way. As a simple example imagine the most natural use of color in bar charts in presentation packages. Users like these to be saturated, undithered colors. In essence they want to use the device colors in specific ways. They don't want the device to dither together lots of primaries and produce ugly patterns, they don't want ICM to get in the way. Please note that this choice is not on a per application basis, or even a per DC basis, a WinWord document might want ICM for bitmap images and device colors for charts and highlights. Windows must have a way of accommodating for this.

It must also be stressed again that we can only approximate uncontrovertable ICM. Each output technology has its own advantages, disadvantages, and color characteristics and gamuts. No image will appear exactly the same across different technologies. The same image will not necessarily appear the same on a particular technology if different image color matchers are used. This could be a problem if cross platform portability was wanted, for example between the Mac and Windows to the same network printer. We cannot solve this problem; it is the usual dichotomy between a device independent representation, and the fact that printers do not have infinite spatial or color resolution. Approximations must be made to the ideal, and different printers require different compromises.

# Implementation details

The concept of ICM is very simple, that images, composed of pixels whose color is communicated through RGB triplets, should appear the same regardless of the target device. It should look the same on the screen as it does on ink-jet as it does on thermal wax, as it does on ink sublimation, as it does on film.

The device independent API's can be broken down into three areas, application support, third party ICMDLLs, and color utilities to create device characterizations.

## Application APIs

An application does not have to do much in order to put device independent color into motion. It only has to use one API, EnableICM() to turn image color matching on and off to a device. If a profile does not exist for the configuration of the device then EnableICM will fail. Windows will keep track of the characterizations of all devices, use a default gamut matching method, and choose the monitor as the default color space. This allows the screen output to be fast and the printer to match the screen.

If an application wishes to work in a color space different from the default, then it can change the color space through the APIs CreateColorSpace() and SetColorSpace(). CreateColorSpace takes a pointer to a logical color space structure. It then sets the color space into a DC. When the application is done with the color space it destroys it with DeleteColorSpace(). To retrieve the color space selected into a DC GetColorSpace() is used. To get the logical definition or characterization file, use GetLogColorSpace(). This is very similar to the object manipulation of SelectObject, DeleteObject, and GetObject for other GDI objects. For validation reasons separate APIs have replaced the object calls. One of the advantages of objects, knowing their lifetime, is important when caching color transforms.

There are three standard gamut matching methods supported in profiles: maintaining saturation, maintaining contrast, and colormetric. For business charts and a lot of computer generated presentations saturation should be preserved when the requested color is out of gamut. Colormetric matching is important when named colors, such as Pantone®, are wanted. Maintaining contrast allows for the best perceptual match between devices, the entire gamuts are smershed around so that the destination white point is used and there are a minimum of many to one color mappings. This is best for photographic images.

To determine if colors are within the gamut of a device, CheckColorsInGamut() is the API of choice. For example a user could be creating an image on the screen, which allows for a larger gamut than printers. Before the user prints the image the application can provide a service that shows what colors will not reproduce exactly when printed. The user can then select other colors that will print.

To do color preview of, say, a print job on the screen, ColorMatchToTarget() is provided. A DC for the target device is used in an intermediate step in the color transformation pipeline. First the image is transformed into the target's color space, with appropriate gamut matching. Then that output is transformed, and displayed, on the preview device. This call cannot be nested.

Although GDI will attempt to manage profiles on behalf of the application, there are limitations. For example GDI will not be able to track all device configurations that might affect characterization. For this reason applications will be able to enumerate all profiles that could be used on a device with a given subsetted configuration with EnumColorProfiles(). This enumerates profiles by attributes and filename of profile with those attributes. EnumColorProfilesProc() is the associated callback routine. GetColorProfile() returns the filename of the current profile GDI is using for image color matching for a given DC. SetColorProfile() allows an application to use a chosen profile.

## ICM control

If an application wishes to use or install a third party image color matcher it is rather simple. As in the case of the default matcher with GDI it is up to the ICMDLL to manage characterization profiles. To load an ICMDLL LoadImageColorMatcher() is used. It takes a pointer to a DLL to load. It returns a handle to the loaded ICMDLL. To unload a loaded ICMDLL FreeImageColorMatcher() is used, with the handle to the ICMDLL as the argument. To select the loaded ICMDLL to do the color transformations a profile specific to that ICMDLL must be used. Internally EnumColorProfiles() calls to all ICMDLLs installed in the system to allow them to enumerate their profiles.

## Device Characterization Utilities

Due to drifts in devices, changing viewing conditions, etc., the characterization of devices changes. It is expected that third parties will supply tools to create and update profiles. Profiles will be kept in a standard windows directory (TBD). Calibration utilities need only copy new profiles to the directory for GDI to find them. To change a profile the current profile should be deleted and then the new one copied over.

Some 24 (and higher) bit display adaptors allow for the downloading of a gamma ramp for hardware gamma correction. MS should actively encourage ALL display adaptor vendors to support this functionality. We have added a new set of APIs to support this, GetDisplayGammaRamp() and SetDisplayGammaRamp(). The hardware gamma ramp is a global property, it affects all running applications, it is not changed on a per DC basis.

## Writing an Image Color Matcher

An installable ICMDLL is a 32-bit Windows DLL. Under Chicago it will run as flat 32-bit code. It must be re-entrant. It is assumed the reader knows how to write a DDL and their behavior.

The set of functionality that GDI depends on is small. Basically the ICMDLL constructs transforms and GDI calls to the ICMDLL to apply those transforms. GDI creates and destroys new transforms when the device characterization changes or the application color space changes. The lifetime of a transform is controlled by the application, an application can create a color space and keep it around for its lifetime. Or it can create a color space just when proofing needs to be done.

The ICMDLL does all of its own memory management, GDI does no allocations on behalf of the ICM.

The ICMDLL must do its own housekeeping with respect to any private characterization files it may have. Either half of the transforms may be the expanded DevMode structure or a pointer to a device characterization file. The DevMode has the description of the device state such as device name, paper, inks, halftoning method, etc. It is up to the ICMDLL to find its corresponding private characterization file.

Device drivers that wish to implement device independent color support just have to support the same color DDI above as installable ICMDLL's. When GDI loads the device driver it will check for the presence of these DDI's. If they are available GDI will use them for the default, they will only be overridden if the application changes the color profile to an ICMDLL.

## Color Profiles

Color profiles are refered to as device characterizations, and also as characterization tables, throughout this document. The exact format is still under consideration and development. It must however be 'attribute based' (also known as tagged based), so that descriptions of the device setup and device environment can be added without a revision of Windows. There must be an attribute that defines what country the text strings are for, the value of the attribute must be the Windows LCID for that country.

# API, flags, #defines, and structures

### Flags

//  values for CMGetInfo()

typedef UINT	INFOFLAGS;

#define CMS_GET_VERSION		0x00000000

#define CMS_GET_IDENT		0x00000001

// flags for CMTranslateRGB(s)

typedef UINT	DIRECTIONFLAG;

#define CMS_FORWARD		0x00000000

#define CMS_BACKWARD		0x00000001

typedef UINT	TRANSLATEFLAGS;

#define CMS_x555WORD		0x00000000

#define CMS_565WORD			0x00000001

#define CMS_RGBTRIPLETS		0x00000002

#define CMS_BGRTRIPLETS		0x00000004

#define CMS_XRGBQUADS		0x00000008

#define CMS_XBGRQUADS		0x00000010

#define CMS_QUADS			0x00000020

// flags for LogColorSpace

typedef UINT	LCSIDENT

#define LCS_DEVICE_RGB			0x00000000

#define LCS_DEVICE_CMYK		0x00000001

#define LCS_CALIBRATED_RGB	0x00000002

typedef UINT	LCSGAMUTMATCH

#define LCS_GM_BUSINESS		0x00000001

#define LCS_GM_GRAPHICS		0x00000002

#define LCS_GM_IMAGES		0x00000004

### #define's

typedef HANDLE	LCOLORSPACE;

typedef		long	FXPT16DOT16, FAR* LPFXPT16DOT16;		// Fixed point 16.16

typedef		long	FXPT2DOT30, FAR* LPFXPT2DOT30;		// Fixed point 2.30

### Structures

Added color definitions:

typedef struct tagCIEXYZ

{

FXPT2DOT30		ciexyzX;

FXPT2DOT30		ciexyzY;

FXPT2DOT30		ciexyzZ;

} CIEXYZ;

typedef CIEXYZ FAR* LPCIEXYZ;

typedef struct tagCIEXYZTRIPLE

{

CIEXYZ	ciexyzRed;

CIEXYZ	ciexyzGreen;

CIEXYZ	ciexyzBlue;

} CIEXYZ;

typedef CIEXYZTRIPLE FAR* LPCIEXYZTRIPLET;

LOGCOLORSPACE

Logical Color Space Information

#define DAVIDS_CONSTANT	256

The LOGCOLORSPACE data structure defines a logical color space. FOR M5 ONLY!

typedef struct tagLOGCOLORSPACE

{

UINT			lcsVersion;

UINT			lcsSize;

LCSIDENT		lcsIdent;

LCSGAMUTMATCH 	lcsGamutMatch;

CIEXYZTRIPLE	lcsEndpoints;

FXPT16DOT16	lcsGammaRed;

FXPT16DOT16	lcsGammaGreen;

FXPT16DOT16	lcsGammaBlue;

char			lcsFilename[DAVIDS_CONSTANT];

}

typedef LOGCOLORSPACE FAR* LPLOGCOLORSPACE;

The LOGCOLORSPACE structure has the following fields:

| Field | Description |
| --- | --- |
| lcsVersion | Specifies the Windows version number for the structure (currently 0x400). |
| lcsSize | Specifies the size of this structure, which is version specific. |
| lcsIdent | Specifies the type of color space to set. It can have the following meanings: |

|  | Constant | Meaning |
| --- | --- | --- |
|  | LCS_DEVICE_RGB | RGBQuads defined in API's are to be interpreted as device RGB's. In effect this turns color matching off. |
|  | LCS_DEVICE_CMYK | RGBQuads defined in API's are to be interpreted as device CMYK. No translation of these values are allowed. The remainder of the structure is ignored in this case. |
|  | LCS_CALIBRATED_RGB | The given CIEXYZTriplet defines the endpoints of the RGB's used in all API's. |

| lcsGamutMatch | Specifies the gamut matching method wanted. It can have the following meanings: |
| --- | --- |

|  | Constant | Meaning |
| --- | --- | --- |
|  | LCS_GM_BUSINESS | Maintain saturation. Used for business charts and the like. |
|  | LCS_GM_GRAPHICS | Colormetric match. Used for graphic designs and named colors. |
|  | LCS_GM_IMAGES | Maintain contrast. Used for photographs and natural images. |

| lcsEndpoints | Definition of the RGB endpoints in CIEXYZ space. |
| --- | --- |
| lcsGammaRed | The scale of the red coordinate. It is fixed point, 16.16. |
| lcsGammaGreen | The scale of the green coordinate. It is fixed point, 16.16. |
| lcsGammaBlue | The scale of the blue coordinate. It is fixed point, 16.16. |
| lcsFilename | NOT REQUIRED. Under certain limited circumstances an application may wish to pass in a device profile for the source color space. For example the scanner used might only output device CMYK's for a specific printer. Instead of forcing an application to transform to calibrated RGB and then to another device space, which leads to artifacts because 8 bits just can't stand the strain, the characterization of the scanner can be passed in. Or a third party color matcher might require a profile. The rest of the structure should be filled in if at all possible, although it may not be 100% accurate. |

## Application APIs

The functions added are:

EnableICM			allows app to force color matching

CreateColorSpace		creates a logical color space

DeleteColorSpace		deletes a color space created with CreateColorSpace

SetColorSpace			defines the endpoints of the logical RGB space in the CIEXYZ space

GetColorSpace			gets the handle of the color space currently set into a DC

GetLogColorSpace		gets the LogColorSpace associated with the handle to that color space

CheckColorsInGamut		determines if the specified colors are in the gamut of the device

ColorMatchToTarget		does preview matching

GetColorProfile			retrieves the current color profile for the given DC

SetColorProfile			sets the color profile for the given DC

EnumColorProfiles		enumerates the different color profiles available on the given DC

EnumColorProfileProc		callback for above

LoadImageColorMatcher	loads an installable color matcher

FreeImageColorMatcher	unloads an installable color matcher

GetDisplayGammaRamp	retrieves the gamma ramp for 24 (and higher) bit displays

SetDisplayGammaRamp		sets the gamma ramp for 24 (and higher) bit displays

### EnableICM

BOOL EnableICM(hDC, fEnableICM)

HDC hdc;			/* handle of device context		*/

BOOL fEnableICM;		/* turns image color matching on and off	*/

The EnableICM function causes image color matching to be performed on the given DC.

| Parameter | Description |
| --- | --- |
| hdc | Identifies a device context that contains the logical palette that was used to initialize the bitmap's colors. |
| fEnableICM | Turns image color matching on and off. TRUE turns it on, FALSE turns it off. |

Returns

The return value is true if the function is successful. Otherwise, it is NULL.

### CreateColorSpace

HCOLORSPACE CreateColorSpace(lpLogColorSpace)

LPLOGCOLORSPACE lpLogColorSpace;	/* pointer to LOGCOLORSPACE data structure */

The CreateColorSpace function creates a logical color space.

| Parameter | Description |
| --- | --- |
| lpLogColorSpace | Points to the LOGCOLORSPACE data structure. |

Returns

If the function is successful, the return value is a handle that identifies a logical color space. Otherwise, it is NULL.

### SetColorSpace

BOOL SetColorSpace(hdc, hColorSpace)

HDC hdc;				/* handle of device context	*/

HCOLORSPACE hColorSpace		/* color space to set		*/

The SetColorSpace defines the endpoints of the logical RGB space in CIEXYZ space.

| Parameter | Description |
| --- | --- |
| hdc | Identifies the device context. |
| hColorSpace | Handle to the logical color space to set. |

Returns

The return value is the handle of the hColorSpace being replaced if successful. Otherwise, it is NULL.

### GetColorSpace

HLOGCOLORSPACE GetColorSpace(hdc)

HDC hdc;				/* handle of device context	*/

The GetColorSpace retrieves the current hLogColorSpace from the given device context.

| Parameter | Description |
| --- | --- |
| hdc | Identifies a device context that is to have its hLogColorSpace retrieved. |

Returns

The return value is the current hLogColorSpace if the function is successful. Otherwise, it is NULL.

### GetLogColorSpace

BOOL GetLogColorSpace(hColorSpace, lpBuffer, nSize)

HLOGCOLORSPACE hLogColorSpace;	/* handle of logical color space		*/

LPVOID lpbuffer;			/* pointer to put logical color space		*/

UINT nSize;				/* size of buffer				*/

The GetLogColorSpace retrieves the definition of the given handle to a logical color space.

| Parameter | Description |
| --- | --- |
| hColorSpace | Handle to LOGCOLORSPACE to retrieve. |
| lpBuffer | Points to buffer that receives the LOGCOLORSPACE. |
| nSize | Specifies the maximum size of the buffer. |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

Comments

This function will copy as much of the LOGCOLORSPACE to the buffer as space allows. To determine the correct size of the buffer to use, applications should check the lcsSize value in the structure.

### DeleteLogColorSpace

BOOL DeleteLogColorSpace(hColorSpace)

HCOLORSPACE hColorSpace		/* color space to delete	*/

The DeleteColorSpace removes and destroys the given logical color space.

| Parameter | Description |
| --- | --- |
| hColorSpace | Handle to the logical color space to delete. |

Returns

The return value is TRUE if successful. Otherwise, it is NULL.

### GetColorProfile

LPCSTR GetColorProfile(hdc)

HDC hdc;			/* handle of device context	*/

The GetColorProfile function retrieves the current color profile for the specified DC.

| Parameter | Description |
| --- | --- |
| hdc | Identifies the device context to retrieve the color profile from. |

Returns

The return value is NULL if the function fails. Otherwise it is a pointer to a fully qualified pathname to the color profile file in use for this device. Image Color Matching must be enabled for the DC in order for this function to succeed.

### SetColorProfile

BOOL SetColorProfile(hdc,lpFileName)

HDC hdc;			/* handle of device context	*/

LPCSTR lpFileName		/* pointer to fully qualified path of profile to use		*/

The SetColorProfile function sets the color profile for the specified DC.

Returns

The return value is TRUE, if successful. Otherwise, it is NULL.

### EnumColorProfiles

int EnumColorProfiles(hdc, lpEnumColorProfilesFunc, lParam)

HDC hdc;						/* device-context handle		*/

ENUMGAMUTMATCHPROC lpEnumGamutMatchFunc;	/* address of callback function	*/

LPARAM lParam;					/* application supplied data	*/

The EnumColorProfiles function enumerates the different color profiles that the system supports for the given device context.

| Parameter | Description |
| --- | --- |
| hdc | Identifies the device context. |
| lpColorProfilesFunc | Specifies the procedure instance address of the application defined callback function. See the description of the EnumColorProfilesProc function. |
| lParam | Application supplied data. The data is passed to the callback function along with the color profile information. |

Returns

A return value of zero specifies the application interrupted the enumeration; -1 specifies that there are no color profiles to enumerate, or that the system is not enabled for ICM. Otherwise the return is the last value returned by the callback function.

### EnumColorProfilesProc

int CALLBACK EnumColorProfilesProc(lpAttributes, lpszFilename, lParam)

LPSTR lpAttributes;		/* pointer to list of attributes	*/

LPCSTR lpszFilename;		/* filename of color profile	*/

LPARAM lparam;		/* application supplied data	*/

The EnumColorProfilesProc function is an application defined callback function that processes color profile data from the EnumColorProfiles function.

| Parameter | Description |
| --- | --- |
| lpAttributes | Points to a collection of attribute strings for the profile. Each string is null terminated. The collection is terminated with a NULL. |
| lpszFilename | Pointer to filename of profile. |
| lParam | Application supplied data passed by EnumColorProfiles. |

Returns

This function must return a positive value to continue enumeration; to stop enumeration, it must return zero. A negative value may not be returned.

### CheckColorsInGamut

BOOL CheckColorsInGamut(hdc, lpaRGBQuad, lpBuffer, nCount)

HDC hdc;				/* handle of device context	*/

LPARGBQUAD lpaRGBQuad;		/* array of RGBQuads to check	*/

LPBYTE dlpBuffer;			/* buffer to put results		*/

UINT nCount;				/* count of elements in array	*/

The CheckColorsInGamut determines if the given RGB lies in the output gamut of the given device.

| Parameter | Description |
| --- | --- |
| hdc | Identifies a device context that is to have the given color space definition. |
| lpaRGBQUAD | Pointer to array of RGBQuads to check. |
| lpBuffer | Pointer to buffer to put results. |
| nCount | Count of elements in array. |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

The lpBuffer holds the results, each byte corresponding to an RGBQuad is CM_IN_GAMUT or CM_OUT_OF_GAMUT.

### ColorMatchToTarget

BOOL ColorMatchToTarget(hdc, hdcTarget, uiAction)

HDC hdc;				/* handle of device context		*/

HDC hdcTarget;				/* handle of device context of target device	*/

UINT uiAction;				/* action (e.g. enable/disable) to take	*/

The ColorMatchToTarget allows for previewing of colors as they would appear on the target device. The color transform for the target device is first performed, and then the transform to the preview device is applied to the results of the first transform. This is mostly useful for checking gamut mapping conditions. The application must have enabled ICM for both DC's.

| Parameter | Description |
| --- | --- |
| hdc | Identifies the device context for previewing, generally the screen. |
| hdcTarget | Identifies the target device context, generally a printer. |
| uiAction | Can have one of the following values: |

|  | Constant | Meaning |
| --- | --- | --- |
|  | CS_ENABLE | Color matching is done through the target. |
|  | CS_DISABLE | Color matching is done in the normal way. |
|  | CS_DELETE_TRANSFORM | Do a disable if enabled and then delete the concatenated transform. |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

Comments

This function cannot be cascaded. While enabled application changes to the color space or gamut matching method are ignored; those changes take effect when this function is disabled. It is not required that an application delete transform using CS_DELETE_TRANSFORM. The transform will be deleted when either of devices are removed from the system, or the application color space is deleted. However if the transform is not going to be used again, an application can free up the space taken by it.

### LoadImageColorMatcher

HANDLE LoadImageColorMatcher(lpszImageColorMatcher)

LPSZSTR lpszImageColorMatcher;		/* pointer to name of dll to load	*/

The LoadImageColorMatcher function loads an installable image color matcher.

| Parameter | Description |
| --- | --- |
| lpszImageColorMatcher | Points to a null-terminated string that names the image color matcher to be loaded. |

Returns

The return value is a handle of the loaded image color matcher if the function is successful. Otherwise, it is an error value less than HINSTANCE_ERROR.This handle can only be used in the FreeImageColorMatcher API, it is not recognized in any other API.

Comments

If the color image matcher has been loaded, LoadImageColorMatcher increments (increases by one) the image color matcher's reference count. If the image color matcher has not been loaded, the function loads it from the specified file. When an application has finished using the image color matcher, it unloads the image color matcher by using the FreeImageColorMatcher function.

For a complete description of path searching and error returns please see LoadLibrary.

### FreeImageColorMatcher

BOOL FreeImageColorMatcher(hImageColorMatcher)

HANDLE hImageColorMatcher;		/* handle of image color matcher to unload	*/

The FreeImageColorMatcher function decrements (decreases by one) the reference count of the loaded image color matcher. When the reference count reaches zero, the image color matcher is unloaded.

| Parameter | Description |
| --- | --- |
| hImageColorMatcher | Handle of image color matcher to unload. |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

Comments

The reference count for a color image matcher is incremented (increased by one) each time an application calls the LoadImageColorMatcher function for the image color matcher.

### GetDeviceGammaRamp

BOOL GetDeviceGammaRamp(hdc, lpRamp)

HDC hdc;				/* device-context handle			*/

LPARAM lpGammaRamp;		/* address of application-supplied data	*/

The GetDeviceGammaRamp gets the gamma ramp on direct color display boards.

| Parameter | Description |  |
| --- | --- | --- |
| hdc | Identifies the device context. |  |
| lpRamp | Points to a set of three arrays of 256 word elements. These arrays are the mapping between RGB values in the frame buffer and DAC values. The first array is red, the next green, and the final one blue. |  |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

Comments

Direct color display modes do not use color lookup tables. The direct color modes are usually 16, 24, or 32 bit. Not all direct color video boards support loadable gamma ramps. This function returns success only for those drivers that support loadable gamma ramps in hardware.

### SetDeviceGammaRamp

BOOL SetDeviceGammaRamp(hdc, lpRamp)

HDC hdc;				/* device-context handle			*/

LPARAM lpGammaRamp;		/* address of application-supplied data	*/

The SetDeviceGammaRamp sets the gamma ramp on direct color display boards.

| Parameter | Description |  |
| --- | --- | --- |
| hdc | Identifies the device context. |  |
| lpRamp | Points to a set of three arrays of 256 word elements. These arrays are the mapping between RGB values in the frame buffer and DAC values. The first array is red, the next green, and the final one blue. |  |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

Comments

Direct color display modes do not use color lookup tables. The direct color modes are usually 16, 24, or 32 bit. Not all direct color video boards support loadable gamma ramps. This function returns success only for those drivers that support loadable gamma ramps in hardware.

## API for Color Matching DLL

Installable image color matchers will be supported under Windows. There can be multiple image color matchers in the system, and applications will have access to all of them. These will be DLL's that the system can load at boot time, or they can be dynamically loaded and unloaded. For dynamic loading please see LoadImageColorMatcher and FreeImageColorMatcher in the previous section. For boot time loading in win.ini GDI will look at

[Windows]

ImageColorMatcher = icm1, icm2, ...

and load the specified DLL's.

typedef HANDLE  HCMTRANSFORM;

typedef LPVOID  LPDEVCHARACTER;

typedef LPVOID  LPDEVMODE;

typedef COLORREF FAR *LPCOLORREF;

The functions added are:

CMGetInfo				gets various information about the ICM

CMCreateTransform			sets up a transform to translate color spaces

CMDeleteTransform			deletes transform created with CMCreateTransform

CMTranslateRGB			translates a give RGB

CMTranslateRGBs			translates an array of RGBs

CMCheckColorsInGamut		determines if the given colors are in the specified gamut

CMEnumColorProfiles			enumerates color profiles supported by DLL

EnumColorProfilesProc			callback for above

### CMGetInfo

ULONG CMGetInfo(dwInfo)

UINT dwInfo;			/* information to be retrieved	*/

The CMGetInfo function retrieves various information about the ICM.

| Parameter | Description |
| --- | --- |
| dwInfo | Values that can have the following meaning: |

|  | Type | Meaning |
| --- | --- | --- |
|  | CMS_VERSION | Retrieves the version of Windows supported. |
|  | CMS_IDENT | Retrieves the identifier of the ICMDLL. |

Returns

For CMS_VERSION CMGetInfo retrieves the version of Windows supported by this module. For modules written to this spec the version should be ??.

### CMCreateTransform

HCMTRANSFORM CMCreateTransform(lpColorSpace, lpDevCharacter, lpDevMode,  lpTargetDevCharacter, lpTargetDevMode)

LPCOLORSPACE lpColorSpace;			/* source color space			*/

LPDEVCHARACTER lpDevCharacter;		/* characterization of output device		*/

LPDEVMODE lpDevMode;			/* device mode for output device		*/

LPDEVCHARACTER lpTargetDevCharacter;	/* characterization of target output device	*/

LPDEVMODE lpTargetDevMode;		/* device mode for target output device	*/

The CMCreateTransform function creates mappings between an application color space and a device color space. The transform allows for previewing of a target device color transformation on a preview device, for example one can preview on the screen how a printed image would appear on the target printer. The lpTargetDevCharacter parameter determines if a straight color transform or a match to target transform is requested. If lpTargetDevCharacter is null then a straight application to device color transform is wanted. In this case the ICM should construct two mappings : 1) a mapping from the application's defined color space to the device's hardware color space, and 2) a mapping from the device's hardware color space to the application's defined color space. If lpTargetDevCharacter is not null then a mapping from the application color space to the target device and then to the preview device is wanted.

| Parameter | Description |
| --- | --- |
| lpColorSpace | Pointer to application's color space. |
| lpDevCharacter | Pointer to device color profile. |
| lpDevMode | Pointer to devmode structure. |
| lpTargetDevCharacter | Pointer to target device color profile. |
| lpTargetDevMode | Pointer to target devmode structure. |

Returns

If the return value is greater than 100 then it is a handle to a color transform.  A value less than 100 is an error return. ERROR_INVALID_PARAMETER denotes that there is an invalid parameter in a color profile passed in.

Comments

The format of the returned transform is completely undefined, it is specific to the image color matching DLL. The handle is used when translating specific color triplets to device color coordinates.

The gamut matching methods are enumerated by the EnumGamutMatchingMethods.

The forward mapping from the application to the device must precomputed at this time, if necessary. Future calls to CMTranslateRGB and CMTranslateRGBs using the forward transform cannot fail due to inability to construct the transform. The backward transform can be delay until a request for its use. The backward transformation is used only to support GetPixel() and GetDIBits(). The forward or backward transform is denoted by a flag passed to CMTranslateRGB(s)().

### CMDeleteTransform

BOOL CMDeleteTransform(hcmTransform)

HCMTRANSFORM hcmTransform;	/* handle of transform to delete	*/

The CMDeleteTransform function deletes and destroys the transforms created with CMCreateTransform.

| Parameter | Description |
| --- | --- |
| hcmTransform | Handle of transform to delete. |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

Comments

The format of the returned transform is completely undefined, it is specific to the image color matching DLL. The handle is used when translating specific color triplets to device color coordinates.

### CMTranslateRGB

BOOL CMTranslateRGB(hcmTransform, ColorRef, lpColorRef, dwFlags)

HCMTRANSFORM hcmTransform;	/* handle of transform to use	*/

COLORREF ColorRef;			/* RGBQuad to translate		*/

LPCOLORREF lpColorRef;		/* buffer to store result		*/

DWORD dwFlags;			/* flags				*/

The CMTranslateRGB function translates an application supplied RGBQuad into the device color coordinate space.

| Parameter | Description |
| --- | --- |
| hcmTransform | Handle of transform to use. |
| RGBQUAD | RGBQuad to translate. |
| lpColorRef | Pointer to buffer to store result. |
| dwFlags | Flags that can have the following meaning |

|  | Type | Meaning |
| --- | --- | --- |
|  | CMS_BACKWARD | Specifies that the backward transform is to be used. |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

Comments

If CMS_BACKWARD is clear then the function uses the forward color transform to do the translation. If it is set then the backwards color transform is to be used.

### CMTranslateRGBs

BOOL CMTranslateRGBs(hcmTransform, lpSrc, dwTransFlags, lpDest, dwTransFlags1, nCount, dwFlags)

HCMTRANSFORM hcmTransform;	/* handle of transform to use		*/

LPVOID lpasrc;				/* pointer to DIB bits to translate		*/

TRANSLATEFLAGS dwTransFlags;	/* translate flags				*/

LPVOID lpadest;			/* buffer to store result			*/

TRANSLATEFLAGS dwTransFlags1;	/* translate flags				*/

UINT nCount;				/* count of elements in array		*/

DWORD dwFlags;			/* flags					*/

The CMTranslateRGBs function translates an array of double bytes, triple bytes, or quad bytes, depending on dwflags.

| Parameter | Description |
| --- | --- |
| hcmTransform | Handle of transform to use. |
| lpasrcDIB | Pointer to DIB bits to translate. |
| dwTransFlags | translate flags |
| lpsdestDIB | Pointer to buffer to store results. |
| dwTransFlag1s | translate flags |
| nCount | Count of elements in array. |
| dwFlags | Describes direction of transform. Flags can have the following meaning: |

|  |  | Type |  | Meaning |
| --- | --- | --- | --- | --- |
|  |  | CMS_BACKWARD |  | Specifies that the backward transform is to be used. |
|  | CMS_x555WORD |  | 16 bit pixels, x555 |  |
|  | CMS_565WORD |  | 16 bit pixels, 565 |  |
|  | CMS_RGBTRIPLETS |  | 24 bit pixels, RGB |  |
|  | CMS_BGRTRIPLETS |  | 24 bit pixels, BGR |  |
|  | CMS_XRGBQUADS |  | 32 bit pixels, xRGB |  |
|  | CMS_XBGRQUADS |  | 32 bit pixels, xBGR |  |
|  | CMS_QUADS |  | 32 bit pixels, CMYK |  |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

Comments

If CMS_BACKWARD is clear then the function uses the forward color transform to do the translation. If it is set then the backwards color transform is to be used.

### CMCheckColorsInGamut

BOOL CMCheckColorsInGamut(hcmTransform, lpaRGBQuad, lpBuffer, nCount)

HCMTRANSFORM hcmTransform;	/* handle of transform to use	*/

LPARGBQUAD lpaRGBQuad;		/* array of RGBQuads to check	*/

LPBYTE lpBuffer;			/* buffer to put results		*/

UINT nCount;				/* count of elements in array	*/

The CMCheckColorsInGamut determines if the given RGBs lie in the output gamut of the given transform.

| Parameter | Description |
| --- | --- |
| hcmTransform | Handle of transform to use. |
| lpaRGBQUAD | Pointer to array of RGBQuads to check. |
| lpBuffer | Pointer to buffer to put results. |
| nCount | Count of elements in array. |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

The lpBuffer holds the results, each byte corresponding to an RGBQuad is CM_IN_GAMUT or CM_OUT_OF_GAMUT.

### CMEnumColorProfiles

int CMEnumColorProfiles(lpEnumColorProfilesFunc, lpDevMode, lpGDIData)

ENUMCOLORPROFILESPROC lpEnumColorProfilesFunc; /* address of callback function	*/

LPDEVMODE lpDevMode;				/* devmode of output device	*/

LPVOID lpGDIData;					/* address of GDI supplied data	*/

The CMEnumColorProfiles function enumerates the different color profiles that the image color matcher supports.

| Parameter | Description |
| --- | --- |
| lpEnumColorProfilesFunc | Specifies the procedure-instance address of the application-defined callback function. See the description of the EnumColorProfilesProc function. If NULL then lParam contains a method to check for validity. |
| lpDevMode | Pointer to devmode. |
| lpGDIData | Points to GDI supplied data. The data is passed to the callback function along with the color information. |

Returns

A return value of zero specifies the application interrupted the enumeration; -1 specifies that there are no gamut matching methods to enumerate. Otherwise the return is the last value returned by the callback function.

### EnumColorProfilesProc

int CALLBACK EnumColorProfilesProc(lpszMethodName, lpuiIdent, lpData)

LPSTR lpAttributes;		/* pointer to collection of attributes		*/

LPCSTR lpFilename;		/* pointer to filename of color profile	*/

LPVOID lpGDIData;		/* GDI supplied data			*/

The EnumColorProfilesProc function is an application defined callback function that processes gamut matching data from the EnumColorProfiles function.

| Parameter | Description |
| --- | --- |
| lpAttributes | Points to a collection of attributes. The attributes strings are null terminated. The collection is null terminated. |
| lpFilename | Points to the filename of the color profile. |
| lpGDIData | Points to the GDI supplied data passed by CMEnumColorProfiles. |

Returns

This function must return a positive value to continue enumeration; to stop enumeration, it must return zero. A negative value may not be returned.

## Entry points, flags, and structures added to the DDI for device independent color.

Device drivers do not have to be revised in order for image color matching to occur at output time. However, if drivers wish to completely take over color matching they can do so simply by supporting the ICM interface described in the previous section. Windows will default to the device ICM if available. If a device supports ICM it must implement the complete interface.

DDITranslateRGB is will be called instead of ColorInfo for printer drivers that have their own image color matchers. This is because colorinfo does not take a pointer to the drawmode. The DEVMODE will be extended to return paper, dithering, ink, viewing, etc, information.

### Structures

The DRAWMODE structure has been modified by the addition of a field at the end, hcmTransform. Drivers use this value to compute RGB translations. The field is NULL if the driver is to do no further color processing on the colors passed in. If non-null the the field holds the hcmTransform to use.

DRAWMODE        struc					;*/ typedef     struct {		/*

Rop2		dw  0	;  The 16-bit encoded logical op	;*/ short		Rop2;		/*

bkMode	dw  0	;  Background Mode (for text only)	;*/short		bkMode;	/*

bkColor	dd   0	;  Physical background Color	;*/DWORD	bkColor;		/*

TextColor	dd   0	;  Physical text (foreground) color	;*/DWORD	TextColor;	/*

TBreakExtra	dw  0	;  Total pixels to stuff into a line	;*/short		TBreakExtra;	/*

BreakExtra	dw  0	;  Div(TBreakExtra, BreakCount)	 ;*/short		BreakExtra;	/*

BreakErr	dw  0	;  Running error term		;*/short		BreakErr;	/*

BreakRem	dw  0	;  Mod(TBreakExtra, BreakCount)	;*/ short		BreakRem;	/*

BreakCount	dw  0	;  Count of breaks in the line	;*/ short		BreakCount;	/*

CharExtra	dw  0	;  Extra pixels to stuff after each char ;*/ short	CharExtra;	/*

LbkColor	dd   0	;  Logical background color	;*/ DWORD	LbkColor;	/*

LTextColor	dd   0	;  Logical Text (foreground) color 	;*/ DWORD	LTextColor;	/*

hcmTransform	dd   0	;  Gamut Matching Method	;*/ DWORD	hcmTransform;	/*

DRAWMODE        ends			;*/ } DRAWMODE, FAR *LPDRAWMODE;	/*

DEVMODE             struc

dmDeviceName	db  CCHDEVICENAME  dup (?)

dmSpecVersion	dw  0

dmDriverVersion	dw  0

dmSize		dw  0

dmDriverExtra	dw  0

dmFields		dd   0

dmOrientation		dw  0

dmPaperSize		dw  0

dmPaperLength	dw  0

dmPaperWidth	dw  0

dmScale		dw  0

dmCopies		dw  0

dmDefaultSource	dw  0

dmPrintQuality	dw  0

dmColor		dw  0

dmDuplex		dw  0

dmYResolution	dw  0

dmTTOption		dw  0

DEVMODE             ends

The functions added are:

DDIGammaRamp			sets and gets display adaptor gamma ramps

### DDIGammaRamp

BOOL DDIGammaRamp(fGetSet, lpRamp)

BOOL fGetSet;				/* flag to determine if a get or set is wanted	*/

LPARAM lpGammaRamp;		/* address of application-supplied data	*/

The DDIGammaRamp sets the ramp on direct color display boards.

| Parameter | Description |
| --- | --- |
| fGetSet | TRUE for getting, FALSE for setting. |
| lpRamp | Points to a set of three arrays of 256 word elements. These arrays are the mapping between RGB values in the frame buffer and DAC values. The first array is red, the next green, and the final one blue. |

Returns

The return value is TRUE if the function is successful. Otherwise, it is NULL.

Comments

Direct color display modes do not use color lookup tables to translate between pixel values and colors, the pixel value is fed directly to the DACs. The direct color modes are usually 16, 24, or 32 bit deep. The display adaptor must have hardware to support loadable gamma ramps for the driver to support this function. If the driver does support this function it must also set the C1_GAMMA_RAMP bit in dpCaps1.

The setting of the display gamma ramp acts globally across all tasks. The gamma ramp is not maintained on a per application basis. Once set, the gamma ramp does not change until the next set call made by an application. It is expected that users will run a calibration utility to set the gamma ramps, and not have applications set it independently of each other.

## Metafiles and device independent color

Metafiles are, philosophically, device independent representations. As discussed before image color matching is specific to devices. Therefore, in general, only the following added API's are metafileable: EnableICM, CreateColorSpace, DeleteColorSpace, SetColorSpace. Compatible bitmaps and DIBs are recorded with no color processing done. In the specific case of GDI creating a metafile for printing purposes, compatible bitmaps and DIBs will be processed to have device dependent color information if an installable image color matcher does the matching. If GDI or the printer driver does the matching then the processing is done at print time, not at metafile accumulation time. The following API's have no relevance to metafiles:

GetColorSpace

GetLogColorSpace

CheckColorsInGamut

ColorMatchToTarget

GetColorProfile

SetColorProfile

EnumColorProfiles

EnumColorProfileProc

LoadImageColorMatcher

FreeImageColorMatcher

GetDisplayGammaRamp

SetDisplayGammaRamp

## DIBS and device independent color !! not quite right yet !!

We add two fields to the BitmapInfoHeader to record the linear RGB space the DIB is recorded against. The last two fields are the added ones.

; /*

BitmapInfoHeader	struc		*/    typedef struct tagBITMAPINFOHEADER {	*/

biSize		dd  0	;	*/	DWORD	biSize;			*/

biWidth		dd  0	;	*/	LONG		biWidth;		*/

biHeight		dd  0	;	*/	LONG		biHeight;		*/

biPlanes		dw  0	;	*/	WORD		biPlanes;		*/

biBitCount	dw  0	;	*/	WORD		biBitCount;		*/

biCompression	dd  0	;	*/	DWORD	biCompression;		*/

biSizeImage	dd  0	;	*/	DWORD	biSizeImage;		*/

biXPelsPerMeter	dd  0	;	*/	LONG		biXPelsPerMeter;	*/

biYPelsPerMeter	dd  0	;	*/	LONG		biYPelsPerMeter;	*/

biClrUsed	dd  0	;	*/	DWORD	biClrUsed;		*/

biClrImportant	dd  0	;	*/	DWORD	biClrImportant;		*/

biColorSpace	dd  0	;	*/	DWORD	biColorSpace;		*/

biRGBSpace	db 36 dup(0) ;	*/	CIEXYZTRIPLE biRGBSpace;		*/

BitmapInfoHeader	ends	;	*/	} BITMAPINFOHEADER;

The values for biColorSpace are the following:

|  | Constant | Meaning |
| --- | --- | --- |
|  | CS_DEVICE_RGB | RGB's are to be interpreted as device RGB's. In effect this turns color matching off. |
|  | CS_DEVICE_CMYK | RGB's are to be interpreted as device CMYK. No translation of these values are allowed. The CIEXYZTriplet is ignored in this case. |
|  | CS_CALIBRATED_RGB | The given CIEXYZTriplet defines the endpoints of the RGB's. |

I am being somewhat subtle here. The RGB triplet used to specify color in Windows today is not really 'logical', although it may be refered to as such. This is because there is no independent measure of what, say, (255,0,0) means. There is no definition of 'red' in terms of any colormetric measures. Contrast this to, say, lines, where if the application asked for a two inch line there is independent verification if the printer satisfied the request correctly.

This is a bit of a simplification, but it is a good working model.

For the Chicago surfaces are limited to things that are writable, it must be write-only or read-write. NT extends the definition to read-only, which is their right, after all they introduced the concept of surfaces.

The speed hit depends on the pixel depth and whether a CLUT is used. For VGAs, an 8-bit CLUT, the hit should not be noticeable.