Chapter 12

Please note: The information contained in this documentation is preliminary and subject to change without notice.

Phone-related Functions

### TSPI_phoneClose

#### Syntax

long TSPI_phoneClose(hdPhone)

This function closes the specified open phone device after completing or aborting all outstanding asynchronous operations on the device.

#### Parameters

HDRVPHONE    hdPhone

Specifies the service provider's opaque handle to the phone to be closed. After the phone has been successfully closed, this handle is no longer valid.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INCOMPATIBLEAPIVERSION

The caller requested a TSPI version that was outside the range supported by the service provider.

·	PHONEERR_INCOMPATIBLEEXTVERSION

The caller requested an Extension version that cannot be supported by the service provider.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	The service provider has the responsibility to (eventually) report completion for every operation it decides to execute asynchronously. If this procedure is called for a phone on which there are outstanding asynchronous operations, the operations should be reported complete with an appropriate result or error code before this procedure returns. Generally, TAPI.DLL would wait for these to complete in an orderly fashion. However, the service provider should be prepared to handle an early call to TSPI_phoneClose in "abort" or "emergency shutdown" situtations.

After this procedure returns the service provider must report no further events on the phone. The service provider's opaque handle for the phone becomes "invalid."

The service provider must relinquish non-sharable resources it reserves while the phone is open. For example, closing a phone accessed through a comm port and modem should result in closing the comm port, making it once available for use by other applications.

This function should always succeed except in extraordinary circumstances. Most callers will probably ignore the return code since they will be unable to compensate for any error that occurs. The specified error returns are more advisory for development diagnostic purposes than anything else.

See Also	Messages:  PHONE_CLOSE

### TSPI_phoneConfigDialog

#### Syntax

LONG TSPI_phoneConfigDialog(dwDeviceID, hwndOwner, lpszDeviceClass)

This function causes the provider of the specified phone device to display a modal dialog as a child window of hwndOwner to allow the user to configure parameters related to the phone device.

#### Parameters

DWORD    dwDeviceID

Specifies the phone device to be configured.

HWND    hwndOwner

Specifies a handle to a parent window in which the dialog window is to be placed.

LPCSTR     lpszDeviceClass

Specifies a far pointer to a NULL-terminated string that identifies a device class name. This device class allows the caller to select a specific subscreen of configuration information applicable to that device class. If this parameter is NULL or an empty string, the highest level configuration dialog should be selected.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_BADDEVICEID

The specified device ID is invalid.

·	PHONEERR_INUSE

The phone device is in use and cannot currently be configured.

·	PHONEERR_INVALPARAM

The specified window handle is invalid.

·	PHONEERR_INVALDEVICECLASS

The specified phone does not support the indicated device class.

·	PHONEERR_NOMEM

Insufficient memory to perform the operation.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

Comments	TSPI_phoneConfigDialog causes the service provider to display a modal dialog as a child window of hWndOwner to allow the user to configure parameters related to the phone specified by dwDeviceID. lpszDeviceClass allows the application to select a specific subscreen of configuration information applicable to the device class in which the user is interested. The permitted strings are the same as for TSPI_phoneGetID. For example, if the phone supports the COMM API, passing "COMM" as lpszDeviceClass causes the provider to display the parameters related specifically to COMM (or, at least, to start at the corresponding point in a multilevel configuration dialog chain, so that the user doesn't have to search to find the desired parameters.) szDeviceClass would be "tapi/phone", "", or NULL to cause the provider to display the highest level configuration for the phone.

The procedure must update the [Windows Telephony] section in WIN.INI and broadcast the WM_WININICHANGE message if it makes any changes to TELEPHON.INI that affect the contents of structures visible to applications (such as PHONECAPS), or if phone devices are created or removed.

See Also	Functions:  TSPI_phoneGetID

### TSPI_phoneDevSpecific

#### Syntax

LONG TSPI_phoneDevSpecific(dwRequestID, hdPhone, lpParams, dwSize)

This function is used as a general extension mechanism to enable a Telephony API implementation to provide features not described in the other operations. The meanings of these extensions are device specific.

#### Parameters

DRV_REQUESTID    dwRequestID

Specifies the identifier of the asynchronous request.

HDRVPHONE    hdPhone

Specifies the handle to the phone on which a device-specific operation is to be performed.

LPVOID    lpParams

Specifies a far pointer to a memory area used to hold a parameter block. Its interpretation is device specific.

DWORD    dwSize

#### Return Value

Returns dwRequestID or a negative error number if an error has occurred. The lResult actual parameter of the corresponding ASYNC_COMPLETION is zero if the function is successful or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_PHONEHANDLE

The specified phone handle is invalid.

·	PHONEERR_INVALPOINTER

The specified pointer parameter is invalid.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

Additional error returns are device specific.

Comments	This operation provides a generic parameter profile. The interpretation of the parameter block is device specific. Indications and replies that are device specific should use the PHONE_DEVSPECIFIC message.

This function is called in direct response to an application calling the TAPI function phoneDevSpecific. TAPI.DLL translates the hPhone parameter used at the TAPI level to the corresponding hdPhone parameter used at the TSPI level. The lpParams buffer is passed through unmodified.

A service provider can provide access to device-specific functions by defining parameters for use with this operation. Applications that want to make use of these device-specific extensions should consult the device-specific (vendor-specific) documentation that describes what extentions are defined. Note that an application that relies on these device-specific extensions will typically not be portable to work with other service provider environments.

See Also	Messages: PHONE_DEVSPECIFIC

### TSPI_phoneGetButtonInfo

#### Syntax

LONG TSPI_phoneGetButtonInfo(hdPhone, dwButtonLampID, lpButtonInfo)

This function returns information about the specified button.

#### Parameters

HDRVPHONE    hdPhone

Specifies the handle to the phone to be queried.

DWORD    dwButtonLampID

Specifies a button on the phone device.

LPPHONEBUTTONINFO    lpButtonInfo

Specifies a far pointer to memory into which the service provider writes a variably sized structure of type PHONEBUTTONINFO. This data structure describes the mode, the function, and provides additional descriptive text corresponding to the button.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALBUTTONLAMPID

The specified button/lamp ID is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

Comments	The service provider should fill in all the fields of the PHONEBUTTONINFO data structure, except for dwTotalSize, which is filled in by TAPI.DLL. The service provider must not overwrite the dwTotalSize field.

See Also	Functions: TSPI_phoneSetButtonInfo

Data Types:  PHONEBUTTONINFO, PHONECAPS

### TSPI_phoneGetData

#### Syntax

LONG TSPI_phoneGetData(hdPhone, dwDataID, lpData, dwSize)

This function uploads the information from the specified location in the open phone device to the specified buffer.

#### Parameters

HDRVPHONE    hdPhone

Specifies the handle to the phone to be queried.

DWORD    dwDataID

Specifies where in the phone device the buffer is to be uploaded from.

LPVOID    lpData

Specifies a far pointer to the memory buffer where the data is to be uploaded to.

DWORD    dwSize

Specifies the size of the data buffer in bytes.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_INVALDATAID

The specified data ID is invalid or the number of bytes specified in dwSize cannot be read from that location.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

Comments	The function uploads a maximum of dwSize bytes from the phone device into lpData. If dwSize is zero, nothing is copied. The size of each data area is listed in the phone's PHONECAPS structure.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneSetData

Data Types:  PHONECAPS

### TSPI_phoneGetDevCaps

#### Syntax

LONG TSPI_phoneGetDevCaps(dwDeviceID, dwTSPIVersion, dwExtVersion, lpPhoneCaps)

This function queries a specified phone device to determine its telephony capabilities.

#### Parameters

DWORD    dwDeviceID

Specifies the phone device to be queried.

DWORD    dwTSPIVersion

Specifies the negotiated TSPI version number. This value has already been negotiated for this device through the TSPI_phoneNegotiateTSPIVersion function.

DWORD    dwExtVersion

Specifies the negotiated Extension version number. This value has already been negotiated for this device through the TSPI_phoneNegotiateExtVersion function.

LPPHONECAPS    lpPhoneCaps

Specifies a far pointer to memory into which the service provider writes a variable sized structure of type PHONECAPS. Upon successful completion of the request, this structure is filled with phone device capabilities information.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INCOMPATIBLEEXTVERSION

The caller requested an Extension version that cannot be supported by the service provider.

·	PHONEERR_NODRIVER

The driver was not installed.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	The service provider should fill in all the fields of the PHONEBUTTONINFO data structure, except for dwTotalSize, which is filled in by TAPI.DLL. The service provider must not overwrite the dwTotalSize field.

If dwExtVersion is zero, no Extension information is requested. If it is non-zero it holds a value that has already been negotiated for this device with the function TSPI_phoneNegotiateExtVersion. The service provider should fill in device and vendor-specific extended information according to the Extension version specified.

See Also	Data Types:  PHONECAPS

Functions:  TSPI_phoneNegotiateExtVersion

### TSPI_phoneGetDisplay

#### Syntax

LONG TSPI_phoneGetDisplay(hdPhone, lpDisplay)

This function returns the current contents of the specified phone display.

#### Parameters

HDRVPHONE    hdPhone

Specifies the handle to the phone to be queried.

LPVARSTRING    lpDisplay

Specifies a far pointer to the memory location where the display content is to be stored by the provider, of type VARSTRING.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	The lpDisplay memory area should be at least (dwNumRows * dwNumColumns) elements in size to receive all of the display information. dwNumRows and dwNumColumns are available in the PHONECAPS structure returned by TSPI_phoneGetDevCaps.

The service provider should fill in all the fields of the VARSTRING data structure, except for dwTotalSize, which is filled in by TAPI.DLL. The service provider must not overwrite the dwTotalSize field.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneSetDisplay

Data Types:  PHONECAPS, VARSTRING

### TSPI_phoneGetExtensionID

#### Syntax

long TSPI_phoneGetExtensionID(dwDeviceID, dwTSPIVersion, lpExtensionID)

This function retrieves the Extension ID that the service provider supports for the indicated phone device.

#### Parameters

DWORD    dwDeviceID

Specifies the phone device to be queried.

DWORD    dwTSPIVersion

Specifies an interface version number that has already been negotiated for this device using TSPI_phoneNegotiateTSPIVersion. This function operates according to the interface specification at this version level.

LPPHONEEXTENSIONID    lpExtensionID

Specifies a far pointer to a structure of type PHONEEXTENSIONID. If the service provider supports provider-specific extensions it fills this structure with the extension ID of these extensions. If the service provider does not support extensions, it fills this structure with all zeros. An extension ID of all zeros is not a legal extension ID, since the all-zeros value is used to indicate that the service provider does not support extensions.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INCOMPATIBLEAPIVERSION

The passed TSPI version did not match an interface version definition supported by the service provider.

·	PHONEERR_BADDEVICEID

The specified phone device ID is out of range.

·	PHONEERR_INVALPOINTER

The specified pointer parameter is invalid.

·	PHONEERR_NODRIVER

The driver was not successfully installed.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Call States	Not applicable

Comments	This function is typically called by TAPI.DLL in response to an application calling the phoneNegotiateAPIVersion function. The result returned by the service provider should be appropriate for use in a subsequent call to TSPI_phoneNegotiateExtVersion.

See Also	Functions:  TSPI_phoneNegotiateTSPIVersion, TSPI_phoneNegotiateExtVersion

### TSPI_phoneGetGain

#### Syntax

LONG TSPI_phoneGetGain(hdPhone, dwHookSwitchDev, lpdwGain)

This function returns the gain setting of the microphone of the specified phone's hookswitch device.

#### Parameters

HDRVPHONE    hdPhone

Specifies the handle to the phone whose gain is to be retrieved.

DWORD    dwHookSwitchDev

Identifies the hookswitch device whose gain level is queried. This parameter can only have a single bit set. Values are:

PHONEHOOKSWITCHDEV_HANDSET

This is the phone's handset.

PHONEHOOKSWITCHDEV_SPEAKER

This is the phone's speakerphone or adjunct.

PHONEHOOKSWITCHDEV_HEADSET

This is the phone's headset.

LPDWORD    lpdwGain

Specifies a far pointer to a DWORD sized location into which the service provider writes the current gain setting of the hookswitch microphone component. dwGain specifies the volume level of the hookswitch device. This is a number in the range 0x00000000 (which is silence) to 0x0000FFFF (which is maximum volume). The actual granularity and quantization of gain settings in this range are service provider specific.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_INVALHOOKSWITCHDEV

The hookswitch device parameter is invalid or unsupported by the phone or provider.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	None.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneSetGain

Data Types:  PHONECAPS

### TSPI_phoneGetHookSwitch

#### Syntax

LONG TSPI_phoneGetHookSwitch(hdPhone, lpdwHookSwitchDevs)

This function returns the current hook switch mode of the specified open phone device.

#### Parameters

HDRVPHONE    hdPhone

Specifies the service provider's opaque handle to the phone whose hook switch mode is to be retrieved.

LPDWORD    lpdwHookSwitchDevs

Specifies a far pointer to a DWORD-sized location into which the service provider writes the mode of the phone's hookswitch devices, of type PHONEHOOKSWITCHDEV. If a bit position is FALSE, the corresponding hookswitch device is on hook. If TRUE, the microphone and/or speaker part of the corresponding hookswitch device is offhook. To find out whether microphone and/or speaker are enabled, TAPI.DLL can use TSPI_phoneGetStatus. Values for lpdwHookSwitchDevs are:

PHONEHOOKSWITCHDEV_HANDSET

This is the phone's handset.

PHONEHOOKSWITCHDEV_SPEAKER

This is the phone's speakerphone or adjunct.

PHONEHOOKSWITCHDEV_HEADSET

This is the phone's headset.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	After the hookswitch state of a device changes, and if hookswitch monitoring is enabled, TAPI.DLL is sent a PHONE_STATE message.

See Also	Functions:  TSPI_phoneSetHookSwitch

Messages:  PHONE_STATE

Data types:  PHONESTATUS, PHONEHOOKSWITCHDEV

### TSPI_phoneGetIcon

#### Syntax

LONG TSPI_phoneGetIcon(dwDeviceID, lpszDeviceClass, lphIcon)

This function retrieves a service phone device-specific (or provider-specific) icon for display to the user.

#### Parameters

DWORD    dwDeviceID

Specifies the phone device whose icon is requested.

LPCSTR    lpszDeviceClass

Specifies a far pointer to a NULL-terminated ASCII string that identifies a device class name. This device class allows the caller to select a specific sub icon applicable to that device class. This parameter is optional and can be left NULL or be empty, in which case the highest level icon associated with the phone device rather than a specified media stream device would be selected.

LPHICON    lphIcon

Specifies a far pointer to a memory location in which the handle to the icon is returned.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALDEVICECLASS

The service provider or phone does not support the indicated device class.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	TSPI_phoneGetIcon causes the provider to return a handle (in lphIcon) to an icon resource (obtained from the Windows LoadIcon function) associated with the specified phone. The icon handle will be for a resource associated with the provider.

lpszDeviceClass allows the provider to return different icons based on the type of service being referenced by the caller. The permitted strings are the same as for TSPI_phoneGetID. For example, if the phone supports the COMM API, passing "COMM" as lpszDeviceClass causes the provider to return an icon related specifically to the COMM device functions of the service provider.

The parameters "tapi/phone", "", or NULL may be used to request the icon for the phone device. A provider may choose to support many icons (selected by lpszDeviceClass and/or phone number), a single icon (such as for the manufacturer, which would be returned for all phoneGetIcon requests regardless of the lpszDeviceClass selected), or no icons, in which case it returns NULL for lphIcon. TAPI.DLL examines lphIcon, and if the provider returns NULL, TAPI.DLL substitutes a generic Windows Telephony Icon included as a resource in TAPI.DLL (the generic "phone" icon).

If the service provider supports no icons, it may leave this function unimplemented, in which case TAPI.DLL will provide a generic “phone” icon for the application.

See Also	Functions:  TSPI_phoneConfigDialog

### TSPI_phoneGetID

#### Syntax

LONG TSPI_phoneGetID(hdPhone, lpDeviceID, lpszDeviceClass)

This function returns a device ID for the given device class associated with the specified phone device.

#### Parameters

HDRVPHONE    hdPhone

Specifies the handle to the phone to be queried.

LPVARSTRING    lpDeviceID

Specifies a far pointer to a data structure of type VARSTRING where the device ID is returned. The format of the returned information depends on the method used by the device class (API) for naming devices.

LPCSTR    lpszDeviceClass

Specifies a far pointer to a NULL-terminated ASCII string that specifies the device class of the device whose ID is requested.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The hdPhone parameter is an invalid handle.

·	PHONEERR_INVALDEVICECLASS

The service provider or phone does not support the specified device class.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	This operation can be used to retrieve a phone device ID given a phone handle. It can also be used to obtain the device ID of the media device (for device classes such as COM, wave, MIDI, phone, line, and mciwave) associated with the opened phone device. This ID can then be used with the appropriate media API (such as mci, midi, and wav) to select the corresponding device.

Some common device class names that are currently used are listed below. The first portion of a name identifies the "API" used to manage the device class, and the second portion is typically used to identify a specific device type extension or subset of the overall API. Names are not case sensitive:

comm	(generic serial-device API; comm port)

comm/datamodem	(not supported in Windows 3.1)

wave	(low-level waveaudio)

mci/midi	(high-level midi sequencer)

mci/wave	(high-level wave device control)

tapi/line	(TAPI line device)

tapi/phone	(TAPI phone device)

ndis	(network driver interface)

The service provider should fill in all the fields of the VARSTRING data structure, except for dwTotalSize, which is filled in by TAPI.DLL. The service provider must not overwrite the dwTotalSize field.

See Also	Data Types:  VARSTRING

### TSPI_phoneGetLamp

#### Syntax

LONG TSPI_phoneGetLamp(hdPhone, dwButtonLampID, lpdwLampMode)

This function returns the current lamp mode of the specified lamp.

#### Parameters

HDRVPHONE    hdPhone

Specifies the handle to the phone whose lamp mode is to be retrieved.

DWORD    dwButtonLampID

Specifies the ID of the lamp to be queried.

LPDWORD    lpdwLampMode

Specifies a far pointer to a memory location into which the service provider writes the lamp mode status of the given lamp, of type PHONELAMPMODE. This parameter can have at most one bit set. Values are:

PHONELAMPMODE_BROKENFLUTTER

Broken flutter is the superposition of flash and flutter.

PHONELAMPMODE_FLASH

Flash means slow on and off.

PHONELAMPMODE_FLUTTER

Flutter means fast on and off.

PHONELAMPMODE_OFF

The lamp is off.

PHONELAMPMODE_STEADY

The lamp is continuously lit.

PHONELAMPMODE_WINK

The lamp is winking.

PHONELAMPMODE_UNKNOWN

The lamp mode is currently unknown.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALBUTTONLAMPID

The specified Button ID is out of range.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	Phone sets that have multiple lamps per button, should be modeled using multiple button/lamps pairs. Each extra button/lamp pair should use a DUMMY button.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneSetLamp

Data Types:  PHONECAPS, PHONELAMPMODE

### TSPI_phoneGetRing

#### Syntax

LONG TSPI_phoneGetRing(hdPhone, lpdwRingMode, lpdwVolume)

This function enables an application to query the specified open phone device as to its current ring mode.

#### Parameters

HDRVPHONE    hdPhone

Specifies the handle to the phone whose ring mode is to be queried.

LPDWORD    lpdwRingMode

Specifies the ringing pattern with which the phone is ringing. Zero indicates that the phone is not ringing.

LPDWORD    lpdwVolume

Specifies the volume level with which the phone is ringing. This is a number in the range 0x00000000 (which is silence) to 0x0000FFFF (which is maximum volume). The actual granularity and quantization of volume settings in this range are service provider specific.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	The service provider defines the actual audible ringing patterns corresponding to each of phone's ring modes.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneSetRing

Data Types:  PHONECAPS

### TSPI_phoneGetStatus

#### Syntax

LONG TSPI_phoneGetStatus(hdPhone, lpPhoneStatus)

This operation queries the specified open phone device for its overall status.

#### Parameters

HDRVPHONE    hdPhone

Specifies the handle to the phone to be queried.

LPPHONESTATUS    lpPhoneStatus

Specifies a far pointer to a variably sized data structure of type PHONESTATUS, into which the service provider writes information about the phone's status.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

#### Fields

The following table indicates which fields of the PHONESTATUS data structure are filled in by the service provider and which fields are filled in by TAPI.DLL. The service provider must preserve (it must not overwrite) the values filled in by TAPI.DLL.

| Field Name | TAPI DLL | Service Provider |
| --- | --- | --- |
| dwTotalSize; | X |  |
| dwNeededSize; |  | X |
| dwUsedSize; |  | X |
| dwStatusFlags; |  | X |
| dwNumOwners; | X |  |
| dwNumMonitors; | X |  |
| dwRingMode; |  | X |
| dwRingVolume; |  | X |
| dwHandsetHookSwitchMode; |  | X |
| dwHandsetVolume; |  | X |
| dwHandsetGain; |  | X |
| dwSpeakerHookSwitchMode; |  | X |
| dwSpeakerVolume; |  | X |
| dwSpeakerGain; |  | X |
| dwHeadsetHookSwitchMode; |  | X |
| dwHeadsetVolume; |  | X |
| dwHeadsetGain; |  | X |
| dwDisplaySize; |  | X |
| dwDisplayOffset; |  | X |
| dwLampModesSize; |  | X |
| dwLampModesOffset; |  | X |
| dwOwnerNameSize; | X |  |
| dwOwnerNameOffset; | X |  |
| dwDevSpecificSize; |  | X |
| dwDevSpecificOffset; |  | X |

Comments	TAPI.DLL can use this function to determine the current state of an open phone device. The status information describes information about the phone device's hook switch devices, ringer, volume, display, and lamps of the open phone.

See Also	Messages:  PHONE_STATE

Data Types:  PHONESTATUS

### TSPI_phoneGetVolume

#### Syntax

LONG TSPI_phoneGetVolume(hdPhone, dwHookSwitchDev, lpdwVolume)

This function returns the volume setting of the specified phone's hookswitch device.

#### Parameters

HDRVPHONE    hdPhone

Specifies the handle to the phone containing the hookswitch device whose volume setting is to be retrieved.

DWORD    dwHookSwitchDev

Identifies a single hook switch device whose volume level is queried. Values are:

PHONEHOOKSWITCHDEV_HANDSET

This is the phone's handset.

PHONEHOOKSWITCHDEV_SPEAKER

This is the phone's speakerphone or adjunct.

PHONEHOOKSWITCHDEV_HEADSET

This is the phone's headset.

LPDWORD    lpdwVolume

Specifies a far pointer to a DWORD sized location into which the service provider writes the current volume setting of the hookswitch device. This is a number in the range 0x00000000 (which is silence) to 0x0000FFFF (which is maximum volume). The actual granularity and quantization of volume settings in this range are service provider specific.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_INVALHOOKSWITCHDEV

The hookswitch device parameter is invalid or not supported by the service provider or phone.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	None.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneSetVolume

Data Types:  PHONECAPS

### TSPI_phoneNegotiateExtVersion

#### Syntax

LONG TSPI_phoneNegotiateExtVersion(dwDeviceID, dwTSPIVersion, dwLowVersion, dwHighVersion, lpdwExtVersion)

This function returns the highest Extension version number the service provider is willing to operate under for this device given the range of possible Extension versions.

#### Parameters

DWORD    dwDeviceID

Identifies the phone device for which interface version negotiation is to be performed.

DWORD    dwTSPIVersion

Specifies an interface version number that has already been negotiated for this device using TSPI_phoneNegotiateTSPIVersion. This function operates according to the interface specification at this version level.

DWORD    dwLowVersion

Specifies the lowest Extension version number under which TAPI.DLL or its client application is willing to operate. The most-significant WORD is the major version number and the least-significant WORD is the minor version number.

DWORD    dwHighVersion

Specifies the highest Extension version number under which TAPI.DLL or its client application is willing to operate. The most-significant WORD is the major version number and the least-significant WORD is the minor version number.

LPDWORD  lpdwExtVersion

Specifies a far pointer to a DWORD. Upon a successful return from this function the service provider fills this location with the highest Extension version number, within the range requested by the caller, under which the service provider is willing to operate. The most-significant WORD is the major version number and the least-significant WORD is the minor version number. If the requested range does not overlap the range supported by the service provider, the function returns PHONEERR_INCOMPATIBLEEXTVERSION.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INCOMPATIBLEAPIVERSION

The passed TSPI version did not match an interface version definition supported by the service provider.

·	PHONEERR_INCOMPATIBLEEXTVERSION

The passed Extension version range did not contain an interface definition version supported by the service provider.

·	PHONEERR_NODRIVER

The driver was not successfully installed.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	This function may be called before or after the device has been opened by TAPI.DLL. If the device is currently open and has an Extension version selected, the function should return that version number if it is within the requested range. If the selected version number is outside the requested range, the function returns PHONEERR_INCOMPATIBLEEXTVERSION.

See Also	None:

### TSPI_phoneNegotiateTSPIVersion

#### Syntax

LONG TSPI_phoneNegotiateTSPIVersion(dwDeviceID, dwLowVersion, dwHighVersion, lpdwTSPIVersion)

This function returns the highest SPI version the service provider is willing to operate under for this device given the range of possible SPI versions.

#### Parameters

DWORD    dwDeviceID

Identifies the phone device for which interface version negotiation is to be performed. Permitted values are strictly within the range of phone devices IDs for this service provider.

DWORD    dwLowVersion

Specifies the lowest TSPI version number under which TAPI.DLL is willing to operate. The most significant WORD is the major version number and the least significant WORD is the minor version number.

DWORD    dwHighVersion

Specifies the highest TSPI version number under which TAPI.DLL is willing to operate. The most significant WORD is the major version number and the least significant WORD is the minor version number.

LPDWORD  lpdwTSPIVersion

Specifies a far pointer to a DWORD. Upon a successful return from this function the service provider fills this location with the highest TSPI version number, within the range requested by the caller, under which the service provider is willing to operate. The most-significant WORD is the major version number and the least-significant WORD is the minor version number. If the requested range does not overlap the range supported by the service provider, the function returns PHONEERR_INCOMPATIBLEAPIVERSION.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INCOMPATIBLEAPIVERSION

The passed TSPI version did not match an interface version definition supported by the service provider.

·	PHONEERR_NODRIVER

The driver was not successfully installed.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available. However, if the service provider supports any phone devices, it must also support this function and the function must not return PHONEERR_OPERATIONUNAVAIL.

Call States	Not applicable.

Comments	TAPI.DLL calls this function early in the initialization sequence for each phone device.

Negotiation of an Extension version is done through the separate procedure TSPI_phoneNegotiateExtVersion.

Comparison	The corresponding function at the TAPI level is an overloaded function that also retrieves the Extension ID, if any, supported by the service provider. At the TSPI level, retrieving the Extension ID is done through a separate procedure, TSPI_phoneGetExtensionID.

See Also	Functions:  TSPI_phoneNegotiateExtVersion, TSPI_phoneGetExtensionID

Constants:  INITIALIZE_NEGOTIATION

### TSPI_phoneOpen

#### Syntax

LONG TSPI_phoneOpen(dwDeviceID, htPhone, lphdPhone, dwTSPIVersion, lpfnEventProc)

This function opens the phone device whose device ID is given, returning the service provider's opaque handle for the device and retaining TAPI.DLL's opaque handle for the device for use in subsequent calls to the PHONEEVENT procedure.

#### Parameters

DWORD    dwDeviceID

Identifies the phone device to be opened.

HTAPIPHONE    htPhone

Specifies TAPI.DLL's opaque handle for the phone device to be used in subsequent calls to the PHONEEVENT callback procedure to identify the device.

LPHDRVPHONE    lphdPhone

A far pointer to a HDRVPHONE where the service provider writes its handle for the phone device to be used by TAPI.DLL in subsequent calls to identify the device.

DWORD    dwTSPIVersion

The TSPI version negotiated through TSPI_phoneNegotiateTSPIVersion under which the service provider is willing to operate.

PHONEEVENT    lpfnEventProc

A far pointer to the PHONEEVENT callback procedure supplied by TAPI.DLL that the service provider will call to report subsequent events on the phone.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INCOMPATIBLEAPIVERSION

The passed TSPI version did not match an interface version definition supported by the service provider.

·	PHONEERR_NODRIVER

The driver was not successfully installed.

·	PHONEERR_INIFILECORRUPT

The TELEPHON.INI file was corrupted.

·	PHONEERR_INUSE

The device is currently in use.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	Opening a phone entitles TAPI.DLL to make further requests on the phone. The phone becomes "active" in the sense that the service provider can report asynchronous events such as hookswitch changes or button presses. The service provider reserves whatever non-sharable resources are required to manage the phone. For example, opening a phone accessed through a comm port and modem should result in opening the comm port, making it no longer available for use by other applications.

If the function is successful, both TAPI.DLL and the service provider become committed to operating under the specified interface version number for this open device. Subsquent operations and events identified using the exchanged opaque phone handles conform to that interface version. This commitment and the validity of the handles remain in effect until TAPI.DLL closes the phone using the TSPI_phoneClose operation or the service provider reports the PHONE_CLOSE event. If the function is not successful, no such commitment is made and the handles are not valid.

See Also	Functions:  TSPI_phoneClose, TSPI_phoneNegotiateTSPIVersion, PHONEEVENT

Messages:  PHONE_CLOSE

### TSPI_phoneSelectExtVersion

#### Syntax

LONG TSPI_phoneSelectExtVersion(hdPhone, dwExtVersion)

This function selects the indicated Extension version for the indicated phone device. Subsequent requests operate according to that Extension version.

#### Parameters

HDRVPHONE    hdPhone

Specifies the handle to the phone for which an Extension version is to be selected.

DWORD    dwExtVersion

Specifies the Extension version to be selected. This version number has been negotiated using TSPI_phoneNegotiateExtVersion. The most-significant WORD is the major version number and the least-significant WORD is the minor version number. Calling this function with a dwExtVersion of zero cancels the current selection.

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INCOMPATIBLEEXTVERSION

The passed TSPI version did not match an Extension version definition supported by the service provider.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Call States	Not applicable.

Comments	This function selects the indicated Extension version. Although the indicated version number may have been successfully negotiated, a different Extension version may have been selected in the interim, in which case this function fails (returning PHONEERR_INCOMPATIBLEEXTVERSION).

Subsequent operations on the phone after an Extension version has been selected behave according to that Extension version. Subsequent attempts to negotiate the Extension version report strictly the selected version or 0 (if the requested range does not include the selected version). Calling this procedure with the special Extension version 0 cancels the current selection. The device becomes once again capable of supporting its full range of Extension version numbers.

TSPI_phoneSelectExtVersion is typically called in two situations: (1) An application requested to open a phone, the application requested that a  particular Extension version be used, and no Extension version was currently selected. (2) The last application using a particular Extension version closed the phone, and the Extension version selection can be cancelled.

See Also	Functions: TSPI_phoneNegotiateExtVersion

### TSPI_phoneSetButtonInfo

#### Syntax

LONG TSPI_phoneSetButtonInfo(dwRequestID, hdPhone, dwButtonLampID, lpButtonInfo)

This function sets information about the specified button on the specified phone.

#### Parameters

DRV_REQUESTID    dwRequestID

Specifies the identifier of the asynchronous request. The service provider returns this value if the function completes asynchronously.

HDRVPHONE    hdPhone

Specifies the handle to the phone for which button info is to be set.

DWORD    dwButtonLampID

Specifies a button on the phone device.

LPPHONEBUTTONINFO    const lpButtonInfo

Specifies a far pointer to a variably sized structure of type PHONEBUTTONINFO. This data structure describes the mode, the function, and provides additional descriptive text to be set for the button.

#### Return Value

Returns the (positive) dwRequestID value if the function will be completed asynchronously, or a negative error number if an error has occurred. The lResult actual parameter of the corresponding ASYNC_COMPLETION is zero if the function is successful or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALBUTTONLAMPID

The specified button/lamp ID is invalid or not supported by the service provider or phone.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	This function sets the meaning and associated descriptive text of a phone's buttons.

See Also	Functions:  TSPI_phoneGetButtonInfo

Data Types:  PHONEBUTTONINFO, PHONECAPS

### TSPI_phoneSetData

#### Syntax

LONG TSPI_phoneSetData(dwRequestID, hdPhone, dwDataID, lpData, dwSize)

This function downloads the information in the specified buffer to the opened phone device at the selected data ID.

#### Parameters

DRV_REQUESTID    dwRequestID

Specifies the identifier of the asynchronous request.

HDRVPHONE    hdPhone

Specifies the handle to the phone into which data is to be downloaded.

DWORD    dwDataID

Specifies where in the phone device the buffer is to be downloaded.

LPVOID    const lpData

Specifies a far pointer to the memory location where the data is to be downloaded from.

DWORD    dwSize

Specifies the size of the buffer in bytes.

#### Return Value

Returns dwRequestID, or a negative error number if an error has occurred. The lResult actual parameter of the corresponding ASYNC_COMPLETION is zero if the function is successful or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALDATAID

The specified data ID is invalid or is not at least dwSize bytes in size.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	The function downloads a maximum of dwSize bytes from lpData to the phone device. The format of the data, its meaning to the phone device, the meaning of the data ID are service provider specific. The data in the buffer or the selection of a data ID may act as commands to the phone device.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneGetData

Data Types:  PHONECAPS

### TSPI_phoneSetDisplay

#### Syntax

LONG TSPI_phoneSetDisplay(dwRequestID, hdPhone, dwRow, dwColumn, lpsDisplay, dwSize)

This function causes the specified string to be displayed on the specified open phone device.

#### Parameters

DRV_REQUESTID    dwRequestID

Specifies the identifier of the asynchronous request.

HDRVPHONE    hdPhone

Specifies the handle to the phone on which the string is to be displayed.

DWORD    dwRow

Specifies the row on the display where the new text is to be displayed.

DWORD    dwColumn

Specifies the column position on the display where the new text is to be displayed.

LPCSTR    lpsDisplay

Specifies a far pointer to the memory location where the display content is stored. The application should have stored the display information in the format specified as dwStringFormat in the phone's PHONECAPS structure.

DWORD    dwSize

Specifies the size in bytes of the information pointed to by lpDisplay.

#### Return Value

Returns dwRequestID, or a negative error number if an error has occurred. The lResult actual parameter of the corresponding ASYNC_COMPLETION is zero if the function is successful or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_INVALPARAM

The row or column parameters are invalid or out of range.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	The specified display information is written to the phone's display, starting at the specified positions. This operation overwrites previously displayed information. If the amount of information exceeds the size of the display, the information will be truncated. The amount of information that can be displayed is at most (dwNumRows * dwNumColumns) elements in size. dwNumRows and dwNumColumns are available in the PHONECAPS structure returned by TSPI_phoneGetDevCaps; they are zero-based.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneGetDisplay

Data Types:  PHONECAPS

### TSPI_phoneSetGain

#### Syntax

LONG TSPI_phoneSetGain(dwRequestID, hdPhone, dwHookSwitchDev, dwGain)

This function sets the gain of the microphone of the specified hook switch device to the specified gain level.

#### Parameters

DRV_REQUESTID    dwRequestID

Specifies the identifier of the asynchronous request.

HDRVPHONE    hdPhone

Specifies the handle to the phone containing the hook switch device whose gain is to be set.

DWORD    dwHookSwitchDev

Identifies the hook switch device whose microphone's gain is to be set. This parameter can have only a single bit set. Values are:

PHONEHOOKSWITCHDEV_HANDSET

This is the phone's handset.

PHONEHOOKSWITCHDEV_SPEAKER

This is the phone's speakerphone or adjunct.

PHONEHOOKSWITCH_HEADSET

This is the phone's headset.

DWORD    dwGain

Specifies a far pointer to a DWORD sized location containing the desired new gain setting of the device. This is a number in the range 0x00000000 (which is silence) to 0x0000FFFF (which is maximum volume). The actual granularity and quantization of gain settings in this range are service provider specific. A value for dwGain that is out of range is clamped to the nearest in-range value.

#### Return Value

Returns dwRequestID, or a negative error number if an error has occurred. The lResult actual parameter of the corresponding ASYNC_COMPLETION is zero if the function is successful or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_INVALHOOKSWITCHDEV

The hookswitch device parameter is invalid, or not supported by the phone or service provider.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	None.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneSetGain

Data Types:  PHONECAPS

### TSPI_phoneSetHookSwitch

#### Syntax

LONG TSPI_phoneSetHookSwitch(dwRequestID, hdPhone, dwHookSwitchDevs, dwHookSwitchMode)

This function sets the hook state of the specified open phone's hookswitch devices to the specified mode. Only the hookswitch state of the hookswitch devices listed is affected.

#### Parameters

DRV_REQUESTID    dwRequestID

Specifies the identifier of the asynchronous request.

HDRVPHONE    hdPhone

Specifies the handle to the phone containing the hookswitch devices whose modes are to be set.

DWORD    dwHookSwitchDevs

Specifies the device(s) whose hookswitch mode is to be set. Values are:

PHONEHOOKSWITCHDEV_HANDSET

This is the phone's handset.

PHONEHOOKSWITCHDEV_SPEAKER

This is the phone's speakerphone or adjunct.

PHONEHOOKSWITCHDEV_HEADSET

This is the phone's headset.

DWORD    dwHookSwitchMode

Specifies the hookswitch mode to set. This parameter can have only a single bit set. Values are:

PHONEHOOKSWITCHMODE_ONHOOK

The device's microphone and speaker are both onhook.

PHONEHOOKSWITCHMODE_MIC

The device's microphone is active, the speaker is mute.

PHONEHOOKSWITCHMODE_SPEAKER

The device's speaker is active, the microphone is mute.

PHONEHOOKSWITCHMODE_MICSPEAKER

The device's microphone and speaker are both active.

#### Return Value

Returns dwRequestID or a negative error number if an error has occurred. The lResult actual parameter of the corresponding ASYNC_COMPLETION is zero if the function is successful or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified phone device handle is invalid.

·	PHONEERR_INVALHOOKSWITCHDEV

The hookswitch device parameter is invalid.

·	PHONEERR_INVALHOOKSWITCHMODE

The hookswitch mode parameter is invalid or not supported by the service provider or phone.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	The hookswitch mode is changed to the specified setting for all devices specified. If different settings are desired, this function can be invoked multiple times with a different set of parameters. A PHONE_STATE message is sent to the application after the hookswitch state has changed.

See Also	Functions:  TSPI_phoneGetHookSwitch

Messages:  PHONE_STATE

Data types:  PHONESTATUS, PHONEHOOKSWITCHDEV, PHONEHOOKSWITCHMODE

### TSPI_phoneSetLamp

#### Syntax

LONG TSPI_phoneSetLamp(dwRequestID, hdPhone, dwButtonLampID, dwLampMode)

This function causes the specified lamp to be set on the specified open phone device in the specified lamp mode.

#### Parameters

DRV_REQUESTID    dwRequestID

Specifies the identifier of the asynchronous request.

HDRVPHONE    hdPhone

Specifies the handle to the phone phone whose lamp is to be set.

DWORD    dwButtonLampID

Identifies the button whose lamp is to be set.

DWORD    dwLampMode

Specifies how the lamp is to be lit. The dwLampMode parameter can only have a single bit set. Values are:

PHONELAMPMODE_BROKENFLUTTER

Broken flutter is the superposition of flash and flutter.

PHONELAMPMODE_FLASH

Flash means slow on and off.

PHONELAMPMODE_FLUTTER

Flutter means fast on and off.

PHONELAMPMODE_OFF

The lamp is off.

PHONELAMPMODE_STEADY

The lamp is continuously lit.

PHONELAMPMODE_WINK

The lamp is winking.

#### Return Value

Returns dwRequestID, or a negative error number if an error has occurred. The lResult actual parameter of the corresponding ASYNC_COMPLETION is zero if the function is successful or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALBUTTONLAMPID

The specified button/lampID is out of range or not supported by the phone or service provider.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_INVALLAMPMODE

The specified lamp mode parameter is invalid or not supported by the phone or service provider..

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	None.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneGetLamp

Data Types:  PHONELAMPMODE

### TSPI_phoneSetRing

#### Syntax

LONG TSPI_phoneSetRing(dwRequestID, hdPhone, dwRingMode, dwVolume)

This function rings the specified open phone device using the specified ring mode and volume.

#### Parameters

DRV_REQUESTID    dwRequestID

Specifies the identifier of the asynchronous request.

HDRVPHONE    hdPhone

Specifies the handle to the phone to be rung.

DWORD    dwRingMode

Specifies the ringing pattern with which to ring the phone.

DWORD    dwVolume

Specifies the volume level with which the phone is to be rung. This is a number in the range 0x00000000 (which is silence) to 0x0000FFFF (which is maximum volume). The actual granularity and quantization of volume settings in this range are service provider specific. A value for dwVolume that is out of range is clamped to the nearest value in range.

#### Return Value

Returns dwRequestID or a negative error number if an error has occurred. The lResult actual parameter of the corresponding ASYNC_COMPLETION is zero if the function is successful or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_INVALRINGMODE

The ring mode parameter is invalid or not supported by the phone or service provider.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	The service provider defines the actual audible ringing patterns corresponding to each of the phone's ring modes.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneGetRing

Data Types:  PHONECAPS

### TSPI_phoneSetStatusMessages

#### Syntax

LONG TSPI_phoneSetStatusMessages(dwRequestID, hdPhone, dwPhoneStates, dwButtonModes, dwButtonStates)

This operation causes the service provider to filter out status messages that are not currently of interest to any application.

#### Parameters

DRV_REQUESTID    dwRequestID

Specifies the identifier of the asynchronous request.

HDRVPHONE    hdPhone

Specifies the opaque handle to the phone whose state-change monitoring filter is to be set.

DWORD    dwPhoneStates

These flags specify the set of phone status changes and events for which TAPI.DLL wishes to receive notification messages. Values are:

PHONESTATE_OTHER

A non-specified phone status information item has changed.

PHONESTATE_CONNECTED

The connection between the phone device and the service provider was just made. This happens when the API is first invoked, or when the wire connecting the phone to the PC is plugged in with the API active.

PHONESTATE_DISCONNECTED

The connection between the phone device and the service provider was just broken. This happens when the wire connecting the phone set to the PC is unplugged while the service provider is active.

PHONESTATE_DISPLAY

The display of the phone has changed.

PHONESTATE_LAMP

A lamp of the phone has changed.

PHONESTATE_REINIT

Items have changed in the configuration of phone devices. To become aware of these changes (as with the appearance of new phone devices) the application should reinitialize its use of TAPI. If specified, the htPhone parameter is left NULL for this state change as it applies to any of the phones in the system.

The PHONEDEVSTATE_REINIT message is never masked. REINIT messages should always be sent when appropriate, regardless of the setting of this flag. The setting of the REINIT flag is simply ignored and generates no error.

PHONESTATE_RINGMODE

The ring mode of the phone has changed.

PHONESTATE_RINGVOLUME

The ring volume of the phone has changed.

PHONESTATE_HANDSETHOOKSWITCH

The handset hookswitch state has changed.

PHONESTATE_HANDSETVOLUME

The handset's speaker volume setting has changed.

PHONESTATE_HANDSETGAIN

The handset's microphone gain setting has changed.

PHONESTATE_SPEAKERHOOKSWITCH

The speakerphone's hookswitch state has changed.

PHONESTATE_SPEAKERVOLUME

The speakerphone's speaker volume setting has changed.

PHONESTATE_SPEAKERGAIN

The speakerphone's microphone gain setting state has changed.

PHONESTATE_HEADSETHOOKSWITCH

The headset's hookswitch state has changed.

PHONESTATE_HEADSETVOLUME

The headset's speaker volume setting has changed.

PHONESTATE_HEADSETGAIN

The headset's microphone gain setting has changed.

PHONESTATE_SUSPEND

TAPI.DLL's use of the phone is temporarily suspended. This may occur when the switch requires exclusive use of the phone device.

PHONESTATE_RESUME

TAPI.DLL's use of the phone device is resumed after having been suspended for some time.

PHONESTATE_DEVSPECIFIC

The phone's device-specific information has changed.

DWORD    dwButtonModes

These flags specify the set of phone button modes for which TAPI.DLL wishes to receive notification messages. Note that if dwButtonModes is 0, then dwButtonStates is ignored. If dwButtonModes has at least one flag set, then dwButtonStates must have at least one bit set. Values are:

PHONEBUTTONMODE_CALL

The button is assigned to a call appearance.

PHONEBUTTONMODE_FEATURE

The button is assigned to requesting features from the switch, such as hold, conference, or transfer.

PHONEBUTTONMODE_KEYPAD

The button is one of the twelve keypad buttons, '0' through '9', '*', and '#'.

PHONEBUTTONMODE_LOCAL

The button is a local function button, such as mute or volume control.

PHONEBUTTONMODE_DISPLAY

The button is a "soft" button associated with the phone's display. A phone set can have zero or more display buttons.

DWORD    dwButtonStates

These flags specify the set of phone button state changes for which TAPI.DLL wishes to receive notification messages. Values are:

PHONEBUTTONSTATE_UP

The button is in the "up" state.

PHONEBUTTONSTATE_DOWN

The button is in the "down" state (pressed down).

#### Return Value

Returns zero if the function is successful, or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified line device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The specified phone states parameter is invalid.

·	PHONEERR_INVALBUTTONMODE

The button mode parameter is invalid.

·	PHONEERR_INVALBUTTONSTATE

The button states parameter is invalid.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	The TAPI defines a number of messages that notify applications about events occurring on phones. The sets of all change messages that all applications are interested may be much smaller than the set of possible messages. This procedure allows TAPI.DLL to tell the service provider the reduced set of messages that should be delivered. The service provider must deliver at least this set. It is permitted to deliver more, but it is discouraged from doing so for performance reasons. All phone status messages except for PHONESTATE_REINIT are disabled by default.

See Also	Messages:  PHONE_STATE, PHONE_BUTTON

Data Types: PHONESTATE

### TSPI_phoneSetVolume

#### Syntax

LONG TSPI_phoneSetVolume(dwRequestID, hdPhone, dwHookSwitchDev, dwVolume)

This function either sets the volume of the speaker component of the specified hookswitch device to the specified level.

#### Parameters

DRV_REQUESTID    dwRequestID

Specifies the identifier of the asynchronous request.

HDRVPHONE    hdPhone

Specifies the handle to the phone containing the speaker whose volume is to be set.

DWORD    dwHookSwitchDev

Identifies the hook switch device whose speaker's volume is to be set.

PHONEHOOKSWITCH_HANDSET

This is the phone's handset.

PHONEHOOKSWITCH_SPEAKER

This is the phone's speakerphone or adjunct.

PHONEHOOKSWITCH_HEADSET

This is the phone's headset.

DWORD    dwVolume

A DWORD specifying the new volume level of the hookswitch device. This is a number in the range 0x00000000 (which is silence) to 0x0000FFFF (which is maximum volume). The actual granularity and quantization of volume settings in this range are service provider specific. A value for dwVolume that is out of range is clamped to the nearest value in range.

#### Return Value

Returns dwRequestID or a negative error number if an error has occurred. The lResult actual parameter of the corresponding ASYNC_COMPLETION is zero if the function is successful or a negative error number if an error has occurred. Possible error returns are as follows:

·	PHONEERR_INVALPHONEHANDLE

The specified device handle is invalid.

·	PHONEERR_INVALPHONESTATE

The phone device is not in a valid state for the requested operation.

·	PHONEERR_INVALHOOKSWITCHDEV

The hookswitch dev parameter is invalid or not supported by the phone or service provider.

·	PHONEERR_NOMEM

Unable to allocate or lock memory.

·	PHONEERR_RESOURCEUNAVAIL

There are not enough resources available to complete the request.

·	PHONEERR_OPERATIONFAILED

The specified operation failed for unspecified reasons.

·	PHONEERR_OPERATIONUNAVAIL

The specified operation is not available.

Comments	None.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneSetVolume

Data Types:  PHONECAPS