Chicago Win32 Communications API

Modem Configuration Extensions

(MCX)

NOTICE:

This is a preliminary API specification subject to change in future releases of Chicago Windows.  Your feedback is appreciated.

MICROSOFT CONFIDENTIAL

This specification is provided under the terms of the non-disclosure agreement between your company and Microsoft Corporation.  Keep all information contained in this document confidential.

Copyright © 1993, Microsoft Corporation.  All Rights Reserved.

Date:	November 15, 1993

Revision:		0.8

Revision History

Revision 0.8	(20 October 1993)

·	Major content revisions

·	Deleted references to Get/SetDefaultCommConfig

·	uuidModemProvider changed to commExtID

·	Most LocalModemOptions fields deleted (MODEMDATAMODE_..., MODEMV42_...)

·	V42BufferedDataDeliveryTimeout, V42MaxRetries fields deleted

·	Preferred/NegotiatedModemOptions fields deleted

·	Modulation definitions simplified, reduced to common industry standards only

·	Added fields Enabled/NegotiatedErrorControl and DataCompression

·	All Preferred/NegotiatedV42... fields deleted

·	SHORT fields changed to DWORD

Revision 0.7	(08 July 1993)

·	Reformatting only, no content changes.

Revision 0.6	(29 March 1993)

·	Minor editorial revisions prior to first external distribution.

Revision 0.5	(22 March 1993)

·	In 1.1, described Comm API configuration functions used by MCX.

·	Added section 1.4, "Interface Overview".

·	Added section 1.5, "Related Documents".

·	Added COMMPROP and COMMCONFIG data structure descriptions to section 2, "Data Structures".

·	Converted to standard Digital Office Systems document template.

Revision 0.4	(19 March 1993)

·	Removed Function calls; changed document to a specification of data structures that will be passed to and from a VCOMM port driver.

·	Since VCOMM has only "GetConfig" and "SetConfig" calls (and not separate "preferences" and "negotiated results" functions), expanded MODEMSETTINGS to contain both the preferred and negotiated settings as separate fields.

·	Expanded MODEMDEVCAPS to include indication as to whether the reporting of negotiated results is meaningful (some modems are unable to report the results of parameter negotiation).

Revision 0.3	(17 March 1993)

·	Clarified that end-to-end data format and parity are settable through COMM DCB.

·	Clarified that end-to-end use of XON/XOFF flow control is settable through COMM DCB.

·	Divided modem modulations into three fields (two for CCITT modulations, one for proprietary) and fleshed out with as many as could be thought of.

·	Added speaker mode and volume (so the modem doesn't have to be treated like a “phone” to control these).

·	Clarified meaning of various options bits in Get/SetPreferences versus GetParameters.

·	Added MNP10 to modem options.

Table of Contents

µ1.	Introduction	1

1.1.	What it is	1

1.2.	What it is not	2

1.3.	Interface Overview	5

1.4.	Related Documents	7

2.	Data Types	8

2.1.	COMMPROP	8

2.2.	COMMCONFIG	8

2.3.	MODEMDEVCAPS	9

2.4.	MODEMSETTINGS	12

2.5.	LOCALMODEMOPTIONS	15

2.6.	MODEMMODULATIONS	16

2.7.	MODEMERRORCONTROL	18

2.8.	MODEMCOMPRESSION	19

# 1	Introduction

In pursuit of a comprehensive solution to the needs of communicating Windows applications, Microsoft has developed, and continues to develop, a series of application programming interfaces under the Windows Open Services Architecture to present hardware-independent, flexible, and easy-to-program communications capabilities. One of these interfaces is the Telephony API, which defines a set of first-party, desktop-focused, basic circuit-switched call control services with a modicum of associated media-identification and media-control capabilities. Continuing study of the related issues identified a need for additional capabilities in the area of configuration of certain features of modems.

The Telephony API does not define the functions for the flow of data (defined as "media streams" in Telephony API). Instead, existing APIs for access to data will be used, such as the Windows Communications API for serial data.

## 2	What it is

This document describes the Modem Configuration Extension (MCX) to the Win32 Communications API. The Comm API  defines classes of communications device drivers, including modems, and allows applications to configure parameters specific to each class through API functions and driver-supplied user interface dialogs.    The Comm API functions used by MCX to configure modem devices are as follows:

·	GetCommProperties:  This function is used to determine the capabilities of the modem function (combined modem service provider and hardware).

·	GetCommConfig:  This function is used to determine the operating characteristics of a modem device for a connection in progress, if any, and/or the preferred operating characteristics for the next connection to be made using the current device handle.  It reflects the result of modem-to-modem parameter negotiation while a connection is in progress or immediately after a connection has been dropped.

·	SetCommConfig:  This function is used to set the operating characteristics of a modem device for a connection in progress, if any, and/or the preferred operating characteristics for the next connection to be made using the current device handle.

·	CommConfigDialog:  This function is used to cause the modem service provider to display a preferences dialog to the user.

The syntax and operation of these functions are documented in the Win32 omm API specifications for Chicago. MCX defines the set of data structures used by these functions for configuring the features of data modems which relate to the end-to-end connectivity across the telephone channel. The configuration of these features conceivably changes on a call-by-call basis, and thus is potentially of interest to application programs (because of impact on higher-layer protocols, performance management and estimation, etc.).  These features include:

Primary mode of operation (async, sync, error control)

Modulation and carrier speed

V.42 error control (including specific parameters)

V.42 bis and MNP5 data compression

Call setup, inactivity, and buffered data delivery timers

In addition, MCX provides to the application the ability to identify the specific modem service provider/port driver to which it is attached (to take advantage of proprietary extensions), to identify the type of modem and related information, and to determine the capabilities of the modem.

## 3	What it is not

MCX does not provide for control or configuration of modem features and functions which are addressed in other APIs, which are "static" and therefore more properly controlled in various service-provider configuration dialogs and setup files, or which are strictly related to the interface between the service provider/port driver and the local modem. Specifically, this document does not address the following:

Management and Maintenance functions which are not typically invoked by applications, which are used only on a occassional basis, and which may be implemented in a modem service provider applet or dialog, such as:

Reset to stored configuration (Zn)

Select power-up parameter set (&Y)

Set to factory-defined configuration (&Fn)

Store current configuration (&W)

Store number to dialing directory (&Z)

Request list of stored number directory (&V)

Request product information (I)

Request configuration summary (&V)

Diagnostic test control

Return to online state and retrain (O1)

Leased line operation (&L)

Transmit level (S91)

Local DTE Interface options which are typically set once at installation time (through a modem service provider dialog and a communications port service provider dialog), which are typically not used directly by applications, which are hidden within the modem service provider/port driver (controlled directly by it for its interface to the modem), or are controlled indirectly through TAPI or other APIs, such as:

Command line termination character (S3)

Response formatting character (S4)

Command line editing character (S5)

Command echo (En)

Response suppression (Qn)

Response format (Vn)

Escape character (S2)

Escape sequence guard time (S12)

Escape to command state (+++)

Return to online state (O)

Response set control (X)

DTE rate ("A" of AT, fixed rate, or maximum limit)

DTE parity and format for responses ("T" of AT)

(parity and format for user data controllable through COMM DCB)

Synchronous clock source (&X)

DCD behavior (&C)

DTR behavior (&D)

CTS behavior (&R)

DSR behavior (&S)

DTR hysteresis (S25)

RTS/CTS delay (S26)

Local flow control type (&K)

(End-to-end XON/XOFF flow control while online is set through

the COMM DCB)

Local flow control high and low water marks (S49, S50)

Specific syntax of events and status reports

Telephony Interface functions, which are controlled through the Telephony API or by the TAPI service provider setup dialog and translated by the service provider/port driver into the appropriate modem commands, such as:

Dial (D) including dial string modifiers

Answer (A)

Hang up (H0)

Go off hook without dialing (H1)

Default dialing mode (T/P)

Automatic answer and ring to answer on (S0)

Pause before blind dialing (S6)

Comma pause time (S8)

DTMF Dialing speed (S11)

Busy and dial tone detection (Xn)

Pulse dialing make/break ratio

Pulse dialing rate

Jack type (&J)

MI/MIC operation (S92)

Incoming call indication (RING)

Remote ring (REMOTE RING)

Connection lost (NO CARRIER)

Stable Modem Settings which control behavior on the telephone line and are typically set once (by national regulation, by the factory, or sometimes by the user) and not changed on a call-by-call basis, such as:

Data calling tone

Carrier loss recovery time (S9)

Automatic disconnect delay (S10)

Long space disconnect (Yn)

Guard tone selection (&Gn)

Auto retrain (&Bn)

## 4	Interface Overview

The following diagram illustrates the relationship and interfaces between modem applications, APIs, drivers, and devices:

The elements of this diagram are as follows:

·	Win32 Comm Applications:  These are applications written to Chicago's Win32 API which use modem devices.  These might be terminal emulators, MAPI service providers, fax applications, etc.  The Telephony and Comm APIs allow these applications to use modems in a device-independent fashion.

·	Telephony API:  Applications will call this to cause modems to dial, answer, hang up, etc.  The Telephony API routes application function calls to the appropriate "service provider" DLL for a modem.

·	Comm API:  Applications will call this to configure modems and perform data I/O through them.  This MCX specification describes the comm API functions and data structures that are used to configure modems.  Note that this diagram does not show the VxD-level "VCOMM" layer, which is not relevant to Win32 comm applications.  In reality, VCOMM sits between the Windows Comm API and modem device drivers, and provides a VxD-level version of the comm API.

·	Modem Service Providers:  These DLLs perform device-specific control of modem devices when called by the Telephony API to dial, answer, hang up, etc.  Modem service providers are also called by the Comm API to display a dialog window allowing the user to configure modem parameters.

·	Modem Device Drivers:  These VxDs are called by their corresponding modem service providers to dial, answer, hang up, etc.  Modem device drivers conform to the Chicago "VCOMM" communications device driver format, and are called by the Comm API to configure modem parameters and send & receive data. The interface between the modem service provider and the modem device driver for exchange of control and status information related to the Telephony API/SPI functions is not defined in this document; that interface is considered “internal” to the service provider, and therefore private.

·	Modem Devices:  These may be modem devices of all types:  internal (ISA, EISA, MCA, etc.), PCMCIA cards, external RS-232, etc.  They may support data calls, fax, voice, etc.  They may or may not be UART compatible.

The diagram shows four paths connecting Win32 comm applications to underlying APIs, drivers, and devices.  These are labeled Control, User Interface, Configuration, and Data I/O, and are meaningful as follows:

·	Control:  This refers to calls made by an application to the Telephony API to control connections made by modem devices.  This includes dialing outbound calls, waiting for inbound calls of a particular type, answering inbound calls, hang up of current calls, etc.  The Telephony API routes control function calls to the appropriate service provider DLL, which in turn calls the modem device driver (through an internal interface not defined in this document) to cause the modem to dial, answer, etc.

·	User Interface:  These are calls made to the Comm API by an application to let the user select the preferred operating characteristics of a modem.  A dialog may include modem parameters such as speaker on/off, baud rate, compression & error correction protocols, etc.  This is similar to specifying printing preferences in a printer driver's "Printer Setup" dialog window.

·	Configuration:  These are the MCX calls made by an application to the Comm API to determine or change preferred and current operating characteristics of a modem.  The Comm API routes configuration function calls to the appropriate modem device driver VxD, which changes the actual hardware settings of the modem.

·	Data I/O:  These are calls made by an application to the Comm API to send and receive data through a modem device.  The Comm API routes these call to the appropriate modem device driver VxD, which performs actual I/O to the modem.

One important aspect of modem device drivers should be noted here.  Many modems are external, attaching to standard serial COM ports or other types of communications adapters.  Or, they may be internal but emulate standard serial UARTs.  In such cases, a modem device driver is expected to call back into the the VxD-level comm API provided by VCOMM.386 to talk through a serial port driver to the modem device.  This is illustrated by the following diagram:

## 5	Related Documents

·	Windows Telephony API Specification:  Documents the API used by applications to dial and answer calls using modems and other telephone devices.

·	Windows Telephony SPI Specification:  Documents the interface between the Telephony API and service providers.

·	Chicago Win32Programming Reference, Communications Functions:  Documents the Communications API for Chicago applications.

·	Device Development Kit (DDK), VCOMM Overview:  Specifies the format of modem and other communications device drivers for Chicago.

# 6	Data Types

This section describes data structures used by MCX and modem device drivers.

## 7	COMMPROP

All generic COMMPROP fields must be set appropriate to the device.  See the COMMPROP documentation for complete details about the fields of this structure.  The following fields require particular values for modem devices:

Member	Description

dwServiceMask	The SP_SERIALCOMM bit is always set for modem providers.

dwProvSubType	Contains PST_DATA_MODEM for data modem devices.

dwProvSpec1	Not used.

dwProvSpec2	Not used.

wcProvChar	Contains a MODEMDEVCAPS structure.

## 8	COMMCONFIG

All generic COMMCONFIG fields must be set appropriate to the device.  See the COMMCONFIG documentation for complete details about the fields of this structure.  The following fields require particular values for modem devices:

Member	Description

pProviderName	Must be "MODEMSETTINGS".

wcProviderData	Contains a MODEMSETTINGS structure.

## 9	MODEMDEVCAPS

typedef struct modemdevcaps_tag {

DWORD		ActualSize;

DWORD		RequiredSize;

DWORD		DevSpecificOffset;

DWORD		DevSpecificSize;

// product and version identification

DWORD		dwLineDeviceID;

UUID		commExtID;// 128 bits

DWORD		dwModemProviderVersion;

DWORD		ModemManufacturerOffset;

DWORD		ModemManufacturerSize;

DWORD		ModemModelOffset;

DWORD		ModemModelSize;

DWORD		ModemVersionOffset;

DWORD		ModemVersionSize;

// local option capabilities

DWORD		LocalModemOptions;			// bitmap

DWORD		CallSetupFailTimer; 			// seconds

DWORD		InactivityTimeout; 			// seconds

DWORD		SpeakerVolume;				// levels

DWORD		SpeakerMode;				// modes

// negotiable option capabilities

DWORD		Modulations;				// bitmap

DWORD		ErrorControl;				// bitmap

DWORD		Compression;				// bitmap

DWORD		MaxDTERate;					// bit/s

DWORD		MaxDCERate;					// bit/s

// Variable portion for proprietary expansion

BYTE		abVariablePortion []

} MODEMDEVCAPS;

The MODEMDEVCAPS structure contains the following fields:

ActualSize

RequiredSize

ActualSize indicates the number of bytes occupied by the data returned to the application. RequiredSize indicates the number of bytes needed to store all of the information. If the application had allocated more than sufficient space, ActualSize will be less than the Size parameter and equal to RequiredSize. If the application allocated exactly enough space, all three will be equal. If the application allocated insufficient space, ActualSize will be equal to Size, and RequiredSize will indicate the total space need; the application may choose to reallocate the space and call the function again to retrieve all of the variable portion (note that if Size was not sufficient to contain even the fixed portion of the structure, the function fails and no information is returned, not even ActualSize and RequiredSize).

DevSpecificOffset

DevSpecificSize

Specifies the offset from the beginning of the fixed portion of the structure to the beginning of, and the length of, a provider-defined series of bytes. The application must identify the provider using uuidModemProvider in order to know the interpretation of these bytes.

dwLineDeviceID

Specifies the TAPI permanent line device ID of the line with which the modem is associated.

commExtID

TBD

dwModemProviderVersion

Version information which may be formatted and used as deemed fit by the provider.

ModemManufacturerOffset

ModemManufacturerSize

Specifies the offset from the beginning of the fixed portion of the structure to the beginning of, and the length of, a text string (not zero-terminated) giving the name of the manufacturer of the attached modem, if the provider could determine the information.

ModemModelOffset

ModemModelSize

Specifies the offset from the beginning of the fixed portion of the structure to the beginning of, and the length of, a text string (not zero-terminated) giving the model of the attached modem, if the provider could determine the information.

ModemVersionOffset

ModemVersionSize

Specifies the offset from the beginning of the fixed portion of the structure to the beginning of, and the length of, a text string (not zero-terminated) giving the version and revision of the attached modem, if the provider could determine the information.

LocalModemOptions

Each bit in this field indicates a feature or capability possessed by the modem function (combination of the service provider and the physical modem). In general, if a bit is on in the MODEMDEVCAPS, it indicates that the provider and/or underlying modem is able to provide that service and that the corresponding bit in the same field in the MODEMSETTINGS structure may be set.

CallSetupFailTimer

InactivityTimeout

SpeakerVolume

SpeakerMode

In each case, indicates the maximum value which may be stored in the corresponding field in the MODEMSETTINGS structure (see the descriptions in the MODEMSETTINGS structure for explanations of the individual features). If a larger value is set in MODEMSETTINGS, the maximum value indicates in MODEMDEVCAPS will be assumed.

Modulations

ErrorControl

Compression

Each bit set in these fields indicates a modulation, error control, or data compression scheme supported by the modem.

MaxDTERate

MaxDCERate

Specifies the maximum DTE and DCE rates supported by the modem, respectively.

abVariablePortion

An array of bytes used to contain the contents of strings pointed to by other fields in the fixed portion of the structure.

## 10	MODEMSETTINGS

typedef struct modemsettings_tag {

DWORD		ActualSize;

DWORD		RequiredSize;

DWORD		DevSpecificOffset;

DWORD		DevSpecificSize;

// static local options (read/write)

DWORD		LocalModemOptions;			// bitmap

DWORD		CallSetupFailTimer; 			// seconds

DWORD		InactivityTimeout; 			// seconds

DWORD		SpeakerVolume;				// levels

DWORD		SpeakerMode;				// modes

// preferences (initial negotiating position; read/write)

DWORD		EnabledModulations;			// bitmap

DWORD		EnabledErrorControl;			// bitmap

DWORD		EnabledCompression;			// bitmap

// negotiated options (read only) for current or last call

DWORD		NegotiatedModulations;			// bitmap

DWORD		NegotiatedErrorControl;			// bitmap

DWORD		NegotiatedCompression;			// bitmap

DWORD		NegotiatedDCERate;			// bit/s

// Variable portion for proprietary expansion

BYTE		abVariablePortion []

} MODEMSETTINGS;

The MODEMSETTINGS structure contains the following fields:

ActualSize

RequiredSize

uActualSize indicates the number of bytes occupied by the data returned to the application. uRequiredSize indicates the number of bytes needed to store all of the information. If the application had allocated more than sufficient space, uActualSize will be less than the Size parameter and equal to uRequiredSize. If the application allocated exactly enough space, all three will be equal. If the application allocated insufficient space, uActualSize will be equal to Size, and uRequired Size will indicate the total space need; the application may choose to reallocate the space and call the function again to retrieve all of the variable portion (note that if Size was not sufficient to contain even the fixed portion of the structure, the function fails and no information is returned, not even uActualSize and uRequiredSize).

DevSpecificOffset

DevSpecificSize

Specifies the offset from the beginning of the fixed portion of the structure to the beginning of, and the length of, a provider-defined series of bytes. The application must identify the provider using uuidModemProvider in order to know the interpretation of these bytes.

LocalModemOptions

See the description of the LOCALMODEMOPTIONS values for the meanings of the individual bits in this field. It controls options which are selected at the local modem and are not negotiated between the modems.

CallSetupFailTimer

Specifies the maximum number of seconds from the completion of dialing of the call that the modem should wait for an indication that the modem-to-modem connection has been established. If the connection is not established within this time, the call is assumed to have failed and will most likely be automatically terminated and placed on hook (with an appropriate DROPPED indication from TAPI). Normal equivalent parameter in Hayes-compatible modems is register S7.

InactivityTimeout

After the connection is established, if the number of seconds specified by this field passes with no data being either transmitted or received, the call will be automatically dropped. Used to prevent running up long distance charges or online service charges if the application unexpectedly locks up or the user walks away.

SpeakerVolume

Indicates the level of the monitor speaker when the speaker is on. The actual meaning of the levels is hardware specific; however, 0 and 1 are usually synonymous and refer to the lowest level, and increasing values mean increasing loudness. The application can present a sliding control or other UI element, based on the maximum value indicated in MODEMDEVCAPS.

SpeakerMode

Indicates when the speaker should be on. Valid values include:

SPEAKERALWAYSOFF		0

SPEAKERONUNTILCONNECT		1

SPEAKERALWAYSON			2

SPEAKEROFFWHILEDIALING		3

Value 3 is the same as 1, but the speaker is turned off when the modem is actually dialing (to prevent load DTMF from filling the room).

EnabledModulations

EnabledErrorControl

EnabledCompression

See the description of the MODEMMODULATIONS, MODEMERRORCONTROL, and MODEMCOMPRESSION values for the meanings of the individual bits in these fields. Multiple bits may be on in these fields, enabling acceptance of any of those modulations (the modems will automatically negotiate to the highest common speed if the MODEMAUTOMODE_ENABLE bit is on in LocalModemOptions).

NegotiatedModulations

NegotiatedErrorControl

NegotiatedCompression

See the description of the MODEMMODULATIONS, MODEMERRORCONTROL, and MODEMCOMPRESSION values for the meanings of the individual bits in these fields. Only one bit will be on in each of these three fields, indicating the modulation, error control, and data compression schemes in use on the current or immediately preceding call.

NegotiatedDCERate

Specifies the connect speed being used for transmission to the remote modem on the current or immediately preceding call.  This will be less than or equal to MaxDCERate in MODEMDEVCAPS.

abVariablePortion

An array of bytes used to contain any proprietary provider-specific information pointed to by uDevSpecificOffset and delimited by uDevSpecificSize.

## 11	LOCALMODEMOPTIONS

The following constants describe the values which can be set in the LocalModemOptions field. All refer to options which are controlled locally and not negotiated between the modems.

MODEMROLE_ANSWERER

MODEMROLE_ORIGINATOR

The meanings of each of these bits is as follows:

MODEMROLE_ANSWERER

MODEMROLE_ORIGINATOR

These bits control the role the modem is to assume on the call. If both bits are set to 0 or both are set to 1, then the modem assumes the role implied by the mechanism used to establish the call through the Telephony API (lineMakeCall or lineAnswer). If only one of these bits is set to 1, then the modem is forced into that role (which is useful when a call is switched from voice to data and the role is based on mutual agreement between the parties rather than by the way that the call was set up.

## 12	MODEMMODULATIONS

The following constants describe the values which can be set in the EnabledModulations and NegotiatedModulations fields.

MODEMMODULATION_V21

MODEMMODULATION_V22

MODEMMODULATION_V22BIS

MODEMMODULATION_V23_TX

MODEMMODULATION_V23_RX

MODEMMODULATION_V26TER

MODEMMODULATION_V32

MODEMMODULATION_V32BIS

MODEMMODULATION_V34

MODEMMODULATION_TDD

MODEMMODULATION_BELL103

MODEMMODULATION_BELL212

MODEMMODULATION_OTHER

The meanings of each of these bits is as follows:

MODEMMODULATION_V21

CCITT V.21 modulation at 300 bit/s.

MODEMMODULATION_V22

CCITT V.22 modulation at 600 or 1200 bit/s.

MODEMMODULATION_V22BIS

CCITT V.22 bis modulation at 2400 bit/s.

MODEMMODULATION_V23_TX

CCITT V.23 modulation with transmission at 600 or 1200 bit/s and reception at 75 bit/s.

MODEMMODULATION_V23_RX

CCITT V.23 modulation with transmission at 75 bit/s and reception at 600 or 1200 bit/s.

MODEMMODULATION_V26TER

CCITT V.26 ter modulation at 1200 or 2400 bit/s.

MODEMMODULATION_V32

CCITT V.32 modulation at 4800 or 9600 bit/s.

MODEMMODULATION_V32BIS

CCITT V.32 bis modulation at 4800, 7200, 9600, 12000, or 14400 bit/s (includes automatic rate renegotiation capability).

MODEMMODULATION_V34

CCITT V.34 (V.fast) modulation at 2400, 4800, 7200, 9600, 12000, 14400, 16800, 19200, 21600, 24400, 26400, or 28800 bit/s.

MODEMMODULATION_TDD

FSK modulation as used in North American text telephones (telecommunications devices for the deaf) at 45.45 bit/s. Some text telephones use CCITT V.21 or Bell 103 modulation, which would be indicated by the appropriate modulation bits.

MODEMMODULATION_BELL103

Bell 103 (AT&T) modulation at 300 bit/s.

MODEMMODULATION_BELL212

Bell 212 (AT&T) modulation at 600 or 1200 bit/s.

MODEMMODULATION_OTHER

Additional modulation schemes available on the modem, if any.

## 13	MODEMERRORCONTROL

The following constants describe the values which can be set in the EnabledErrorControl and NegotiatedErrorControl fields.

MODEMERRORCONTROL_NONE

MODEMERRORCONTROL_FALLBACK

MODEMERRORCONTROL_LAPM

MODEMERRORCONTROL_ALT

MODEMERRORCONTROL_MNP10

MODEMERRORCONTROL_OTHER

The meanings of each of these bits is as follows:

MODEMERRORCONTROL_NONE

No error control scheme can/should be negotiated.

MODEMERRORCONTROL_FALLBACK

If this bit is set in EnabledErrorControl, the modem will still connect even if none of the enabled error control protocols is negotiated.  Applications may want to clear this bit if they require error control for proper communications.

This bit is not used in NegotiatedCompression.

MODEMERRORCONTROL_LAPM

MODEMERRORCONTROL_ALT

V.42 protocols which can/should be reported/enabled for a call.  Note:  The V.42 Alternative Protocol ("ALT") is functionally equivalent to MNP Classes 2-4.

MODEMERRORCONTROL_MNP10

MNP10 "Adverse Channel Enhancements" (for cellular telephone connections) when the Alternative protocol is established.

MODEMERRORCONTROL_OTHER

Additional error control schemes available on the modem, if any.

## 14	MODEMCOMPRESSION

The following constants describe the values which can be set in the EnabledCompression and NegotiatedCompression fields.

MODEMCOMPRESSION_NONE

MODEMCOMPRESSION_FALLBACK

MODEMCOMPRESSION_V42BIS

MODEMCOMPRESSION_MNP5

MODEMCOMPRESSION_OTHER

The meanings of each of these bits is as follows:

MODEMCOMPRESSION_NONE

No compression scheme can/should be negotiated.

MODEMCOMPRESSION_FALLBACK

If this bit is set in EnabledCompression, the modem will still connect even if none of the enabled data compression protocols is negotiated.  Applications may want to clear this bit if they require data compression for proper communications.

This bit is not used in NegotiatedCompression.

MODEMCOMPRESSION_V42BIS

V.42 bis data compression.

MODEMCOMPRESSION_MNP5

MNP5 data compression when the Alternative protocol is established.

MODEMCOMPRESSION_OTHER

Additional compression schemes available on the modem, if any.