Chapter 1

About MCI Command Messages

VCR Command Messages

This reference defines the messages and associated structures for video cassette recorder (VCR) devices. You can use these messages and associated structures with the mciSendCommand function that supports the MCI command-message interface. For information about using command messages, see the Multimedia Programmer’s Reference in the Microsoft Windows 32 Software Development Kit.

All MCI commands use two parameters to pass information to the MCI VCR driver. The first parameter, lParam1, contains flags associated with each command that is sent. The second parameter is a long pointer to a data structure containing information needed to carry out a command, or information returned by the command.

All commands accept the optional flags MCI_NOTIFY, MCI_WAIT, and MCI_TEST flags as described in the MCI VCR command set.

The VCR driver uses the VCR keyword to identify the device type. The constant MCI_DEVTYPE_VCR is defined for this device type.

# Reference

## Command Message Reference

The VCR command set is made of the following command messages and structures:

### MCI_CLOSE

This message releases access to a device or device element. All MCI devices respond to this message.

MCI_CLOSE applies to each instance of a digital-video device driver. Closing an instance releases all resources associated with it.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_NOTIFY

Specifies that MCI should post the MM_MCINOTIFY message when this command completes. The window that receives this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the close operation should finish before MCI returns control to the application.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure. (Devices with extended command sets might replace this data structure with a device-specific data structure.

### MCI_CUE

This message prepares a device instance for playback or recording with minimum delay.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_CUE_PREROLL

Cue the device to current position, or the dwFrom position, minus the preroll duration. This will allow the device to get “up to speed” before entering record or playback mode.

MCI_VCR_CUE_REVERSE

The direction of the upcoming play or record command is reverse.

MCI_VCR_CUE_INPUT

Prepare for recording.

MCI_VCR_CUE_OUTPUT

Prepare for playing. If neither MCI_VCR_CUE_INPUT nor MCI_VCR_CUE_OUTPUT is specified, MCI_VCR_CUE_OUTPUT should be assumed.

MCI_FROM

dwFrom contains the starting position specified in the current time format.

MCI_TO

dwTo contains the ending (pausing) position specified in the current time format.

LPMCI_VCR_CUE_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_CUE_PARMS data structure.

### MCI_FREEZE

This message freezes the input or output image of the device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_FREEZE_INPUT

Freeze the input image (used for recording).

MCI_VCR_FREEZE_OUTPUT

Freeze the output image (used with frame capture).

MCI_VCR_FREEZE_FIELD

Freeze only one field of the current frame.

MCI_VCR_FREEZE_FRAME

Freeze both fields of the current frame.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_GENERIC_PARMS

The MCI_GENERIC_PARMS structure contains parameters used by the following messages for digital-video cassette recorders: MCI_FREEZE, MCI_INDEX, MCI_MARK, MCI_PAUSE, MCI_RESUME, MCI_STOP, and MCI_UNFREEZE. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_GENERIC_PARMS {
	DWORD	dwCallback;

} MCI_GENERIC_PARMS;

The MCI_GENERIC_PARMS structure has the following field:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

### MCI_GETDEVCAPS

This message obtains static information about a device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_GETDEVCAPS_ITEM

Specifies that the dwItem field of the data structure contains a constant specifying which device capability to obtain. The following constants are recognized:

MCI_GETDEVCAPS_CLOCK_INCREMENT_RATE

Sets dwReturn to the number of increments per second.

MCI_GETDEVCAPS_CAN_RECORD

Sets dwReturn to TRUE.

MCI_GETDEVCAPS_HAS_AUDIO

Sets dwReturn to TRUE.

MCI_GETDEVCAPS_HAS_VIDEO

Sets dwReturn to TRUE.

MCI_VCR_GETDEVCAPS_HAS_TIMECODE

Sets dwReturn to TRUE if the device has timecode capability or if this capability is unknown.

MCI_GETDEVCAPS_USES_FILES

Sets dwReturn to FALSE.

MCI_GETDEVCAPS_COMPOUND_DEVICE

Sets dwReturn to FALSE.

MCI_GETDEVCAPS_DEVICE_TYPE

Sets dwReturn to MCI_DEVTYPE_VCR.

MCI_GETDEVCAPS_CAN_EJECT

Sets dwReturn to TRUE if the device has programmatic eject a videocassette.

MCI_GETDEVCAPS_CAN_PLAY

Sets dwReturn to TRUE.

MCI_GETDEVCAPS_CAN_SAVE

Sets dwReturn to FALSE.

MCI_VCR_GETDEVCAPS_CAN_PREROLL

Sets dwReturn to TRUE if the device can preroll the tape in a videocassette.

MCI_VCR_GETDEVCAPS_CAN_PREVIEW

Sets dwReturn to TRUE if the device supports previewing with the record command.

MCI_VCR_GETDEVCAPS_CAN_MONITOR_SOURCES

Sets dwReturn to TRUE if the device can monitor input audio and video signals.

MCI_VCR_GETDEVCAPS_CAN_DETECT_LENGTH

Sets dwReturn to TRUE if the device can determine the length of tape in a videocassette or if this capability is not known for a device.

MCI_VCR_GETDEVCAPS_CAN_FREEZE

Sets dwReturn to TRUE if the device is capable of freezing the output image.

MCI_VCR_GETDEVCAPS_SEEK_ACCURACY

Sets dwReturn to the seek accuracy of the device.

MCI_VCR_GETDEVCAPS_HAS_CLOCK

Sets dwReturn to TRUE if the device supports an external clock.

MCI_VCR_GETDEVCAPS_CAN_REVERSE

Sets dwReturn to TRUE if the device can play a videocassette in the reverse direction.

MCI_VCR_GETDEVCAPS_NUMBER_OF_MARKS

Sets dwReturn to the maximum number of marks the device can place on a videocassette tape.

MCI_VCR_GETDEVCAPS_CAN_TEST

Sets dwReturn to TRUE.

LPMCI_GETDEVCAPS_PARMS   lParam2

Specifies a far pointer to the MCI_GETDEVCAPS_PARMS data structure.

### MCI_GETDEVCAPS_PARMS

The MCI_GETDEVCAPS_PARMS structure contains parameters used by the MCI_GETDEVCAPS message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_GETDEVCAPS_PARMS {
	DWORD	dwCallback;
	DWORD	dwReturn;
	DWORD	dwItem;

} MCI_GETDEVCAPS_PARMS;

The MCI_GETDEVCAPS_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwReturn

Contains the return information on exit.

dwItem

Identifies the capability being queried.

### MCI_INDEX

This message turns the on-screen display on or off. The information presented in the on-screen display is controlled by the set index command.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_SET_ON

Turns on-screen display on.

MCI_SET_OFF

Turns on-screen display off.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_INFO

This message obtains static string information from the device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_INFO_PRODUCT

Sets lpstrReturn to point to the product name, and also sets dwRetSize equal to the length of the string pointed to.

MCI_VCR_INFO_VERSION

Sets lpstrReturn to point to the version number, and also sets dwRetSize equal to the length of the string pointed to.

LPMCI_INFO_PARMS   lParam2

Specifies a far pointer to the MCI_INFO_PARMS data structure.

### MCI_INFO_PARMS

The MCI_INFO_PARMS structure contains parameters used by the MCI_INFO message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_INFO_PARMS {
	DWORD	dwCallback;
	LPSTR	lpstrReturn;
	DWORD	dwRetSize;
} MCI_INFO_PARMS;

The MCI_INFO_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

lpstrReturn

Specifies a long pointer to a user-supplied buffer for the return string.

dwRetSize

Specifies the size in bytes of the buffer for the return string.

### MCI_LIST

This message obtains information about the number and types of inputs available to the device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_LIST_VIDEO_SOURCE

Inquire about the types of video video inputs for the device.

MCI_VCR_LIST_AUDIO_SOURCE

Inquire about the types of audio audio inputs for the device.

MCI_VCR_LIST_COUNT

Set dwReturn to the total number of video or audio inputs.

MCI_VCR_LIST_NUMBER

Set dwReturn to the type of the video or audio input specified by dwNumber.

LPMCI_VCR_LIST_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_LIST_PARMS data structure.

### MCI_MARK

This message records or erases marks that can be used with MCI_SEEK for high-speed searches.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_MARK_WRITE

Write a mark at the current location.

MCI_VCR_MARK_ERASE

Erase a mark at the current location if one exists.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_OPEN

This MCI command message initializes an instance of the device or device element. Each open creates an independent instance of the device driver, which can be referenced by its device name. There might be device-specific restrictions on how many device instances can be opened simultaneously.

DWORD  lParam1

Parameters

The following flags apply to all devices:

MCI_NOTIFY

Post the MM_MCINOTIFY message when this command completes. The window that receives this message is specified in the dwCallback field of the data structure identified by lParam2.

MCI_WAIT

Specifies that the open operation should finish before MCI returns control to the application.

MCI_OPEN_ALIAS

Specifies that an alias is referenced in the lpstrAlias field of the data structure identified by lParam2. The alias identifies an instance of the device driver. The open is rejected if the alias specified is already in use by this application. If the flag is omitted, the default alias is the specified filename.

MCI_OPEN_TYPE

Specifies that a device-type constant or a pointer to a device-type name is included in the lpstrDeviceType field of the data structure identified by lParam2.

MCI_OPEN_TYPE_ID

Specifies that the low-order word of the lpstrDeviceType field of the data structure identified by lParam2 contains a standard MCI device type ID and the high-order word optionally contains the ordinal index for the device. Use this flag with the MCI_OPEN_TYPE flag.

LPMCI_OPEN_PARMS  lParam2

Specifies a far pointer to the MCI_OPEN_PARMS data structure. (Devices with extended command sets might replace this data structure with a device-specific data structure.)

### MCI_PAUSE

This message pauses the current action of the device.

DWORD   lParam1

Parameters

Specifies the MCI_NOTIFY, MCI_WAIT, and MCT_TEST flags.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_PLAY

This message plays the device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_FROM

dwFrom contains the starting play location specified in the current time format.

MCI_TO

dwTo contains the ending (pausing) play location specified in the current time format.

MCI_VCR_PLAY_REVERSE

The play direction is reverse.

MCI_VCR_PLAY_SCAN

Play as fast as possible while maintaining video output.

MCI_VCR_PLAY_AT

dwAt contains a time when the entire command should begin, or if the device is cued when the device should be playing across the from position given by the cue command.

LPMCI_VCR_PLAY_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_PLAY__PARMS data structure.

### MCI_RECORD

This message causes the device to start recording.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_FROM

dwFrom contains the starting play location specified in the current time format.

MCI_TO

dwTo contains the ending (pausing) play location specified in the current time format.

MCI_RECORD_INSERT

Returns error.

MCI_RECORD_OVERWRITE

This is assumed by default, and therefore is not needed.

MCI_VCR_RECORD_INITIALIZE

Seek the device to the start of the media, begin recording blank video and audio, and record timecode, if possible.

MCI_VCR_RECORD_AT

dwAt contains a time when the entire command should begin, or if the device is cued when the device should be playing across the from position given by the cue command.

MCI_VCR_RECORD_PREVIEW

Don’t record, rather, play to the cued “in-point” and switch to displaying the selected source where the recording would begin. After using MCI_RECORD with this flag, the device must be cued again (using MCI_CUE) for the record to be carried out.

LPMCI_VCR_RECORD_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_RECORD_PARMS data structure.

### MCI_RESUME

This message resumes a play, record, or seek command that was interrupted by a pause.

DWORD   lParam1

Parameters

Specifies the MCI_NOTIFY, MCI_WAIT, and MCT_TEST flags.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_SEEK

This message positions and cues the workspace to the specified position.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_SEEK_TO_START

Seek to the start of the video-cassette tape.

MCI_SEEK_TO_END

Seek to the end of the media.

MCI_TO

dwTo contains the position, in the current time format, to seek to.

MCI_VCR_SEEK_REVERSE

Seek direction is reverse; this is only used with the mark flag.

MCI_VCR_SEEK_MARK

dwMark contains the numbered mark to search for.

MCI_VCR_SEEK_AT

dwAt contains a time when the entire command should begin.

LPMCI_VCR_SEEK_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SEEK_PARMS data structure.

### MCI_SET

This message sets device information or state.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_SET_TIME_FORMAT

dwTimeFormat contains a constant specifying the new time format to be used with play, record, seek, cue, and status position commands. The following are valid constants:

MCI_FORMAT_MILLISECONDS

Time format is milliseconds.

MCI_FORMAT_FRAMES

Time format is frames.

MCI_FORMAT_SMPTE_30DROP

Time format is smpte 30 drop.

MCI_FORMAT_SMPTE_30

Time format is smpte 30 non-drop.

MCI_FORMAT_SMPTE_25

Time format is smpte 25.

MCI_FORMAT_SMPTE_24

Time format is smpte 24.

MCI_FORMAT_HMS

Time format is hours, minutes, seconds.

MCI_FORMAT_TMSF

Time format is tracks, minutes, seconds, frames.

MCI_FORMAT_MSF

Time format is minutes, seconds, frames.

MCI_SET_AUDIO

Unsupported.

MCI_VCR_SET_TIME_MODE

dwTimeMode contains a constant indicating the new positional time mode. The following constants are valid:

MCI_VCR_TIME_DETECT

Each time a new video-cassette tape is inserted into the device, or the mode changes from not ready to ready, attempt to determine if there is timecode available, on the tape. If timecode is available then use timecode in all subsequent commands that specify positions. Otherwise, use the counter.

MCI_VCR_TIME_TIMECODE

Forces the device to use timecode exclusively.

MCI_VCR_TIME_COUNTER

Forces the device to use counter exclusively.

MCI_SET_DOOR_OPEN

Ejects the tape.

MCI_VCR_SET_ASSEMBLE_RECORD

Sets the device to record in assemble or insert modes (when assemble is off, insert is on, and viceversa).

MCI_SET_ON

Sets to assemble record, insert is turned off. Records all video, audio and timecode tracks.

MCI_SET_OFF

Sets assemble record off, and turn insert record on. When assemble record is off, individual tracks of video, audio, and timecode can be selected for recording.

MCI_VCR_SET_RECORD_FORMAT

dwRecordFormat contains a constant describing the record speed, which must be one of the following:

MCI_VCR_FORMAT_SP

Records at standard speed.

MCI_VCR_FORMAT_LP

Records at medium-slow speed.

MCI_VCR_FORMAT_EP

Records at slow speed.

MCI_VCR_SET_SPEED

dwSpeed contains the new speed setting, where 1000 is normal speed, 2000 is double, etc., and 500 is 1/2 speed, 250 is 1/4, etc.

MCI_VCR_SET_TAPE_LENGTH

dwTapeLength contains the new length of the tape, provided that the length of the tape is undetectable.

MCI_VCR_SET_CLOCK

dwClock contains the new clock time.

MCI_VCR_SET_COUNTER_FORMAT

dwCounterFormat contains a constant specifying the new counter-time format to be used by the status counter. For a list of valid constants, see MCI_VCR_SET_TIME_FORMAT.

MCI_VCR_SET_COUNTER_VALUE

dwCounterValue contains the new counter value.

MCI_VCR_SET_INDEX

dwIndex contains a constant indicating the contents of the on-screen display (for more information, see MCI_INDEX), and must be one of the following:

MCI_VCR_INDEX_TIMECODE

Displays timecode.

MCI_VCR_INDEX_COUNTER

Displays counter.

MCI_VCR_INDEX_DATE

Displays date.

MCI_VCR_INDEX_TIME

Displays time.

MCI_VCR_SET_POWER

Sets the power on or off, and must be used with one of the following flags:

MCI_SET_ON

Turns power on.

MCI_SET_OFF

Turns power off.

MCI_VCR_SET_PREROLL_DURATION

dwPrerollDuration contains the videotape length, in the current time format, needed to stabilize the VCR output.

MCI_VCR_SET_POSTROLL_DURATION

dwPostrollDuration contains the videotape length, in the current time format, needed to brake the VCR transport when a stop or pause command is issued.

MCI_VCR_SET_PAUSE_TIMEOUT

dwPauseTimeout contains the maximum duration, in milliseconds, of a pause command.

MCI_VCR_SET_TRACKING

Tune the speed of the VCR transport with a fine adjustment, and must be used with one of the following flags:

MCI_VCR_PLUS

Increases the transport speed.

MCI_VCR_MINUS

Decreases the transport speed.

MCI_VCR_RESET

Returns the tracking adjustment to zero.

LPMCI_VCR_SET_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SET_PARMS data structure.

### MCI_SETAUDIO

This message sets values associated with audio recording, monitoring, and input selection.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_SETAUDIO_RECORD

Sets the audio recording to on or off, which is used in conjunction with one of following flags:

MCI_SET_ON

Audio recording on.

MCI_SET_OFF

Audio recording off.

MCI_TRACK

dwTrack specifies which track is affected by the command.

MCI_SET_ON

Enable audio output.

MCI_SET_OFF

Disable audio output.

MCI_VCR_SETAUDIO_SOURCE

Sets the audio source, and must be used with the MCI_VCR_SETAUDIO_TO flag.

MCI_VCR_SETAUDIO_MONITOR

Sets the audio source monitor, and must be used with the MCI_VCR_SETAUDIO_TO flag.

MCI_VCR_SETAUDIO_TO

dwTo contains a constant describing the type of input or monitored input, and must be one of the following:

MCI_VCR_SRC_TYPE_TUNER

Type is tuner.

MCI_VCR_SRC_TYPE_LINE

Type is line.

MCI_VCR_SRC_TYPE_AUX

Type is auxiliary.

MCI_VCR_SRC_TYPE_GENERIC

Type is generic.

MCI_VCR_SRC_TYPE_MUTE

Type is mute. This can only be used with the MCI_VCR_SETAUDIO_SOURCE flag.

MCI_VCR_SRC_TYPE_OUTPUT

MCI_VCR_SETAUDIO_NUMBER

dwNumber contains the audio input (of the type specified by the MCI_VCR_SETAUDIO_TO parameter) to use.

LPMCI_VCR_SETAUDIO_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SETAUDIO_PARMS data structure.

### MCI_SETTIMECODE

This message enables or disables timecode recording for a VCR.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_SETTIMECODE_RECORD

Sets the timecode track recording to on or off. This flag is used in combination with one of the following additional flags.

MCI_SET_ON

Timecode recording on.

MCI_SET_OFF

Timecode recording off.

LPMCI_VCR_SETAUDIO_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_SETTUNER

This message sets the current channel on the tuner.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_SETTUNER_CHANNEL_UP

Increments the tuner channel.

MCI_VCR_SETTUNER_CHANNEL_DOWN

Decrements the tuner channel.

MCI_VCR_SETTUNER_CHANNEL_SEEK_UP

Searches in the increasing channel-number direction for a valid channel.

MCI_VCR_SETTUNER_CHANNEL_SEEK_DOWN

Searches in the increasing channel-number direction for a valid channel.

MCI_VCR_SETTUNER_CHANNEL

dwChannel contains the new channel number.

MCI_VCR_SETTUNER_NUMBER

dwNumber specifies which logical tuner to affect with this command.

LPMCI_VCR_SETTUNER_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SETTUNER_PARMS data structure.

### MCI_SETVIDEO

This message sets values associated with video recording, monitoring, and input selection.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_SETVIDEO_RECORD

Sets the video recording to on or off, and is used in conjunction with one of following flags:

MCI_SET_ON

Video recording on.

MCI_SET_OFF

Video recording off. It might be necessary to first turn off the assemble recording (using set with the MCI_VCR_SET_ASSEMBLE_RECORD flag set to off) before the video recording can be turned off.

MCI_TRACK

dwTrack specifies which track is affected by the command.

MCI_SET_ON

Enable video output.

MCI_SET_OFF

Disable video output.

MCI_VCR_SETVIDEO_SOURCE

Sets the video source, and must be used with the MCI_VCR_SETVIDEO_TO flag.

MCI_VCR_SETVIDEO_MONITOR

Sets the video source monitor, and must be used with the MCI_VCR_SETVIDEO_TO flag.

MCI_VCR_SETVIDEO_TO

dwTo contains a constant describing the type of input or monitored input. For a description of the applicable constants, see the MCI_SETAUDIO flag under MCI_VCR_SETAUDIO.

MCI_VCR_SRC_TYPE_TUNER

MCI_VCR_SRC_TYPE_LINE

MCI_VCR_SRC_TYPE_AUX

MCI_VCR_SRC_TYPE_GENERIC

MCI_VCR_SRC_TYPE_MUTE

MCI_VCR_SRC_TYPE_OUTPUT

MCI_VCR_SRC_TYPE_RGB

MCI_VCR_SETVIDEO_NUMBER

dwNumber contains the video input (of the type specified by the MCI_VCR_SETVIDEO_TO parameter) to use.

LPMCI_VCR_SETVIDEO_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SETVIDEO_PARMS data structure.

### MCI_SIGNAL

This message marks one or more positions in a workspace (for example, video-cassette tape).

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_SIGNAL_AT

Specifies a signal position is included in the dwPosition field of the data structure identified by lParam2.

MCI_DGV_SIGNAL_EVERY

Specifies a signal period value is included in the dwEvery field of the data structure identified by lParam2.

MCI_VCR_SIGNAL_USERVAL

Specifies a data value is included in the dwUserParm field of the data structure identified by lParam2. The data value is reported back with the Windows message.

MCI_VCR_SIGNAL_CANCEL

Removes the signal position specified by the value associated with the MCI_VCR_SIGNAL_USERVAL flag.

MCI_VCR_SIGNAL_POSITION

Specifies that the device should send the position value with the next Window message instead of the user-specified value.

LPMCI_VCR_SIGNAL_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_SIGNAL_PARMS data structure.

### MCI_STATUS

This message obtains information about an instance of an MCI device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_STATUS_ITEM

Specifies that the dwItem field of the data structure contains a constant indicating which information to obtain. The following constants are recognized:

MCI_STATUS_LENGTH

Sets dwReturn to the length of the viceo-cassette tape.

MCI_STATUS_POSITION

Sets dwReturn to the current device position in the current time format.

MCI_STATUS_NUMBER_OF_TRACKS

Sets dwReturn to 1.

MCI_VCR_STATUS_NUMBER_OF_VIDEO_TRACKS

Sets dwReturn to the number of video tracks that are independently selectable.

MCI_VCR_STATUS_NUMBER_OF_AUDIO_TRACKS

Sets dwReturn to the number of audio tracks that are independently selectable.

MCI_STATUS_MODE

Sets dwReturn to a constant indicating the current mode of the device. The mode will be one of the following:

MCI_MODE_NOT_READY

MCI_MODE_STOP

MCI_MODE_PLAY

MCI_MODE_RECORD

MCI_MODE_SEEK

MCI_MODE_PAUSE

MCI_MODE_OPEN

MCI_STATUS_MEDIA_PRESENT

Sets dwReturn to TRUE if the media is present.

MCI_VCR_STATUS_TIMECODE_PRESENT

Sets dwReturn to TRUE if timecode is present at the current position on the media.

MCI_STATUS_TIME_FORMAT

Sets dwReturn to the current time format. For a description of the time formats, see MCI_SET flag under MCI_SET_TIME_FORMAT.

MCI_VCR_STATUS_TIME_MODE

Sets dwReturn to one of the following (for more information, see MCI_SET, flag MCI_VCR_SET_TIME_MODE):

MCI_VCR_TIME_TIMECODE

MCI_VCR_TIME_COUNTER

MCI_VCR_TIME_DETECT

MCI_STATUS_READY

Sets dwReturn to TRUE if the device is ready to accept a play or record command.

MCI_STATUS_CURRENT_TRACK

Sets dwReturn to 1.

MCI_VCR_STATUS_FRAME_RATE

Sets dwReturn to the current native frame rate of the device.

MCI_VCR_STATUS_SPEED

Sets dwReturn to the current speed. (For more information, see MCI_SET, flag MCI_SET_SPEED).

MCI_VCR_STATUS_MEDIA_TYPE

Sets dwReturn to one of the following:

MCI_VCR_MEDIA_8MM

MCI_VCR_MEDIA_HI8

MCI_VCR_MEDIA_VHS

MCI_VCR_MEDIA_SVHS

MCI_VCR_MEDIA_BETA

MCI_VCR_MEDIA_EDBETA

MCI_VCR_MEDIA_OTHER

MCI_VCR_STATUS_RECORD_FORMAT

Sets dwReturn to one of the following:

MCI_VCR_FORMAT_SP

MCI_VCR_FORMAT_LP

MCI_VCR_FORMAT_EP

MCI_VCR_FORMAT_OTHER

MCI_VCR_STATUS_PLAY_FORMAT

Sets dwReturn to one of the following:

MCI_VCR_FORMAT_SP

MCI_VCR_FORMAT_LP

MCI_VCR_FORMAT_EP

MCI_VCR_FORMAT_OTHER

MCI_VCR_STATUS_PREROLL_DURATION

Sets dwReturn to length of tape, in the current time format, needed to stabilize the VCR output.

MCI_VCR_STATUS_POSTROLL_DURATION

Sets dwReturn to length of tape, in the current time format. Needed to brake the VCR transport from a stop or pause command.

MCI_VCR_STATUS_PAUSE_TIMEOUT

Sets dwReturn to the maximum duration, in milliseconds, of a pause command. The return value of zero indicates that no time-out will occur.

MCI_VCR_STATUS_AUDIO_SOURCE_NUMBER

Sets dwReturn to the number of the currently selected audio-source type.

MCI_VCR_STATUS_AUDIO_SOURCE

Sets dwReturn to a constant, indicating the current audio-source type.

MCI_VCR_STATUS_VIDEO_SOURCE_NUMBER

Sets dwReturn to the number of the currently selected video-source type.

MCI_VCR_STATUS_VIDEO_SOURCE

Sets dwReturn to a constant indicating the currently selected video-source type.

MCI_VCR_STATUS_AUDIO_MONITOR_NUMBER

Sets dwReturn to the number of the currently selected audio-monitor type.

MCI_VCR_STATUS_AUDIO_MONITOR

Sets dwReturn to a constant, indicating the currently selected audio-monitor type.

MCI_VCR_STATUS_VIDEO_MONITOR_NUMBER

Sets dwReturn to the number of the currently selected video-monitor type.

MCI_VCR_STATUS_VIDEO_MONITOR

Sets dwReturn to a constant, indicating the currently selected video-monitor type.

MCI_VCR_STATUS_INDEX_ON

Sets dwReturn to TRUE if the on-screen display is on.

MCI_VCR_STATUS_INDEX

Sets dwReturn to a constant, describing the current contents of the on-screen display, and will be one of the following:

MCI_VCR_INDEX_TIMECODE

MCI_VCR_INDEX_COUNTER

MCI_VCR_INDEX_DATE

MCI_VCR_INDEX_TIME

MCI_VCR_STATUS_COUNTER_FORMAT

Sets dwReturn to a constant describing the current counter format. (For more information, see MCI_SET flag, MCI_SET_TIME_FORMAT).

MCI_VCR_STATUS_COUNTER_RESOLUTION

Sets dwReturn to a constant describing the resolution of the counter, and will be one of the following values:

MCI_VCR_COUNTER_RES_SECONDS

Counter has resolution of seconds.

MCI_VCR_COUNTER_RES_FRAMES

Counter has resolution of frames.

MCI_VCR_STATUS_TIMECODE_TYPE

Sets dwReturn to a constant, describing the type of timecode which is natively supported by the device, and will be one of the following:

MCI_VCR_TIMECODE_TYPE_SMPTE

This device uses smpte timecode.

MCI_VCR_TIMECODE_TYPE_SMPTE_DROP

This device uses smpte drop timecode.

MCI_VCR_TIMECODE_TYPE_OTHER

MCI_VCR_TIMECODE_TYPE_NONE

MCI_VCR_STATUS_COUNTER_VALUE

Sets dwReturn to the current counter reading, in the current counter-time format.

MCI_VCR_STATUS_TUNER_CHANNEL

Sets dwReturn to the current channel number.

MCI_VCR_STATUS_NUMBER

dwNumber contains the logical-tuner number this command applies to.

MCI_VCR_STATUS_WRITE_PROTECTED

Sets dwReturn to TRUE if the media is write protected.

MCI_VCR_STATUS_TIMECODE_RECORD

Sets dwReturn to TRUE if the timecode will be recorded when the next record command is given.

MCI_VCR_STATUS_ASSEMBLE_RECORD

Sets dwReturn to TRUE if assemble mode is on.

MCI_VCR_STATUS_VIDEO_RECORD

Sets dwReturn to TRUE if video will be recorded when the next record command is given.

MCI_TRACK

dwTrack contains the track this inquiry applies to.

MCI_VCR_STATUS_AUDIO_RECORD

Sets dwReturn to TRUE if audio will be recorded when the next record command is given.

MCI_TRACK

dwTrack contains the track this inquiry applies to.

MCI_VCR_STATUS_TIME_TYPE

Sets dwReturn to a constant describing the current time type in use (used by play, record, seek, etc.), and will be one of the following:

MCI_VCR_TIME_TIMECODE

Timecode is in use.

MCI_VCR_TIME_COUNTER

Counter is in use.

MCI_VCR_STATUS_POWER_ON

Sets dwReturn to TRUE if the power is on.

MCI_VCR_STATUS_CLOCK_ID

Sets dwReturn to a number which uniquely describes the clock in use.

MCI_VCR_STATUS_CLOCK

Sets dwReturn to the current clock value, in total clock increments.

MCI_STATUS_START

Sets dwReturn to 0.

LPMCI_VCR_STATUS_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_STATUS_PARMS data structure.

### MCI_STEP

This message steps the device.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_STEP_FRAMES

dwFrames contains the number of frames to step by.

MCI_VCR_STEP_REVERSE

The step direction is reverse.

LPMCI_VCR_STEP_PARMS   lParam2

Specifies a far pointer to the MCI_VCR_STEP_PARMS data structure.

### MCI_STOP

This message stops the device.

DWORD   lParam1

Parameters

Specifies the MCI_NOTIFY, MCI_WAIT, and MCT_TEST flags.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_UNFREEZE

This message unfreezes the input or output and indicates that the device should resume displaying normal output or receiving normal input.

DWORD   lParam1

Parameters

The following flags apply to the VCR driver:

MCI_VCR_UNFREEZE_INPUT

Unfreeze the input.

MCI_VCR_UNFREEZE_OUTPUT

Unfreeze the output.

LPMCI_GENERIC_PARMS   lParam2

Specifies a far pointer to the MCI_GENERIC_PARMS data structure.

### MCI_VCR_CUE_PARMS

The MCI_VCR_CUE_PARMS structure contains parameters used by the MCI_CUE message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_CUE_PARMS {
	DWORD	dwCallback;
	DWORD	dwFrom;
	DWORD	dwTo;
} MCI_VCR_CUE_PARMS;

The MCI_VCR_CUE_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwFrom

Specifies the position to cue from.

dwTo

Specifies the position to cue to.

### MCI_VCR_LIST_PARMS

The MCI_VCR_LIST_PARMS structure contains parameters used by the MCI_LIST message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_LIST_PARMS {
	DWORD	dwCallback;
	DWORD	dwReturn;
	DWORD	dwNumber;
} MCI_VCR_LIST_PARMS;

The MCI_VCR_LIST_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwReturn

Contains the return information on exit.

dwNumber

Specifies the number of video or audio inputs supplied with the VCR.

### MCI_VCR_PLAY_PARMS

The MCI_VCR_PLAY_PARMS structure contains parameters used by the MCI_PLAY message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_PLAY_PARMS {
	DWORD	dwCallback;
	DWORD	dwFrom;
	DWORD	dwTo;
	DWORD	dwAt;
} MCI_VCR_PLAY_PARMS;

The MCI_VCR_PLAY structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwFrom

Specifies the beginning play position specified in the current time format.

dwTo

Specifies the ending play position specified in the current time format.

dwAt

Specifies a time when the play command should begin, or when a device is cued. When the device should play the dwFrom position is given with the MCI cue command.

### MCI_VCR_RECORD_PARMS

The MCI_VCR_RECORD_PARMS structure contains parameters used by the MCI_RECORD message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_RECORD_PARMS {
	DWORD	dwCallback;
	DWORD	dwFrom;
	DWORD	dwTo;
	DWORD	dwAt;
} MCI_VCR_RECORD_PARMS;

The MCI_VCR_RECORD_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwFrom

Specifies the beginning play position specified in the current time format.

dwTo

Specifies the ending play position specified in the current time format.

dwAt

Specifies a time when the play command should begin, or when a device is cued. When the device should play the dwFrom position is given with the MCI cue command.

### MCI_VCR_SEEK_PARMS

The MCI_VCR_SEEK_PARMS structure contains parameters used by the MCI_SEEK message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SEEK_PARMS {
	DWORD	dwCallback;
	DWORD	dwTo;
	DWORD	dwMark;
	DWORD	dwAt;
} MCI_VCR_SEEK_PARMS;

The MCI_VCR_SEEK_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwTo

Specifies a position to seek to in the current time format.

dwMark

Specifies a numbered mark to search for.

dwAt

Specifies a time when the seek command should begin.

### MCI_VCR_SET_PARMS

The MCI_VCR_SET_PARMS structure contains parameters used by the MCI_SET message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SET_PARMS {
	DWORD	dwCallback;
	DWORD	dwTimeFormat;
	DWORD	dwAudio;
	DWORD	dwTimeMode;
	DWORD	dwRecordFormat;
	DWORD	dwCounterFormat;
	DWORD	dwIndex;
	DWORD	dwTracking;
	DWORD	dwSpeed;
	DWORD	dwLength;
	DWORD	dwCounter;
	DWORD	dwClock;
	DWORD	dwPauseTimeout;
	DWORD	dwPrerollDuration;
	DWORD	dwPostrollDuration;
} MCI_VCR_SET_PARMS;

The MCI_VCR_SET_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwTimeFormat

Specifies the time format used with the play, record, seek cue, and status commands.

dwAudio

Not used.

dwTimeMode

Contains a constant that specifies the timing source used by the device. The timing source is either a timecode recorded on tape, or the counters in the device that sense tape movement.

dwRecordFormat

Specifies the recording rate used by the device.

dwCounterFormat

Specifies the format of a new counter-time value.

dwIndex

Specifies the contents of the on-screen display.

dwTracking

Specifies the speed adjustment used when tracking the VCR playback rate.

dwSpeed

Specifies the playback rate used by the device as an integer. Normal playback speed is 1000, double speed is 2000, and half speed is 500.

dwLength

Specifies the tape length when the tape length is undetectable by the device.

dwCounter

Specifies a new counter value.

dwClock

Specifies a new clock time.

dwPauseTimeout

Specifies a new timeout value for the pause command.

dwPrerollDuration

Specifies the videotape length needed to stabilize the VCR output.

dwPostrollDuration

Specifies the videotape length needed to brake the VCR transport when a stop or pause command is issued.

### MCI_VCR_SETAUDIO_PARMS

The MCI_VCR_SETAUDIO_PARMS structure contains parameters used by the MCI_SETAUDIO message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SETAUDIO_PARMS {
	DWORD	dwCallback;
	DWORD	dwTrack;
	DWORD	dwTo;
	DWORD	dwNumber;
} MCI_VCR_SETAUDIO_PARMS;

The MCI_VCR_SETAUDIO_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwTrack

Specifies the track that is affected by the command.

dwTo

Specifies the type of input or monitored input.

dwNumber

Specifies the audio input (of the type specified in the dwTo field) to use.

### MCI_VCR_SETTUNER_PARMS

The MCI_VCR_SETTUNER_PARMS structure contains parameters used by the MCI_SETTUNER message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SETTUNER_PARMS {
	DWORD	dwCallback;
	DWORD	dwChannel;
	DWORD	dwNumber;
} MCI_VCR_SETTUNER_PARMS;

The MCI_VCR_SETTUNER_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwChannel

Specifies the new channel number.

dwNumber

Specifies the logical tuner that the MCI_SETTUNER command affects.

### MCI_VCR_SETVIDEO_PARMS

The MCI_VCR_SETVIDEO_PARMS structure contains parameters used by the MCI_SETVIDEO message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SETVIDEO_PARMS {
	DWORD	dwCallback;
	DWORD	dwTrack;
	DWORD	dwTo;
	DWORD	dwNumber;
} MCI_VCR_SETVIDEO_PARMS;

The MCI_VCR_SETVIDEO_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwTrack

Specifies the track that is affected by the command.

dwTo

Specifies the type of input or monitored input.

dwNumber

Specifies the video input (of the type specified in the dwTo field) to use.

### MCI_VCR_SIGNAL_PARMS

The MCI_VCR_SIGNAL_PARMS structure contains parameters for the MCI_SIGNAL message used by digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_SIGNAL_PARMS {
	DWORD	dwCallback;
	DWORD	dwPosition;
	DWORD	dwPeriod;
	DWORD	dwUserParm;
} MCI_VCR_SIGNAL_PARMS;

The MCI_VCR_STATUS_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwPosition

Specifies the position to be marked.

dwPeriod

Specifies the interval between adjacent position marks.

dwUserParm

Specifies a value associated with the signals being set.

### MCI_VCR_STATUS_PARMS

The MCI_VCR_STATUS_PARMS structure contains parameters used by the MCI_STATUS message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_STATUS_PARMS {
	DWORD	dwCallback;
	DWORD	dwReturn;
	DWORD	dwItem;
	DWORD	dwTrack;
	DWORD	dwNumber;
} MCI_VCR_STATUS_PARMS;

The MCI_VCR_STATUS_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwReturn

Specifies the value returned by the MCI status command. The return value varies according to the inquiry of the status command. For additional information, see the description of the MCI_STATUS message.

dwItem

Specifies the type of information the status command supplies.

dwTrack

Specifies the audio or video track that will store information during the next recording. This field is used to return information when the status command inquires about the video or audio recording status.

dwNumber

Specifies the logical tuner that the current channel is associated with. This field is used to return information when the status command inquires about the current channel number.

### MCI_VCR_STEP_PARMS

The MCI_VCR_STEP_PARMS structure contains parameters used by the MCI_STEP message for digital-video cassette recorders. When assigning data to the fields in the following data structure, set the corresponding MCI flags in the lparam1 parameter of mciSendCommand to validate each field:

typedef struct tagMCI_VCR_STEP_PARMS {
	DWORD	dwCallback;
	DWORD	dwFrames;
} MCI_VCR_STEP_PARMS;

The MCI_VCR_STEP_PARMS structure has the following fields:

Fields

dwCallback

The low-order word specifies a window handle used for the MCI_NOTIFY flag.

dwFrames

Specifies the number of frames to jump (the length of a single step) as the step command steps forward or backward through the media.