Chapter 7

In Windows Telephony, Supplementary functions are functions whose form and functionality has been defined by the API description, but which are not shipped with Telephony in the sample Dialer application. They are functions that developers of telephony applications and service providers may choose to implement to suit the design of their custom products. That is, in contrast to Basic Telephony functions, Supplementary functions are optional, not mandatory.

Supplementary Line Functions

This chapter describes the Supplementary functions used with TAPI-defined line devices. For convenience, they are grouped into general functional categories.

## Bearer Mode and Rate

The notion of bearer mode corresponds to the quality of service requested from the network for establishing a call. It is important to keep the concept of bearer mode separate from that of media mode. The media mode of a call describes the type of information that is exchanged over a specific call of a given bearer mode. As an example, the analog telephone network (PSTN) provides only 3.1 kHz voice-grade quality of service—this is its bearer mode. However, a call with this bearer mode can support a variety of different media modes such as voice, fax, or data modem. In other words, media modes require certain bearer modes. TAPI manages the bearer modes only by passing the bearer mode parameters on to the network. Media modes are fully managed through the appropriate media mode APIs, although some limited support is provided in TAPI.

The bearer mode of a call is specified when the call is set up, or is provided when the call is offered. With line devices able to represent channel pools, it is possible for a service provider to allow calls to be established with wider bandwidth. The rate (or bandwidth) of a call is specified separately from the bearer mode, allowing an application to request arbitrary data rates.

The bearer modes defined in TAPI are:

·	Voice  Regular 3.1 kHz analog voice service. Bit integrity is not assured.

·	Speech  G.711 speech transmission on the call.

·	Multiuse  As defined by ISDN.

·	Data  Unrestricted data transfer. The data rate is specified separately.

·	Alternate speech and data  The alternate transfer of speech and unrestricted data on a call (ISDN).

·	Non call-associated signaling  This provides a clear signaling path from the application to the service provider.

Although support for changing a call's bearer mode or bandwidth is limited in networks today, TAPI provides an operation to request a change in the bearer mode or the data-rate parameters of an existing call. This function is lineSetCallParams.

## Monitoring Operations

### Media Monitoring

When a call is in the connected state, information can be transported over the call. A call's media mode provides an indication of the type of information (for example, its data type, higher-level protocol) of this media stream. TAPI allows applications to be provided with a callback notification about changes in a call's media mode. The notification provides an indication of the call's new media mode. Note that the service provider decides how it wants to make this determination. For example, the service provider could use signal processing of the media stream to determine the media mode, or it could rely on distinctive ringing patterns assigned to different media streams, or on information elements passed in an out-of-band signaling protocol. Independent of how the media mode determination is done, the application is simply informed about media mode changes on an existing call.

The media modes defined by TAPI include:

·	Unknown  The media mode of the call is not currently known—the call is unclassified.

·	Interactive voice  Voice energy was detected on the call, and the call is handled as an interactive voice call with a person at the application's end.

·	Automated voice  Voice energy was detected on the call, and the call is handled as a voice call but with no person at the application's end, such as with an answering machine application.

·	Stored voice  Voice energy was detected on the call, and the call is handled as a voice call but with no person at the application's end, such as with an answering machine application.

·	Digital data  A digital data stream of unspecified format.

·	G3 fax  A group 3 fax session on the call.

·	G4 fax  A group 4 fax session on the call.

·	Data modem  A modem session on the call. Note that current modem protocols require the called station to initiate the handshake. For an inbound data modem call, the application can typically make no positive detection. How the service provider makes this determination is its choice. For example, a period of silence just after answering an inbound call may be used as a heuristic to decide that this might be a data modem call.

·	Teletex, Videotex, mixed  These corresponds to the telematic services with the same names.

·	TDD  The call's media stream uses the Telephony Devices for the Deaf protocol.

·	ADSI  An Analog Display Services Interface session on the call. ADSI enhances voice calls with alphanumeric information downloaded to the phone and the use of soft buttons on the phone.

An application can enable or disable media monitoring on a specified call with the function lineMonitorMedia. The application specifies which media modes it is interested in monitoring. When media monitoring is enabled, the detection of a media mode change causes the application to be notified with the LINE_MONITORMEDIA callback. This callback provides the call handle on which the media mode change was detected as well as the new media mode.

There is a distinction between the media mode of a call as reported by lineGetCallInfo and the media mode event reports by LINE_MONITORMEDIA messages. A call's media mode is determined exclusively by owner applications of the call and is not automatically changed by media monitoring events. The one exception is the initial media mode determination that may be performed by TAPI’s implementation to select the first owner of a call. One could argue that in that case, the TAPI implemention is the owner of the call.

Default media mode monitoring is performed for the media modes for which the line device has been opened. This allows an inbound call's media mode to be determined before the call is handed to an application based on what the applications demand.

Note that the scope of the media monitoring of a call is bound by the lifetime of the call. Media monitoring on a call ends as soon the call disconnects or goes idle.

An application can obtain device IDs for various Windows device classes associated with an opened line by invoking lineGetID. This function takes a line handle, address, or call handle and a device class description. It returns the device ID for the device of the given device class that is associated with the open line device, address, or call. If the device class is line, then the device ID of the line device is returned. If the device class is mci waveaudio, then the device ID of an mci waveaudio device is returned (if supported), which allows activities such as the recording or playback of audio over the call on the line.

The application can use the returned device ID with the corresponding media API to query the device's capabilities and subsequently open the media device. For example, if your application needs to use the line as a waveform device, it will first have to call waveInGetDevCaps and/or waveOutGetDevCaps to determine the waveform capabilities of the device. The typical wave form data format supported by telephony in North America is 8-bit m-law at 8000 samples per second, although the wave device driver can convert this sample rate and companding to other more common multimedia audio formats.

To subsequently open a line device for audio playback using the waveform API, an application calls the waveOutOpen function. The implementation of the waveOutOpen call is device-specific, and each implementation has a number of options for implementing this function.

### Digit Monitoring

Digit monitoring monitors the call for digits. TAPI provides for digits to be signaled according to two methods (digit modes):

·	Pulse  Digits are signaled as pulse or rotary sequences. Note that for detection, these pulses manifest themselves as nothing more than sequences of audible clicks. Valid pulse digits are '0' through '9'.

·	DTMF  Digits are signaled as DTMF (Dual Tone Multiple Frequency) tones. Valid DTMF digits are '0' through '9', 'A'. 'B', 'C', 'D', '*', and '#'. Both the beginning and the down edge of DTMF digits can be monitored.

An application can enable or disable digit monitoring on a specified call with the function lineMonitorDigits. When digit monitoring is enabled, detected digits cause the application to be notified with the LINE_MONITORDIGITS callback. This callback provides the call handle on which the digit was detected as well as the digit value and the digit mode.

Note that the scope of digit monitoring is bound by the lifetime of the call. Digit monitoring on a call ends as soon the call disconnects or goes idle.

### Tone Monitoring

Tone monitoring monitors the media stream of a call for specified tones. A tone is described by its component frequencies and cadence. An implementation of the API may allow several different tones to be monitored simultaneously. An application can "tag" each tone in order to be able to distinguish the different tones for which it requests detection.

An application can enable or disable tone monitoring on a specified call with the function lineMonitorTones. With this function, the application specifies which tones to detect on a specified call. When tone monitoring is enabled, detected digits cause the application to be notified with the LINE_MONITORTONE callback. This callback provides the call handle on which the tone was detected as well as the application's "tag" for the tone.

The scope of tone monitoring is bound by the lifetime of the call. Tone monitoring on a call ends as soon the call disconnects or goes idle.

Note that the monitoring of tones, digits, or media modes often requires the use of resources of which the service provider may only have a finite amount. A request for monitoring may be rejected if resources needed are not available. For the same reason, an application should disable any monitoring that is not needed.

## Media Control

An application can request the execution of a limited set of media control operations on the call's media stream triggered by telephony events. Although an application is encouraged to use the media API specifically defined for the media mode, media control can yield a significant performance improvement for client-server implementations. The function lineSetMediaControl is used to set up a call's media stream for media control by allowing an application to specify a list of tuples specifying a telephony event and the associated media control action. The telephony events that can trigger media control activities are:

·	Detection of a digit  The application provides a list of specific digits and the media control action each of them triggers.

·	Detection of a media mode  The application provides a list of media modes and the media control actions a transition into the media mode triggers.

·	Detection of a specified tone  The application specifies a list of tones and the media control action each tone detection triggers.

·	Detection of a call state  The application specifies a list of call states and the media control action each transition to the call state triggers.

The media control actions are defined generically for the different media modes. Not all media streams may provide meaningful interpretations of the media control actions. The operations should map well to audio streams.

·	Start  Starts the media stream.

·	Reset  Resets the media stream.

·	Pause  Stops or pauses the media stream.

·	Resume  Starts or resumes the media stream.

·	Rate up  Increases the rate (speed) of the media stream by a implementation-defined amount.

·	Rate down  Decreases the rate (speed) of the media stream by a implementation-defined amount.

·	Rate normal  Returns the rate (speed) to normal.

·	Volume up  Increases the volume (amplitude) of the media stream.

·	Volume down  Decreases the volume (amplitude) of the media stream.

·	Volume normal  Returns the volume (amplitude) to normal.

The scope of media control is bound by the lifetime of the call. Media control on a call ends as soon the call disconnects or goes idle. Only a single media control request can be outstanding on a call across all applications.

## Digit Gathering

Besides enabling digit monitoring and being notified of digits one at a time, an application can also request that multiple digits be collected in a buffer. Only when the buffer is full or when some other termination condition is met is the application notified. Digit gathering is useful for functions such as credit card number collection. It is peformed when an application invokes lineGatherDigits, specifying a buffer to fill with digits. Digit gathering terminates when one of a number of conditions is true.

·	The requested number of digits has been collected.

·	One of multiple termination digits is detected. The termination digits are specified to lineGatherDigits, and the termination digit is placed in the buffer as well.

·	One of two timeouts expires. The timeouts are a first digit timeout, specifying the maximum duration before the first digit must be collected, and an inter-digit timeout, specifying the maximum duration between successive digits.

·	Digit gathering is canceled explictly by invoking lineGatherDigits again with either another set of parameters (which would amount to a new buffer) to start a new gathering request or by using a NULL digit buffer parameter to simply cancel.

When terminated for whatever reason, a LINE_GATHERDIGITS message is sent to the application that requested the digit gathering. Note that only a single digit gathering request can be outstanding on a call at any given time across all applications that are owners of the call.

Note that digit gathering and digit monitoring may be enabled on the same call at the same time. In that case, the application will receive a LINE_MONITORDIGITS message for each detected digit and a separate LINE_GATHERDIGITS message when the buffer is sent back.

## Generating Inband Digits and Tones

Once a call is in the connected state, information can be transmitted over it. Two operations are provided that allow end-to-end inband signaling between the application and remote station equipment such as an answering machine. One operation is lineGenerateDigits, which generates inband digits on a call, signaling them over the voice channel. Digits can be signaled as either rotary/pulse sequences or as DTMF tones. The other operation is lineGenerateTone, which enables the application to generate one of a variety of multi-frequency tones inband (over the media stream). This generates telephony tones, such as ringback, beep, and busy, as well as arbitrary multi-frequency, multi-cadenced tones.

Only one digit or tone generation can be in progress on a call at any one time. When digit or tone generation completes, a LINE_GENERATE message is sent to the application that requested the generation. In the case of generation of multiple digits, only a single message is sent back after all digits have been generated. Calling lineGenerateDigits or lineGenerateTone while digit or tone generation is in progress will abort the generation currently in progress and send the LINE_GENERATE message to the application whose generation was aborted with a cancel indication.

## Call Operations

### Call Accept, Reject, and Redirect

In environments like ISDN, call offering is separate from alerting. In fact, after a call has been offered to an application, a time window exists during which the application has a number of options.

·	The application can simply answer the call using lineAnswer, which accepts an offered call and starts alerting both the called party (ring) and the caller (ringback) .

·	The application can reject the offering call using lineDrop, which will revert the offering call to the idle state.

·	The application can accept the call using lineAccept, which will initiate alerting to both the caller as ringback, and the called party as ring.

·	The application can redirect the call using lineRedirect, which deflects the offering call to another address. The call will revert to the idle state.

### Call Hold

Most PBXs can associate multiple calls with a single line. A call can be placed on hard hold. This frees up the user's line/address to make other calls. An application can place a call on hard hold by invoking lineHold. An application may retrieve an call on hard hold by invoking lineUnhold.

Hard hold is different from a consultation hold. A call is automatically placed on consultation hold, for example, when a call is prepared for transfer or conference.

### Call Transfer

TAPI provides two mechanisms for call transfer: blind transfer and consultation transfer.

·	In blind transfer (or single-step transfer), an existing call is transferred to a specified destination address in one phase using lineBlindTransfer.

·	Under a consultation transfer, the existing call is first prepared for transfer to another address using lineSetupTransfer. This places the existing call on consultation hold, and identifies the call as the target for the next transfer-completion request. lineSetupTransfer also allocates a consultation call that can be used to establish the consultation call with the party to be transferred to. The application can dial the extension of the destination party on the consultation call (using lineDial), or it can drop and deallocate the consultation call and instead activate an existing held call (using lineUnhold), if supported by the switch.

While the initial call is on consultation hold and the consultation call is active, the application can toggle between these calls with the function lineSwapHold.

Finally, the application completes the transfer in one of two ways using lineCompleteTransfer:

·	Transfer the call on transfer hold to the destination party. Both calls will revert to the idle state.

·	Enter a three-way conference. A new call handle is created to represent the conference and this handle is returned to the application.

### Call Conference

Conference calls are calls that include more than two parties simultaneously. Conference calls can be set up using either a switch-based conference bridge or an external server-based bridge. Typically, only switch-based conferencing will allow the level of conference control provided by the API. In server-based conference calls all participating parties dial into the server which mixes all the media streams together and sends each participant the mix; there may be no notion of individual parties in the conference call, only that of a single call between the application and the bridge server.

A conference call can be established in a number of ways, depending on device capabilities:

·	A conference call may be able to start out as a regular two-party call, such as a call established with lineMakeCall. Once the two-party call exists, additional parties can be added, one at a time. Invoking lineSetupConference prepares a given call for the addition of another party, and this action establishes the conference call. This operation takes the original two-party call as input, allocates a conference call, connects the original call to the conference, and allocates a consultation call whose handle is returned to the application.

Note that the capabilities of the addressed line device may limit the number of parties conferenced in a single call and whether or not a conference starts out with a normal two-party call.

·	The application can then use lineDial on the consultation call to establish a connection to the next party to be added. lineDrop can be used to abandon this call attempt. The third party is added with the function lineAddToConference, which specifies both the conference call and the consultation call.

·	To add additional parties to an existing conference call, the application uses linePrepareAddToConference. When invoking this function, the application , supplies the handle of an existing conference call. The function allocates a consultation call that can later be added to the conference call and returns a consultation call handle to the application. This conference call is then placed on conference hold. Once the consultation call exists, it can be added to the existing conference call with lineAddToConference.

·	A conference call may have to be established with the function lineSetupConference without an existing two-party call. This returns a handle for the conference call and allocates a consultation call. After a period of consultation, the consultation call can be added with lineAddToConference. Additional parties are added with linePrepareAddToConference followed by lineAddToConference.

·	The final mechanism for establishing a three-way conference call is by resolving a transfer request for three-way conference. In this scenario, a two-party call is established as either an inbound or outbound call. Next the call is placed on transfer hold with the function lineSetupTransfer which returns a consultation call handle. After a period of consultation, the application may have the option to resolve the transfer setup by selecting the three-way conference option which conferences all three parties together in a conference call with lineCompleteTransfer with the conference option (instead of the transfer option). Under this option, a conference call handle representing the conference call is allocated and returned to the application.

Once a call becomes a member of a conference call, the member's call state reverts to conferenced. The state of the conference call typically becomes connected. The call handle to the conference call and all the added parties remain valid as individual calls. LINE_CALLSTATE events can be received about all calls. For example, if one of the members disconnects by hanging up, an appropriate call-state message may inform the application of this fact; such a call is no longer a member of the conference.

As is the case with call transfer, the application may toggle between the call consultation call and the conference call using lineSwapHold.

Use the call handle for the member calls to later remove the call from the conference. Do this by invoking lineRemoveFromConference on the call handle. Note that this operation is not commonly available in its fully general form. Some switches may not allow it at all, or only allow the most recently added party to be removed. The line's device capabilities describe which flavor of lineRemoveFromConference is possible.

#### Removing a Party

When canceling the consultation call to the third party for a conference call or when removing the third party in a previously established conference call, the service provider (and switch) may release the conference bridge and revert the call back to a normal two-party call. If this is the case, hConfCall will transition to the idle state, and the only remaining participating call will transition to the connected state.

### Call Park

Two forms of call parking are provided: directed call park and non-directed call park. In directed call park, the application specifies the destination address where the call is to be parked. This roughly behaves like a call transfer to the destination address, but it doesn't alert or time out as a transfer would.

In non-directed call park, the switch returns to the application the address where it parked the call. In either case, the function linePark is used to park a call.

A parked call can later be retrieved with lineUnpark. The application specifies the park address to lineUnpark which returns a call handle to the unparked call. Appropriate LINE_CALLSTATE messages will be sent to the application as the call is reconnected.

### Call Forwarding

Forwarding affects the treatment by the switch or network of incoming calls destined for a given address. The application can specify call forwarding conditions based on origin of call (internal, external, selective based on caller ID), status of the address (busy, no answer, unconditionally), and destination address where calls are to be forwarded. When the specified conditions are met for an incoming call, the switch deflects the incoming call to the specified destination number. Note that because the switch performs the forwarding action, the application will typically not know when a call has been forwarded.

The lineForward function provides a combination of call forwarding (by setting call-forwarding requests) and a do-not-disturb feature. LineForward can also cancel any or all of the forwarding requests currently in effect. Some switches require that a call be established to the forwarding address in order for call forwarding to be initiated. On such systems, lineForward allocates a consultation call and returns the handle for it to the application. The consultation call can be used as any other call. After the connection is established, when forwarding is in effect, forwarding confirmation is received from the switch, and the call is dropped (using lineDrop). A LINE_ADDRESSSTATE message with a forwarding indication informs the application about changes in an address' forwarding status.

Note that it may be impossible for a service provider to know at all times what forwarding is in effect for an address. Forwarding may be canceled or changed in ways that make it impossible for a service provider to be informed of this fact.

### Call Pickup

Call pickup allows an application to answer a call that is alerting at another address. The application invokes linePickup by identifying the target of the pickup and is returned a call handle for the picked-up call. There are several ways to specify the target of the pickup request. First, specify the address (extension) of the alerting party. Second, if no extension is specified and the switch allows it, the application can pick up any ringing phone in its pickup group. Third, some switches require a group ID to identify the group to which the ringing extensions belongs.

After the call has been picked up, it is diverted to the application and the application is sent appropriate LINE_CALLSTATE messages for the call. An application can invoke lineGetCallInfo to find out information about the picked-up call, if provided by the switch.

### Call Completion

When making an outbound call, the unavailability of certain resources may prevent the call from reaching the connected state, as when the destination party is busy or doesn't answer. Unavailable resources include trunk circuits as well as the destination party's station. The lineCompleteCall function lets the application specify how its wants to complete a call that cannot be completed normally—this is called placing a call-completion request. The application has the following options:

·	Camp on to queue the call until the call can be completed.

·	Call back requests the called station to return the call when the station returns to idle. Answering the call back may automatically re-initiate (redial) the connection request.

·	Intrude allows the application to barge in to the existing call.

·	Message (aka "leave word calling") allows the application to send one of a small number of predefined messages to the destination. These messages can be text shown on the phone's display, a voice message left for the user, and so forth.

A call completion request can be canceled with lineUncompleteCall. Multiple call completion requests can potentially be outstanding for a given address at any one time. To identify individual requests, the implementation returns a completion ID. When a call completion request completes and results in a new call, then the call completion ID is available in the LINECALLINFO data structure returned by lineGetCallInfo. Canceling a call completion request in progress also uses this call completion ID.

## Extended Line Functions

Extended Line Services (or Device Specific Line Services) include all service-provider defined extensions to the API. The API defines a mechanism that enables service-provider vendors to extend TAPI using device specific extensions. The API only defines the extension mechanism, and by doing so provides access to device specific extensions, but the API does not define their behavior. Behavior is completely defined by the service provider.

TAPI consists of scalar and bit-flag constant definitions, data structures, functions, and callback messages. Procedures are defined that enable a vendor to extend most of these as follows:

For extensible scalar data constants, a service-provider vendor may define new values in a specified range. As most data constants are DWORDs, typically the range 0x00000000 through 0x7FFFFFFF is reserved for common future extensions, while 0x80000000 through 0xFFFFFFFF are available for vendor-specific extensions. The assumption is that a vendor would define values that are natural extensions of the data types defined by the API.

For extensible bit-flag data constants, a service-provider vendor may define new values for specified bits. As most bit-flag constants are DWORDs, typically a specific number of the lower bits are reserved for common extensions while the remaining upper bits are available for vendor-specific extensions. Common bit flags are assigned from bit zero up; vendor-specific extensions should be assigned from bit 31 down. This provides maximum flexibility in assigning bit positions to common extensions versus vendor-specific extensions. A vendor is expected to define new values that are natural extensions of the bit flags defined by the API.

Extensible data structures have a variably sized field that is reserved for device-specific use. Being variably sized, the service provider decides the amount of information and the interpretation. A vendor that defines a device-specific field is expected to make these natural extensions of the original data structure defined by the API.

Two functions, lineDevSpecific and lineDevSpecificFeature, and two related messages, LINE_DEVSPECIFIC and LINE_DEVSPECIFICFEATURE, are provided whose only purpose is to provide a vendor-specific extension mechanism. The lineDevSpecific operation and associated LINE_DEVSPECIFIC message enables an application to access device-specific line, address, or call features that are unavailable with the Basic or Supplementary Telephony services. The parameter profile of the lineDevSpecific function is generic in that little interpretation of the parameters is made by the API. The interpretation of the parameters is defined by the service provider and must be understood by an application that uses them. The parameters are simply passed through by the Telephony DLL form the application to the service provider. An application that relies on device-specific extensions will not generally work with other service providers; however, applications written to the Basic and Supplementary Telephony Services will work with the extended service provider.

For convenience, a more specialized escape function is also provided. It is similar to lineDevSpecific, but places interpretation on some of the parameters. The function lineDevSpecificFeature is a device-specific escape function to allow sending switch features to the switch. The message LINE_DEVSPECIFICFEATURE is the device-specific message sent to the application's callback as an indication of features sent to the switch. This function and its associated message allow an application to emulate button presses at the line's feature phone. As feature phones and the meanings of their buttons are vendor-specific, feature invocation using lineDevSpecificFeature is also vendor specific.

As mentioned earlier, there is no central registry for manufacturer IDs. Instead a unique ID generator is made available as part the Telephony Developer Toolkit.