Chapter 14

Please note: The information contained in this documentation is preliminary and subject to change without notice.

Phone-related Data Structures

### PHONEBUTTONFUNCTION

The PHONEBUTTONFUNCTION scalar constants describe the functions commonly assigned to buttons on telephone sets.

PHONEBUTTONFUNCTION_UNKNOWN	0x00000000
PHONEBUTTONFUNCTION_CONFERENCE	0x00000001
PHONEBUTTONFUNCTION_TRANSFER	0x00000002
PHONEBUTTONFUNCTION_DROP	0x00000003
PHONEBUTTONFUNCTION_HOLD	0x00000004
PHONEBUTTONFUNCTION_RECALL	0x00000005
PHONEBUTTONFUNCTION_DISCONNECT	0x00000006
PHONEBUTTONFUNCTION_CONNECT	0x00000007
PHONEBUTTONFUNCTION_MSGWAITON	0x00000008
PHONEBUTTONFUNCTION_MSGWAITOFF	0x00000009
PHONEBUTTONFUNCTION_SELECTRING	0x0000000A
PHONEBUTTONFUNCTION_ABBREVDIAL	0x0000000B
PHONEBUTTONFUNCTION_FORWARD	0x0000000C
PHONEBUTTONFUNCTION_PICKUP	0x0000000D
PHONEBUTTONFUNCTION_RINGAGAIN	0x0000000E
PHONEBUTTONFUNCTION_PARK	0x0000000F
PHONEBUTTONFUNCTION_REJECT	0x00000010
PHONEBUTTONFUNCTION_REDIRECT	0x00000011
PHONEBUTTONFUNCTION_MUTE	0x00000012
PHONEBUTTONFUNCTION_VOLUMEUP	0x00000013
PHONEBUTTONFUNCTION_VOLUMEDOWN	0x00000014
PHONEBUTTONFUNCTION_SPEAKERON	0x00000015
PHONEBUTTONFUNCTION_SPEAKEROFF	0x00000016
PHONEBUTTONFUNCTION_FLASH	0x00000017
PHONEBUTTONFUNCTION_DATAON	0x00000018
PHONEBUTTONFUNCTION_DATAOFF	0x00000019
PHONEBUTTONFUNCTION_DONOTDISTURB	0x0000001A
PHONEBUTTONFUNCTION_INTERCOM	0x0000001B
PHONEBUTTONFUNCTION_BRIDGEDAPP	0x0000001C
PHONEBUTTONFUNCTION_BUSY	0x0000001D
PHONEBUTTONFUNCTION_CALLAPP	0x0000001E
PHONEBUTTONFUNCTION_DATETIME	0x0000001F
PHONEBUTTONFUNCTION_DIRECTORY	0x00000020
PHONEBUTTONFUNCTION_COVER	0x00000021
PHONEBUTTONFUNCTION_CALLID	0x00000022
PHONEBUTTONFUNCTION_LASTNUM	0x00000023
PHONEBUTTONFUNCTION_NIGHTSRV	0x00000024
PHONEBUTTONFUNCTION_SENDCALLS	0x00000025
PHONEBUTTONFUNCTION_MSGINDICATOR	0x00000026
PHONEBUTTONFUNCTION_REPDIAL	0x00000027
PHONEBUTTONFUNCTION_SETREPDIAL	0x00000028
PHONEBUTTONFUNCTION_SYSTEMSPEED	0x00000029
PHONEBUTTONFUNCTION_STATIONSPEED	0x0000002A
PHONEBUTTONFUNCTION_CAMPON	0x0000002B
PHONEBUTTONFUNCTION_SAVEREPEAT	0x0000002C
PHONEBUTTONFUNCTION_QUEUECALL	0x0000002D
PHONEBUTTONFUNCTION_NONE	0x0000002E

Extensibility	Values in the range 0x80000000 to 0xFFFFFFFF can be assigned for device-specific extensions; values in the range 0x00000000 to 0x7FFFFFFF are reserved.

Comments	The PHONEBUTTONFUNCTION constants have values commonly found on current telephone sets. These button functions can be used to invoke the corresponding function from the switch using TSPI_lineDevSpecificFeature. Note that TAPI does not define the semantics of the button functions, it only provides access to the corresponding function.

See Also	Functions:  TSPI_phoneGetButtonInfo, TSPI_phoneSetButtonInfo, TSPI_lineDevSpecificFeature

Messages:  PHONE_BUTTON

### PHONEBUTTONINFO

The PHONEBUTTONINFO structure contains information about a button on a phone device.

typedef struct phonebuttoninfo_tag {
	DWORD	dwTotalSize;
	DWORD	dwNeededSize;
	DWORD	dwUsedSize;

	DWORD	dwButtonMode;
	DWORD	dwButtonFunction;

	DWORD	dwButtonTextSize;
	DWORD	dwButtonTextOffset;

	DWORD	dwDevSpecificSize;
	DWORD	dwDevSpecificOffset;
} PHONEBUTTONINFO, FAR *LPPHONEBUTTONINFO;

#### Fields

The PHONEBUTTONINFO structure contains the following fields:

dwTotalSize

The total size in bytes allocated to this data structure.

dwNeededSize

The size in bytes for this data structure that is needed to hold all the returned information.

dwUsedSize

The size in bytes of this data structure that contains useful information.

dwButtonMode

Defines the mode or general usage class of the button, of type PHONEBUTTONMODE. Values are:

PHONEBUTTONMODE_DUMMY

This value is used to describe a button/lamp position that has no corresponding button, but has only a lamp.

PHONEBUTTONMODE_CALL

The button is assigned to a call appearance.

PHONEBUTTONMODE_FEATURE

The button is assigned to requesting features from the switch, such as hold, conference, and transfer.

PHONEBUTTONMODE_KEYPAD

The button is one of the twelve keypad buttons, ‘0’ through ‘9’, ‘*’, and ‘#’.

PHONEBUTTONMODE_LOCAL

The button is a local function button, such as mute or volume control.

PHONEBUTTONMODE_DISPLAY

The button is a “soft” button associated with the phone’s display.  A phone set can have zero or more display buttons.

dwButtonFunction

Specifies the function assigned to the button, of type PHONEBUTTONFUNCTION (not listed here).

dwButtonTextSize
dwButtonTextOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized field containing descriptive text for this button. The format of this information is as specified in the dwStringFormat field of the phone’s device capabilities.

dwDevSpecificSize
dwDevSpecificOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized device-specific field.

Extensibility	Device-specific extensions should use the DevSpecific (dwDevSpecificSize and dwDevSpecificOffset) variably sized area of this data structure.

Comments	None.

See Also	Functions:  TSPI_phoneGetButtonInfo, TSPI_phoneSetButtonInfo

Data Types:  PHONEBUTTONMODE, PHONEBUTTONFUNCTION

### PHONEBUTTONMODE

The PHONEBUTTONMODE bit flag constants describe the button classes.

PHONEBUTTONMODE_DUMMY	0x00000001
PHONEBUTTONMODE_CALL	0x00000002
PHONEBUTTONMODE_FEATURE	0x00000004
PHONEBUTTONMODE_KEYPAD	0x00000008
PHONEBUTTONMODE_LOCAL	0x00000010
PHONEBUTTONMODE_DISPLAY	0x00000020

#### Values

The PHONEBUTTONMODE constants have the following values:

PhoneButtonMode_dummy

This value is used to describe a button/lamp position that has no corresponding button, but has only a lamp.

PhoneButtonMode_call

The button is assigned to a call appearance.

PhoneButtonMode_feature

The button is assigned to requesting features from the switch, such as hold, conference, and transfer.

PhoneButtonMode_keypad

The button is one of the twelve keypad buttons, ‘0’ through ‘9’, ‘*’, and ‘#’.

PhoneButtonMode_local

The button is a local function button, such as mute or volume control.

PhoneButtonMode_display

The button is a “soft” button associated with the phone’s display. A phone set can have zero or more display buttons.

Extensibility	None. All 32 bits are reserved.

Comments	This enumeration type is used in the PHONECAPS data structure to describe the meaning associated with the phone’s buttons.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneGetButtonInfo, TSPI_phoneSetButtonInfo

Messages:  PHONE_BUTTON

Data Types:  PHONECAPS, PHONEBUTTONINFO

### PHONEBUTTONSTATE

The PHONEBUTTONSTATE bit flag constants describe various button positions.

PHONEBUTTONSTATE_UP	0x00000001
PHONEBUTTONSTATE_DOWN	0x00000002

#### Values

The PHONEBUTTONSTATE constants have the following values:

PhoneButtonState_up

The button is in the “up” state.

PhoneButtonState_down

The button is in the “down” state (pressed down).

Extensibility	None. All 32 bits are reserved.

Comments	None.

See Also	Functions:  TSPI_phoneGetDevCaps, TSPI_phoneSetStatusMessages

Messages:  PHONE_BUTTON

Data Types:  PHONECAPS

### PHONECAPS

The PHONECAPS structure describes the capabilities of a phone device.

typedef struct phonecaps_tag {
	DWORD	dwTotalSize;
	DWORD	dwNeededSize;
	DWORD	dwUsedSize;

	DWORD	dwProviderInfoSize;
	DWORD	dwProviderInfoOffset;

	DWORD	dwPhoneInfoSize;
	DWORD	dwPhoneInfoOffset;

	DWORD	dwPermanentPhoneID;
	DWORD	dwPhoneNameSize;
	DWORD	dwPhoneNameOffset;
	DWORD	dwStringFormat;

DWORD	dwPhoneStates;
	DWORD	dwHookSwitchDevs;
	DWORD	dwHandsetHookSwitchModes;
	DWORD	dwSpeakerHookSwitchModes;
	DWORD	dwHeadsetHookSwitchModes;

	DWORD	dwVolumeFlags;
	DWORD	dwGainFlags;
	DWORD	dwDisplayNumRows;
	DWORD	dwDisplayNumColumns;
	DWORD	dwNumRingModes;
	DWORD	dwNumButtonLamps;

	DWORD	dwButtonModesSize;
	DWORD	dwButtonModesOffset;

	DWORD	dwButtonFunctionsSize;
	DWORD	dwButtonFunctionsOffset;
	DWORD	dwLampModesSize;
	DWORD	dwLampModesOffset;

	DWORD	dwNumSetData;
	DWORD	dwSetDataSize;
	DWORD	dwSetDataOffset;

	DWORD	dwNumGetData
	DWORD	dwGetDataSize;
	DWORD	dwGetDataOffset;

	DWORD	dwDevSpecificSize;
	DWORD	dwDevSpecificOffset;


} PHONECAPS, FAR *LPPHONECAPS;

#### Fields

The PHONECAPS structure contains the following fields:

dwTotalSize

The total size in bytes allocated to this data structure.

dwNeededSize

The size in bytes for this data structure that is needed to hold all the returned information.

dwUsedSize

The size in bytes of this data structure that contains useful information.

dwProviderInfoSize
dwProviderInfoOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized field containing service-provider-specific information.

The dwProviderInfoSize/Offset field is intended to provide information about the provider hardware and/or software, such as the vendor name and version numbers of HW and SW. This information can be useful when a user needs to call customer service with problems regarding the provider.

dwPhoneInfoSize
dwPhoneInfoOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized device field containing phone-specific information.

The dwPhoneInfoSize/Offset field is intended to provide information about the attached phone device, such as the phone-device manufacturer, the model name, the software version, and so on. This information can be useful when a user needs to call customer service with problems regarding the phone.

dwPermanentPhoneID

Specifies the permanent DWORD identifier by which the phone device is known in the system’s configuration.

dwPhoneNameSize
dwPhoneNameOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized device field containing user configurable name for this phone device. This name can be configured by the user when configuring the phone device’s service provider and is provided for the user’s convenience.

dwStringFormat

Specifies the string format to be used with this phone device, of type STRINGFORMAT. Values are:

STRINGFORMAT_ASCII

This is ASCII string format using one byte per character.

STRINGFORMAT_DBCS

This is DBCS string format using two bytes per character.

STRINGFORMAT_UNICODE

This is unicode string format using two bytes per character.

dwPhoneStates

Specifies the state changes for this phone device for which the application can be notified via a PHONE_STATE callback message, of type PHONESTATE. Values are:

PHONESTATE_OTHER

An other phone status information item not listed below has changed.

PHONESTATE_CONNECTED

The connection between the phone device and TAPI was just made. This happens when TAPI is first invoked, or when the wire connection the phone to the PC is plugged in with TAPI active.

PHONESTATE_DISCONNECTED

The connection between the phone device and TAPI was just broken. This happens when the wire connecting the phone set to the PC is unplugged while TAPI is active.

PHONESTATE_OWNER

The number of owners for the phone device has changed.

PHONESTATE_MONITORS

The number of monitors for the phone device has changed.

PHONESTATE_DISPLAY

The display of the phone has changed.

PHONESTATE_LAMP

A lamp of the phone has changed.

PHONESTATE_RINGMODE

The ring mode of the phone has changed.

PHONESTATE_RINGVOLUME

The ring volume of the phone has changed.

PHONESTATE_HANDSETHOOKSWITCH

The handset hookswitch state has changed.

PHONESTATE_HANDSETVOLUME

The handset’s speaker volume setting has changed.

PHONESTATE_HANDSETGAIN

The handset’s mic gain setting has changed.

PHONESTATE_SPEAKERHOOKSWITCH

The speakerphone’s hookswitch state has changed.

PHONESTATE_SPEAKERVOLUME

The speakerphone’s speaker volume setting has changed.

PHONESTATE_SPEAKERGAIN

The speakerphone’s mic gain setting state has changed.

PHONESTATE_HEADSETHOOKSWITCH

The headset’s hookswitch state has changed.

PHONESTATE_HEADSETVOLUME

The headset’s speaker volume setting has changed.

PHONESTATE_HEADSETGAIN

The headset’s mic gain setting has changed.

PHONESTATE_REINIT

Items have changed in the configuration of phone devices. To become aware of these changes (such as for the appearance of new phone devices) the application should reinitialize its use of TAPI. If specified, the htPhone parameter is left NULL for this state change as it applies to any of the phones in the system.

PHONESTATE_SUSPEND

The application’s use of the phone is temporarily suspended.

PHONESTATE_RESUME

The application’s use of the phone device is resumed after having been suspended for some time.

PHONESTATE_DEVSPECIFIC

The phone’s device-specific information has changed.

dwHookSwitchDevs

This field specifies the phone’s hookswitch devices, of type PHONEHOOKSWITCHDEV. Values are:

PHONEHOOKSWITCHDEV_HANDSET

This is the ubiquitous, hand held ear and mouth piece.

PHONEHOOKSWITCHDEV_SPEAKER

A built-in loudspeaker and microphone. This could also be an externally connected adjunct to the telephone set.

PHONEHOOKSWITCHDEV_HEADSET

This is a headset connected to the phone set.

dwHandsetHookSwitchModes
dwSpeakerHookSwitchModes
dwHeadsetHookSwitchModes

This field specifies the phone’s hookswitch mode capabilities of the handset, speaker, or headset respectively, of type PHONEHOOKSWITCHMODE. The field is only meaningful if the hookswitch device is listed in dwHookSwitchDevs. Values are:

PHONEHOOKSWITCHMODE_ONHOOK

The device’s mic and speaker are both onhook.

PHONEHOOKSWITCHMODE_MIC

The devicer’s mic is active, the speaker is mute.

PHONEHOOKSWITCHMODE_SPEAKER

The device’s speaker is active, the mic is mute.

PHONEHOOKSWITCHMODE_MICSPEAKER

The device’s mic and speaker are both active.

dwVolumeFlags

This field specifies the volume setting capabilities of the phone device’s speaker components. If the bit in position PHONEHOOKSWITCHDEV is TRUE then the volume of the corresponding hookswitch device’s speaker component can be adjusted via TSPI_phoneSetVolume; otherwise FALSE.

dwGainFlags

This field specifies the gain setting capabilities of the phone device’s mic components. Of the bit position PHONEHOOKSWITCHDEV is TRUE then the volume of the corresponding hookswitch device’s mic component can be adjusted via the TSPI_phoneSetGain; otherwise FALSE.

dwDisplayNumRows

This field specifies the display capabilities of the phone device by describing the number of rows in the phone display. dwDisplayNumRows and dwDisplayNumColumns are both zero for a phone device without a display.

dwDisplayNumColumns

This field specifies the display capabilities of the phone device by describing the number of columns in the phone display. dwDisplayNumRows and dwDisplayNumColumns are both zero for a phone device without a display.

dwNumRingModes

This field specifies the ring capabilities of the phone device. The phone is able to ring with dwNumRingModes different ring patterns, identified as 1, 2, ¼ dwNumRingModes - 1. A ring patterns of zero is meant to use silence.

dwNumButtonLamps

This field specifies the number of button/lamps on the phone device that are detectable in TAPI. Button/lamps are identified by their ID. Valid button/lamp IDs range from zero to dwNumButtonLamps - 1. The keypad buttons ‘0’, through ‘9’, ‘*’, and ‘#’ are assigned the IDs 0 through 12.

dwButtonModesSize
dwButtonModesOffset

The size in bytes and the offset from the beginning of this data structure in bytes of the variably sized field containing the button modes of the phone’s buttons, of type PHONEBUTTONMODE. The array is indexed by button/lamp ID. Values are:

PHONEBUTTONMODE_DUMMY

This value is used to describe a button/lamp position that has no corresponding button, but has only a lamp. If the phone provides any non-DUMMY buttons, the PHONE_BUTTON message will be sent to the application if a button is pressed at the phone device.

PHONEBUTTONMODE_CALL

The button is assigned to a call appearance.

PHONEBUTTONMODE_FEATURE

The button is assigned to requesting features from the switch, such as hold, conference, and transfer.

PHONEBUTTONMODE_KEYPAD

The button is one of the twelve keypad buttons, ‘0’ through ‘9’, ‘*’, and ‘#’.

PHONEBUTTONMODE_LOCAL

The button is a local function button, such as mute or volume control.

PHONEBUTTONMODE_DISPLAY

The button is a “soft” button associated with the phone’s display.  A phone set can have zero or more display buttons.

dwButtonFunctionsSize
dwButtonFunctionsOffset

The size in bytes and the offset from the beginning of this data structure in bytes of the variably sized field containing the button modes of the phone’s buttons, of type PHONEBUTTONFUNCTION. The array is indexed by button/lamp ID.

dwLampModesSize
dwLampModesOffset

The size in bytes and the offset from the beginning of this data structure in bytes of the variably sized field containing the lamp modes of the phone’s lamps, of type PHONELAMPMODE. The array is indexed by button/lamp ID. Values are:

PHONELAMPMODE_BROKENFLUTTER

Broken flutter is the superposition of flash and flutter.

PHONELAMPMODE_FLASH

Flash means slow on and off.

PHONELAMPMODE_FLUTTER

Flutter means fast on and off.

PHONELAMPMODE_OFF

The lamp is off.

PHONELAMPMODE_STEADY

The lamp is continuously lit.

PHONELAMPMODE_WINK

The lamp is winking.

PHONELAMPMODE_DUMMY

This value is used to describe a button/lamp position that has no corresponding lamp.

dwNumSetData

The number of different download areas in the phone device. The different areas are referred to using the data IDs 0, 1, ¼, dwNumSetData - 1. If this field is zero, the phone does not support the download capability.

dwSetDataSize
dwSetDataOffset

The size in bytes and the offset from the beginning of this data structure in bytes of the variably sized field containing the sizes (in bytes) of the phone’s download data areas. This is an array with DWORD-sized elements, indexed by data ID.

dwNumGetData

The number of different upload areas in the phone device. The different areas are referred to using the data IDs 0, 1, ¼, dwNumGetData - 1. If this field is zero, the phone does not support the upload capability.

dwGetDataSize
dwGetDataOffset

The size in bytes and the offset from the beginning of this data structure in bytes of the variably sized field containing the sizes (in bytes) of the phone’s upload data areas. This is an array with DWORD-sized elements, indexed by data ID.

dwDevSpecificSize
dwDevSpecificOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized device-specific field.

Extensibility	Device-specific extensions should use the DevSpecific (dwDevSpecificSize and dwDevSpecificOffset) variably sized area of this data structure.

Comments	None.

See Also	Functions:  TSPI_phoneGetDevCaps

Data types:  PHONEBUTTONMODE, PHONELAMPMODE, PHONEHOOKSWITCHDEV

### PHONEERR

This is the list of error codes that the implementation may return when invoking operations on phone devices. Consult the individual function descriptions to determine which of these error codes each function may return.

PHONEERR_ALLOCATED	0x90000001
PHONEERR_BADDEVICEID	0x90000002
PHONEERR_INCOMPATIBLEAPIVERSION	0x90000003
PHONEERR_INCOMPATIBLEEXTVERSION	0x90000004
PHONEERR_INIFILECORRUPT	0x90000005
PHONEERR_INUSE	0x90000006
PHONEERR_INVALAPPHANDLE	0x90000007
PHONEERR_INVALAPPNAME	0x90000008
PHONEERR_INVALBUTTONLAMPID	0x90000009
PHONEERR_INVALBUTTONMODE	0x9000000A
PHONEERR_INVALBUTTONSTATE	0x9000000B
PHONEERR_INVALDATAID	0x9000000C
PHONEERR_INVALDEVICECLASS	0x9000000D
PHONEERR_INVALEXTVERSION	0x9000000E
PHONEERR_INVALHOOKSWITCHDEV	0x9000000F
PHONEERR_INVALHOOKSWITCHMODE	0x90000010
PHONEERR_INVALLAMPMODE	0x90000011
PHONEERR_INVALPARAM	0x90000012
PHONEERR_INVALPHONEHANDLE	0x90000013
PHONEERR_INVALPHONESTATE	0x90000014
PHONEERR_INVALPOINTER	0x90000015
PHONEERR_INVALPRIVILEGE	0x90000016
PHONEERR_INVALRINGMODE	0x90000017
PHONEERR_NODEVICE	0x90000018
PHONEERR_NODRIVER	0x90000019
PHONEERR_NOMEM	0x9000001A
PHONEERR_NOTOWNER	0x9000001B
PHONEERR_OPERATIONFAILED	0x9000001C
PHONEERR_OPERATIONUNAVAIL	0x9000001D
PHONEERR_RESOURCEUNAVAIL	0x9000001F
PHONEERR_REQUESTOVERRUN	0x90000020
PHONEERR_STRUCTURETOOSMALL	0x90000021
PHONEERR_UNINITIALIZED	0x90000022
PHONEERR_REINIT	0x90000023

Extensibility	The values 0xC0000000 through 0xFFFFFFFF are available for device-specific extensions, the values 0x80000000 through 0xBFFFFFFF are reserved, and 0x00000000 through 0x7FFFFFFF are used as request IDs.

Comments	None.

See Also	Messages:  PHONE_REPLY

### PHONEEXTENSIONID

The PHONEEXTENSIONID structure describes an extension ID. Extension IDs are used to identify service-provider-specific extensions for phone device classes.

typedef struct phoneextensionid_tag {
	DWORD	dwExtensionID0;
	DWORD	dwExtensionID1;
	DWORD	dwExtensionID2;
	DWORD	dwExtensionID3;
} PHONEEXTENSIONID, FAR *LPPHONEEXTENSIONID;

#### Values

The PHONEEXTENSIONID structure contains the following fields:

dwExtensionID0
dwExtensionID1
dwExtensionID2
dwExtensionID3

These four DWORD-sized fields together specify a universally unique extension ID that identifies a phone device class extension.

Extensibility	None.

Comments	Extension IDs are generated using an SDK-provided generation utility.

See Also	Functions:  TSPI_phoneNegotiateTSPIVersion

### PHONEHOOKSWITCHDEV

The PHONEHOOKSWITCHDEV bit flag constants describe various audio I/O devices each with its own hookswitch controllable from the PC.

PHONEHOOKSWITCHDEV_HANDSET	0x00000001
PHONEHOOKSWITCHDEV_SPEAKER	0x00000002
PHONEHOOKSWITCHDEV_HEADSET	0x00000004

#### Values

The PHONEHOOKSWITCHDEV constants have the following values:

PhoneHookSwitchDev_handset

This is the ubiquitous, hand held ear and mouth piece.

PhoneHookSwitchDev_speaker

This is a built-in loudspeaker and microphone. This could also be an externally connected adjunct speaker to the telephone set.

PhoneHookSwitchDev_headset

This is a headset connected to the phone set.

Extensibility	None. All 32 bits are reserved.

Comments	These constants are used in the PHONECAPS data structure to indicate the hookswitch device capabilities of a phone device. The PHONESTATUS structure reports the state of the phone’s hookswitch devices. The function TSPI_phoneSetHookSwitch and TSPI_phoneGetHookSwitch use it as a parameter to select the phone’s I/O device.

See Also	Functions:  TSPI_phoneSetHookSwitch, TSPI_phoneGetHookSwitch, TSPI_phoneGetDevCaps, TSPI_phoneGetStatus

Data Types:  PHONECAPS, PHONESTATUS

### PHONEHOOKSWITCHMODE

The PHONEHOOKSWITCHMODE bit flag constants describe the mic and speaker components of a hookswitch device.

PHONEHOOKSWITCHMODE_ONHOOK	0x00000001
PHONEHOOKSWITCHMODE_MIC	0x00000002
PHONEHOOKSWITCHMODE_SPEAKER	0x00000004
PHONEHOOKSWITCHMODE_MICSPEAKER	0x00000008
PHONEHOOKSWITCHMODE_UNKNOWN	0x00000010

#### Values

The PHONEHOOKSWITCHMODE constants have the following values:

PhoneHookSwitchMode_onHook

The device’s mic and speaker are both onhook.

PhoneHookSwitchMode_mic

The devicer’s mic is active, the speaker is mute.

PhoneHookSwitchMode_speaker

The device’s speaker is active, the mic is mute.

PhoneHookSwitchMode_micSpeaker

The device’s mic and speaker are both active.

PhoneHookSwitchMode_unknown

The device’s hookswitch mode is currenly unknown.

Extensibility	None. All 32 bits are reserved.

Comments	These constants are used to provide individual level of control over the mic and speaker components of a phone device.

See Also	Functions:  TSPI_phoneSetHookSwitch, TSPI_phoneGetHookSwitch, TSPI_phoneGetDevCaps, TSPI_phoneGetStatus

Data structures:  PHONECAPS, PHONESTATUS

### PHONELAMPMODE

The PHONELAMPMODE bit flag constants describe various ways in which a phone’s lamp can be lit.

PHONELAMPMODE_DUMMY	0x00000001
PHONELAMPMODE_OFF	0x00000002
PHONELAMPMODE_STEADY	0x00000004
PHONELAMPMODE_WINK	0x00000008
PHONELAMPMODE_FLASH	0x00000010
PHONELAMPMODE_FLUTTER	0x00000020
PHONELAMPMODE_BROKENFLUTTER	0x00000040
PHONELAMPMODE_UNKNOWN	0x00000080

#### Values

The PHONELAMPMODE constants have the following values:

PhoneLampMode_dummy

This value is used to describe a button/lamp position that has no corresponding lamp.

PhoneLampMode_off

The lamp is off.

PhoneLampMode_steady

Steady means the lamp is continuously lit.

PhoneLampMode_wink

Wink means normal rate on and off.

PhoneLampMode_flash

Flash means slow on and off.

PhoneLampMode_flutter

Flutter means fast on and off.

PhoneLampMode_brokenflutter

Broken flutter is the superposition of flash and flutter.

PhoneLampMode_unknown

The lamp mode is currently unknown.

Extensibility	The high order 16 bits can be assigned for device-specific extensions. The low order 16 bits are reserved.

Comments	Where the exact on and off cadences may differ across phone sets from different vendors, mapping of actual lamp lighting patterns for most phones onto the values listed above should be straightforward.

See Also	Functions:  TSPI_phoneSetLamp, TSPI_phoneGetLamp, TSPI_phoneGetDevCaps, TSPI_phoneGetStatus

Data Types:  PHONECAPS, PHONESTATUS

### PHONESTATE

The PHONESTATE bit flag constants describe various status items for a phone device.

PHONESTATE_OTHER	0x00000001
PHONESTATE_CONNECTED	0x00000002
PHONESTATE_DISCONNECTED	0x00000004
PHONESTATE_OWNER	0x00000008
PHONESTATE_MONITORS	0x00000010
PHONESTATE_DISPLAY	0x00000020
PHONESTATE_LAMP	0x00000040
PHONESTATE_RINGMODE	0x00000080
PHONESTATE_RINGVOLUME	0x00000100
PHONESTATE_HANDSETHOOKSWITCH	0x00000200
PHONESTATE_HANDSETVOLUME	0x00000400
PHONESTATE_HANDSETGAIN	0x00000800
PHONESTATE_SPEAKERHOOKSWITCH	0x00001000
PHONESTATE_SPEAKERVOLUME	0x00002000
PHONESTATE_SPEAKERGAIN	0x00004000
PHONESTATE_HEADSETHOOKSWITCH	0x00008000
PHONESTATE_HEADSETVOLUME	0x00010000
PHONESTATE_HEADSETGAIN	0x00020000
PHONESTATE_SUSPEND	0x00040000
PHONESTATE_RESUME	0x00080000
PHONESTATE_DEVSPECIFIC	0x00100000
PHONESTATE_REINIT	0x00200000

#### Values

The PHONESTATE constants have the following values:

PhoneState_other

A non-specified phone status information item has changed.

PhoneState_connected

The connection between the phone device and TAPI was just made. This happens when TAPI is first invoked, or when the wire connection the phone to the PC is plugged in with TAPI active.

PhoneState_disconnected

The connection between the phone device and TAPI was just broken. This happens when the wire connecting the phone set to the PC is unplugged while TAPI is active.

PhoneState_owner

The number of owners for the phone device.

PhoneState_monitors

The number of monitors for the phone device.

PhoneState_display

The display of the phone has changed.

PhoneState_lamp

A lamp of the phone has changed.

PhoneState_ringmode

The ring mode of the phone has changed.

PhoneState_ringvolume

The ring volume of the phone has changed.

PhoneState_handsetHookSwitch

The handset hookswitch state has changed.

PhoneState_handsetVolume

The handset’s speaker volume setting has changed.

PhoneState_handsetGain

The handset’s mic gain setting has changed.

PhoneState_speakerHookSwitch

The speakerphone’s hookswitch state has changed.

PhoneState_speakerVolume

The speakerphone’s speaker volume setting has changed.

PhoneState_speakerGain

The speakerphone’s mic gain setting state has changed.

PhoneState_headsetHookSwitch

The headset’s hookswitch state has changed.

PhoneState_headsetVolume

The headset’s speaker volume setting has changed.

PhoneState_headsetGain

The headset’s mic gain setting has changed.

PhoneState_suspend

The application’s use of the phone is temporarily suspended.

PhoneState_resume

The application’s use of the phone device is resumed after having been suspended for some time.

PhoneState_devspecific

The phone’s device-specific information has changed.

PhoneState_reinit

Items have changed in the configuration of phone devices. To become aware of these changes (as for the appearance of new phone devices) the application should reinitialize its use of TAPI.

Extensibility	None. All 32 bits are reserved.

Comments	None.

See Also	Functions:  TSPI_phoneSetStatusMessages

Messages:  PHONE_STATE

Data Types:  PHONESTATUS

### PHONESTATUS

The PHONESTATUS structure describes the current status of a phone device.

typedef struct phonestatus_tag {
	DWORD	dwTotalSize;
	DWORD	dwNeededSize;
	DWORD	dwUsedSize;

	DWORD	dwStatusFlags; 
	DWORD	dwNumOwners;
	DWORD	dwNumMonitors;
	DWORD	dwRingMode;
	DWORD	dwRingVolume;

	DWORD	dwHandsetHookSwitchMode;
	DWORD	dwHandsetVolume;
	DWORD	dwHandsetGain;

	DWORD	dwSpeakerHookSwitchMode;
	DWORD	dwSpeakerVolume;
	DWORD	dwSpeakerGain;

	DWORD	dwHeadsetHookSwitchMode;
	DWORD	dwHeadsetVolume;
	DWORD	dwHeadsetGain;

	DWORD	dwDisplaySize;
	DWORD	dwDisplayOffset;

	DWORD	dwLampModesSize;
	DWORD	dwLampModesOffset;

	DWORD	dwOwnerNameSize;
	DWORD	dwOwnerNameOffset;

	DWORD	dwDevSpecificSize;
	DWORD	dwDevSpecificOffset;
} PHONESTATUS, FAR *LPPHONESTATUS;

#### Fields

The PHONESTATUS structure has the following fields:

dwTotalSize

The total size in bytes allocated to this data structure.

dwNeededSize

The size in bytes for this data structure that is needed to hold all the returned information.

dwUsedSize

The size in bytes of this data structure that contains useful information.

dwStatusFlags

This field provides a collection of status flags for this phone device, of type PHONESTATUSFLAGS. Values are:

PHONESTATUSFLAGS_CONNECTED

Specifies whether the phone is currently connected to TAPI. TRUE if connected, FALSE otherwise.

PHONESTATUSFLAGS_SUSPENDED

Specifies whether TAPI’s manipulation of the phone device is suspended or not. TRUE is suspended, FALSE otherwise. An application’s use of a phone device may be temporarily suspended when the switch wants to manipulate the phone in a way that cannot tolerate interference from the application.

dwRingMode

Specifies the current ring mode of a phone device.

dwRingVolume

Specifies the current ring volume of a phone device. This is a value between 0x00000000 (silence) and 0x0000FFFF (maximum volume).

dwHandsetHookSwitchMode

Specifies the current hook switch mode of the phone’s handset, of type PHONEHOOKSWITCHMODE. Values are:

PHONEHOOKSWITCHMODE_ONHOOK

The device’s mic and speaker are both onhook.

PHONEHOOKSWITCHMODE_MIC

The devicer’s mic is active, the speaker is mute.

PHONEHOOKSWITCHMODE_SPEAKER

The device’s speaker is active, the mic is mute.

PHONEHOOKSWITCHMODE_MICSPEAKER

The device’s mic and speaker are both active.

dwHandsetVolume

Specifies the current speaker volume of the phone’s handset device. This is a value between 0x00000000 (silence) and 0x0000FFFF (maximum volume).

dwHandsetGain

Specifies the current mic gain of the phone’s handset device. This is a value between 0x00000000 (silence) and 0x0000FFFF (maximum gain).

dwSpeakerHookSwitchMode

Specifies the current hook switch mode of the phone’s speakerphone, of type PHONEHOOKSWITCHMODE. Values are:

PHONEHOOKSWITCHMODE_ONHOOK

The device’s mic and speaker are both onhook.

PHONEHOOKSWITCHMODE_MIC

The devicer’s mic is active, the speaker is mute.

PHONEHOOKSWITCHMODE_SPEAKER

The device’s speaker is active, the mic is mute.

PHONEHOOKSWITCHMODE_MICSPEAKER

The device’s mic and speaker are both active.

dwSpeakerVolume

Specifies the current speaker volume of the phone’s speaker device. This is a value between 0x00000000 (silence) and 0x0000FFFF (maximum volume).

dwSpeakerGain

Specifies the current mic gain of the phone’s speaker device. This is a value between 0x00000000 (silence) and 0x0000FFFF (maximum gain).

dwHeadsetHookSwitchMode

Specifies the current hook switch mode of the phone’s headset, of type PHONEHOOKSWITCHMODE. Values are:

PHONEHOOKSWITCHMODE_ONHOOK

The device’s mic and speaker are both onhook.

PHONEHOOKSWITCHMODE_MIC

The devicer’s mic is active, the speaker is mute.

PHONEHOOKSWITCHMODE_SPEAKER

The device’s speaker is active, the mic is mute.

PHONEHOOKSWITCHMODE_MICSPEAKER

The device’s mic and speaker are both active.

dwHeadsetVolume

Specifies the current speaker volume of the phone’s headset device. This is a value between 0x00000000 (silence) and 0x0000FFFF (maximum volume).

dwHeadsetGain

Specifies the current mic gain of the phone’s headset device. This is a value between 0x00000000 (silence) and 0x0000FFFF (maximum gain).

dwDisplaySize
dwDisplayOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized field containing the phone’s current display information.

dwLampModesSize
dwLampModesOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized field containing the phone’s current lamp modes.

dwOwnerNameSize
dwOwnerNameOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized field containing the name of the application that is the the current owner of the phone device. The name is the application name provided by the application when it invoked phoneInitialize. If no application name was supplied, the application’s module name is used instead. If the phone currently has no owner, dwOwnerNameSize is zero.

dwDevSpecificSize
dwDevSpecificOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized device-specific field.

Extensibility	Device-specific extensions should use the DevSpecific (dwDevSpecificSize and dwDevSpecificOffset) variably sized area of this data structure.

Comments	None.

See Also	Functions:  TSPI_phoneGetStatus

Data types:  PHONELAMPMODE, PHONEHOOKSWITCHDEV, PHONEHOOKSWITCHMODE, PHONESTATUSFLAGS, PHONESTATE

### PHONESTATUSFLAGS

The PHONESTATUSFLAGS bit flag constants describe a variety of phone device status information.

PHONESTATUSFLAGS_CONNECTED	0x00000001
PHONESTATUSFLAGS_SUSPENDED	0x00000002

#### Values

The PHONESTATUSFLAGS constants have the following values.

PhoneStatusFlags_connected

Specifies whether the phone is currently connected to TAPI. TRUE if connected, FALSE otherwise.

PhoneStatusFlags_suspended

Specifies whether TAPI’s manipulation of the phone device is suspended or not. TRUE is suspended, FALSE otherwise. An application’s use of a phone device may be temporarily suspended when the switch wants to manipulate the phone in a way that cannot tolerate interference from the application.

Extensibility	None. All 32 bits are reserved.

Comments	None.

See Also	Data Types:  PHONESTATUS

### STRINGFORMAT

The STRINGFORMAT enumeration constants describe different string formats.

STRINGFORMAT_ASCII	0x00000001
STRINGFORMAT_DBCS	0x00000002
STRINGFORMAT_UNICODE	0x00000003
STRINGFORMAT_BINARY	0x00000004

#### Values

The STRINGFORMAT constants have the following values.

StringFormat_ASCII

Specifies standard ASCII character format using one byte per character.

StringFormat_DBCS

Specifies standard DBCS character format using two bytes per character.

StringFormat_unicode

Specifies standard Unicode character format using two bytes per character.

StringFormat_binary

This is an array of unsigned characters; could be used for numeric values.

Extensibility	None. All 32 bits are reserved.

Comments	None.

See Also	Data Types:  VARSTRING

### VARSTRING

The VARSTRING structure is used for returning variably sized strings. It is used both by the line device class and the phone device class.

typedef struct varstring_tag {
	DWORD	dwTotalSize;
	DWORD	dwNeededSize;
	DWORD	dwUsedSize;

	DWORD	dwStringFormat;
	DWORD	dwStringSize;
	DWORD	dwStringOffset;
} VARSTRING, FAR *LPVARSTRING;

#### Fields

The VARSTRING structure contains the following fields:

dwTotalSize

The total size in bytes allocated to this data structure.

dwNeededSize

The size in bytes for this data structure that is needed to hold all the returned information.

dwUsedSize

The size in bytes of this data structure that contains useful information.

The size in bytes and the offset in bytes from the beginning of the containing data structure of a variably sized field.

dwStringFormat

Specifies the format of the string, of type STRINGFORMAT. Values are:

STRINGFORMAT_ASCII

This is ASCII string format using one byte per character. The actual string is a NULL-terminated ASCII string with the terminating NULL included in the string size. If dwStringFormat is STRINGFORMAT_ASCII, the actual string is a NULL-terminated ASCII string with the terminating NULL included in the string size.

STRINGFORMAT_DBCS

This is DBCS string format using one or two bytes per character.

STRINGFORMAT_UNICODE

This is unicode string format using two bytes per character.

STRINGFORMAT_BINARY

This is an array of unsigned characters; could be used for numeric values.

dwStringSize
dwStringOffset

The size in bytes and the offset in bytes from the beginning of this data structure of the variably sized device field containing the string information.

Extensibility	None.

Comments	None.

See Also	Data Types:  STRINGFORMAT