This specification defines standard behavior of the video-casette recorder (VCR) device class for the Media Control Interface (MCI), or MCI VCR. It applies to low-end equipment, such as VCR players, and high-end equipment, such as VTRs. This specification was developed in collaboration with OEMs to ensure that it reflects the inherent characteristics of video-tape recorders. However, this specification is not limited to a single manufacturer’s hardware and is expected to be applicable to a wide range of hardware implementations.

Overview

This document is not self-contained. In particular, it assumes you have a general familiarity with the documentation for Windows with multimedia extensions, as well as a general familiarity with Windows programming. This document contains the following sections:

·	The overview summarizes the documentation and lists available resources.

·	Chapter 1, “Video-Cassette Recorder Commands for MCI,” provides an overview of the characteristics of a VCR device and introduces the digital-VCR command set.

·	Chapter 2, “VCR Command Strings,” lists the MCI command strings available for VCR devices.

·	Chapter 3, “VCR Command Messages,” lists the MCI command messages available for VCR devices.

·	Appendix A, “VCR MCI Command String Syntax,” summarizes the digital-video commands and their proper usage for digital-video devices that use files.

The major purpose of this document is to provide clear guidelines to any developer of a VCR device driver as to how to implement specific features. Because a feature is described herein does not imply that the developer of a compliant VCR device driver must support that feature. All VCR device drivers should use the following guidelines:

(a) A VCR device driver that does not support a specific aspect of the specification should return an MCIERR_UNSUPPORTED_FUNCTION error code.

(b) A VCR device driver that supports functionality described by this specification should use those aspects of the specification relevant to performing that functionality.

(c) All command strings and MCI messages should support the test flag. This lets an application check whether a device driver supports a proposed action.

This standard presents new and updated information for the MCI command set. This document relies on the information previously published for MCI commands and programming. The following sources contain this information:

1.	The Microsoft Multimedia Development Kit (MDK) 1.0 Programmer’s Reference

2.	The Microsoft Windows 3.1 Software Development Kit (SDK) Multimedia Programmer’s Reference

3.	The Microsoft Windows 3.1 Software Development Kit (SDK) Multimedia Programmer’s Guide

4.	The Multimedia Programmer’s Reference from Microsoft Press

5.	The Multimedia Programmer’s Workbook from Microsoft Press

6.	The Microsoft Windows 3.1 Device Development Kit (DDK) Multimedia Device Adaptation Guide (this guide is needed only by developers creating device drivers)

# Where to Look for Information

Current versions of this document as well as other technical updates and technical notes and sample code are available from the following sources:

1.	Compuserve WINSDK forum.

2.	Microsoft Multimedia BBS at (206) 936-4082 in the files library in the specs section in the DVMCI1.ZIP file. Technical notes are available in the technote section. BBS modem settings are 9600 baud, no parity, 8 data bits, 1 stop bit.

3.	Via anonymous FTP on ftp.uu.net in the VENDORS\MICROSOFT\MULTIMEDIA directory. Sample code is in the SAMPLES directory and technical notes are in the TECHNOTE directory.

4.	Current versions of any document can be ordered (currently free) by calling (206) 936-8644.

# Versions of this Document

This document is continually being updated and expanded. Eventually the information presented in this document will be included in the standard references from Microsoft, such as the Multimedia Programmer’s Reference from Microsoft Press.

When referring to standards defined in this document, please refer to the data and version number printed on the cover page.

Versions of this document with the same version number are just expanded editions of the same information.

# If You Have Questions

If you have questions, requests, or problems with this technical update, you should send your question to the following address:

Microsoft Corp.
Multimedia Product Management
One Microsoft Way
Redmond, WA 98052-6399

FAX (206) 93MSFAX