Chapter 13

Please note: The information contained in this documentation is preliminary and subject to change without notice.

Phone-related Messages

### PHONE_BUTTON

This message is sent to notify the TAPI DLL that it has detected a button press on a phone device that the TAPI DLL has open and for which button press monitoring is enabled.

#### Sent to

PHONEEVENT

#### Parameters

HTAPIPHONE    htPhone

Specifies the TAPI DLL’s opaque object handle to the phone device.

DWORD    dwMsg

The value PHONE_BUTTON

DWORD    dwParam1

Specifies the button/lamp ID of the button that was pressed. Note that the button IDs 0 through 11 are always the KEYPAD buttons. ‘0’ is button ID 0, ‘1’ is button ID 1, and so on, through button ID 9. ‘*’ is button ID 10, and ‘#’ is button ID 11. Additional information about a button ID is available via TSPI_phoneGetDevCaps, and TSPI_phoneGetButtonInfo.

DWORD    dwParam2

Specifies the button mode of the button, of type PHONEBUTTONMODE. Values are:

PHONEBUTTONMODE_CALL

The button is assigned to a call appearance.

PHONEBUTTONMODE_FEATURE

The button is assigned to requesting features from the switch, such as hold, conference, or transfer.

PHONEBUTTONMODE_KEYPAD

The button is one of the twelve keypad buttons, ‘0’ through ‘9’, ‘*’, and ‘#’.

PHONEBUTTONMODE_LOCAL

The button is a local function button, such as mute or volume control.

PHONEBUTTONMODE_DISPLAY

The button is a “soft” button associated with the phone’s display. A phone set can have zero or more display buttons.

DWORD    dwParam3

Specifies whether this is a button down event or a button up event, of type PHONEBUTTONSTATE. Values are:

PHONEBUTTONSTATE_UP

The button is in the “up” state.

PHONEBUTTONSTATE_DOWN

The button is in the “down” state (pressed down).

Comments	The set of phone button events that is reported is selected by the function TSPI_phoneSetStatusMessages. The service provider must send PHONE_BUTTON messages for at least the set of button press events selected through that procedure. The service provider may send more than this set, however, it should try to limit its messages to this set for performance reasons.

A PHONE_BUTTON message is sent whenever a button changes state. The service provider must guarantee that each button DOWN event is strictly paired with a button UP event, in that order. Note that a service provider that can detect only one of the button press or release events should expand the event it does detect into a button DOWN message followed by a button UP message.

See Also	Data Types:  PHONESTATUS, PHONEBUTTONMODE, PHONEBUTTONSTATE, PHONEBUTTONFUNCTION

Functions: TSPI_phoneGetDevCaps, TSPI_phoneGetButtonInfo, TSPI_phoneSetStatusMessages

### PHONE_CLOSE

This message is sent when an open phone device has been forcibly closed.  The phone device handle is longer valid once this message has been sent. The service provider guarantees that all asynchronous requests on the phone have been reported complete by calling ASYNC_COMPLETION for each outstanding request before this message is sent. The TAPI DLL must not request any future operations using this phone handle.

#### Sent to

PHONEEVENT

#### Parameters

HTAPIPHONE    htPhone

Specifies the TAPI DLL’s opaque object handle to the phone device.

DWORD    dwMsg

The value PHONE_CLOSE

DWORD    dwParam1

Unused.

DWORD    dwParam2

Unused.

DWORD    dwParam3

Unused.

Comments	This message is only sent to the TAPI DLL after an open phone has been forcibly closed. This may be done, for example, when taking the phone out of service or reconfiguring the service provider. Whether or not the phone can be reopened immediately after a forced close is up to the service provider.

An open phone device may be forcibly closed after the user has modified the configuration of that phone or its driver. If the user wants the configuration changes to be effective immediately (as opposed to after the next system restart), and they affect the application’s current view of the device (such as a change in device capabilities), then a service provider may forcibly close the phone device.

See Also	Functions:  TSPI_phoneClose

### PHONE_DEVSPECIFIC

This message is sent to notify the TAPI DLL about device-specific events occurring at the phone. The meaning of the message and the interpretation of the parameters is implementation-defined.

#### Sent to

PHONEEVENT

#### Parameters

HTAPIPHONE    htPhone

Specifies the TAPI DLL’s opaque object handle to the phone device.

DWORD    dwMsg

The value PHONE_DEVSPECIFIC

DWORD    dwParam1

Device specific.

DWORD    dwParam2

Device specific.

DWORD    dwParam3

Device specific.

Comments	The TAPI DLL sends the PHONE_DEVSPECIFIC message to applications in response to receiving this message from a service provider. The htPhone is translated to the appropriate hPhone as the hDevice parameter at the TAPI level. The dwParam1, dwParam2, and dwParam3 paramters are passed through unmodified.

See Also	Functions: TSPI_phoneDevSpecific

### PHONE_REPLY

Comparison	This message is not used at the TSPI level. Completion of an asynchronous request is reported using the ASYNC_COMPLETION callback.

### PHONE_STATE

This message is sent to the TAPI DLL whenever a phone device’s status changes.

#### Sent to

PHONEEVENT

#### Parameters

HTAPIPHONE    htPhone

Specifies the TAPI DLL’s opaque object handle to the phone device.

DWORD    dwMsg

The value PHONE_STATE

DWORD    dwParam1

Specifies the phone state that has changed, of type PHONESTATE. Values are:

PHONESTATE_OTHER

An other phone status information item not listed below has changed.

PHONESTATE_CONNECTED

The connection between the phone device and the API was just made. This happens when the API is first invoked, or when the wire connection the phone to the PC is plugged in with the API active.

PHONESTATE_DISCONNECTED

The connection between the phone device and the API was just broken. This happens when the wire connecting the phone set to the PC is unplugged while the API is active.

PHONESTATE_DISPLAY

The display of the phone has changed.

PHONESTATE_LAMP

A lamp of the phone has changed.

PHONESTATE_REINIT

Items have changed in the configuration of phone devices. To become aware of these changes (as with the appearance of new phone devices) the application should reinitialize its use of TAPI. If specified, the htPhone parameter is left NULL for this state change as it applies to any of the phones in the system.

PHONESTATE_RINGMODE

The ring mode of the phone has changed.

PHONESTATE_RINGVOLUME

The ring volume of the phone has changed.

PHONESTATE_HANDSETHOOKSWITCH

The handset hookswitch state has changed.

PHONESTATE_HANDSETVOLUME

The handset’s speaker volume setting has changed.

PHONESTATE_HANDSETGAIN

The handset’s mic gain setting has changed.

PHONESTATE_SPEAKERHOOKSWITCH

The speakerphone’s hookswitch state has changed.

PHONESTATE_SPEAKERVOLUME

The speakerphone’s speaker volume setting has changed.

PHONESTATE_SPEAKERGAIN

The speakerphone’s mic gain setting state has changed.

PHONESTATE_HEADSETHOOKSWITCH

The headset’s hookswitch state has changed.

PHONESTATE_HEADSETVOLUME

The headset’s speaker volume setting has changed.

PHONESTATE_HEADSETGAIN

The headset’s mic gain setting has changed.

PHONESTATE_SUSPEND

The application’s use of the phone is temporarily suspended.

PHONESTATE_RESUME

The application’s use of the phone device is resumed after having been suspended for some time.

PHONESTATE_DEVSPECIFIC

The phone’s device-specific information has changed.

DWORD    dwParam2

Specifies phone state dependent information detailing the status change. This field is not used if multiple flags are set in dwParam1, since multiple status items have changed. The TAPI DLL should invoke TSPI_phoneGetStatus to obtain complete set of information.

If dwParam1 is PHONESTATE_LAMP, dwParam2 contains the button/lamp ID of the lamp that has changed.

If dwParam1 is PHONESTATE_RING, dwParam2 contains the new ring mode.

If dwParam1 is PHONESTATE_HANDSET, SPEAKER or HEADSET, dwParam2 contains the new hookswitch mode of that hookswitch device, of type PHONEHOOKSWITCHMODE. Values are:

PHONEHOOKSWITCHMODE_ONHOOK

The device’s mic and speaker are both onhook.

PHONEHOOKSWITCHMODE_MIC

The device’s mic is active, the speaker is mute.

PHONEHOOKSWITCHMODE_SPEAKER

The device’s speaker is active, the mic is mute.

PHONEHOOKSWITCHMODE_MICSPEAKER

The device’s mic and speaker are both active.

DWORD    dwParam3

Unused.

Comments	The sending of this message is controlled by TSPI_phoneSetStatusMessages. By default, this message is disabled for all state changes.

Comparison	The service provider never reports changed values for PHONESTATE_OWNER or PHONESTATE_MONITOR, since the notion of privilege does not appear at the TSPI level.

See Also	Functions:  TSPI_phoneGetStatus, TSPI_phoneSetStatusMessages

Data Types: PHONESTATE