Simply LANtastic(R) Information Update File (README.DOC)

------------------------------------------------

(C) Copyright 1993 Artisoft, Inc.

(Edition 1)

Windows 3.0 Help Support

========================

You must be running Windows 3.1 to use Simply LANtastic's on-line Help

feature in Windows.

Changing Your Permanent Connections

===================================

Simply LANtastic now has an easier way to save your permanent network

connections.  These connections will be available to you when running in

DOS or Windows.

1. Run SETNET from your Simply LANtastic installation directory.  At

the DOS prompt type:

CD \LANTASTI

SETNET

The NET Main menu appears.

2. From the NET menu, choose "Connect to Other Computer's Drives" or

"Connect to Other Computer's Printers." to set the drive and

printer connections you want to be available each time you start

your computer.  Detailed instructions appear in Chapters 3 and 4

of your Simply LANtastic User's Manual.

3. When you're finished setting up your connections, press Esc to

exit each menu.

After you exit the Main menu, your connections are saved in a file

called CONNECT.BAT.  When you restart your computer, all the drive and

printer connections that you set up in step 2 above will be available.

NOTE:  You can still use the steps listed in Chapter 6 to change your

permanent drive and printer connections.  Using SETNET, however, is

much easier.

Preventing Simply LANtastic from Running

========================================

You can now use the INSTALL program to prevent the Simply LANtastic

software from running when you start your computer.  This is convenient

when you want to temporarily remove your computer from the network.

1. Run the Simply LANtastic INSTALL program:

-  If you use DOS only, run INSTALL from DOS.  For example:

CD \LANTASTI

INSTALL

-  If you use Windows only or both Windows and DOS, run Windows

and choose Simply LANtastic Setup.

2. In DOS or Windows, choose "DISABLE... Simply LANtastic on Next

Reboot."

3. Exit INSTALL or Simply LANtastic Setup.

The next time you start your computer, the Simply LANtastic software will

not run.  You can re-enable Simply LANtastic by completing the steps

listed above and choosing "ENABLE... Simply LANtastic on Next Reboot" in

step 2.

NOTE:  You can still use the steps listed in Chapter 6 to stop Simply

LANtastic from running at startup; however, the method described above is

usually easier.

Stopping Your Computer from Sharing

===================================

You can now stop your computer from sharing its drives and printers by

creating a file called "NOSHARE" in the directory where you installed

Simply LANtastic.

1. Open a text editor such as Edit and specify the name NOSHARE on

the command line.  For example:

CD \LANTASTI

EDIT NOSHARE

2. Save the file.  There doesn't need to be any information in the

file, it can be blank.

When you restart your computer, Simply LANtastic will see the file and

know not to share your drives and printers.  You can still connect to

the drives and printers on other computers.  If you ever want to share

your drives and printers again, simply delete the NOSHARE file and

restart your computer.

NOTE:  You can still use the steps listed in Chapter 6 to stop your

computer from sharing.  The method described above, however, is much

easier.

Connecting the B: Drive on Single Floppy Systems

================================================

If you don't have a B: drive physically installed in your computer, you

can't use B: as drive letter to connect to another computer's disk drive.

Network Disconnect Problems with Enhanced-Mode Windows

======================================================

If you have problems with occasional loss of all network connections

while using multiple DOS windows in Microsoft(R) Windows(Tm) enhanced

mode, make sure INSTALL has correctly specified your network adapter IRQ

setting in the SYSTEM.INI file.

1. Use a text editor to open your SYSTEM.INI file.  For example,

at the DOS prompt type:

EDIT C:\WINDOWS\SYSTEM.INI

2. Locate the [LANtastic] section of your SYSTEM.INI file.  It

usually appears near the end of the file.

3. Make sure the Network_IRQ= setting in the [LANtastic] section

corresponds to the IRQ setting of your adapter.  For example:

Network_IRQ=15

The factory pre-set for Artisoft adapters is IRQ 15.  If you have

multiple adapters installed, repeat the Network_IRQ line for each

adapter (each adapter must use a unique IRQ).  See your adapter

documentation for more information.

For Central Station and external (pocket) Ethernet adapters, use

the IRQ that corresponds to the parallel or serial port where the

adapter is installed.  Here are the default IRQ settings for LPT

and COM ports:

PORT      IRQ             PORT      IRQ

----      ---             ----      ---

LPT1       7              COM1       4

LPT2       5              COM2       3

3. Save the file and close the text editor.

NET PRINT Error Message Explanations

====================================

If you issue the NET PRINT command and receive the message "No more files,

print operation has failed," it means that the name of the file you

entered on the NET PRINT command line doesn't exist.  The message "File

not found" is usually returned if you don't specify which printer you want

the file to go to.  For example if you type:

NET PRINT README.DOC            (incorrect)

instead of:

NET PRINT README.DOC LPT1       (correct)

You will receive the message "File not found."

These messages will be changed in future releases of Simply LANtastic to

explain the error condition more clearly.

About the Installation Program

==============================

The installation program used to install Simply LANtastic, INSTALL,

is based on licensed software provided by Knowledge Dynamics Corp.,

P.O. Box 780068, San Antonio, Texas 78278 (USA).  INSTALL is

Copyright (C) 1987-1993 by Knowledge Dynamics Corp., which reserves

all copyright protection worldwide.  INSTALL is provided to you for

the exclusive purpose of installing Simply LANtastic.  Artisoft has

modified the software as provided by Knowledge Dynamics Corp., and

thus the performance and behavior of the INSTALL program shipped

with Simply LANtastic may not represent the performance and behavior

of INSTALL as shipped by Knowledge Dynamics Corp.  Artisoft, Inc. is

exclusively responsible for the support of Simply LANtastic, including

support during the installation phase.  In no event will Knowledge

Dynamics Corp. provide any technical support for Simply LANtastic.