DigiBoard

6400 Flying Cloud Drive

Eden Prairie, MN 55344

(612) 943-9020

Installation Guide

and

Reference Manual

for

DigiCHANNEL PC/Xe, PC/Xi, MC/Xi

Novell AIO Driver

DBI P/N 92000115B

DigiBoard, DigiCHANNEL, PC/Xe, PC/Xi and MC/Xi are trademarks of

DigiBoard.  NetWare, NetWare Access and Novell are registered

trademarks of Novell, Incorporated.  All other brand and product

names are the trademarks of their respective holders.

Copyright (c) Digi International, Inc 1992

All Rights Reserved

Information in this document is subject to change without notice

and does not represent a commitment on the part of DigiBoard.

DigiBoard provides this document "as is", without warranty of any

kind, either expressed or implied, including, but not limited to,

the particular purpose.  DigiBoard may make improvements and/or

changes in this manual or in the product(s) and/or the program(s)

described in this manual at any time.

This product could include technical inaccuracies or typographical

errors.  Changes are periodically made to the information herein;

these changes may be incorporated in new editions of the publication.

RESTRICTED RIGHTS:

For non-U. S. Government use:

These programs are supplied under a license.  They may be used,

disclosed, and/or copied only as permitted under such license

agreement.  Any copy must contain the above copyright notice and

this restricted rights notice.  Use, copying, and/or disclosure

of the programs is strictly prohibited unless otherwise provided

in the license agreement.

For U.S. Government use:

Use, duplication, or disclosure by the Government is subject to

restrictions as set forth in sub-paragraph (c)(1)(ii) of the

Rights in Technical Data and Computer Software clause of DFARS

52.227-7013.

1.0  Introduction

1.1  Overview

DigiBoard's support for Novell compatible asynchronous

applications is provided through a universal driver

conforming to the "AIO" (Asynchronous Input/Output)

specification.  Use the AIO driver when DigiBoard hardware

is being installed in the file server (or machine with run-

time NetWare 3.X).  When DigiBoard hardware is installed in

a machine other than the server (for instance NetWare Access

Server v1.2) use the specific driver for that application.

AIO drivers are loaded on the file server from the system

console (or RCONSOLE) command line or the AUTOEXEC.NCF file.

Installation is simply a matter of copying of the driver

files AIODGXI.NLM and AIODGMEM.NLM.

1.2  Compatibility

The DigiBoard AIO driver supports the DigiCHANNEL PC/Xi,

PC/Xe and MC/Xi boards and therefore can be used on ISA,

EISA and MCA servers.

The driver will support up to 7 DigiCHANNEL PC/Xe or PC/Xi

boards on ISA and EISA bus servers.  These DigiCHANNEL

boards may be configured to use the same memory address

("stacked") or may be configured for unique addresses.  The

driver supports both 64k and 128k versions, although your

server may not have an open memory space for the 128k

DigiCHANNEL board.

On MCA bus servers the driver will support up to 4 32k

DigiCHANNEL MC/Xi boards.  Your server may be limited to 2

boards due to conflicts with the network adapter card and

ESDI hard disk controller.

2.0  Installation

To install the DigiBoard AIO driver, simply copy the relevant

files to your file server (although the driver can be loaded

from the file server floppy, this will not be a typical way to

set-up a system).  The DigiBoard AIO driver files (AIODGXI.NLM

and AIODGMEM.NLM) should both be copied to the system directory

of the file server, typically called F:\SYSTEM.  Copy the files

from a workstation to the file server with the commands:

COPY A:\AIODGXI.NLM F:\SYSTEM

and

COPY A:\AIODGMEM.NLM F:\SYSTEM

2.1  PC/Xe and PC/Xi Installation

DigiCHANNEL PC/Xe and PC/Xi boards may be installed in either

ISA or EISA platforms.  In either case, the driver must be

loaded for each board you wish to support.  The syntax for

loading the driver is as follows:

load aiodgxi port=xxx mem=xxxxx [name=bname] [node=0]

[altp[=mask]]

port=xxx :

This parameter is the board's 3 digit hex I/O

port address.

For example: port=320.

Valid port addresses are 100, 110, 120, 200, 220,

300 and 320.

Each board must have a unique I/O port address, which

must not be used by any other device in the system.

mem=xxxxx :

This parameter is the 5 digit hex starting address of

the DigiCHANNEL board's dual-ported memory.

For example: mem=d0000.

Memory addresses supported by the driver are

80000, 90000, A0000, B0000, C0000, and D0000.

Typically C0000 and D0000 will be the only

addresses available in your server.

All PC/Xe and PC/Xi boards may share the same

memory addresses, since only one board's

memory is turned on at a time.

name=bname :

This parameter is optional and allows you to

specify up to a 10 character name to identify

the board.  bname may consist of any combination

of alphanumerical characters.  If you do not

specify a name, one will be assigned by the driver.

Following is a list of the board's I/O port

addresses and the associated default names and

board numbers.

Port         Default    Board

Address      Name       Number

-------      --------   ------

100h         DigiXi00     0

110h         DigiXi01     1

120h         DigiXi02     2

200h         DigiXi03     3

220h         DigiXi04     4

300h         DigiXi05     5

320h         DigiXi06     6

node=0 :

This parameter must be used if you are using

AIO.NLM version 1.0.2 or earlier.  These

versions of AIO need sequential board numbers

based on the load order of the driver.  (The

first board loaded will be assigned #0, the

second board will be assigned #1 and so on.)

Using the node=0 parameter informs the driver

to assign board numbers sequentially.  If

this parameter is excluded, the driver will

assign board numbers based on the I/O port

address as defined above.

altp :

The standard wiring for DigiCHANNEL boards

equipped with RJ-45 connectors places the

modem control lines RI (Ring Indicator) and

DCD (Data Carrier Detect) on pins 1 and 10,

respectively, which makes them inaccessible

to an 8-pin RJ-45 plug.  While RI is seldom

used, DCD is necessary for most modem applica-

tions.  To facilitate the use of 8-pin RJ-45

plugs, a software feature called "ALTPIN" has

been included with this device driver.  When

ALTPIN is active, DCD is swapped with DSR

(Data Set Ready), which is on pin 1 of an 8-

pin RJ-45 plug.

This parameter is optional and allows you to

specify that you want to activate the driver's

ALTPIN option.  When this parameter is entered

as just "altp", all ports on the board will

operate with the ALTPIN option active.   When

entered as "altp=mask" the driver uses the hex

mask to determine which ports will use the ALTPIN

feature.  See example on next page:

altp=f0

F0 (hex) = 11110000 (binary)

||||||||

|||||||+----Port 1

||||||+-----Port 2

|||||+------Port 3

||||+-------Port 4

|||+--------Port 5

||+---------Port 6

|+----------Port 7

+-----------Port 8

This will activate ALTPIN on the board's last

four ports. The mask can be up to 4 hex

digits long to activate ALTPIN on up to 16

ports.

IMPORTANT!  IRQs on PC/Xe and PC/Xi boards should be disabled.

Set all eight switches in DIP switch bank DS2 to "OFF"

(set away from the board).

2.1.1  PC/Xe and PC/Xi DIP Switch Settings

Memory Start Address:

DS1, Switch  1   2   3   4   5   6   7   8

Address     --- --- --- --- --- --- --- ---

080000h     ON  ON  ON  OFF ON  ON  ON  ON

090000h     OFF ON  ON  OFF ON  ON  ON  ON

0A0000h     ON  OFF ON  OFF ON  ON  ON  ON

0B0000h     OFF OFF ON  OFF ON  ON  ON  ON

0C0000h     ON  ON  OFF OFF ON  ON  ON  ON

0D0000h     OFF ON  OFF OFF ON  ON  ON  ON

I/O Port Address:

DS1, Switch  9  10  11

Address     --- --- ---

100h        ON  OFF OFF

110h        OFF ON  OFF

120h        ON  ON  OFF

200h        OFF OFF ON

220h        ON  OFF ON

300h        OFF ON  ON

320h        ON  ON  ON

IRQ:

DS2, Switches 1-8

Set all switches in DS2 to "ON" (interrupts disabled).

2.2  MC/Xi Installation

The driver will also support up to 4 DigiCHANNEL MC/Xi

boards on MCA (Micro Channel) bus servers.

2.2.1  Configuring the MC/Xi for 32K Operation

Due to memory restrictions, the MC/Xi boards must be

set for 32K operation.  This is accomplished by changing

a jumper, J1, on the board.  J1 must be installed between

pins 2 and 3 (the right-hand two pins).

Please note that only the newer MC/Xi boards (Part

#30001224) are supported by this software.  The part

number can be found next to the serial number of the

Processor board (the MC/Xi board is actually two boards

joined together.  The Processor board is the bottom

segment, which plugs into the Micro Channel slot):

Jumper J1 is located above the MC/Xi board's edge

connector.

2.2.2  Loading the AIO Driver

There are two ways to load the driver on these servers:

You may load the driver for one board at a time or you

may autoload the driver for all boards in the system.

The syntax for loading the driver is as follows :

load aiodgxi[port=xxxx mem=xxxxx [name=bname]] [node=0]

altp[=mask[,mask[,mask[,mask]]]]]

port=xxxx :

This parameter is optional and specifies the

MC/Xi board's 4 digit hex I/O port address.

For example : port=F1F0

Valid port addresses are F1F0, F2F0, F4F0 and

F8F0.

This parameter is used when you are loading

the driver to support just one specific board

and must be used with the parameter "mem=".

If you do not use these two parameters, the

driver will autoload support for all MC/Xi

boards found in the system.

mem=xxxxx :

This parameter is optional and specifies the

MC/Xi board's 5 digit hex memory starting

address.

For example : mem=d0000

Memory addresses supported by the driver are

C0000, C8000, D0000 and D8000.  This

parameter is used when you are loading the

driver to support just one specific board and

must be used with the parameter "port=". If

you do not use these two parameters, the

driver will autoload support for all MC/Xi

boards found in the system.

name=bname :

This parameter is optional when using the

"mem=" and "port=" parameters, and allows you

to specify up to a 10 character name to

identify the board.  bname may consist of any

combination of alphanumerical characters.  If

you do not specify a name, one will be

assigned by the driver.  Following is a list

of the board's I/O port addresses and the

associated default names and board numbers.

Port         Default    Board

Address      Name       Number

-------      --------   ------

F1F0h        DigiXi00     0

F2F0h        DigiXi01     1

F4F0h        DigiXi02     2

F8F0h        DigiXi03     3

node=0 :

This parameter must be used if you are using

AIO.NLM version 1.0.2 or earlier.  These

versions of AIO need sequential board numbers

based on the load order of the driver.  (The

first board loaded will be assigned #0, the

second board will be assigned #1 and so on.)

Using the node=0 parameter informs the driver

to assign board numbers sequentially.  If

this parameter is excluded, the driver will

assign board numbers based on I/O port

address as defined above.

altp :

The standard wiring for DigiCHANNEL boards

equipped with RJ-45 connectors places the

modem control lines RI (Ring Indicator) and

DCD (Data Carrier Detect) on pins 1 and 10,

respectively, which makes them inaccessible

to an 8-pin RJ-45 plug.  To facilitate the

use of 8-pin RJ-45 plugs, a software feature

called "ALTPIN" has been included with this

device driver.  When ALTPIN is active, DCD is

swapped with DSR (Data Set Ready), which is

on pin 1 of an 8-pin RJ-45 plug.

This parameter is optional and allows you to

specify that you want to activate the driver's

ALTPIN option.  When this parameter is entered

as just "altp", all ports on the board will

operate with the altpin option active.  When

entered as "altp=mask" the driver uses the hex

mask to determine which ports will use the

ALTPIN feature.

For example : altp=f0

F0 (hex) = 11110000 (binary)

||||||||

|||||||+----Port 1

||||||+-----Port 2

|||||+------Port 3

||||+-------Port 4

|||+--------Port 5

||+---------Port 6

|+----------Port 7

+-----------Port 8

This will activate ALTPIN on the board's last

four ports.

When autoloading support for all DigiCHANNEL

boards in the server, you may specify up to

four masks, one for each possible board.

For example:

altp=1,8,10,80

1 (hex) = 00000001 (binary)

8 (hex) = 00001000 (binary)

10 (hex) = 00010000 (binary)

80 (hex) = 10000000 (binary)

||||||||

|||||||+----Port 1

||||||+-----Port 2

|||||+------Port 3

||||+-------Port 4

|||+--------Port 5

||+---------Port 6

|+----------Port 7

+-----------Port 8

In this example the first port on the first

board, the fourth port on the second board,

the fifth port on the third board and the

eighth port on the fourth board will use

ALTPIN.  The mask can be up to 4 hex digits

long to activate ALTPIN on up to 16 ports.

When autoloading, the driver uses the MCA

bus's POS (Programmable Option Select)

capabilities to determine the MC/Xi boards

that are installed in your server, starting

with slot 1.  The first ALTPIN mask will

activate ALTPIN on the board in the lowest

numbered slot, the second mask for the next

lowest slot, and so on.

3.0  Error Messages

The following error messages may occur while the driver is

parsing the parameters.

*Usage Error: AIODGXI port=XXXX mem=XXXXX [name=bname].

This error will appear if you load the driver with no

parameters on ISA and EISA bus servers.

*Usage Error: First parameter not 'port='.

The driver expects the first parameter to be port=xxxx,

but this was not the case.

*Error: Invalid Port Address 0x310.

This message indicates that an invalid port address was

specified.

*Error: Port Address 0x320 is already supported.

This  message indicates that the driver is already

supporting a DigiCHANNEL board at this port address.

*Usage Error: Second parameter not 'mem='

The driver expects the second parameter to be mem=xxxxx,

but this was not the case.

*Error: Invalid Memory Address 0xF0000.

This message indicates that an invalid memory address was

specified.

*Error: Memory Address 0xC0000 is already supported.

This message indicates that the driver is already supporting

a DigiCHANNEL board at the specified memory address.  This

message will only be seen on MCA bus servers since MC/Xi

boards must use different memory addresses.

The following error messages may occur while the driver is

performing some initial tests on the DigiCHANNEL board.

Board 4: Port 220 Mem D0000 Port 220 not found.

This message indicates that the driver could not find a

board with port 220h.  Check to insure that you have

specified the correct I/O port or that you have properly

configured the board's port address.

Board 4: Port 220 Mem D0000 Hardware Reset Fails.

This message indicates that the driver could not place

the board into a reset state.  This is most likely the

result of an I/O port address conflict or a faulty board.

Be sure that there are no other I/O ports in your system

at the indicated I/O port address.

Board 4: Port 220 Mem D0000 PC/Xi Memory size too Large.

This message indicates that your PC/Xi board has more

than 128k and is not supported by the driver.

Board 4: Port 220 Mem D0000 Memory not found error x.

This message indicates that the driver could not find  the

board's memory.  x indicates the number of memory errors

found in a 256 byte block.

Board 4: Port 220 Mem D0000 Memory error x.

This message indicates that the driver has found the

board's memory, but it appears to be faulty.  x indicates

the number of errors found in a 256 byte block.

Board 4: Port 220 Mem D0000 Memory fails disable.

This message indicates that the board's memory could not

be disabled.  This message will not occur on MC/Xi boards.

The following messages may occur during the drivers attempt to

initialize the board.

Board 4: Port 220 Mem D0000 BIOS fails initialization.

This message indicates that the board's on-board BIOS did

not initialize properly.

Board 4: Port 220 Mem D0000 BIOS passes init but reports 0 ports.

This message indicates that the BIOS initialized successfully,

but could not find any of the asynchronous channels.

Board 4: Port 220 Mem D0000 NMI Fails during FEPOS code move.

This message indicates a failure during transfer of the

on-board operating system code.

Board 4: Port 220 Mem D0000 NMI Fails during FEPOS execution.

This message indicates a failure during startup of the

on-board operating system.

Board 4: Port 220 Mem D0000 FEPOS Fails Initialization.

This message indicates that the on-board operating system

did not initialize successfully.

Board 4: Port 220 Mem D0000 Command failure during Initialization.

This message indicates that the on-board operating system

encountered a failure during initialization of the asynchronous

channels.