__________________________________________________________________________

LabWindows MEMORY.DOC File

National Instruments LabWindows Software Version 2.3a

(c) Copyright 1993 National Instruments Corporation.

All rights reserved.

Product names are trademarks of their respective manufacturers.

Company names are trademarks or trade names of their respective companies.

__________________________________________________________________________

Overview

--------

The following topics are discussed in this file:

* PC Memory and LabWindows

* Specific Memory Managers

* LabWindows and Memory Intensive Devices

* Microsoft Windows Interaction

* Configuration Examples

PC Memory and LabWindows

------------------------

Operating LabWindows on the PC requires that system memory be properly

configured.  Each system has different requirements which depend on system

resources available and applications being run, making the best setup a

balance reached on a system by system basis.  This document details the

memory prerequisites for operating LabWindows.  Topics discussed are

Conventional, Extended, Expanded, and Virtual Memory, Specific Memory

Managers, Memory Intensive Devices, and Microsoft Windows Interaction with

LabWindows.  Finally, Configuration Examples are provided for various system

configurations.  For more information about memory management in general,

contact National Instruments and request the LabWindows Memory Management

Application Note.

The following table summarizes LabWindows memory requirements and potential

uses.  Each memory type is then discussed in detail.

Memory Type           LabWindows Usage        Amount Required or

Recommended

-----------------------------------------------------------------------------

Conventional         Required for program     640 kilobytes installed

(0-640k)             operation

High DOS             Not used directly by     Fixed amount in computer

(640k-1M)            LabWindows

Extended Memory      Required for program     1.4 megabytes required

(VCPI compatible)    operation                4 megabytes recommended

Expanded-Memory      Virtual memory if RAM    Determined by available

Board and driver     is exhausted             RAM and application size

Emulated Expanded    Used in place of         1.4 megabytes required

Memory               extended memory          4 megabytes recommended

(VCPI Compatible)

Virtual Memory       Used when RAM is         Determined by available

(Hard Disk)          exhausted                RAM and application size

Conventional and High DOS Memory

--------------------------------

LabWindows requires 640 kilobytes of installed conventional memory to run

LW.EXE.  On rare occasions, if too much conventional memory is used to

load the DOS system, drivers, and TSR programs, you may encounter some

difficulty loading LabWindows.  LabWindows does not directly access high

DOS Memory.  If LabWindows is using an expanded-memory board for

virtual memory, the expanded-memory manager will make use of this

area. See the Expanded Memory Board section below for a more detailed

explanation.

Standalone executables using LabWindows libraries require only 640

kilobytes of conventional memory to run.  The conventional memory minus

the memory required for device drivers and the DOS system determines the

executable size limit.  Because of the need to store variables and

graphics information, standalone programs often require more memory than

is indicated by the executable size.  Additional conventional memory can

be freed by loading device drivers and a portion of the DOS system into

high DOS memory.  This requires the installation of a memory manager, a

subject that is addressed below.

Extended Memory

---------------

LabWindows requires a minimum of 1.4 megabytes of extended memory to

operate, but it is recommended that four megabytes be available.  Using

more extended memory reduces virtual memory requirements and improves

LabWindows' performance.  All extended memory must conform to the VCPI

memory standard to be compatible with DOS/16M, the Rational Systems,

Inc. DOS extender used by LabWindows.

Standalone executables created with LabWindows cannot use extended

memory.

Expanded-Memory Boards

----------------------

After exhausting extended memory, LabWindows can use an expanded-memory

board conforming to the LIM 4.0 specification.  This memory is used

as swap space and cannot replace the extended memory LabWindows requires

to load initially.  The Virtual Memory section below provides more

information on the use of expanded-memory boards.

Standalone executables created with LabWindows cannot use expanded-memory

boards.

Expanded Memory Emulation

-------------------------

Expanded-Memory managers (EMM) provide expanded memory support by

swapping blocks from extended memory instead of from an expanded memory

board.  LabWindows can coexist with expanded memory managers if they

conform to the VCPI specification.  LabWindows uses the expanded memory

instead of the extended memory normally required for operation.

LabWindows cannot access the residual extended memory.  Therefore, the

EMM should always be installed with at least 1.4 megabytes configured

as expanded memory.

Standalone executables created with LabWindows cannot use expanded

memory.

Virtual Memory

--------------

LabWindows contains a Virtual Memory Manager (VMM) that enables you

to use your hard disk as a memory device. VMM swaps segments of your

program to and from your hard disk as needed. If you have an expanded

memory board in your system, VMM can use it as a swap device instead

of, or in addition to, your hard disk. Using EMS memory for virtual

memory is faster than using a hard disk.

The LW.VMC file contains the virtual memory configuration parameters

for LabWindows. A typical file might contain:

swapname=c:\lw\swap\lw_swap.tmp

swapsize=2048

ems=0

In this example, virtual memory consists of a two megabyte file called

LW_SWAP.TMP and no expanded memory. Refer to the LabWindows User Manual

for additional information on virtual memory in LabWindows.

Note: Virtual memory does not take the place of the required extended

memory for startup.

Corrupt Memory

--------------

Since most DOS applications do not use extended memory, bad memory chips

often go unnoticed until a program using a DOS extender exhibits

unexplainable problems.  If LabWindows crashes with DOS/16M errors or RAM

parity errors, you should run a memory checking program.

If a section of bad memory exists, LabWindows can operate in a restricted

range of memory using the DOS16M environment variable.  For example, to

restrict LabWindows to operation within the memory range between one

megabyte and three megabytes, enter the following at the DOS prompt or

in your AUTOEXEC.BAT file:

SET DOS16M=@1-3M

Determining Available Memory

----------------------------

LabWindows includes a standalone utility called PMINFO.EXE to display

memory information.  This utility presents available memory (conventional

and extended) and switch rate information for the processor in protected

mode.  Refer to the LabWindows User Manual for information about the

PMINFO utility.

Specific Memory Managers

------------------------

DOS extender software accesses memory beyond 1 megabyte on 80286 and 80386

computers.  Memory managers, however, do not follow a common standard and

access memory in different ways. For instance, memory managers accommodate

memory requirements in either the Dos Protected Mode Interface (DPMI) or the

Virtual Control Program Interface (VCPI) standard, and may provide available

memory as either extended memory (XMS) or expanded memory (EMS) emulated

using extended memory.  Because of this, LabWindows compatibility differs

from one memory manager to another.  This section discusses the LabWindows

compatibility of several memory managers.

HIMEM  and EMM386

-----------------

DOS 5.0 includes both HIMEM.SYS and EMM386.EXE software.  These managers

are widely used because of their Microsoft Windows compatibility and the

significant number of DOS 5.0 operating systems in use.  HIMEM.SYS is

independent of EMM386.EXE, but EMM386.EXE requires HIMEM.SYS to operate.

HIMEM.SYS manages memory allocation in extended memory, including the

first 64 kilobytes known as the High Memory Area (HMA) on 80286 and 80386

machines.  It prevents multiple applications from attempting to access

the same memory addresses.  The ability to load part of the DOS system

into the HMA and save conventional memory is another major benefit of

HIMEM.SYS.  Examples of applications that access extended memory include

Microsoft Windows, RAM disks, cache software, and EMM386.EXE.

EMM386.EXE can provide expanded-memory support while using extended

memory.  This driver can also load device drivers and applications into

high DOS memory, freeing conventional memory.  Access to high DOS

memory requires the use of the RAM or NOEMS flag.  EMM386.EXE operates

only on machines with an 80386 or higher microprocessor that have

HIMEM.SYS installed.

LabWindows is compatible with HIMEM.SYS and the DOS=HIGH command.  No

special configuration is required for the programs to coexist.

Although LabWindows does not require EMM386.EXE for operation, the two

can coexist with some modifications to the EMM386 configuration.

LabWindows accesses only the expanded memory the driver sets aside, not

the residual extended memory.  In contrast, Microsoft Windows standard

and enhanced modes do not use expanded memory, causing a possible

conflict for memory resources.  If you require access to high DOS

memory, LabWindows only works with the RAM flag that provides access

to the upper memory area and expanded memory.  LabWindows does not

operate with the NOEMS flag that prevents access to expanded memory but

does provide access to high DOS memory.

In its default configuration, the installation of EMM386 in the

CONFIG.SYS file appears as:

DEVICE=EMM386.EXE

This configuration defaults to an expanded memory area of 256 kilobytes,

less than the 1.4 megabytes required by LabWindows.  Three methods

provide EMM386 compatibility:

* Use an expanded memory flag in the device, such as:

DEVICE=EMM386.EXE 4096

The 4096 flag reserves four megabytes of expanded memory.  This

reserved memory is not accessible to programs requiring extended

memory but will support LabWindows operation.

* Use the ON and OFF switching option from the DOS prompt.

EMM386.EXE provides an option for disabling and enabling the

memory manager. Entering

EMM386 OFF

at the DOS prompt disables the manager and returns the memory to a

LabWindows compatible state.  Upon completing the LabWindows tasks,

entering the command

EMM386 ON

reinitializes EMM386.EXE to its original parameters.  The major

drawback of this option is that about 400 kilobytes of the manager

always remains resident in memory, and is inaccessible to LabWindows.

* Develop multiple CONFIG.SYS files for different system

configurations.  Create one file for LabWindows operation and a

separate file for other programs requiring a different setup.

See the Configuration Examples Section for additional details on specific

EMM386.EXE configurations.

QEMM386

-------

Quarterdeck Expanded-Memory Manager-386 (QEMM386) provides versatile

memory management for 80386 and greater machines.  It works in place of

both HIMEM.SYS and EMM386.EXE and is compatible with Windows 3.x and

LabWindows.  The QEMM386 system accommodates the DOS=HIGH command and can

load drivers and applications into high DOS memory.

LabWindows is compatible with QEMM386 in most configurations, except when

the "noems" switch is used.  LabWindows can operate with or without

the "ram" switch that provides access to high Dos memory.  Finally, the

QEMM386 "frame=none" option frees an additional 64k of high Dos memory

by removing the expanded-memory page frame.  This option is compatible

with LabWindows and Windows 3.x because they do not require the page

frame.  Omit the "frame=none" flag if applications require true

expanded memory support.

See the Configuration Examples section for additional information on

configuring QEMM386 for use with LabWindows.

386MAX

------

The 386MAX program from Qualitas provides memory management and utilities

for 80386 and higher machines.  This manager accommodates the DOS=HIGH

function and can load device drivers and applications into high memory.

The program automatically converts all but 64 kilobytes of the extended

memory to expanded specification.  LabWindows can use all the converted

memory.  If the expanded-memory conversion is overridden, sufficient

memory should always remain in expanded form for LabWindows operation.

386MAX Version 6.0 is compatible with LabWindows 2.2.x and 2.3, and

386MAX Version 5.1 is compatible with LabWindows 2.x.

See the Configuration Examples section for additional information on

configuring 386MAX for use with LabWindows.

32-Bit Extended-Memory Managers

-------------------------------

Memory managers are available that support more than 16 megabytes of

memory.  One example is the PharLap 386 DOS|Extender.  These programs

require 32-bit support from compatible applications.  LabWindows

libraries are not compiled under this standard and are therefore not

compatible.

LabWindows and Memory Intensive Devices

---------------------------------------

Many utilities enhance the speed and storage capabilities of PC-based

applications.  Common examples include RAM disks and cache programs.

Utilities such as these were among the first programs to use the extended

memory capabilities of the 80286 and 80386 microprocessors.  This use of

memory beyond the first one megabyte can produce conflicts with LabWindows

memory management.

One area of possible memory incompatibility arises from the standard employed

for protected mode operation.  Utilities using extended-memory or expanded

memory emulation require a memory manager for protected mode access to any

memory beyond 1024 kilobytes.  Recall that two standards exist for protected

mode operation: VCPI and DPMI.  Because LabWindows adheres to the VCPI

standard, any memory manager required for your utility programs must also be

VCPI compatible to prevent conflicts.

Another area of incompatibility lies in the allocation of memory resources to

applications and utilities.  This factor is of greater importance with PC

machines that have limited RAM available.  LabWindows requires a minimum of

1.4 megabytes of free VCPI-standard extended memory for proper operation.  At

this limited memory level, LabWindows may run significantly slower for large

applications because it has to use virtual memory.  Efficient operation can

require up to four megabytes of memory.

Many utilities require or recommend setting aside large portions of extended

memory.  Decisions must be made on which programs to operate and how much

memory to allocate to each.  In extreme cases, it is possible to alternate

system configurations by switching memory manager parameters or changing the

CONFIG.SYS file.  The final decisions on memory allocation depend on

available memory, program requirements, and the priority of each application.

Two types of memory intensive utilities, RAM disks and cache programs, are

discussed in more detail below.

RAM Disks

---------

A RAM disk employs part of the system memory to emulate a very fast hard

disk.  LabWindows can coexist with compatible RAM disks.

You must configure some RAM disks such as VDISK.SYS in VCPI memory.

Other types, such as RAMDRIVE.SYS, are compatible with LabWindows when

configured in either DPMI or VCPI memory.  In either case, the memory set

aside for the RAM disk is not accessible to LabWindows as startup memory.

Sufficient memory must remain unallocated after RAM disk installation for

operating LabWindows.

The installation of a RAM disk provides two potential performance

benefits for LabWindows.  First, a RAM disk supplies a high-speed storage

device.  Because a RAM disk is significantly faster than standard hard

disks, high speed data storage from LabWindows programs is possible.  The

RAM disk also accommodates LabWindows temporary files through the TMP

environment variable, increasing program execution speed.  Refer to the

RAM disk user manual for specific installation instructions.

A potential memory conflict exists for users of VDISK.SYS.  VDISK

uses memory starting at one megabyte, while LabWindows accesses

memory from the upper limit.  To prevent possible conflicts, restrict

LabWindows from accessing the VDISK memory addresses by setting the

environment variable DOS16M. For example, if you have 4 megabytes of

total RAM and VDISK uses one megabyte, entering the following line at the

DOS prompt prevents memory errors:

SET DOS16M=:2M

This statement restricts LabWindows to the upper 2 megabytes of extended

memory.

Cache Software

--------------

Cache software reduces the amount of time programs spend writing to and

reading from the hard disk and other peripherals.  The software uses

extended or expanded memory as a buffer for information.  This memory is

not accessible to LabWindows.  Sufficient memory must remain unallocated

after installation of the caching program in order to operate LabWindows.

SMARTDRIVE.SYS is one example of cache software.  It can place a buffer

in extended or expanded memory.  The installation line in the CONFIG.SYS

file might be:

device=SMARTDRIVE.SYS 2048 256

In the example above, the SMARTDRIVE loads with a buffer size of 2048

kilobytes.  The 256 flag restricts the SMARTDRIVE buffer to a size no

smaller than 256 kilobytes.  In contrast to Windows 3.x, LabWindows

cannot reduce the buffer size.  The initial buffer size determines the

amount of memory inaccessible to LabWindows.

Microsoft Windows Interaction

-----------------------------

Compatibility questions concerning the interaction of Windows and LabWindows

involve issues such as memory management, Windows operating modes, and proper

Windows setup.  The following section addresses these compatibility issues

and presents setup suggestions.

Microsoft Windows Operation

---------------------------

Microsoft Windows provides a graphical environment for running DOS-based

and Windows applications on PC-DOS systems.  One of the major advantages

of the Windows environment is the ability to multitask DOS and Windows

applications on capable systems.  This support for multiple applications

arises from the way Windows uses the HIMEM.SYS memory manager in

conjunction with a virtual memory manager.

Windows 3.1 operates in one of two possible modes: Standard, and

Enhanced.  The Windows setup routine selects the operating mode during

the installation process.  The following switches override the default

selection if included after the win statement:

/S      standard mode

/3      enhanced mode

Note: Windows 3.1 no longer supports the Real mode of Windows 3.0.

The selection of operating modes determines the performance of Windows,

the level of multitasking, and the type of memory access.

Windows standard mode accesses conventional and extended memory to

multitask of Windows applications.  Windows employs a task switching

method in which only one full screen DOS application can be operated at a

time.  DOS applications in standard mode can access VCPI or DPMI memory

allocated in the program information file (PIF), addressed in more detail

below.  Because of memory allocation, Windows standard mode requires an

XMS driver such as HIMEM.SYS for operation.  Also, many applications run

faster in standard mode than in enhanced mode, making standard mode

preferable if the enhanced features are not required.

Windows enhanced mode is the more versatile and powerful of the two

operating modes.  Enhanced mode is the only Windows mode capable of

multitasking DOS and Windows applications.  In the case of DOS

applications, the system creates multiple virtual DOS environments

to run the different programs.  Enhanced mode uses extended memory

and virtual memory to support the multitasking operations. Extended

memory handling for the enhanced mode features requires an XMS driver

be present on the system.

A Program Interface File (PIF) is used to control DOS applications

running under Windows. This file determines how the Windows program

allocates system resources to the application.  If certain applications

have particular resource needs, the PIF indicates the requirements

allowing proper operation. You create a PIF using the PIF editor in

the Main Group in Windows.  This editor provides a number of options

for customizing application resources.  These options differ between

those required for standard mode and those required for enhanced mode.

LabWindows and Windows

----------------------

LabWindows is compatible with Windows standard mode, but not with the 386

enhanced mode.  This is because LabWindows requires a VCPI memory

manager, while Windows requires a DPMI memory manager when running in

386 enhanced mode. Windows standard mode does not require DPMI

compatibility and is therefore compatible with LabWindows.  LabWindows

operates in Windows standard mode as a full screen DOS application only.

Note: If you are running Windows 3.0, LabWindows will also operate in

Real mode.

LabWindows includes a PIF file named LW.PIF, which facilitates the use

of LabWindows under Windows. The elements of the LabWindows PIF file

are discussed below:

Program Filename       - provides the path name of the .EXE file for

the application.  This must point to the

LabWindows executable file.

Window Title           - supplies the descriptive name that appears

under the Windows icon, in this case,

LabWindows.

Optional Parameters    - specifies the startup flags for the

application. For example, to start

LabWindows in the C language mode, enter

"-c".

Startup Directory      - specifies the current directory when the

application is started.

Video Mode             - determines the video display mode and

reserves the required video memory.  For

LabWindows, setting this option to text mode

is sufficient. The Graphics/Multiple Text

selection would only be necessary for

saving the graphics screen during switches

back and forth between Windows. This switch

is not allowed when running LabWindows

(see Prevent Program Switch option in this

section).

Memory Requirements    - KB required defines a minimum amount of

available conventional memory required for

operation of the application.  LabWindows

requires at least 256 kilobytes of

conventional memory.

XMS Memory             - KB required sets the minimum amount of

extended memory required to start the

application. At least 2048 kilobytes of

extended memory for LabWindows operation.

Excessive use of virtual memory or memory

errors may occur if the minimum value is

inadequate.

KB limit defines the maximum amount of

extended memory the Windows environment

allows the application to access.  For

LabWindows, set this option to -1, which

provides the program with all the extended

memory required.

Directly Modifies      - specifies whether the application controls

the COM ports or the keyboard.  LabWindows

requires control of the COM ports.

No Screen Exchange     - prohibits copying the screen display to

the Windows clipboard for use in other

applications.

Prevent Program Switch - prohibits switching from the application

to Windows or other applications.  LabWindows

does not allow this program switching for

two major reasons:

* LabWindows reserves certain key

combinations used for program switching

by Windows.

* Program switching interrupts GPIB and

RS-232 data transfers.

Close Window on Exit   - returns control to Windows after you quit

LabWindows.

No Save Screen         - retains screen information for switching

between applications.  Application switching

is not allowed with LabWindows.

Reserve Shortcut Keys  - reserves the selected key combinations

for use in the application, not the Windows

environment. LabWindows requires the selected

key combinations for internal use.

Launching LabWindows with the PIF

---------------------------------

You can launch LabWindows from the PIF in two different ways.  Selecting

the LW.PIF file in the Windows File Manager launches the program.  Also,

adding a program element to one of the Windows groups allows LabWindows

startup from an application icon.

Neither of these methods will work with Windows in enhanced mode.  Use

the /S switch by entering:

WIN /S

at the DOS prompt to change to standard mode prior to LabWindows

operation.

Configuration Examples

----------------------

A Gateway 2000 486/25 Mhz computer produced the numbers and files presented

in this appendix.  This machine had 8 megabytes of installed RAM and was

running DOS Version 5.0.  The values indicated may differ from machine to

machine and are provided for comparison purposes only.  The PMINFO utility

supplied with LabWindows Version reported the memory values presented below.

Remember that you must reboot for the changes in CONFIG.SYS to take effect.

HIMEM.SYS and EMM386.EXE

------------------------

HIMEM.SYS and EMM386.EXE are memory management utilities supplied with

recent versions of DOS or the Windows operating system.  HIMEM.SYS

manages the extended memory while EMM386.EXE provides expanded-memory

support.

A basic version of a CONFIG.SYS file using HIMEM.SYS and EMM386.EXE

might include:

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\DOS\EMM386.EXE

FILES=20

BUFFERS=30,8

DEVICE=C:\AT-GPIB\GPIB.COM

In this case, PMINFO yields the following information on available memory

for LabWindows:

Conventional:   487             DOS16/M Extended:       240

In this configuration, EMM386.EXE provides a default expanded-memory pool

of 256 kilobytes.  When EMM386.EXE is activated, LabWindows uses the

expanded-memory pool.  This example is not compatible with LabWindows

because of insufficient memory.

Turning the EMM386.EXE driver off at the DOS prompt by entering the

command,

EMM386 OFF

allows LabWindows to access the extended memory except for the resident

portion of the EMM386.EXE driver.  When used with the above

configuration, the EMM386 OFF option results in the following memory

configuration as reported by PMINFO:

Conventional:   488             DOS 16/M Extended:      6640

Removing the EMM386.EXE driver from the CONFIG.SYS file altogether allows

LabWindows to access all the extended memory.  If you remove the

EMM386.EXE driver call from the CONFIG.SYS file above, the following

memory information is reported by PMINFO:

Conventional:   494             DOS 16/M Extended:      7040

Notice that no expanded-memory support is provided if the EMM386.EXE

driver is removed.  This provides LabWindows with more memory, but may

cause problems with other applications requiring expanded-memory support.

If the EMM386.EXE driver is not installed, LabWindows accesses the

extended memory.

A more detailed LabWindows-compatible CONFIG.SYS file might include the

following:

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\DOS\EMM386.EXE 4096 ram

DOS=HIGH,UMB

FILES=20

BUFFERS=30,8

DEVICE=C:\AT-GPIB\GPIB.COM

DEVICEHIGH=C:\DOS\RAMDRIVE.SYS 1024 /a

This example sets aside four megabytes of expanded memory and reserves

one megabyte of the expanded memory for the creation of a RAM drive.

Part of the DOS system and all of RAMDRIVE.SYS loads into high memory.

The detailed CONFIG.SYS file above creates the following memory

configuration as reported by PMINFO:

Conventional:   553             DOS 16/M Extended:      3056

The available DOS 16/M memory is the amount of expanded memory minus the

expanded-memory RAM drive (1 megabyte).  Remember that LabWindows uses

the EMM386.EXE compatible expanded memory instead of the extended memory.

Refer to the MS-DOS User's Guide or the Microsoft Windows guide for more

information on EMM386.EXE and HIMEM.SYS.

The HIMEM.SYS and EMM386.EXE combination can be configured in a number of

ways not addressed in this section.  Use the PMINFO utility to monitor

the effect of memory management changes on LabWindows operation.

LabWindows requires at least 1.4 megabytes of DOS 16/M extended memory to

operate, and four megabytes are recommended.

QEMM386

-------

QEMM386 is a DPMI and VCPI compatible memory manager for DOS systems

produced by Quarterdeck Office Systems.  It takes the place of EMM386.EXE

and HIMEM.SYS and is compatible with LabWindows and Windows 3.x.

A basic version of a QEMM386 equipped CONFIG.SYS file might include:

DEVICE=QEMM386.SYS

FILES =20

BUFFERS=30,8

DEVICE=C:\AT-GPIB\GPIB.COM

The PMINFO utility reports the following information on available memory

in this case:

Conventional:   495             DOS 16/M Extended:      7392

The following line can be substituted for the DEVICE=QEMM386.SYS line:

DEVICE=QEMM386.SYS ram

The "ram" flag provides access to the upper memory areas and allows

devices to be loaded high.  It does use some available memory as the

following PMINFO result shows:

Conventional:   495             DOS 16/M Extended:      7248

One additional flag is useful if you plan on operating LabWindows under

Windows with the QEMM386 memory manager.  The "EMBMEM=xxxx" flag included

after the EMM386.SYS statement prevents Windows from taking more than

xxxx kilobytes of memory.  The remaining memory is available for

LabWindows.

A more detailed version of CONFIG.SYS incorporating QEMM386 might contain

the following:

DEVICE=QEMM386.SYS frame=none ram

DOS=HIGH

FILES=20

BUFFERS=30,8

DEVICE=C:\AT-GPIB\GPIB.COM

DEVICE=C:\QEMM\LOADHI.SYS /R:3 C:\DOS\RAMDRIVE.SYS 1024 /a

This file loads part of DOS and RAMDRIVE.SYS into the high memory area.

The QEMM "ram" flag provides access to the upper memory blocks while the

"frame=none" flag removes the expanded page frame.  Removing the page

frame provides more accessible memory but prevents support of applications

requiring true expanded memory.  The LOADHI.SYS statements are created

automatically using the Optimize utility that accompanies the QEMM386

system.  The detailed CONFIG.SYS creates the following memory

configuration as reported by PMINFO:

Conventional:   548             DOS 16/M Extended:      7136

The QEMM386 memory manager has numerous flags and options.  If you

require options not listed in this section, use the PMINFO utility to

monitor the effect of changes made to the memory manager.  Remember

PMINFO must show at least 1.4 megabytes of DOS 16/M extended memory for

LabWindows operation, and four megabytes is recommended.

386MAX

------

386MAX is a VCPI and DPMI compatible memory management program for DOS

systems produced by Qualitas.  It is compatible with Windows 3.1 and

LabWindows 2.2.x and 2.3.

A basic version of a CONFIG.SYS file incorporating 386MAX might include

the following:

DEVICE=C:\386MAX\386MAX.SYS pro=C:\386MAX\386MAX.PRO

FILES=20

BUFFERS=30,8

DEVICE = C:\AT-GPIB\GPIB.COM

The 386MAX.PRO file addressed in the first line of the file contains the

following information:

USE=B000-B200   ; INSTALL ==> Recover RAM in MDA region

USE=F400-F800   ; INSTALL ==> Recover fill regions in ROM

; This profile created automatically by INSTALL

Installing 386MAX automatically arranges both of these files.  The

configuration above creates the following memory results as reported by

PMINFO:

Conventional:   497             DOS 16/M Extended:      6752

A more complete CONFIG.SYS file might contain:

DEVICE=C:\386MAX\386MAX.SYS pro=C:\386MAX\386MAX.PRO

DOS=HIGH

FILES=20

BUFFERS=30,8

DEVICE = C:\AT-GPIB\GPIB.COM

DEVICE=C:\386MAX\386LOAD.SYS size=5888 prgreg=3 flexframe

prog=C:\DOS\RAMDRIVE.SYS 1024 /a

This file loads part of DOS into high memory and loads RAMDRIVE.SYS into

the upper memory region.  The MAXIMIZE utility creates the RAMDRIVE

device command.  PMINFO reports the following information for this

configuration:

Conventional:   549             DOS 16/M Extended:      5728

The 386MAX memory manager has numerous flags and options.  If you require

options not listed in this section, use the PMINFO utility to monitor the

effect of changes made to the memory manager.  Remember PMINFO must show

at least 1.4 megabytes of DOS 16/M extended memory for LabWindows

operation, and four megabytes is recommended.

Note: If you use the EMS flag, you must specify at least 1500 kilobytes

for expanded memory.