ATPUTIL 3.x Tutor

=================

(c) 1994 by Stamatis Vellis

with kind permission to be included in ATPUTIL package

a) Please note that anything enclosed within [ ] means a keypress of whatever

is enclosed.  ATPUTIL recognizes only lower case letters for commands.

b) Bear in mind that an enviroment like DESQView, or even WINDOWS, is ideal

for such work, cause you can switch to/from ATP and ATPUTIL on the fly.

c) I suggest you backup your entire ATP subdir.

d) I'll be numbering my paragraphs, so that you may ask questions referring

to that number.

1.

Invoke ATPUTIL and choose [n] and [p]. This will bring you to the Primaries

Menu. Hit [ENTER] and [F3]. Move the cursor to the Primary airport you wish

to have replaced by BUF. Place the cursor there, and hit [ENTER] twice.

This will bring up the Primary Airport editing screen.

PRIMARY AIRPORTS SCREEN

The screen looks like this (I have left the entries empty on purpose so as

not to confuse you:

+---------------------------------------------------------+

|                    Primary Airports                     |

+---------------------------------------------------------+

|Short Name:     Airport/Center Name:                     |

|                               City:                     |

|Terminal Pos. North:                   East:             |

|Dir:                                                     |

|Takeoff Position North:                East:             |

|Dir:                           Departure Altitude:       |

|                                                         |

|Frequency ATIS:         Clearance:        Ground:        |

|         Tower:               Approach/Departure:        |

|                                                         |

|ATC controlled runways:                                  |

|Uncontrolled runways:                                    |

+---------------------------------------------------------+

2.

"Short Name:"          Enter [BUF] and hit [TAB] to advance.

"Airport/Center Name:" Enter the name of the Airfield [Greater Buffalo Intl.]

and hit [TAB].

"City:"                Enter [Buffalo, NY] hit [TAB].

3.

Now go into ATP, USA scenery EAST, and using SMS ([F7]-[9]) place yourself at

Buffalo Intl. Slew to the *exact* location in front of the terminal building,

where you wish to be brought each time you will be starting an Assignment

from Buffalo. Hit [F7] [3] and note down the North/East coordinates with an

accuracy of 4 decimal points, and the displayed Heading to an accuracy of 2

decimal points. Now go back to ATPUTIL.

NOTE: If you are not using DESQView or WINDOWS, note down everything in

advance, and then go to ATPUTIL to edit everything at one go. In my

explanation, I assume either DESQView or WINDOWS is being used.

4.

"Terminal Pos. North:" Enter the North coords you just noted down above,

with 4 decimals. Hit [TAB].

"East:"                Enter the East coords with 4 decimals. Hit [TAB].

"Dir:"                 Enter the heading value you noted above, with 2

decimals. Hit [TAB].

In the BUF case I would enter the following:

Terminal Pos. North: 17844.6327

East:                19447.0816

Dir:                 187.85

5.

Now we come to the Takeoff position entry. Personally, I never use this

because I never use Auto-Taxi. I taxi manually. Thus, I enter 0s. But if you

want to use it, go back to ATP, enter the SMS menu, ([F7]-[9]) and choose

Buffalo Airport and subsequently the specific runway of your choice for

Auto-taxi. When SMS places you at the threshold, hit [F7]-[3] and again note

down the exact N/E coords to 4 decimals, and the heading to 2 decimals. Now,

go back to ATPUTIL.

6.

"Takeoff Position North:" Enter the North coords you just noted down above,

with 4 decimals. Hit [TAB].

"East:"                   Enter the East coords from para 5 above. Hit [TAB].

"Dir:"                    Enter the Heading as noted in para 5 above, to 2

decimals. Hit [TAB].

In our case, BUF, if I wanted to fill in such data I would have entered:

Takeoff Position North: 17842.3280

East:                   19443.7767

Dir:                    53.49

7.

"Departure Altitude:"

Enter the altitude (in feet) that you wish to be cleared to after takeoff.

This is not your Cruise Altitude, just your initial level-off altitude.

I suggest using the altitude written in the SID charts for this airfield.

If none available, you can use a very high altitude, say [18000], so as to

avoid initial level-off and climb straight to the Jet Route MEA. If the MEA

for the specific J-Route portion you'll be initially flying along is higher,

use that higher altitude instead.

In our case, the SID calls for an initial altitude of 10,000ft or lower, so

we enter:

Departure Altitude :    10000

NOTE: When an airport is used as a destination, the altitude entered above,

minus 1000 feet, will be the intermediate descent altitude for ATC. In other

words, it will be the altitude you will be cleared to after descending to

10000 ft., and before being cleared to the Glideslope intercept altitude.

So, it is always recommended that before starting any flight, both Airports,

Departure and Destination, are quickly checked via this ATPUTIL menu for this

Altitude setting, and edited as appropriate.

8.

"Frequency ATIS:" "Clearance:" "Ground:" "Tower:" "Approach/Departure:"

Note all the above frequencies down from the IAP charts and enter them here.

You can use 1 or 2 decimals.

In our example we would enter:

Frequency ATIS:     135.35

Clearance:          124.70

Ground:             121.90

Tower:              120.50

Approach/Departure: 123.80 (I could not find a Departure Control frequency,

so I use the Approach Control)

After the last entry, hit [TAB] to move into the ATC controlled Runways

screen.

NOTE: Some airports have different frequencies for Approach Control and

Departure Control, or even for Approach ATIS and Departure ATIS. In such

cases I recommend editing the correct frequency each time before flying,

depending on whether the airport will be used as Departure or Destination.

This check can be done at the same time when doing the "Altitude" check

recommended in para 7 above.

ATC CONTROLLED RUNWAYS SCREEN

9.

Hit [F3] to start editing the table. I think everything is self explanatory.

+-------------------------------------------------+

|             ATC controlled runways              |

+-------------------------------------------------+

|  Name:  Dir:  Freq.ILS: Appr.Alt.: North: East: |

|  |                                              |

|  |                                              |

|  |                                              |

|  |                                              |

|  |                                              |

|  |                                              |

+-------------------------------------------------+

"Name"               is the Runway designator.

"Dir:"               is the Course to be flown during the approach (not

necessarily coinciding with the Runway heading, except

for ILS approaches)

"Freq.ILS:"          is the ILS (or VOR) frequency to be used for the

Approach.

"Appr.Alt.:"         is the last altitude you will be commanded to descend

to, before being cleared to land. For an ILS approach

use the GS intercept altitude. For a VOR approach use

the highest altitude depicted in the verical plan of the

IAP.

"North:" and "East:" are the runway threshold coordinates.

I recommend using the ATP SMS menus to place yourself at the threshold of

those runways and read the N/E coordinates and the Runway heading from the

[F7]-[3] menu in ATP. N/E coords need no decimals here, but the Heading

should be copied with 2 decimals again. The reason I suggest using this

heading, as opposed to that shown in the IAP, is because sometimes the ATP

Runway headings differ from those in the IAP charts, by a few degrees. On

very rare occasions, the difference can be significant. I also suggest using

the Glideslope intercept altitude as shown in the IAP, for the "Appr.Alt:"

entry.

NOTE: You can enter Runways with any Instrument Approach here, even if the

Runway does not have an ILS approach. Just use the coordinates of the Runway

threshold and the heading as depicted in the IAP for the approach being used.

Sometimes the course flown in such an Approach is not the Runway's heading.

Also, when using a LDA approach, remember to use the heading of the Approach

plate, not the Runway heading, so that ATC will "design" the correct approach

pattern and vector you accordingly. When entering a non-ILS approach, leave

the "Freq.ILS:" field empty, or enter whatever VOR frequency you will be

using during Approach. *Don't* enter an ILS frequency though.

Again, for "Appr.Alt. :" use an appropriate altitude as read from your IAP

chart for that approach.

In our BUF example, we would enter the following for the BUF ILS Runways.

I also include an example of a VOR-A Approach for Runway 32:

Name:   Dir:    Freq.ILS:   Appr.Alt.:  North:  East:

5       53.49   108.50      2300        17842   19443

23      233.49  111.30      2200        17849   19449

32      288.00  116.40      2900        17845   19448

Note that the "Dir:" for the 32 VOR-A approach is 288^ and not 316.221^ which

is the Runway's heading.

Also note that obviously you will not be marked during your VOR-A Approach

because there is no Localizer and Glideslope to judge you by.

When you finish, hit [ENTER] to get back to the previous screen, and hit

[TAB] and [F3] to edit the "List of Uncontrolled Runways".

LIST OF UNCONTROLLED RUNWAYS

10.

Here, you just enter the remaining Airport Runways that do not support an

Instrument Approach, but that you would like to see in the selection of

Runways when choosing a different takeoff runway.

Hit [INS],enter the Runway Designator, and [ENTER] twice.

If you want to delete an existing entry, place the cursor on that entry and

hit [DEL] and [ENTER] as the cursor is on the "Yes".

In our BUF example, since we have entered Runways 5, 23 and 32, we only need

to enter Runway 14:

Hit [INS], enter [14] hit [ENTER] twice and you are done.

Now hit [ESC] to return to the initial "Primary Airports" menu.

11.

Hit [ENTER]. Hit [u]. Remember, Upper Case is not accepted, it has to be a

lower case letter to activate ATPUTIL menus.

12.

Now repeat exactly the same procedure for EWR. Just pay special attention to

the "Departure Altitude:" field, which to you now really means "Arrival

Altitude". Set it a few thousand feet above you Glideslope Intercept altitude

for the EWR runways, and definitely higher than the MSA reported in the IAP

charts. Also take care to use the proper Arrival frequencies.

You are now done with your airports. Remember to check each of the airports

you will be using in order to edit the "Departure Altitude:" and the various

"frequency" fields depending on whether the airport will be a Departure or

Destination.

ROUTE ENTRY AND EDITING

13.

First task is to Browse NODES.BIN to examine which, if any, of our waypoints

already exists in the database. To do this, when in the initial ATPUTIL menu

hit [n] [v] [ENTER]. Now start typing each waypoint, and note down its

existence or absence from the file. As my NODES file is heavily modified

already, I cannot remember which, if any, of the waypoints of your suggested

Route exist in the default NODES.BIN.

NOTE: After typing a waypoint in the "View" mode, before typing the next you

must hit the [UP] or [DOWN] arrow once, otherwise the program will think you

are still completing the name of the previous waypoint. So, remember to hit

[UP] or [DOWN] before commencing typing for a new search.

We hit [ESC] and hit [n] [e] and [ENTER]. We are now in EDIT mode, ready to

enter the missing waypoints of our Route.

14.

Hit [INS]. Type [GEE] and hit [TAB] to move to the next field. Now hit [F3]

to select VOR type. You can hit [TAB] to browse possible selections. When the

correct type has appeared, hit [ENTER]. Next enter the Frequency for GEE.

Hit [TAB] and enter the range for that VOR, depending on its type (usually 70

or 132). Hit [TAB] again to reach the coordinates entry fields.

INPORTANT NOTE: Whatever you enter in these fields does *NOT* affect the

actual scenery files, only the NODES.BIN file which is used solely for ATC

purposes. In other words, if you enter a different VOR frequency or VOR

range, this does *not* mean that the actual frequency or range of the VOR has

changed. Simply, ATC will be fed with wrong info, and things may go a bit

crazy <g>.

15.

Now you must go back to ATP and find the N/E coordinates for GEE using SMS

([F7]-[9]), or, you can enter a [0] in the "North:" field hit [TAB] and enter

[0] in the "East:" field. Without hitting ENTER, hit GREY [+] from the

numeric keypad. A new menu will appear, and you can enter the Lat/Lon

coordinates instead of the N/E coordinates. Now, you can open your maps and

very accurately enter the location of GEE in Degrees, Minutes and Seconds.

I personally prefer the second, Lat/Lon method.

NOTE: Be careful to convert the "Degrees Minutes.Tenths-of-minutes" format to

Degrees Minutes Seconds" for ATPUTIL.

This is what we will have entered for GEE:

Node Name:  VOR-Type:   Freq.:  Range:  North:  East:

GEE         VORTAC      108.20  70      17840   19767

I found the coordinates by using the Lat/Lon window, where I entered the

following:

+-----------------------------------+

|        Lat/Lon Coordinates        |

+-----------------------------------+

|Latitude:   42  ^ 50  '  3.60  " N |

|Longitude:  77  ^ 43  ' 58.90  " W |

+-----------------------------------+

I got the exact coordinates from my Flight Planner. Incidentally, the

N/E coordinates listed in the SMS database are exactly the same.

We now repeat exactly the same procedure for HNK and SAX. (Naturally, if they

were already in the database, we would have discovered them in the browsing

process described in para 12 above, and there wouldn't be a need to do

anything.)

IMPORTANT !!

If a navaid to be used in the flight plan, be it a VOR, a NDB, an ILS, or a

Marker, does not exist in the scenery file to be used for the flight, then

entering them in NODES.BIN is not enough: NODES.BIN takes care of the ATC

part of the flight, so that ATC can monitor if you are heading towards the

correct direction.

For YOU, the PILOT though, you need to create these missing navaids, via

SCENEDIT. Only then will the missing navaids be tunable during your flight.

16.

We now enter the Intersection and Fixes. These are simpler, cause all we need

to enter is the "Node Name:" field and the "North:" and "East:" fields.

Again, we can either find the N/E coords from the USA SMS database in ATP,

(if the fixes are entered there) or copy the Lat/Lon coordinates from our

maps or SID/STAR charts, using the GREY [+] option.

NOTE: Remeber to always use the [TAB] key when finishing an entry, and at the

end, when the cursor advances to the next line below, use the [INS] key to

bring up a new empty line in order to enter the next waypoint. If we hit

[ENTER] we quit the Editing Menu.

This is what we would heve entered for EXTOL:

Node Name: VOR-Type:  Freq.: Range:  North:   East:

EXTOL                                17810   20128

This is how the Lat/Lon table was entered:

+-----------------------------------+

|        Lat/Lon Coordinates        |

+-----------------------------------+

|Latitude:   42  ^ 39  ' 14.40  " N |

|Longitude:  76  ^ 37  ' 17.90  " W |

+-----------------------------------+

Again, the SMS database has exactly the same N/E coordinates stored.

When we have finished entering all the waypoints, hit [ENTER] instead of

[INS]: That will bring us back to the initial ATPUTIL screen.

Hit [n] and then [u].

We have just completed the NODES.BIN editing.

EDITING ROUTES

17.

Hit [n] and then [r] and then [ENTER]. You will have entered the RouteEditor.

now type [BUF] , [ENTER] and [EWR]. the next screen of the RouteEditor will

appear, listing Departure: and Destination:, as well as the whole Route, from

Waypoint to Waypoint. Delete all entries, by hit [DEL] as many times as

required. We are now ready to enter our route.

18.

Place the cursor under the "Nav.Aids:" column, and type the Identification of

your Departure airport [BUF] and hit [ENTER]. the cursor is now under the

"Radial:" column. Now type the course *AWAY* from BUF, towards the next

waypoint, in this case towards GEE: It is 106^.

Continue entering all the Waypoints the same way, always writing the course

FROM that waypoint to the next alongside each Waypoint.

19.

In the column labelled "Airway:" we can enter the word [DIRECT], the name of

the Jet Route to be followed from that waypoint onwards, e.g. [J80], or the

name of the SID or STAR to be followed from that waypoint onwards,

e.g. [SHAFF2].

Please keep the following in mind:

a) Whatever we enter under "Airway:" comes into effect, and gets pronounced

in Clearance Delivery, AFTER the waypoint itself. After whatever is

written in the "Airway:" column has been pronounce during the Clearance

Delivery, the next thing to be pronounced will be the first waypoint whose

"Airway:" column is *not* empty, followed by whatever happens to be

written in the "Airway:" column. Anywaypoints in between will NOT be

pronounced, as in real life. However, they *must* be followed during the

flight.

b) When we enter an Airway name od SID/STAR, we should still include all the

waypoints along that Airway,SID or STAR because in reality ATP's ATC knows

NOTHING about such routes, and cannot figure out the course an Airway, SID

or STAR follows, unless we include it in our Route. We should NOT repeat

entering the same designator for every entry along that same airway, SID

or STAR. Write the next "Airway:" entry, whatever that may be, to the left

of the LAST waypoint belonging to the Airway, SID or STAR entered, and

right ABOVE the first waypoint to be entered NOT belonging to that Airway,

SID or STAR.

c) Whatever you enter as "Airway:" in the first row, ATP will replace it with

[Direct], because that's the way ATP functions towards the first waypoint

in any route: Radar vectors. You can leave that cell blank, because ATP

will enter the [Direct] anyway.

d) ATP's ATC only knows how to pronounce DIRECT and the J route designators:

i.e. J80 will be pronounced as "Jet eight zero". All other entries will be

pronounced phonetically, letter after letter, unless ATPVOICE has been

used.

e) Contrary to FA, always start the Route entries with the Departure

Airport's ID, or with a VOR very near to that airport (I suggest less than

10nm away). If you use a nearby VOR, make sure that the "Radial" entry

corresponds to the radial from *that* VOR to the first Waypoint, and not

from the Departure airport.  Likewise, finish your route preferably with

the Dstination airport's ID, or a co-located VOR.

f) ATP rounds down all "Radial:" entries to the nearest even number, so don't

worry if you later examine your flight plan and notice that all odd

radials have been reduced by 1 unit.

g) I suugest you enter the radials as read from real aviation maps/charts,

or as computed by professional real-time flight planners.

20.

Upon finishing the Route, it should look like this:

+-------------------------------------+

|             Routeeditor             |

+-------------------------------------+

|Departure: BUF    Destination: EWR   |

|                                     |

|    Airway:  Nav.Aids:   Radial:     |

|    |Direct    BUF        106        |

|    |SHAFF2    GEE        111        |

|    |          EXTOL      133        |

|    |          HNK        128        |

|    |          HELON      209        |

|    |          CRANK      209        |

|    |          SHAFF      209        |

|    |DIRECT    SAX        154        |

|    |          EWR                   |

|    |                                |

|    |                                |

|    |                                |

+-------------------------------------+

NOTE: It is not really necessary to enter FLOSI, CRANK and SHAFF because the

Course from HELON to SAX is a constant 209^.

NOTE: Also note that by BUF here we mean the BUF VOR. If we wanted to use the

airport itself, we should have entered a new NODE in NODES.BIN by a different

name, say KBUF, following the procedure in para 15 above, and use KBUF

instead of BUF in the Route. Since BUF is quite near KBUF though, there is no

such need. WE must pay attention though to enter the Radial from the BUF VOR

to GEE, and not the Course from KBUF to GEE. I have personally entered

various Kxxx nodes, to use as airports, when there are VORs with the same

Ident at more than 10nm apart.

You can actually enter a second NODE by the name of BUF and enter the Airport

coordinates, but this means you now have two BUF entries in your NODES.BIN

file and you shouldn't be able to tell the airport apart from the VOR, so I

suggest using the "KBUF" name.

After typing in the last entry in the Route Editor hit [ENTER]. If you see

the following message, choose the "No" if you are sure about your radials:

+--------------------------------------------+

|  Radials of given route out of tolerance!  |

|         Change them automatically?         |

+--------------------------------------------+

|Yes                                         |

|No                                          |

+--------------------------------------------+

Then hit [n] and [u], and you are done with your Route Editing.

EDITING ASSIGNMENTS

21.

First, download the Wather Report for your trip. I use AEROBRIEF (GO

AEROBRIEF) here at CIS. Have the Weather report in front of you when editing

the Assignment.

Now we are ready to Edit the Assignment itself. Type [a] [e] and [ENTER].

You will see this table:

+---------------------------+

| Career Assignment Number  |

+---------------------------+

| Number of Assignment: 1   |

+---------------------------+

I suggest accepting "1" and hitting [ENTER]. You will be faced with the

Assignment screen editor, whose left page you should edit as follows:

+---------------------------------+

| Career Assignment No.:        1 |

| Origin Airport       :      BUF |

| Destination Airport  :      EWR |

| Aircraft             :     XXXX |

| Payload              :    XXXXX |

| Flight date          : 02/28/94 |

| Dep. Time            :    XX:XX |

| Navigation Methode   :    XXXXX |

| Flightlevel          :      XXX |

| Landing Runway       :      XXX |

| Fuel Consumption     :     XXXX |

| Missed approach      :       XX |

| Zero Zero Sight      :       XX |

|                                 |

| Departure Weather    :          |

| Weather enroute      :          |

| Destination Weather  :          |

+---------------------------------+

Replace the "X"s with your own choices. Hit [F3] to find out available

options at each field. This is a fairly straightforward exercise.

I suggest flying a real leg, taken out of OAG, using the actual type,

preferrably the B737, and the exact Departure Time of the real flight.

Naturally, it is nice to download the weather to be REALLY prevailing during

the time of the actual trip.

NOTE: I suggest entering a "Flightlevel" of 120, and requesting your real

cruise FL when airborne. This way you will not be instructed to "Descend and

maintain 10000 feet" when 140 nm from destination <g>  You can request

permission to descend when you think it optimum for your Efficiency.

NOTE: What you enter in "Fuel Consumption" will be used by ATC as your

Reference Fuel for this trip, so set a realistic target based on your

experience, data etc. Take into consideration prevailing Winds when doing so.

NOTE: Choose your "Landing Runway" from the [F3] menu choices based on the

Surface Winds prevailing at destination. (See your Weather Report)

DEPARTURE WEATHER EDITING

22.

Now place the cursor at the "Deaprture Weather" field, and hit [F3]. You will

be faced with the following menu (I only present here the right page):

+------------Weather------------+

| Altimeter          :     30.40|

| Temperature        :       51 |

|                               |

| Cloud Coverage     :     5/10 |

| Bottom of Clouds   :     2500 |

| Top of Clouds      :     3000 |

|                               |

| Dir. Surfacewinds  :      140 |

| Speed Surfacewinds :       11 |

| Depth Surfacewinds :     4000 |

| Dir. Windlayer 1   :      283 |

| Speed Windlayer 1  :       40 |

| Bottom Windlayer 1 :     4952 |

| Top Windlayer 1    :    34700 |

| Dir. Windlayer 2   :      258 |

| Speed Windlayer 2  :       52 |

| Bottom Windlayer 2 :    34700 |

| Top Windlayer 2    :    45000 |

+-------------------------------+

I have left some numbers as they were entered from a previous Assignment of

mine, cause I think it is easier to understand it this way. Let me briefly

explain what I'm doing here:

Altimeter          :    I copy the value of the most recent SA report

Temperature        :    I deduct the ground Temp from the reported Temp aloft

at 18000 feet. I use a special calculator for this

deduction.

Cloud Coverage     :    I read it off the FA weather report for the Estimated

Time of my Departure.

Bottom of Clouds   :    Same as above.

Top of Clouds      :    I enter it arbitrarily. Usually 1000 ft higher than

the Bottom.

Dir. Surfacewinds  :    From the FA report, for the time of my Departure.

Speed Surfacewinds :    Same as above

Depth Surfacewinds :    I always set it to 4000 ft.

NOTE: For winds aloft I enter two sets: One is the prevailing winds reported

at 18000 feet MSL (approximately a midpoint for your climb), and the other is

the reported winds for my Cruise FL. I do the same for Destination Weather

too, using the Destination Airport reports.

Dir. Windlayer 1   :    The winds at 18000 MSL above my Departure airport.

Taken from the Weather Report.

Speed Windlayer 1  :    As above.

Bottom Windlayer 1 :    The top of the limit of my Surface Winds, in MSL.

Top Windlayer 1    :    My Cruise FL, less 300 feet.

Dir. Windlayer 2   :    The Wind Direction Prevailing at my Cruise FL for the

time of my passage.

Speed Windlayer 2  :    The wind speed as above.

Bottom Windlayer 2 :    My Cruise FL less 300 ft.

Top Windlayer 2    :    Usually 45000 feet.

After finishing with our DEPARTURE WEATHER, we hit [ESC] and move the cursor

down to the Weather Enroute field and hit [F3] again.

WEATHER ENROUTE EDITING

23.

We are faced with exactly the same page.

Here is how I fill it in:

Altimeter          : I copy the setting from the SA report for ENROUTE

WEATHER. I usually pick the mid-point of my flight, but

any point will do.

Temperature        : I deduce and enter the Ground Temp that will result in

my Cruise FL temp being the same as reported in the

Winds Aloft reports. Ground Elevation must be taken into

consideration here. Again, the Ground Elevation at

mid-point will do, or the average Elevation between

Departure and Destination.

Cloud Coverage     : I take it from the FA report for a mid-point in my

flight, for the time of my trip.

Bottom of Clouds   : I take it from the FA report for a mid-point in my

flight, for the time of my trip.

Top of Clouds      : Usually 1000 ft higher than above.

Dir. Surfacewinds  : 0

Speed Surfacewinds : 0

Depth Surfacewinds : 0

Dir. Windlayer 1   : The same as in Departure Weather

Speed Windlayer 1  : The same as in Departure Weather

Bottom Windlayer 1 : 0

Top Windlayer 1    : The same as in Departure Weather

Dir. Windlayer 2   : The same as in Departure Weather

Speed Windlayer 2  : The same as in Departure Weather

Bottom Windlayer 2 : The same as in Departure Weather

Top Windlayer 2    : The same as in Departure Weather

After finishing with our WEATHER ENROUTE, we hit [ESC] and move the cursor

down to the "Destination weather" field and hit [F3] again.

DESTINATION WEATHER EDITING

24.

We now have the Destination Weather page, which I fill-in exactly as per my

Departure Weather page:

Altimeter          :    I copy the value of the most recent SA report

Temperature        :    I deduct the ground Temp from the reported Temp aloft

at 18000 feet. I use a special calculator for this

deduction.

Cloud Coverage     :    I read it off the FA weather report for the Estimated

Time of my Arrival.

Bottom of Clouds   :    Same as above.

Top of Clouds      :    I enter it arbitrarily. Usually 1000 ft higher than

the Bottom.

Dir. Surfacewinds  :    From the FA report, as predicted for the time of my

Arrival.

Speed Surfacewinds :    Same as above

Depth Surfacewinds :    I always set it to 4000 ft.

Dir. Windlayer 1   :    The winds at 18000 MSL above my Arrival airport.

Taken from the Weather Report, for the estimated time

of my arrival.

Speed Windlayer 1  :    As above.

Bottom Windlayer 1 :    The top of the limit of my Surface Winds, in MSL.

Top Windlayer 1    :    My Cruise FL, less 300 feet.

Dir. Windlayer 2   :    The Wind Direction Prevailing at my Cruise FL for the

time of my passage.

Speed Windlayer 2  :    The wind speed as above.

Bottom Windlayer 2 :    My Cruise FL less 300 ft.

Top Windlayer 2    :    Usually 45000 feet.

We now hit [ESC], [ENTER], [a], [s] and [ENTER]. Now hit [ESC], [a] and [u].

We have just Updated our Assignment files.

25.

Now we should be tweaking our TRAFFIC CENTERS to correspond exactly to the

real Center boundaries for our trip, but I am too tired to write how I do

that right now! <G>

Instead, we will proceed by hitting [p], and [ENTER]. You should be facing

the following screen:

+---------------------------+

| Career Assignment Number  |

+---------------------------+

| Number of Assignment: 0   |

+---------------------------+

Enter [1], which was the number of the assignment we edited, and hit [ENTER]

twice. Now hit [ENTER] again, and then [r] and [ENTER].

You will be faced with your "Route Information" table.

Hit [ESC] and you will see the "Frequencis" table.

Hit [ESC] again, and you will be prompted to PRINT your Plan. Choose YES, and

hit [ENTER].

Now hit [ENTER], [f] and [ENTER] again, and you will see the "Estimate Fuel"

table, automatically computed by ATPUTIL for the optimum FL.

NOTE: I personally disregard this choice and Cruise at the FL of my choice.

The reason is that my flying habits differ from those specified in the Fuel

table, both in Cruise speed, but also in Descent Profile. In any event, the

Fuel specified at this table gives a very good indication of what Reference

Fuel target you should set in your Assignment menu: Difficult to beat, easy

to exceed if the Surface Winds have you being vectored all the way around for

an approach from the other side of the field! <g>

Hit [ENTER], and when prompted choose YES for printing your "Estimate Fuel"

table.

WE HAVE FINISHED!!

YOU ARE READY NOW TO FLY YOUR REALISTIC TRIP!

WHEW!!!

Best Regards

Stamatis (CIS: 71020,2502, INTERNET: 71020.2502@compuserve.com)

PS Regarding CENTERS, I am sure you can figure it out yourself, but in case

you face difficulties drop me a line.