Dayna Communications Inc's LocalTalk MC Card Installation Notes

===============================================================

1. IMPORTANT NOTES

---------------

To use this MLID, DL2.COM is the only software driver you need to run

PhoneNET PC in your system.

2. GENERAL INFORMATION

-------------------

The LocalTalk MC Card uses I/O ports and interrupts.  The card does not

use DMA channels or shared memory.

The LocalTalk MC Card uses one of 6 I/O ports.  The card's I/O address

can be set by using the reference diskette that comes with your

computer system and the .ADF file that comes with the adapter.

The install program allows the following possible values for I/O ports:

340H, 320H, 2A0H, 260H, 240H, 200H

The LocalTalk MC Card uses one of 3 IRQs.  The card's IRQ number can be

set by using the same files mentioned above.  The following IRQ

numbers are allowed:

3, 4, 5

For more details about configuring the LocalTalk MC Card and installing

the driver, please refer to the "LocalTalk Cards User's Guide" that

comes with your adapter.