Intel EtherExpress 16TP LAN Adapter Installation Notes

======================================================

1. IMPORTANT NOTES

---------------

You MUST run the SOFTSET.EXE program supplied by Intel to configure the

EtherExpress 16TP LAN adapter before you load the driver EXP16ODI.COM.

Be sure that the configuration settings you select when you run

SOFTSET.EXE match the settings when you run the PhoneNET PC INSTALL

software in your machine.  Do NOT load the EXP16ODI.COM driver before

running SOFTSET.EXE, since doing so could damage your card.

2. GENERAL INFORMATION

-------------------

The EtherExpress 16TP LAN Adapter will work on either Twisted Pair

Ethernet cabling (connected via an RJ-45 connector) or thick Ethernet

cabling (connected via a DIX connector).  The connector used by the

adapter is selected with the SOFTSET program.

For more information about installing and configuring your adapter,

please refer to the manual "The Complete Guide to installing and

configuring the Intel EtherExpress 16 and 16TP Network Adapters for ISA

computers" that comes with your adapter.