Farallon PhoneNET Card PC Installation Notes

============================================

1. IMPORTANT NOTES

---------------

If you use this MLID, LTALKP.COM is the only driver you need to run

PhoneNET PC in your system.

This MLID is preferred to the MLID LTALK.COM for use with the PhoneNET

Card PC for LocalTalk.

2. GENERAL INFORMATION

-------------------

The default settings for this card as it comes out of the package are

switches 2, 5, 7, and 8 ON, and 1, 3, 4, 6 OFF.  This indicates DMA

channel 1, interrupt level 3, and base I/O port address 240.

The LTALKP MLID does not use interrupts, it is a "polled" driver.  It

will therefore not conflict with other cards that use interrupt levels

IRQ 4 and 3, since the MLID does not enable the interrupt in software.

You can select one of two DMA channels, 1 or 3.  Switches 5 and 7

should be ON to use channel 1, and switches 4 and 6 should be ON to use

channel 3.  When 5 and 7 are on, 4 and 6 must be off, and vice-versa.

Switch 8 sets the port I/O address.  If it is ON, address 240 will be

used, otherwise, address 220 is used.

Do not use DMA channel 2 if you have a floppy disk drive, since the

floppy uses the same DMA channel.

For more information, consult your Farallon PhoneNET Card PC for

LocalTalk manual.