Xircom Creditcard Ethernet Adapter Installation Notes

=====================================================

1. IMPORTANT NOTES

---------------

DO NOT USE PhoneNET PC IN WINDOWS (3.1) STANDARD MODE WITH

THIS ADAPTER.  Connections to AFP servers may drop unexpectedly when

using the Creditcard driver in Windows Standard mode.

To use this adapter card, CEODI.COM is the only driver you need.  You

must run the program CECFG.EXE, located on the floppy that came with

your adapter.  This will determine the proper configuration to use.

Write down the 'Int=X, Mem=0xX000, Port=0xX00' values after running

CECFG.EXE, as you will need to provide them to the PhoneNET installer.

IMPORTANT:  You must copy the program CECFG.EXE to your hard drive,

then you must edit your AUTOEXEC.BAT file to run this program prior

to loading PhoneNET PC.

2. GENERAL INFORMATION

-------------------

The installer will prompt you for an Interrupt Line, an I/O Port, and

a Shared Memory Location (all of which are obtained through CECFG.EXE).

For more information about installing and configuring your adapter,

please refer to the manual that comes with your adapter.