8/10/94am

Overview

========

LZM is a Zmodem download driver.  It will NOT upload files, it will only

download them.  It has numerous bells-and-whistles which make waiting for

downloads much more tolerable.

Features

========

Percentage complete bar graph, runs full length of screen

Support for batch downloads

Unique "Leech" cancelation

Configuration

=============

LZM is called from DOS or your favorite terminal program with the following

parameters:

LZM D=[path] P=[port] B=[baud] C=[canmet]

Uppercase/lowercase or parameter order does not matter.  LZM assumes default

values for any parameters not specified.  These defaults can be configured with

the LZMCNF program.  Here is an explanation of each parameter:

D=[path]   This is the directory into which files you download will be

put.  If you enter "!" as a path, all files will be put in

the default directory.

P=[port]   The communications port LZM should use.  Port addresses

follow established standards and are non-configurable.  You

can specify 1 through 4 here.

B=[baud]   Baud rate to run the transfer at.  Sadly, LZM will not

automatically detect the current baud rate yet.  You may

enter a baud rate of 300 to 38400 bps.

C=[canmet] This is explained in detail below, in the "Leech" section.

LZMCNF lets you set defaults for all these parameters.  You may also configure

the screen colors with LZMCNF.

Leech

=====

LZM, as previously mentioned, supports a rather unique feature: Leech download.

This handy option will let you download entire files while convincing the BBS

you are on that the transfer aborted.  [Canmet] is a character value, and

can range from A to Z.  Currently supported cancelation methods are:

N -  No cancelation.  The download runs as normally.

S -  ZSKIP cancelation.  A ZSKIP header is sent just before EOF.  This

method has been defeated by DSZ's "snailbait" option.  Batch

downloads are supported with this method.

C -  ZCAN cancelation.  A ZCAN header is sent before EOF.  This does

not support batch download, as the entire transfer is canceled.

DSZ's "slugbait" detection will defeat this scheme.

H -  Half abort.  A ZSKIP header is sent somewhere between %30 and

70% of the way through the transfer.  This method is not used

much, expect future development.

R -  Rewind skipping.  After all data is received, the sender's

transfer pointer is rewound to a random point in the file and a

ZSKIP header is sent.  The sender will think that only data has

only been sent up to that random point.

Disclamer

=========

Use this program at your own risk.   It may have serious bugs in it that could

cause unpredictible results.  I did not, however, write any code intended to

cause damage to the user's system.  I do not [officially] advocate use of the

Leech feature, it is merely a curiousity.  Ahem.  Enjoy.

-Black Star