## Printing Improvements

Chicago offers several changes in how printing is handled by Windows.  The goals are to address requests made from our customers and from independent software and hardware vendors.  We aimed to make improvements in three major areas:

- Improved performance.

- Chicago has a new 32-bit printing architecture which supports preemptive multitasking and improves overall performance.

- Making it even easier to use.

- Printing is easier now because of improvements made in Chicago’s user interface.  Chicago also features Plug and Play support for installing new printers.

- Improved integration of network printing.

Network printing integration is better because of Chicago has extended the local printing architecture to the network environment. In addition, Chicago ties together installation enhancements to shared network printers.

This section of the Guide describes the printing architecture used in Chicago and discusses the different areas where printing has been improved over Windows 3.1.

### Summary of Improvements over Windows 3.1

The primary improvements in printing for Chicago are:

- New 32-bit Print subsystem modeled after Windows NT providing smooth background printing

- Increased printing performance by decreasing time needed to return control to application through the use of enhanced metafile (EMF) spooling

- Support for over 800 different printer models (versus over 300 for Windows 3.1) through the development of new printer mini-drivers

- Support for PostScript Level II printers

- Spooling of MS-DOS–based application print jobs along with Windows–based applications and solves conflicts when MS-DOS and Windows–based applications try to print at the same time

- Image color matching support providing better WYSIWYG between color in images displayed on-screen and color generated on an output device

- Deferred printing for mobile computer users, allowing users to print while undocked and not connected to a printer, then automatically starting print job once docked into a docking station

- Simplified printer driver installation, configuration, ease of use, and ease of support, through new consolidated user interface

- System support for new bi-directional printers and ports providing improved I/O performance with new fast parallel ports (ECP) and error status reporting

- Better integration of network printing support including point-and-print support for automatic installation of printer drivers from Chicago, Windows NT, or Novell NetWare servers

- Plug and Play support for printers for simpler installation and configuration

### 32-bit Print Subsystem

Chicago features a 32-bit Print subsystem that includes a multi-threaded, preemptive spooler architecture providing improved printing performance, smoother background printing, and quicker “return to application” time once a print job is initiated by a user in an application.  The architecture of the Print subsystem is compatible with the Windows NT 3.1 Print subsystem.

#### 32-bit Preemptive Spooler

In Windows 3.1, print spooling functionality was handled by Print Manager and was supported by code in several different Windows components.  In Chicago, the print spooler is implemented as a series of 32-bit virtual device drivers and consolidates the spooler functionality into a single architecture.

- The new spooler provides smooth background printing.

In Windows 3.1, Print Manager passed a fixed amount of information down to the printer, whether the print was ready to receive it or not.  If the printer wasn’t ready to receive more data, the system would be suspended until the printer was ready.  Unlike Print Manager, the Chicago spooler passes data to the printer only when the printer is ready to receive more information.  This helps to reduce what often seemed like “jerkiness” when printing documents with Windows 3.1 Print Manager.

- The new spooler provides quick “return to application” time.

- Due to the smooth background printing, made possible by the new 32-bit print subsystem, Chicago spools enhanced metafiles (EMF) when printing from Windows–based applications rather than raw printer data to result in quicker return to application time.  Once spooled, the EMF information is interpreted in the background by the printer driver, and output is then sent to the printer.  For more details, see the following “Enhanced Metafile Spooling” section.

- The new spooler is much more powerful and flexible.

- It allows the user to select printer attributes on a per printer basis instead of requiring global printing attributes as in Windows 3.1.  For example, each printer can have a different separator page and the option of printing direct printing via a queue.

##### Enhanced Metafile Spooling

EMF spooling results in quicker “return to application” time for returning control to the user after initiating a print job in a Windows–based application (Win16 or Win32).

Before discussing how EMFs fit into the printing architecture used by Chicago, it is worth reviewing how print jobs are handled by Windows 3.1.  The improvements present in Chicago result in great printing performance over Windows 3.1.

Figure 1.  Spooler Relationship to Printing in Windows 3.1

In Windows 3.1, all interpretation of print API calls were handled by the Windows printer driver before the information was spooled to Print Manager.  The interpretation of print information for printers was the most time-consuming operation in the print process.  Postscript printers were not impacted by this as the printer driver sends high-level Page Description Language (PDL)-based information to the printer rather than raw image data, where it was interpreted by the printer itself.  Users of non-Postscript printers saw a delay in “return to application” time under Windows 3.1 once the print job was initiated while the GDI print API calls are processed by the printer driver.  Once the output image file was created by the printer driver, the Print Manager spooler took over and control was returned to the user’s application.  Background printing under Windows 3.1 often seemed choppy.

Figure 2.  Spooler Relationship to Printing in Chicago

Chicago greatly improves the “return to application” time by spooling high-level command information generated by the GDI print API, collectively referred to as an enhanced metafile, rather than spooling raw printer data generated by the printer driver.  For example, if a document contains a solid black rectangle, the EMF would contain a command to draw a rectangle with the given dimensions, that should be filled in solid, with the color black.  Once the EMF is created, control is returned to the user, and the EMF file is interpreted in the background by the 32-bit print subsystem spooler and sent to the printer driver.  This results in control being returned to the user in significantly less time than having to wait for the print calls to be fully interpreted by the printer driver directly.

### Improved Printing Support for MS-DOS–based Applications

Chicago improves on support for printing from an MS-DOS–based application in the Windows environment over that provided by Windows 3.1 by allowing MS-DOS-based applications to spool print jobs to the Chicago Print subsystem 32-bit spooler.  With Windows 3.1, users printing from MS-DOS applications could not take advantage of the Windows–based spooling functionality offered by Print Manager, and encountered device contention issues when trying to print from MS-DOS–based applications at the same time as printing from Windows–based applications.

Chicago addresses the print issues in Windows 3.1 by incorporating the functionality for an MS-DOS–based application to spool directly to the 32-bit Chicago print spooler.  This support is integrated into a print spooler virtual device, which takes the output destined for a printer port and first places it in the Chicago print spooler before sending the data to the printer.  This functionality works with all existing MS-DOS–based applications, and results in quicker “return to application” through the use of the spooling mechanism.  While MS-DOS–based applications do not benefit from EMF spooling, which is supported only for printing from Windows–based applications, users won’t encounter device contention issues, but will benefit from smoother background printing and from improved printing performance in Chicago.  The print spooling functionality for use with MS-DOS–based applications is automatically installed and configured and handling is transparent to the user.

### Support for Deferred Printing

To benefit mobile computer users, the Chicago Print subsystem also features support for deferred printing.  This capability allows users not connected to a printer to generate print jobs, which are stored on their local computers.  This feature is handy for users working in a location away from the printer, and for users in the office who temporarily lose printer connections because of network or printer problems, for example.  Mobile users can create print jobs from Windows–based applications (Win16 or Win32 applications) or MS-DOS–based applications while on the road, then print on a physical printer once reaching home or office.  Items not immediately printed are held in the print queue until the user reconnects to a printer.

### Image Color Matching Support

Chicago includes Image Color Matching (ICM) support, enabling applications to offer better consistency between the color of images displayed on the screen and the color of images generated by an output device.

Using technology licensed from Kodak, Chicago will include image color matching (ICM) support for display and printer drivers.  ICM provides consistent (predictable) color rending from input, through monitor preview, to output.  Applications that utilize ICM functionality enables portability of color information across applications that manipulate the graphic information, across users to provide consistent use of colors, and across platforms allowing color information to be easily moved to different systems where the ICM technology has been implemented.

Image color matching support in Chicago provides the following benefits to application vendors, which in turn result in benefits to users:

- Easily enable color-aware applications

- Allows for color What You See Is What You Get (WYSIWYG)

- Provides for consistent color output across devices, and

Since Windows 3.1 did not provide ICM support as part of the operating system or in an external driver, image color matching support was implemented in a proprietary manner by an application vendor—the burden was on the application vendor to properly map colors generated on a display device to the colors generated by a printer device.  Chicago simplifies this process by including ICM support as part of the operating system, allowing application vendors to integrate ICM functionality into their applications, and thus take advantage of this new system service.  To provide support for device-independent color matching, colors used in applications are tied to international (CIE-based) colorimetric standards, rather than in device-dependent form to specific hardware devices.  The operating system will then do the appropriate color transformations to map the device-independent color representations to the colors supported by the physical device.

The key to ICM support is the use of a profile, which represents the color properties of a monitor, printer, or scanner device.  The profile format used by the ICM support in Chicago, is the result of an industry consortium called ColorSync 2.0 and is made up of many industry hardware vendors (including Kodak, Microsoft, Apple Computer Inc., Sun, and Silicon Graphics, among others) and industry standard-setting bodies.  The ColorSync efforts provide for a consistent cross-platform color standardization process that will result in industry-wide standards for defining ICM properties of output and display devices.

### Installing and Configuring a Printer

The first thing you will notice when looking at printer support in Chicago are some changes to the user interface.  Print Manager and the Print icon in Control Panel are gone.  Gone also is the confusion of which tool to use when you wanted to manage a print job, install a new printer, create a queue, or perform some other task related to printing.  Chicago consolidates the printer and printing functions into a single location called the Printers Folder.

Figure 3.  Chicago Printer Folders

The Printers Folder provides the user easy access to adding a new printer, configuring an existing printer, and managing print jobs.

#### Easy New Printer Setup

Chicago makes it easy to install new printers by supporting the following installation mechanisms:

- Plug and Play Printer Detection

For Plug and Play printers, Chicago will automatically detect the printer at installation time, or during the Chicago boot process.  The Plug and Play detection code will prompt the user for the appropriate driver files if they are not resident in the Windows directory.

- New Device Installation Wizard

Chicago includes support for Wizards that walk the user through the printer installation process.  Installing a printer is now even easier under Chicago, whether that printer is connected to the local PC, or shared on another PC on the network.  The following figure shows the process that the New Device Installation Wizard uses for installing a printer.

Figure 4.  New Device Installation Wizard Walks User Through Installing a Printer

- Connecting to a Network Printer—”Point and Print” Printing

Chicago makes it easy for users to connect to and use a printer shared on another Chicago PC, a Novell NetWare server, or a Windows NT Server.  If a user connects to a printer shared on another Chicago PC, Chicago will automatically copy down and install the proper driver for the shared printer from the remote Chicago PC.  Chicago also supports installing the appropriate printer drivers on a Novell NetWare server or Windows NT Server for automatic downloading and installation of the printer driver.  This allows users to simply connect to the remote printer and once the drivers have been copied from the network and installed on the local Chicago PC, begin printing.  Network Point and Print printing is discussed in the Chicago Networking section of this guide.

#### Configuring a Printer

Configuring a printer in Chicago is also greatly simplified over Windows 3.1.  All printer configuration is consolidated into a single property sheet for the printer, and can be accessed from the Printers folder.  The property sheet provides common access to printer parameters such as the printer port (or network path) the printer is connected to, the paper options for the printer, the fonts built into the printer, and device options specific to the printer model.

Figure 5.  Sample Properties for the Hewlett-Packard LaserJet IIIsi Printer

To further simplify printer configuration, Chicago will support bi-directional communications between compatible printers and printer ports, allowing Chicago to query the characteristics and configuration options directly from the printer.  This allows the printer driver to be automatically configured to exactly match the configuration of the printer including the amount of memory installed, the paper options available, and fonts installed in the printer.

### Managing Print Jobs

Managing print jobs is improved in Chicago over the capabilities offered by Print Manager in Windows 3.1 and Windows for Workgroups.  Improvements provided in Chicago include:

- Direct Integration with the Chicago User Interface

- The Printers folder serves as the centralized location for interacting or configuring with printer devices.  Switching to Details view will show additional information about the number of jobs presently residing in the printer.  Opening the given printer will show detailed information about the contents of active print jobs or jobs that are waiting in the queue including the name of the document, the status of the document, the owner of the document, when the document was submitted to the print queue, the number of pages in the document (when printing, the status of the print job down to the page that is being printed, is displayed), the size of the document, and the priority of the print job.

Figure 6.  Detailed Remote Print Queue Status

- Ability to Manage Print Jobs Locally and Remotely

- Under Chicago, a user has the ability to pause or cancel printing of print jobs residing in a remote Chicago print queue.  Under Windows for Workgroups, for example, a user needed to physically walk over to the remote PC to cancel any printing operations.  In addition, if you are given administrator access to the remote Chicago PC that is sharing a printer, you have the ability to remotely manage and administer the print queue with the same UI and functionality available for a locally connected printer.  It is no longer necessary to walk over to the remote machine where the queue resides, in order to terminate print jobs or to resume the printer if an error occurs.

### Network Printing Improvements

Chicago provides improved support over Windows 3.1 for printing in a networked environment.  These enhancements include:

- Network point-and-print functionality

- Under Chicago, users can print to a shared network printer connected to a computer running Chicago, Windows NT Advanced Server, or Novell NetWare, and have the appropriate printer driver automatically copied down from the remote computer and configured on the local Chicago computer.  This simplifies the printer installation process, and ensures that the proper printer driver is installed to match the remote printer.

- Microsoft Print Server for NetWare

Chicago provides a 32-bit Netware-compatible print server, offering functionality equivalent to the Novell Pserver utility and allows a computer running Chicago to despool print jobs from the queue on a NetWare server.  As a 32-bit application, the Microsoft Print Server for NetWare provides smooth background processing and printing of jobs, as well as robust operation.

- Remote Administration of Print Jobs

Chicago provides full remote administration of print jobs for shared printers on another computer running Chicago.  With the appropriate access privileges, these operations include the ability to hold a print job, cancel a print job, or resume printing when the print queue is paused.

More information about network printing enhancements present in Chicago is provided in the Networking section of this guide.

### Plug and Play Support

Installing and configuring printers in Chicago is greatly simplified over Windows 3.1.  As with other components of the Chicago system, setting up new printers in Chicago benefits from Plug and Play capabilities.  Chicago detects Plug and Play compatible printers that return device ID values as described in the IEEE 1284 Specification through bi-directional parallel communications.  Bi-directional parallel communications with the printer will also aid in the query of other physical attributes of the device.

Chicago detects a Plug and Play printer in one of several ways—when Chicago is first installed on a user’s PC, during the boot cycle each time Chicago is started, or when a user explicitly requests a detection to be made.

Figure 7.  New Device Found Dialog Box Showing Detection of Plug and Play Printer

Plug and Play for printers works this way:

When Chicago is first installed on a user’s PC and when Chicago starts up, the Plug and Play detection code attempts to identify a printer connected to a bi-directional communications port.  If the printer connected is not presently configured in the Chicago system, the user is asked as to whether the printer should be installed.  If the user says yes and the appropriate printer driver is already present on the system, Chicago automatically installs and configures the driver for the new printer.  If the printer driver is not already present, Chicago prompts the user for the appropriate Chicago Setup and Installation disk.  If the printer is not recognized by the Chicago system, the user is prompted to insert a disk containing the printer driver provided by the printer manufacturer.

#### Examples of Plug and Play-Compatible Printers

While there will be much broader support and demand for Plug and Play printers at the time Chicago ships, several printers on the market today provide varying degrees of Plug and Play awareness.  For example, the Hewlett-Packard’s LaserJet 4P, 4MP, and 4ML, which are presently shipping, are Plug and Play peripherals.

### Try It!

To see how the improvements made to printing support in Chicago will help users that print from MS-DOS and Windows-based applications, you’ve got to try it!

#### Quick Return to Application Time

To demonstrate the improved quick return to application time for printing from MS-DOS or Windows-based applications, try the following:

- Under Windows 3.1, start Print Manager.  Turn off background printing if supported by your application (for example, Word for Windows version 6.0).  Print from your application - how long did it take before control was returned?  Perform the same task under Chicago - how long did it take before control was returned?

- Quick return to application time is also evident when printing from MS-DOS–based applications.  Try printing from an MS-DOS–based application under both Windows 3.1 and Chicago - how long did it take before control was returned?

#### Spooling from an MS-DOS–based Application

In addition to providing quick return to application time when printing from MS-DOS–based applications, print jobs generated by MS-DOS–based applications show up in the Chicago print queue and can be manipulated like print jobs from Windows–based applications.  To see this, try the following.

- Pause the Chicago print queue for your printer and then print from an MS-DOS–based application and note that it shows up in the print queue.

#### Plug and Play Support

To try Plug and Play support for printers with Chicago, connect one of the supported Hewlett-Packard LaserJet printers (4P, 4MP, and 4ML) that is Plug and Play aware to your computer before starting Chicago.  During the Chicago boot process, the Plug and Play printer will automatically be detected and will prompt you to install the appropriate printer driver.

## Communications

Chicago features a new 32-bit communications subsystem that provides higher throughput, better reliability, and greater device independence for communications operations than Windows 3.1.  The new communications subsystem serves as the underlying architecture on which Chicago provides communication services for supporting remote network access, Microsoft At Work fax services, access to on-line information services, and remote access to mail.

The communications architecture addresses problems that users have encountered with communications support in Windows 3.1 to provide a powerful, robust, and flexible communications architecture.

### Summary of Improvements over Windows 3.1

Changes made in the kernel and communications architecture in Chicago provide improvements and benefits to the Windows 3.1 user including:

- Robust and reliable high baud rate communications throughput

- Better multitasking of communications applications

- Simpler centralized setup and configuration

- Broader device support, and

- Better support for sharing communication devices on a PC (e.g., modems) among different communication applications

### Communications Architecture

Around the time when Windows 3.0 was first developed, 2400 baud modems were the mainstream and 9600 baud modems were just becoming affordable.  Windows was able to handle receiving data at these relatively slow rates without much difficulty.  However, as mechanisms to transfer communications information at faster rates (e.g., higher baud rates or the through the use of data compression) are becoming more popular, the communications architecture of Windows needed to be examined closely.

Around the time when Windows 3.1 shipped, 9600 baud modems were extremely popular, and communications under Windows 3.1 had many barriers that limited the overall effectiveness of reliable high data throughput and support for multitasking when running communications applications.  These barriers included high interrupt latency and overhead affecting high speed communications, and a monolithic driver architecture that made it necessary for some third-parties to replace the communications driver provided with Windows to allow their devices to run efficiently in the system.

Chicago greatly improves upon the Windows 3.1 architecture to support communications applications and supports high speed communications, as well as a modular communications architecture to allow third-parties and communications device manufacturers to easily plug in new communications device drivers.  This section describes the communications architecture used in Chicago.

#### Chicago Communication Goals

The goals of communications support in Chicago are focused around supporting an architecture that delivers better performance than Windows 3.1, and supports ease-of-use enhancements through Plug and Play communications.  The communications architecture of Chicago delivers the following performance benefits over Windows 3.1:

- High-speed reliability

- Chicago supports reliable high-speed communications by keeping up with data coming in from the communications port, and thus resulting in no lost characters due to interrupt latency.  In addition, the use of a 32-bit protected mode file system and network architecture results in less impact on the communications system by reducing required mode transitions and interrupt latency.

- Higher data throughput

- The 32-bit communications subsystem leverages the preemptive multitasking architecture of Chicago to provide better responsiveness to communications applications, and thus supporting higher data throughput.  Communications transfers in 32-bit applications are not as affected by other tasks running in the system as Win16–based applications under Windows 3.1.

- Provide support for time critical protocols

Chicago’s communications architecture provides support for time critical protocols and allows for real-time serial device control.

The Plug and Play initiative provides ease-of-use enhancements system-wide in Chicago and communications support is no different.  Plug and Play support for communications delivers:

- Broad device support

- Chicago features a new communication driver architecture that makes it easier for third-parties to extend the communications support provided as part of the operating system, without sacrificing functionality or stability.  In addition, the new communications architecture features APIs that are extended to support more robust communications devices beyond base RS-232 devices (e.g., ISDN).

- Easy to install and use communication devices

- Chicago features centralized modem installation and configuration support to simplify setup for end-users, and simplify communications development efforts for application developers.  Chicago leverages the use of a single universal modem driver (UNIMODEM) to provide a consistent mechanism for communicating with modem devices.  Chicago also provides detection support for Plug and Play modems.  It also provides support for existing hardware by including mechanisms for detecting legacy modems.

- Enables device sharing among communications applications

Through the use of the Telephony API (TAPI), Chicago provides consistent device-independent access to control communication devices for operations such as dialing and answering incoming calls.  Arbitration for sharing of communication ports and devices is also handled through TAPI.  For example, while Chicago remote access services is waiting for an incoming call, a TAPI-aware fax communications application can send an outgoing fax without having to first terminate the already running communications application.

To further understand the improvements related to the new 32-bit communications subsystem in Chicago, we’ll examine the components that make up the communications support.

#### Chicago Kernel Improvements

When data is coming into the system from a serial communications port, an interrupt occurs telling the system that a piece of data has just been received. Under Windows 3.1, if information is being received at a high rate it was possible that the system could not keep up with the incoming data, thus resulting in errors or lost information at the port.

What is unique about serial communications I/O is that one interrupt occurs on the system for each incoming character, versus disk or network I/O in which they manipulate blocks of information at a time.  The burden on the communications driver to keep up is quite high.

To support high speed throughput of information from a communications device, the system must be able to respond quickly to incoming data.  In Windows 3.1, real-mode drivers sometimes disabled system-wide interrupts for a “long” period of time (usually in terms of milliseconds), during which no incoming information can be received.

To directly address the issue of supporting higher sustained communications throughput, the Chicago development team focused on areas in the Windows kernel that resulted in periods of time that interrupts were disabled by the system.  In Windows 3.1, the Windows kernel and other components was limited to reliable serial communications at rates of 9,600 bps or slightly higher (dependent upon CPU speed), due to high interrupt latency or other systems design limitations.  In addition, the use of real-mode file system and networking drivers would block the system when Windows 3.1 had to execute real-mode code, thus preventing the system from being able to keep up with incoming data.

To improve performance and the rate at which the system can accept incoming data reliably, the Chicago team reduced code that can only be used by one process at a time (critical sections), and reduced interrupt latency in the core system.  In addition, the use of new 32-bit protect mode components for the implementation of the file system and network subsystem also helped to improve the system responsiveness.  Chicago is now truly limited in baud rate only by the hardware characteristics of the PC such as the processor speed, and type of communications port.

#### Driver Architecture

The communications subsystem consists of a modular 32-bit protect mode architecture with new communications drivers.  VCOMM is a new layer that provides protected mode services that allow Windows-based applications and device drivers to use ports and modems.  To conserve system resources, communications device drivers are loaded into memory only when in use by applications.  Also, VCOMM uses the new Chicago Plug and Play services to assist with configuration and installation of communications devices.

Figure 8.  Windows 3.1 Communications Architecture

Windows 3.1 uses a monolithic communications driver, COMM.DRV, that provides an API interface for Windows–based applications to interact with communications devices and the code that serves as the communications port driver.  The monolithic approach made it necessary to completely replace the Windows communication driver if new functionality was required by a hardware device.  Figure 66 shows the relationship between the COMM.DRV driver and the hardware device in Windows 3.1.

Chicago provides a more flexible communications architecture than Windows 3.1, separating communications operations into three primary areas—Win32 communication APIs and TAPI, the universal modem driver, and communication port drivers.

Figure 9.  Chicago Communications Architecture

Figure 67 shows the relationship between the VCOMM communications driver and the port drivers to communicate with hardware devices.  The flow path for a Win16–based application is also illustrated to show how compatibility is maintained for existing Windows–based applications.  Compatibility is maintained for IHVs and ISVs that replace the Windows 3.1 COMM.DRV driver, however the vendor-specific communications driver communicates directly with the I/O port, rather than going through VCOMM.

A description of the primary areas that make up the architecture are described below.

	Win32 Communication APIs and TAPI

Chicago’s Win32 Communications APIs provide an interface to use modems and communications devices in a device-independent fashion.  Applications will call the Win32 Communication APIs to configure modems and perform data I/O through them.  Through the Telephony API, applications will be able to control modems for operations such as dialing, answering, or hanging up a connection, in a standard way.  TAPI-aware communication applications no longer need to provide their own modem support list, as interaction with a modem is now centralized by Chicago.  The communications functionality provided with Chicago utilizes these services.

	Universal Modem Driver

Also new in Chicago is the universal modem driver, UniModem, which is a layer for providing services for data and fax modems, and voice.  Users and application developers will not have to learn or maintain difficult modem “AT” commands to dial, answer, and configure modems.  Rather, UniModem does these tasks automatically, using mini-drivers written by modem hardware vendors.  Application developers can utilize TAPI to perform modem control operations in a modem-independent manner.

	Port Drivers

Port drivers are specifically responsible for communicating with I/O ports.  I/O ports are accessed through the VCOMM driver and provide a layered approach to device communications.  For example, Chicago will provide a port driver to communicate with serial communications and parallel ports, and third-parties and IHVs can provide port drivers to communicate with their own hardware adapter such as multiport communications adapters.  With the port driver model in Chicago, it will no longer be necessary for third-parties to replace the communications subsystem as they did in Windows 3.1.

### Telephony API (TAPI)

The Windows Telephony API is part of the Microsoft Windows Open Services Architecture (WOSA), which provides a single set of open-ended interfaces to enterprise computing services.  WOSA encompasses a number of APIs, providing applications and corporate developers with an open set of interfaces to which applications can be written and accessed.  WOSA also includes services for data access, messaging, software licensing, connectivity and financial services.

WOSA services such as the Windows Telephony API consist of two interfaces.  Developers write to an applications programming interface (API).  The other interface, referred to as the service provider interface (SPI), is used to establish the connection to the specific telephone network.  This model is similar to the computer industry model whereby printer manufacturers provide printer drivers for Windows-based applications.  Figure 68 shows the relationship between the “front-end” Windows Telephony API and the “back-end” Windows Telephony SPI.

Figure 10.  Windows Seamlessly Integrates Applications and Telephone Networks

The Windows Telephony API provides a standard way for communication applications to control telephony functions for data, fax, and voice calls.  This includes such basic functions as establishing, answering and terminating a call.  It also includes supplementary functions, such as hold, transfer, conference and call park found in PBXs and other phone systems.  The API also provides access to features that are specific to certain service providers, with built-in extensibility to accommodate future telephony features and networks as they become available.

Through the use of the TAPI services, applications that support communication services have a device-independent means for interacting with communications devices.  TAPI also provides a common access mechanism for requesting the use of communication ports and communication devices, thus providing a means for communications-based applications to share a single modem (voice, fax, or data) in the computer.

Chicago includes TAPI support in the base operating system, thus allowing application developers to leverage this functionality in their Chicago-aware applications.  In addition, all communication components included as part of Chicago are TAPI clients.

#### Better Sharing of Communication Devices Between Communication Applications and Services

Through the TAPI interface, communications applications can ask for access to the modem or telephone device, allowing the Chicago Communications subsystem to arbitrate device contention and allow applications to share a communications device in a cooperative manner.

Win32–based applications can utilize TAPI functionality to allow applications to be able to make outgoing calls, while others are waiting for inbound calls.  For example, while a remote network access service is configured for auto-answer mode and is waiting for an incoming call, a Win32–based communications application can call the TAPI services to request the use of the modem and perform an outgoing call.  Of course, only one call can be performed at a time, but users no longer have to terminate other applications that are using a communications port in order to run a different communication.  The TAPI services arbitrate requests to share communication ports and devices.

### Centralized Modem Setup and Configuration

Support for installing and configuring a modem under Chicago is greatly simplified over Windows 3.1.  No longer is it necessary to configure each individual communications program for the proper serial port, modem type, and other related modem configuration parameters.  Chicago provides central configuration of communications devices through an icon in Control Panel.  Win32–based applications that take advantage of the TAPI services implemented in Chicago can completely leverage the user’s configuration of their communications hardware, making subsequent configuration of communications-based applications easy.

Chicago brings the following benefits to modem configuration:

	Easy modem configuration for use by entire system for new communications applications

	Centralized communications port status and configuration

	Supported by TAPI & Win32 Communication APIs

	Support for 100+ modems in Chicago

#### Modem Configuration in Windows 3.1

To understand the implications of improved modem support in Chicago, let’s first examine the issue of modem configuration under Windows 3.1.  Under Windows 3.1, when a user adds a new communications program to their computer, the user must first configure the application to communicate with their modem by specifying the COM port to use, the type of modem they have, in addition to other communication parameters.  Communications and modem configuration is either handled by the applications vendor and specified as a series of default modem “AT” command statements, or it is up to the user to read through his/her modem manual and type in the appropriate command strings.  Figure 69 shows the Modem Commands dialog box in Terminal provided with Windows 3.1.  Furthermore, given the number of modems available on the market, many Windows 3.1–based communications applications support a limited set of recognized modems because of the increased burden on the applications developer to provide this support.

Figure 11.  Modem Configuration in Windows 3.1 Terminal

#### Modem Configuration in Chicago

As with support for printers, the use of modems in Chicago is centralized by the operating system.  When a user first installs Chicago, the user is prompted to detect or identify the modem device that they have connected to or installed in their computer. Once a modem has been selected and configured, any communications application that supports TAPI services can interact with the modem in a device-independent way.  Users no longer need to know or understand complicated “AT” command sequences to customize their communications application.

Configuring a modem under Chicago is as easy as performing three simple steps: identifying the new modem device, configuring the modem device, and configuring the Telephony services.

##### Identifying the New Modem Device

If the user doesn’t select a Modem when Chicago is first installed, a new modem can be identified from the Modems control panel icon.  When the Modems dialog box is displayed, the user can either select a modem from the list of known manufacturers and models, or can choose the option to have Chicago detect the modem connected to the PC.  The detect option makes it easy for users by using Plug and Play detection or querying the modem to configure the correct device.  If Chicago can not detect the device, then the user can manually select the proper modem to use.

Figure 12.  Selecting a Modem Device

##### Configuring a Modem Device

Once the proper modem has been selected, the user can optionally change the properties for the device to set configuration parameters such as the volume for the modem speaker, the time to wait for the remote computer to answer the call, and the maximum baud rate to use.  (The maximum baud rate is limited by the speed of the PC’s CPU and the speed supported by the communications port.)

Figure 13.  Sample Modem Property Sheet

##### Configuring Telephony Services

In addition to configuring the modem device, the user configures telephony services to identify the various dialing parameters associated with the different locations where the computer will be used.

For each location where the PC will be used, information is stored for use by TAPI-aware applications to simplify dialing a local call, a long distance call, the area code for the location for use in determining whether the call is inside or outside the calling area code, and calling card information.  For a desktop PC, the location would commonly use the “default location” or perhaps change the default name to “in the office,” whereas a mobile user would add several different locations to match where the laptop computer is commonly used.

For example, a mobile user may use the computer in the office, on the road, or in a remote city as part of his/her business needs.  The following figure shows several different locations selected.

Figure 14.  Dial Assistant Property Sheet for Configuring Location Information

### Improved Device/Hardware Support

Chicago provides improved communications device and hardware support over Windows 3.1.  A few areas of improvement are discussed below.

#### 16550A UART FIFO Support

Chicago will provide greater robustness and performance at high baud rates for MS-DOS-based and Windows-based communications applications using local serial ports with 16550A compatible UARTs.  The 16550A UART contains a 16 byte FIFO buffer to prevent character overflow due to interrupt latency, and helps to reduce interrupt overhead overall.  The Windows 3.1 communications driver did not fully support the use of the 16550A UART, requiring some third-party communications vendors to replace the Windows 3.1-provided communications driver.  Communications in Chicago should alleviate the need for third-party vendors to replace communications driver components.

#### More Ports Supported

Windows 3.1 imposed a limit to the number of logical names that could be used to address or refer to serial communication ports and to parallel ports, of 9 serial ports and 4 parallel ports.  The communication APIs present in Chicago have been enhanced to support the same number of logical ports as MS-DOS, which is 128 serial ports, and 128 parallel ports.  This limit inhibited the use of multi-port serial devices in Windows 3.1.  The actual limitations to the number of ports usable is still based on the physical number of ports available to the system.

#### Support for Future Parallel Modems

Chicago also provides support for Enhanced Capabilities Ports (ECP) to facilitate higher speed communications than is possible over a serial device.  This support allows the use of future parallel port modems.

### Plug and Play Support

Plug and Play support for communications devices in Chicago facilitates the detection of connected modem devices and assigning of system resources (e.g., IRQs and I/O addresses for communication ports), simplifying configuration and setup.  In addition to Plug and Play detection, Chicago provides for manual detection of non-Plug and Play communication devices such as modems.  Since there is presently no standard for obtaining device information using the “AT” modem command strings, detection of legacy modems is handled by performing a manual query of the modem device and checking information returned against a database of  known modem information.  Microsoft is working with other leading industry manufacturers to standardize the modem command set as part of a Telecommunications Industry Association (TIA) proposed standard called IS-131.  When this proposal is adopted, Chicago will support the standardized command set and this will aid detection of legacy modems.

#### Modems

Plug and Play detection for external modems requires new firmware in the modem to return the required Plug and Play ID information, while internal Plug and Play modems utilize the ISA Plug and Play specification.  Chicago supports the use of PCMCIA communication devices as part of the Plug and Play services for the PCMCIA specification.  Some modem manufacturers will improve their communications product offerings by revising their existing modem lines, whereas others will produce a new line of Plug and Play modems.  Detection for Plug and Play serial devices such as modems is handled when Chicago is initially installed, during the Chicago boot process, or when a new modem device is connected to the system.  As with other Plug and Play devices, the user is notified that the new device has been detected and is asked to confirm the installation and configuration of the device.

Support for legacy modems is provided by using device-specific information about a modem to provide a manual detection mechanism, or providing a list of supported modems from which a user can choose the appropriate one.  Once the modem is identified for the system, it is available for use by TAPI-enabled communication applications including remote access services, Microsoft At Work fax services, and the new HyperTerminal communications application.

### New Communications Application: HyperTerminal

Chicago includes a new 32-bit communications application called HyperTerminal that demonstrates what it’s like to be a good communication application under Chicago. HyperTerminal offers the same base communication capabilities as Terminal included with Windows 3.1, but integrates well with the Chicago UI and demonstrates how the Win32 communication APIs and TAPI services support more flexible communications applications than Windows–based applications under Windows 3.1.

Good communication applications under Chicago will utilize the following services and capabilities to offer a more robust and powerful solution:

- Win32–based application that uses the Win32 Communication APIs.

- Internal architecture that uses multiple threads of execution to provide good responsiveness to the user and great error-free high speed communications.  Multiple threads allows for full preemptive multitasking of communication tasks, supporting concurrent interaction with the user, downloading of remote data, and display of communication status.

- Support for TAPI services for making remote connections and controlling the modem device.

### Try It!

To see how the improvements made to communications support in Chicago will help users of existing MS-DOS and Windows-based communications applications, you’ve got to try it!

#### Background Multitasking of Communications Applications

To see how support for communications application in Chicago is improved over Windows 3.1, try the following under both Windows 3.1 and under Chicago:

	Run an MS-DOS–based communications application in the background with other foreground activities.

	Run a Win16–based communications application and perform other CPU or disk intensive tasks, such as copying files, accessing a network, or accessing/formatting a floppy disk.

	Run the 32-bit HyperTerminal communications application and perform other CPU or disk intensive tasks, such as copying files, accessing a network, or accessing/formatting a floppy disk.

## Mobile Computing Services

As personal computing moves beyond its traditional fetters of a single desktop, Microsoft is committed to leading the market as the critical catalyst in this change.  With Windows “Chicago,”  Microsoft is in a unique position to deliver system services and end-user functionality that takes mobile computing to the next level of opportunity.

### Vision of Chicago Mobile Computing

The goal for mobility in Chicago is to allow users—wherever they are and whatever computing they want to do—to do it easily. Our strategy for delivering on this vision is based on the following tenets:

- Mobile computing encompasses anyone that moves away from their desktop PC and wants computing capabilities.  It includes everyone, from the people who moves from meeting to meeting in an office building, to those who shuttle between their homes and offices, to those who have no office at all and move from customer site to customer site.

- The tasks people want to do away from their desks are fundamentally similar to those that they want to do on their desktop.  On desktop PCs, people want to draft a memo, review a budget spreadsheet, browse email, peruse a presentation on the network, look at their schedule for the day.  Away from their office and desktop PC, people want to continue browsing email, drafting a memo, or perusing a presentation up on the network.

- The “mobile” computer environment is fundamentally different than the desktop environment.  When users move away from their desktop PC, their computing environment changes dramatically. Portable PC users are operating in a power constrained environment with a video display often half the size of their desktop display.  Their hardware changes regularly as they plug in/unplug different components to deal with the task at hand.  They can’t participate in their workgroup so they can’t access a file on a server or receive email.  In this way, the mobile computing environment can be constrictive to users.

### Chicago Mobile Framework

Mobile computing restrictions stem from two problems:

- Communicating

- At their desk, users have a wide array of communication capabilities to keep them connected to the people in their network workgroup and people outside of their workgroup.  They have access to the LAN and all its services such as email, file sharing, and print sharing.  There is a phone, a fax machine, and perhaps a shared modem close at hand.  When they leave their desks, these users become communication islands.  They are cut off from their workgroup and all its LAN services.  Phones, faxes, and modems are not readily available.  Being mobile is a constant struggle to stay in touch.

To realize seamless mobility, users must be able to easily communicate regardless of location whether it be in the office, at a customer site, or at home.  Communication is between their portable and their desktop PC, between themselves and the rest of their workgroup, and between themselves and the broader community of PC users.

Chicago provides compelling end-user communication capabilities and an open, extensible set of services for applications to establish these connections.

- Adapting to hardware changes and differences

Desktop PCs operate in a fairly constant environment.  Mobile computers do not.  For example, docking station owners usually change video resolution, pointing device,  and network state every time they change location (for example, when they undock).  To change locations means tweaking configuration files, contending with error messages and restarting their computer a lot.  Being mobile is a constant struggle to get the hardware to adapt to the new conditions the user is computing in.

A portable PC user’s hardware configuration is constantly changing as he or she move from location to location.  To achieve seamless mobility, these changes in hardware must be seamless to the user.

Microsoft’s Plug and Play architecture, defined in partnership with other industry leaders like Intel and Compaq, provides an infrastructure to effectively tackle these problems.

Chicago’s development investments for mobile computing have focused on delivering solutions for these two problems.  More specifically, the development focus has been on the following:

- Keeping mobile PC users connected to and communicating with people both within and outside of their workgroup (LAN).

- Seamlessly adapting to hardware changes when users demand different hardware as they move from location to location, task to task.

The following table shows features meant to address the communication and hardware adaptation needs of users:

|  | Communication | Hardware Adaptation |
| --- | --- | --- |
| Empowering | Remote Access | Warm Docking |
| Users | The Briefcase: File Synchronization | Power Management |
|  | Local Connections: Serial, Parallel, Wireless | PCMCIA Support |
|  | Remote Mail | Dynamic Networking |
|  | At Work Fax | Deferred Printing |
|  | Accessing Internet and Online Services | Dynamic Video Resolution |
|  | VoiceView | Enhanced Mouse Support |
|  | User Profiles |  |
|  | Password Management |  |
|  | Document Viewers |  |
| Enabling | Chicago Networking Architecture | Plug and Play Architecture |
| Independent | Slow Link and Remote Access Services | Device Detection and Setup |
| Software Vendors | OLE 2: File Synchronization Interfaces | Plug and Play messages |
| (ISVs) | Telephony API (TAPI) | Enhanced Registry |
|  | Unimodem |  |
|  | Messaging API (MAPI) |  |

The following sections detail how Chicago provides improved communication and hardware-adaptability features for the mobile user.

### Remote Network Access

In the office, well over 50% of PC users have become accustomed to full workgroup computing capabilities—printing to a network printer, sending and receiving email, and accessing shared files.  However, when users leave the office, while they can take their computer home, they cannot take all of the shared resources from their workgroup environment with them.

Chicago’s remote access feature gives users complete workgroup computing capabilities whether it be running a client-server application accessing a customer database, downloading and/or browsing email, or accessing shared files.  It is smoothly integrated into the Chicago shell so it doesn’t seem like users are running a separate application.  Establishing a remote connection works the same as establishing a connection in the office—the user simply double-clicks on the network object to connect.  Similarly, if the user double-clicks on Mail, a remote connection is automatically established.

The Remote Access client software component, like the rest of Chicago networking, connects to a broad set of networks and provides an open architecture for universal connectivity.  Chicago’s remote access functionality includes support for IPX, NetBEUI, and TCP/IP network protocols using industry standard point-to-point protocol (PPP) over the wire, as shown in Figure 73.  Because remote access support uses the dynamic, 32-bit, protected-mode network architecture of Chicago, users don’t have to reboot their computers or exit Windows to continue working after establishing or ending a connection.

Figure 15.  Remote Access Functionality in Chicago Supports TCP/IP and IPX

On the host computer—that is, the one into which the user is dialing—Chicago provides an easy to use, single-port host, capable of multi-protocol routing for IPX and NetBIOS with pass-through user-level security.  This security uses Windows NT or NetWare authentication mechanism and user database to validate the user.  Of course, share-level security is also an available option.  Using Chicago’s desktop management capabilities, an administrator can disable dial-in access so users cannot dial into a particular desktop PC, or remotely access the entire network.  (For more details about Chicago’s desktop management infrastructure, see the Networking section earlier in this guide.)  If the user chooses to use a host system such as Windows NT, Shiva Netmodem/LanRover, or NetWare Connect, Chicago’s remote access feature offers full connectivity.  This also means that the Chicago desktop PC can be used as a convenient access point to a small LAN or simply to the desktop PC itself.  (The “Daytona” release of Windows NT Advanced Server supplements the remote network access support in Chicago to provide a large network solution that allows for as many as 256 simultaneous dial-in sessions.)

Figure 16.  Chicago Supports Flexible Remote Connectivity Options and Broad Network Access

Chicago provides a modular, open architecture to support applications establishing a “pipeline” to the network remotely.  Part of the Win32 API, is the Remote Access API, which provides ISVs with services to initiate resume a remote connection as well as gather information about the type and status of the connection.

The second critical element of these services is the Remote Access subsystem.  This open subsystem is network- and device-independent to enable universal connectivity.  This means, for example, that Chicago supports ISDN boards, PBX modems, and so on.  This capability is accomplished through service providers—software components that manage physical connections, and network traffic over the remote media. Finally, the Remote Access subsystem has a modular authentication provider that can be supplemented or replaced to provide custom security services.  For instance, suppose the BrandX Company wanted to provide its own custom services.  That company can replace Chicago’s authentication DLL with its own to take advantage of BrandX-specific security features.

The following diagram highlights the various components of the remote access subsystem that can be replaced by third-party service providers.  The shadowed items can be replaced to add functionality not offered by the basic provider.

Figure 17.  Authentication DLL Integrates Directly Into PPP Connectivity

#### Telephony API

To communicate in a mobile environment, users and applications must dial phones or modems.  Applications can use Chicago’s TAPI to dial, whether the device is a phone on a PBX system, an ISDN board, or a modem.  TAPI provides services to allow applications to share a line so that more than one application can wait for an incoming call while another dials out.  TAPI itself is extensible so third-party developers can write TAPI service providers to extend the dial support of TAPI.  One such TAPI service provider is Unimodem (discussed in detail in the following section).

TAPI also provides Dial Guide (this is the preliminary name for this user interface that applications can call) to guide users through the process of defining the correct phone number.  Dial Guide gives users the opportunity to define phone numbers in a location independent fashion.  The user enters an area code and seven digits out of their Address Book, for example, and using Dial Guide applies location specific parameters to the number, such as a prefix to get an outside line.  When users dial this number from different location, they simply switch their Dial Guide locations, instead of reentering the entire number.

#### Unimodem

Similar to Window’s infrastructure for printers, Chicago provides an easy, central, extensible mechanism for installing and configuring modems.  It is important to note that TAPI and Unimodem use Chicago’s extensible 32-bit communications architecture.  See the Communications Architecture section of this guide for additional information.  Chicago automatically detects the modem and provide a default configuration for it.  Once the user installs a modem, it is available to all applications.  Applications don’t have to store modem commands or the capabilities of the different modems. Chicago ships with support for the top 200 modems worldwide.  Adding new modems is as simple as supplying the appropriate .INF file. ( Microsoft certifies the .INF files for each new modem and the modem includes a logo identifying it as Windows compatible.)

### “Local” Connections

Roughly 70% of portable PC users also own a desktop PC.  As a result, they constantly need to connect their desktop and portable PCs “locally” (that is, from a range of one to ten feet.).  Chicago makes this process significantly easier.

Like remote access, establishing a local connection is seamlessly integrated into the shell and provides full participation on a variety of networks.

Wireless technologies like Infrared (IR) are another form of local connections.  Using Chicago’s extensible device driver architecture, Microsoft is working closely with creators of wireless devices to develop and ship Windows drivers for these new technologies.

### File Synchronization: The Chicago Briefcase

Portable PC owners who also have a desktop PC constantly work to keep the most up-to-date files on the computer they are currently using.

The Chicago Briefcase minimizes these headaches by keeping track of the relationships between the files on their two computers using a simple metaphor that users are already comfortable with—a briefcase.

Figure 18.  Initial Briefcase Dialog Box Explaining Briefcase Process

After installing the Chicago Briefcase software on the portable PCs, a user can specify which files and directories he or she wants to take keep up-to-date between the two PCs by dragging and dropping those objects into the Briefcase.  When the user reconnects the portable and desktop PCs, Briefcase automatically replaces the unmodified files with the recently modified files.  If both files have changed, Chicago calls the appropriate application to merge the disparate files.

Figure 19.  Sample Briefcase Contents Showing Document Status

Chicago provides a set of OLE 2 interfaces that allow applications to bind reconciliation handlers to it, track the contents of the Briefcase, and define the outcome of any reconciliation on a class by class basis. For example, when both the file in the Briefcase and its synched copy outside have changed, Chicago calls the appropriate reconciliation handler to merge the two files. These APIs will also serve as the foundation for Cairo’s reconciliation APIs.  As a result, ISVs writing to the Chicago reconciliation APIs can leverage that investment as they write Cairo applications in the future.

### Remote Mail

Historically, a user leaving the office also left behind email capabilities.  Microsoft, Lotus and other email vendors are changing this. Chicago delivers the next generation of remote mail. With Chicago, mail is completely integrated mail into the Chicago shell so that a user’s Inbox appears as just another folder rather than a separate application.  When out of the office, users don’t need to alter their behavior. They simply connect a phone line to their modem and start using mail.  The remote connection is established automatically for them using Remote Access services.

While connecting remotely is seamless to the user, Chicago has minimized the cost of slow links for the user.  Performance over the wire has been enhanced, users can browse headers, downloading only specific messages and getting an estimated time to download and status of the download process.

#### Messaging API

MAPI provides a critical layer for Chicago’s mobile story. Mobile users more than any other class of users need multiple messaging providers and the ability to seamlessly move between these providers.

MAPI is an open, extensible messaging infrastructure standard.  This standard ensures complete independence for Windows applications and client software from underlying messaging systems while enabling vendors to supply a wide array of providers.  MAPI provides the support to dynamically switch in and out providers and associate multiple providers and preferences with a “profile.”

### Microsoft At Work Fax

Fax is one of the primary means in which mobile users send messages and documents.  Rich fax services are seamlessly integrated into the Chicago email client.  Users send a fax the same way they send any other electronic message.

For example, these services enable users to send a fax and attach a binary file just the way they do in mail today.  Depending on the capabilities of the recipient’s PC or fax machine, the message appears as a message in their inbox with an attachment or for those people with a Class 3 fax machine, the attached document is rendered and printed with a cover sheet. If users want to send faxes when they are not connected, they can spool them to their outbox and when they connect, the faxes are automatically be sent.  Microsoft At Work Fax provides security to ensure the correct recipient via an RC4 encrypted password or public key and private key encryption.

Microsoft At Work Fax uses the open, extensible architecture of MAPI, plugging in as a transport provider and then leveraging the user interface provided by the Chicago client.

### Accessing Online Services

Online services are a critical network for PC users to gather information and communicate with other PC users.

Chicago integrates a variety of  access points into online services.  The Info Center will allow Chicago users to send and receive mail through a variety of mail service providers from a single integrated mail client.  Chicago’s HyperTerminal communications application will support dial-up connections to services including CompuServe, Dow Jones, and America Online.

### User Profiles

Many users share portable and desktop PCs, but each user wants individualized settings for user interface appearance and operating system defaults.  Ideally, as users move from one PC to another, these settings move with them.  This is possible with Chicago’s user profiles.

A user profile is a list of user-specific settings, kept on a per-user basis, in the HK_Current_User key of the Registry.  Because user profiles are per-user, even users who share a computer can each maintain separate user profile of customized settings for Chicago.

User profiles can also be stored up on the network to address the needs of the roving PC user.  When the user logs onto Chicago (and thus the network) on a different PC, their settings can be downloaded from the network.

### Password Management

Users constantly strive to protect the data on their portables from prying eyes and hands.  This is no easy chore.  Password protection at boot-up, after a suspend (reduced power) state, and at network logon means users must often contend with scattered user interface and multiple passwords.

Chicago’s Security Control Panel provides a central, extensible mechanism for users to manage their security.  Chicago’s Master Password gives users the opportunity to unify all their different passwords around a single password regardless of the physical store of the password.  This user interface and security are open and extensible.  As a result, ISVs and portable PC manufacturers can add their own property sheets to the Security Control Panel and hook their password services to the Master Password.

### Document Viewers

Portable PC users often need to exchange and view documents with customers or people in a different work environment then their own.  Many times, however, they don’t have the application used to create that file so they can’t view the file..

Seamlessly integrated into the Chicago shell is an extensible, replaceable file viewer technology.  Users simply select a file and choose “Quick View” to view the file. Chicago supports viewing more than 30 different file types out of the box and publishes interfaces to allow applications to add support for additional formats and even to add their own viewer.

### Docking Support for Mobile Computing Hardware

Many portable PC users have had to compromise storage, extensibility, and display size and resolution in favor of mobility.  Docking stations provide users with both the mobility of the portable PC and the storage, extensibility, and versatile display capabilities of a desktop PC. However, users with docking stations spend a lot of time synchronizing files on the desktop and portable computers.

Chicago makes this kind of synchronization automatic for the user.

Microsoft forged partnerships with leading portable vendors like Toshiba and Compaq and BIOS vendors like Phoenix Technologies to achieve a level of integration between hardware and software never achieved before.  On the hardware side, the challenges were to electrically enable docking without powering off the computer.  On the software side, Chicago must detect the changes in configuration anticipate the resulting changes in hardware, manage any conflicts, and load the appropriate drivers.

The results are exciting.  Chicago and an accompanying family of hardware support docking without powering the computer off.  Chicago ships with a full complement of dynamically loadable drivers from network drivers to mouse drivers that load and unload depending on whether the user is docked or undocked.

To the user, they simply choose to “Eject PC” from the System menu.  Chicago checks for any potential problems before undocking (for example, Chicago lets a user know if he or she has a file open on a network drive at this time), and the system undocks, without powering down if the user chooses.  Once undocked, the system automatically reconfigures for the different hardware (for example, changed video resolution down to 640x480).

#### New Message Support

The Windows Plug and Play initiative provides is a new set of Windows messages that alert applications and device drivers to changes in the hardware so they can react intelligently.  These messages include:

- Docking:
About to change configuration (for example, when the user is about to undock)
Device about to be removed
Configuration changed (for example, when the user just undocked)
Device about to be added

- Power Management:
System about to suspend
System suspended
System resumed

- PCMCIA:
Device inserted
Device removed

- Miscellaneous:
New device inserted (for a device that needs to be set up)
Serial mouse inserted 
Parallel cable inserted

These messages expose a plethora of opportunities for applications and system services to better support the portable PC user.  Chicago itself takes full advantage of these messages.  Take the Config_Changed message as an example.  Applications shipping with Chicago alone uses the message this way: the Briefcase uses it to try to start updating, the print spooler to print all deferred print jobs, and Mail to try to reestablish a network connection.

#### The Registry

The Chicago Registry provides a  centralized, dynamic data store for all Windows settings that ISVs can use.  The Chicago Registry defines a branch called HKCurrent_Config to better serve the needs of a mobile user.  The Config branch stores information on a per-configuration basis.  For example, the Desktop Control Panel stores per-configuration information about video resolution changes and Print Manager stores per-configuration information about the default printer.

Configurations are created when Chicago queries the BIOS for a dock serial ID, asking the user for a friendly name for the configuration, and then storing hardware and software associated with this configuration.  Applications access and store information for each of the different hardware configurations that the mobile user uses.

### PCMCIA Support

The emergence of PCMCIA cards has been one of the most exciting advances in the portable market.  However, to date, users were never sure if the card was compatible with their portable, they had to struggle through installation, and insertion and removal was anything but dynamic.

Through the Plug and Play architecture, Chicago’s PCMCIA support brings power, compatibility, ease of installation, and dynamic card insertion and removal to the PCMCIA experience.  The drivers are robust, 32 bit, dynamically loadable virtual device with zero-memory footprint.  Chicago ships with an updated version of card and socket services  and a compatibility testing/logo program ensures compatibility with these standards.  Installation is as simple as inserting the card.  Finally, insertion and removal of cards happens dynamically.  For example, when the user plugs in a PCMCIA network card instead of having to reboot the computer, the portable detects the network card, load the network stack, establish a network connection (assuming one is available).  Then the shell updates its user interface to reflect that the mapped network drives are now active.

### Dynamic Networking

Historically, network users with a portable have dealt with CONFIG.SYS files and a regular stream of error messages as they connected and disconnected from the network.

To adapt to changes in link speed and configuration, Chicago’s network solution is completely dynamic, regardless of whether the user is using the NetWare compatible components, or a Windows networking components.  All the underlying drivers, transports, and redirectors are robust, 32 bit, dynamically-loadable, protected-mode virtual devices that support Plug and Play.  This enables Chicago to load and unload components of the network stack as demanded by hardware events.  For example,  when the user docks a portable PC, the network components are loaded and connections are established without user interaction.  This is seamless mobility.  Even assigning a TCP/IP address is now dynamic, using the Dynamic Host Configuration Protocol (DHCP) servers to allocate addresses on demand.

### Power Management

One of the curses of a portable user’s existence is battery life.  Chicago supports Advanced Power Management (APM) 1.1 which represents a major step forward from APM 1.0.

From an end-user perspective, there are three major changes.  First, integrated into the Chicago shell is a battery meter that lets the user constantly monitor an accurate representation of the battery life they have remaining. Second, from the System menu, the user will be able to put their system in “Suspend” as opposed to going to a hardware control.  Third, users will have the option of automatically powering their PC off when they shut down Windows.  (In the past of course, the user had to shut down Windows, then use the hardware power switch to shut off the PC.)

From a software vendor’s perspective, Plug and Play APM messages allow applications to react to changes in the power state and battery life.

### Flexible Video Resolution Support

In one focus group after another, display was cited as the number one limitation with portables.  To overcome that limitation, portable vendors are putting high-end video on the motherboard, and users are plugging a monitor to their portable when they are at their desks.

Chicago stores video resolution on a per configuration basis and supports dynamic resolution changes.  As a result, when the user is docked, for example, they can set their video resolution at 1024x768.  Whenever they return to their docked configuration, the resolution will automatically return to 1024x768.

### Deferred Printing

Users want to generate print jobs regardless of where they are.  Chicago supports deferred print jobs so the user can generate print jobs regardless of whether a printer is connected.  When a printer becomes available, Chicago detects this connection and automatically spool the print jobs as a background process.

Extending this, Chicago gives users the ability to print to a generic printer. So if they aren’t sure which printer they will be connected to next, users can still queue the print jobs, then specify the printer only when they connect to the printer.

Finally to better support the mobile user, Chicago stores the default printers on a per configuration basis.  If the user has a different printer at home then they do the office, when Chicago detects the change in location (for example, docked versus undocked state on a computer), it will change the default printer.

### Pointing Devices

In our focus groups, portable PC users often described difficulties in switching between the integrated pointing device on their portable to a desktop pointing device when they are at their desk.

Chicago improves on this in two ways.  First, when the user changes configurations, Chicago automatically detects which pointing device is available to use, and enables it (or both).  Extending this, without changing configuration, users can plug in a Plug and Play serial mouse and the system will detect the new mouse, and dynamically reconfigure so the user can it.

## Chicago’s Messaging and Information Center

### A Single View on a World of Information

Customers today are using their PCs for an increasingly wide range of tasks, beyond simply creating and editing documents.  Electronic mail has not only become a primary communication vehicle within many organizations, but also among companies, organizations, and individuals.  Many companies and departments have begun installing so-called workgroup systems, which provide team information sharing capabilities beyond basic email.  Finally, the world of online information services has begun to dramatically increase—witness the astounding 15% per month growth rate seen by the Internet, in addition to the rapid growth in commercial services, such as CompuServeÒ and others.

A problem users face today is that each of these different information sources and services comes with its own unique software and user interface.  Users often have software for an email client such as Microsoft MailÒ, a groupware client such as Lotus NotesÒ, and an online services client such as CompuServe Information ManagerÒ, and perhaps some Electronic Fax software they received with their modem—all in addition to the basic File Manager they use for accessing and manipulating documents.  Users with email accounts on multiple services face an even more complicated problem, having to log into to several different email applications simply to check their mail.

Figure 20.  Today's Dilemma Involves Integrating Disparate Information Sources

To work with different types of information, the user today must acquire and learn four, five, or even more separate applications.  Often there’s very little integration between programs beyond rudimentary cut and paste.

To make using the PC interface easier, Chicago consolidates the previously separate applications functionality provided by File Manager, Program Manager, and Print Manager into a single UI to provide common and consistent access.  Chicago does this by extending the Windows shell beyond just files and documents in the users’ local and network file system.  Users now have a single tool—Windows—that serves as a platform that provides easy, common access to a wide variety of email systems, workgroup applications, and information services.

### The Info Center

As described earlier, the Chicago shell features a single namespace that represents all of the data accessible locally and on the network.  Through the new user interface, new users are protected from the complexities of a large hierarchy of information by Chicago’s simple folder metaphor.  Advanced users can choose to make full use of this powerful namespace by using the Explorer tool to see the entire hierarchy at once.  The Chicago namespace is initially divided into three fundamental “places”:

- My Computer—all of the files, documents stored physically on the user’s PC.

- Network Neighborhood—all of the files and documents that reside on other PCs on the network.

- Info Center—email messages, forms, documents, and folders that have been stored by some information service, such as an email system or workgroup application.  Items in the Info Center needn’t be MS-DOS files on a hard disk—the actual storage mechanism can be an email system or a workgroup database server.

Messaging and information services “plug in” to the Info Center.  It provides a common interface in Windows where users access all of the services they use.  Users can choose to view the Info Center resources as regular folders (the default), or they may “explore” the Info Center.  The figure below shows the Info Center opened with the Explorer, in this case open to the user’s Inbox folder.

Figure 21.  Chicago Desktop Showing Info Center

Within the Info Center, the user can:

- Read and Send Email on a number of different email systems, while maintaining a single inbox for all incoming messages.  Drivers will be available from a wide range of vendors, including LAN-based systems, host-based systems, and various online information services.

- Send and Receive Faxes electronically with the use of a fax modem supported by the Microsoft At Work Fax software.  Faxes are sent just like any email message, and incoming faxes are received in the same inbox with other email.

- Access Workgroup Servers including email, shared databases, forms packages, document stores, and workflow systems with the use of appropriate MAPI drivers.

- Easily Move Messages and Documents back and forth between folders.  Users can store information the way they want to—email messages can be mixed with word-processing documents and spreadsheets.  The Windows shell provides a common user interface and toolset for managing these different types of information.

- Organize All Types of Information for most effective retrieval and use.  Folders stored under the Info Center allow the user to create customized views of their information—users can choose which fields (or “properties”) to display in these folders, and how to sort and filter the stored items.

### Info Center Components

Chicago’s Info Center system is the part of the Windows operating system that allows Windows to handle all these different types of information.  It consists of a number of components:

- A Set Of Basic MAPI Drivers.  Chicago includes MAPI drivers for Microsoft Mail as well as Microsoft at Work Fax services. Additional drivers will be available from 3rd parties.

- The Info Center “Viewer”.  This is an advanced, extensible messaging and workgroup client that is built into the Chicago shell.  It can be the front-end to any email or information system that has appropriate MAPI drivers - including Microsoft Mail.  It can be easily customized to display advanced features when connected to an advanced messaging system.  It includes an OLE 2.0-compatible rich text editor used for composing and reading messages, as well as powerful custom views, searching, and filtering.

- Common Address Book.  The Windows Address Book contains not only email addresses, but names, phone/fax numbers, mailing addresses, and other personal contact information.  Through the open MAPI interfaces, it is accessible from a wide variety of applications.  Through the use of MAPI drivers , the Address Book is also the user interface for corporate email and information services directories.  The Windows Address Book can store addresses for multiple email systems at the same time.

- Personal Information Store.  The Windows Messaging System includes a sophisticated local “database” that allows users to store messages, forms, documents, and other information in a common place.  Its rich organizing capabilities include using long filenames, plus sorting and filtering on various fields of the stored objects.  Users can create and save any number of custom views on the information in their personal store.  It also functions as the user’s Mailbox—including a universal Inbox and Outbox that work with any connected messaging system.

- MAPI 1.0.  The core system components that connect the Viewer and other applications to the various information services.  MAPI’s namesake component is the Messaging Application Programming Interface — the set of services that any mail-enabled or workgroup application can make use of.  MAPI also defines a Service Provider Interface (SPI) that allows drivers (or “providers”) to be written for many different messaging and workgroup services.

### Open Architecture for Open Connectivity

The Info Center is designed to work with virtually any messaging or workgroup system—whether it’s LAN-based, host-based, or an online service requiring dial-in access.  The key to this open architecture is MAPI—sometimes called the “Windows Messaging System.”

Figure  SEQ Figure \* ARABIC 39.  MAPI Provides an Open Architecture

Any information or messaging service provider can develop a MAPI “driver” (see below) so that the Info Center can be used with their system.  There are currently over 60 different vendors working on creating MAPI drivers for different email, voicemail, workgroup, and information systems—including LAN-based, Host-based, and Wireless systems.  A partial list of these providers includes:

| - Apple | - MCI |
| --- | --- |
| - AT&T | - Microsoft |
| - Banyan | - Novell |
| - CompuServe | - Octel |
| - Delrina | - RAM Mobile Data |
| - Isocor | - Skytel |
| - Lotus | - WordPerfect |

#### Profiles

A Chicago user can install a combination of drivers so that their Info Center can be used for multiple email or workgroup systems at the same time.  To make it easy to use different information services, Chicago allows users to set up profiles.  A user’s profile specifies which messaging and information services the user will have access to, along with preference information and settings.  In the example below, the user has set up a profile that contains both CompuServe Mail as well as Microsoft Mail.  This user will be able to send and receive on both systems simultaneously—while sharing a common inbox and address book. Profiles are stored in the Chicago Registry on a per-user basis.

Figure 22.  Configuring a Profile with Multiple Services

### A Quick Tour

To best understand the role the Info Center would play in a user’s day-to-day access to electronic mail and online services, a sample scenario is presented as follows.

To read their email in Chicago, a user would first log onto Windows.  With many of the MAPI drivers under no separate logon is required for the mail system—it has been unified with the “system” logon.  For simple access to the user’s common tasks, Chicago has a Start button.  One of the items on the Start button is “View Inbox.”

This immediately takes the user to the inbox (see Figure 79 above) where he or she can read any new mail received.  Users can compose new messages or choose to reply to existing ones.  There is also a “Compose New Message” item on the Start Menu.  The Info Center includes an OLE 2.0-compatible rich text message editor that lets the user get the point across in an effective way using a combination of fonts, font styles, and font attributes.

Figure 23.  Rich Text Message Editor

Note that this same message editor can be used with any of the back-end messaging systems that have MAPI drivers available.  If the underlying messaging system doesn’t support rich text, the message can be sent as plain text to maintain compatibility.  Or, if the destination reader also uses the Info Center Viewer the rich text information can be automatically encapsulated and sent as a binary file.

The user can address this message to any user on any of the email systems to which he or she is connected.  Additionally, the message may be sent as a Fax by choosing a recipient from their address book who has a Fax address, or simply entering [FAX:phone#] on the TO: line.

Messages received in the inbox can be saved for future reference, if the user so chooses.  The user simply drags the messages into any of the other folders in the mailbox (message stores)—or the user can drag the message to any folder on their local or network hard drives.  In the latter case, the message becomes a .MSG file—but maintains all of the messaging-specific fields such as Sender, Recipient, and so on.

#### Information Stores

Sets of folders in the Info Center are also called Information Stores.  Users can also drag items from the file system into information store folders.  Information Stores go beyond the basic MS-DOS file system in many ways.  They can be physically stored in local files, or represent a database on a network server.  One of the differences between the file system and an information store is the set of fields -- called object properties -- that are stored along with each item.  Object properties include not only messaging-related properties like Sender, Recipients, Date Received, Subject, and so on—but also custom properties including:

- Fields Defined In Custom Forms.  MAPI provides an open method for forms developers to register their forms and related data fields.  Once registered, these fields become object properties suitable for searching, categorizing, and so on.  This facility is available to any forms package or development tool.

- Standard OLE 2.0 Document Properties.  Applications that are compatible with the OLE 2.0 object standard can choose to store their documents in a standard compound storage format.  Compound documents of this type have a number of pre-defined standard properties, such as Author, Title, Keywords, Comments, and Number of Pages.  Users can simply drag a compound document into an information store, and the document properties are automatically extracted so they can be used in views, searching, and so on.

The figure below shows the OLE properties of a compound document that has been dragged into a folder in the user’s information store:

Figure 24.  OLE Document Properties

- Custom OLE 2.0 Document Properties.  Many applications allow their users to define custom properties; for example, Case Number for a law firm or Total for an expense report.  Like the pre-defined properties above, these will be available in the views, filters, searching, and so on.

The data in a Personal Information Store is kept in a single file, making it easy for companies to distribute rich documents and messages in a standard format.  Every Chicago user then has a built-in tool (Info Center Viewer) that can browse, search, and organize the information in rich ways.  Personal Information Stores can be encrypted and password-protected for security.  Besides the Personal Information Store, the MAPI architecture makes it possible to plug in many different types of servers and databases as information stores.

#### The Info Center - Summary

The Info Center architecture then, provides Chicago with a single toolset and user interface for accessing, exchanging, and organizing information—regardless of data type, including E-mail, faxes, documents, and others.  Through MAPI, it provides an extremely open platform—making the Windows desktop the “place to live” regardless of the back-end services in use.