==PCB.DOC======================TANGO-PCB=================Version 2.00==

INTRODUCTION

Tango-PCB Version  2.00 has  many new features and bug fixes, described

below.   Version 2.00  PCB files  are  not  backwards  compatible  with

versions prior  to Version  2.00: while PCB files produced with version

1.XX programs  can be  loaded by  version 2.00, files created with 2.00

cannot be  read by  version 1.XX  programs.  This will also be true for

Route version  2.00: Route 2.00 files will not be compatible with Route

1.XX files.   Please  make sure  you take  proper  precautions:  backup

design files  before using  the new 2.00 version, just in case you wish

to use these files at another station running version 1.XX.

The same  is true for the PCB definitions file (PCB.DFN).  Version 1.XX

PCB.DFN files  will be  read, but  your user-defined  palette codes for

Lines and  Text (formerly  Tracks and  Strings) may  produce unexpected

results when  used.   To fix  them, either  delete  and  redefine  them

through the  Setup Palette command, or, using a text editor, change the

leading character  in the  item code  (the second quoted string) in the

PALETDEF records from T and S, to L and T, respectively.

INSTALLATION INSTRUCTIONS

ACCEL is standardizing on an automatic installation program, INSTALL,

which uses compressed files on the distribution diskettes.  The Tango-

PCB Reference Manual contains complete instructions for using this

utility.  The minimum free disk space required for a full installation

is 2.5 MB.

GENERAL

1.   LIBRARY  COMPATIBILITY   -  PCBMAIN.LIB,   PCBCONN.LIB,   and

PCBSMT.LIB have  all been revised so that all pin designators

match their equivalents in Tango Schematic libraries.

2.   MOVE LINE  -  This  command  (formerly  Move  Track)  is  now

consistent with other commands that drag tracks.

3.   NEW LIBRARY  COMPONENTS -  Support has  been  added  for  the

following component  patterns: TO220,  TO99, TO247, 44/52/68-

pin PLCC PGA sockets, and plugs/sockets for all connectors.

BUG FIXES

1.   ARCS -  Domain Errors  no longer occur when placing, editing,

or selecting arcs.

2.   COMPONENT -  Creating a  component from  an existing  one and

placing a  track between  pads used  to  cause  the  pads  to

disappear after a move component.

Page 1

==PCB.DOC======================TANGO-PCB=================Version 2.00==

3.   CURRENT PAD  - Rapidly  deleting all  current PADS  caused  a

system hang.   Similar  problems, that may have caused random

graphics on the display, have also been fixed.

4.   OUTPUT CAM  - Offsetting an N/C Drill file enough to generate

negative  coordinates,   would  result   in  the  minus  sign

overwriting the first zero, if not suppressed.

5.   OUTPUT  PLOT/PRINT  (PLOT)  -  When  the  rotate  option  was

enabled, the same offset for different layers would result in

different locations  on the  paper.   Enabled layers  are now

registered correctly.

6.   OUTPUT PLOT/PRINT  (PRINT) -  Draft mode  now works  with all

printer drivers;   Drill  Drawings now print much faster; and

short, wide lines now print correctly.

7.   PALETTE -  Items can  no longer  be  placed  in  the  palette

region.

8.   SETUP GRID  - Absolute grid is on 25 mils at (75,75).  Change

it to relative grid of 100 mils you could not get your cursor

back to the origin: it would jump off the grid.

KNOWN PROBLEMS

1.   MACROS - Under extreme low-memory conditions, you may receive

two out-of-memory  error messages  while recording  a  macro.

After acknowledging  the first,  the program may appear to be

hung but  it is  actually waiting for another acknowledgment:

hit <Enter> to return to normal processing.

2.   MOVE REROUTE  - If  the layer is changed before any lines are

placed, a  via may  not be  inserted;  however,  it  will  be

inserted correctly after any line is placed.

3.   PLACE COMPONENT  - After a netlist is loaded, always initiate

manual placement  with a  refdes that is part of the netlist.

Starting  off   with  a  non-netlist  refdes  may  result  in

duplication of the component.

4.   OUTPUT PLOT/PRINT  - When  printing in Draft Mode, additional

stray lines may be generated.

5.   POLYGONS -  "Busy" polygons  are those with many simultaneous

vertical edges;  e.g., a polygon that looks like a horizontal

saw, with  all of  its points at the same y-coordinate.  Busy

polygons outline  correctly, but do not fill correctly on the

display.  Also, there may not be enough memory to output busy

polygons, depending  on the  size, scaling factors, available

memory, and  driver user  (the  PostScript  drivers  are  not

affected).  If you run out of memory while outputing, you can

try purging  the Undo  information (File  DOS will  do this),

Page 2

==PCB.DOC======================TANGO-PCB=================Version 2.00==

break the  polygon into  two or  more  smaller  polygons,  or

rotate the image to change the vertical edges into horizontal

ones.   Also, extremely  narrow polygons may not be displayed

at some zoom levels.

7.   TIGA GRAPHICS  DRIVER -  Not enough  memory for both the TIGA

TSR and  PCB may  result in  unexpected behavior.   Also, the

File DOS command changes the PCB cursor to a random shape.

OTHER CONSIDERATIONS

1.   COMPONENTS -  Version 2.00  requires more netlist information

than is  contained in  version 1.XX  files; be sure to reload

your netlist file to insure proper processing.

2.   MACROS -  If a  macro contains  two or  more separate editing

commands, UNDO  will only  affect the  last one;  it will not

affect the entire macro.

Page 3