==PCB.DOC======================TANGO-PCB=================Version 2.02==

INTRODUCTION

Tango-PCB Version  2.02 is  a maintenance release with enhancements and

bug fixes,  described below.   Version 2.02 PCB files are not backwards

compatible with  versions  prior  to  Version  2.00:  while  PCB  files

produced with  version 1.XX  programs can  be loaded  by version  2.02,

files created  with 2.02 cannot be read by version 1.XX programs.  This

will also  be true for Route version 2.02: Route 2.02 files will not be

compatible with  Route 1.XX  files.   Please make  sure you take proper

precautions: backup  design files  before using  the new  2.02 version,

just in  case you  wish to  use these  files at another station running

version 1.XX.

The same  is true for the PCB definitions file (PCB.DFN).  Version 1.XX

PCB.DFN files  will be  read, but  your user-defined  palette codes for

Lines and  Text (formerly  Tracks and  Strings) may  produce unexpected

results when  used.   To fix  them, either  delete  and  redefine  them

through the  Setup Palette command, or, using a text editor, change the

leading character  in the  item code  (the second quoted string) in the

PALETDEF records from T and S, to L and T, respectively.

INSTALLATION INSTRUCTIONS

ACCEL is standardizing on an automatic installation program, INSTALL,

which uses compressed files on the distribution diskettes.  The Tango-

PCB Reference Manual contains complete instructions for using this

utility.  The minimum free disk space required for a full installation

is 2.5 MB.

ENHANCEMENTS

In this update, in order to support our newest Tango product,

Tango-Route PRO,  we have  added the  8.3 fractional grid and

modified the  17.6 fractional  grid.   The 8.3  grid  (8-9-8)

allows twice  the line  density of  the 17.6  grid.  The 17.6

grid was  changed from  a 16-17-17 placement scheme to 17-16-

17.

BUG FIXES

1.   COMPONENTS -  You  can  now  create  a  component  containing

multiple polygons.

2.   COMPONENTS  AND  LINES  -  Components  and  lines  no  longer

disappear from the database.

3.   DELETE POLYGON  - You  can now select any side of the polygon

to delete the polygon.

4.   JUMP PAD CENTER - Now works correctly with Move Reroute.

5.   MOVE COMPONENT  - When  moving an  SMT component, the polygon

reference block now moves with the component correctly.

Page 1

==PCB.DOC======================TANGO-PCB=================Version 2.02==

6.   MOVE POLYGON  - Now  works correctly.  You can now select any

side of the polygon to move it.

7.   NETS CLEAR  - Component  string corruption  no longer  occurs

after moving a component.

8.   NETS GENERATE,  NETS HIGHLIGHT,  NETS VERIFY  -  Now  support

polygons correctly.

9.   NETS VERIFY - Now correctly reports single node routes.

10.  NETS VERIFY  - Now  verifies and reports all mismatched power

and ground nodes correctly.

11.  OUTPUT CAM  - When generating an NC Drill file, Output CAM no

longer looks for a T00 assignment.

12.  OUTPUT PLOT,  OUTPUT CAM  - Small  polygons are now generated

correctly.

13.  OUTPUT PLOT - Targets and Mounting Holes now plot correctly.

14.  OUTPUT PLOT - Polygons now scale correctly.

15.  OUTPUT PLOT/PRINT  - 250-mil holes are now being generated in

the Drill Drawing.

16.  OUTPUT REPORTS - The Bill of Materials report now reports the

correct values.

17.  OUTPUT  REPORTS  -  The  Statistics  Report  information  now

correctly matches the Info Status information.

18.  PLACE  COMPONENT   -  Replacing  a  component  now  correctly

rebuilds connections.

19.  SQUARE VIAS - These are again supported.

20.  TEXT STRINGS  - A  modification has been made to correct text

string problems.

21.  LEADING ZEROS:  TEXT, COMPONENTS - You can now place text and

component information with leading zeros.

22.  SECURITY DEVICE  - There is no retry (no second chance) given

if there is a security key problem or if device is not found.

23.  PIN DESIGNATOR  CORRUPTION -  Component Move followed by Nets

Load) Ref Des, Type, Value, and Pin Designators no longer get

corrupted.

24.  PCBEVAL.LIB -  This is  now  compatible  with  the  Schematic

Evaluation Library, SCHEVAL.LIB.

Page 2

==PCB.DOC======================TANGO-PCB=================Version 2.02==

KNOWN PROBLEMS

1.   MACROS - Under extreme low-memory conditions, you may receive

two out-of-memory  error messages  while recording  a  macro.

After acknowledging  the first,  the program may appear to be

hung but  it is  actually waiting for another acknowledgment:

hit <Enter> to return to normal processing.

2.   MOVE REROUTE  - If  the layer is changed before any lines are

placed, a  via may  not be  inserted;  however,  it  will  be

inserted correctly after any line is placed.

3.   OUTPUT PLOT/PRINT  - When  printing in Draft Mode, additional

stray lines may be generated.

4.   POLYGONS -  "Busy" polygons  are those with many simultaneous

vertical edges;  e.g., a polygon that looks like a horizontal

saw, with  all of  its points at the same y-coordinate.  Busy

polygons outline  correctly, but do not fill correctly on the

display.  Also, there may not be enough memory to output busy

polygons, depending  on the  size, scaling factors, available

memory, and  driver used  (the  PostScript  drivers  are  not

affected).   If you  run out  of memory while outputting, you

can try purging the Undo information (File DOS will do this),

break the  polygon into  two or  more  smaller  polygons,  or

rotate the image to change the vertical edges into horizontal

ones.   Also, extremely  narrow polygons may not be displayed

at some zoom levels.

5.   TIGA GRAPHICS  DRIVER -  Not enough  memory for both the TIGA

TSR and  PCB may  result in  unexpected behavior.   Also, the

File DOS command changes the PCB cursor to a random shape.

OTHER CONSIDERATIONS

1.   COMPONENTS -  Version 2.02  requires more netlist information

than is  contained in  version 1.XX  files; be sure to reload

your netlist file to insure proper processing.

2.   MACROS -  If a  macro contains  two or  more separate editing

commands, UNDO  will only  affect the  last one;  it will not

affect the entire macro.

3.   OVERLAY FILES  - The  overlay files  (.OV1 and  .OV2) are  no

longer  used.  Delete  PCB.0V1  and  PCB.0V2  from  your  PCB

directory.

4.   ROUTE AND  PCB -  Route V2.02  must ship  concurrent with PCB

V2.02 for grid compatibility with 8.3 and 17.6.

Page 3

==PCB.DOC======================TANGO-PCB=================Version 2.02==

5.   ARC -  Wrong message  displayed: "Line  width cannot exceed 2

times the  radius" should  display instead  of "Memory  limit

reached".

DOCUMENTATION ADDITIONS AND CORRECTIONS

1.   ERROR 900:  INITIALIZATION (missing  from PCB manual) - Tango

PCB aborted.  An error occurred during program initialization

that does  not allow  Tango PCB  to run.  Correct the problem

described in  the error message (generally that the specified

graphics driver  could not be found) and run PCB again.  Type

"PCB /g:<graphics driver name>" to change the graphics driver

(see manual:  "Changing Graphics  Drivers", p.  3-7, 3-8  for

more information).

2.   MOVE BLOCK  (ARCS) -   You must include the centerpoint of an

ARC in order to move it.

3.   MACROS -  F11 and F12 are not legal macro keys.                                           ___

4.   MACROS -   Alt-K  can be  used to stop recording a macro when

you are in a dialog box.

5.   DRILL SYMBOLS  -  You can have a maximum of 24 drill symbols.

The program  allows you  to assign more than 24, but the 25th

will wrap and use the value that is assigned to 0, 26th to 1,

etc.

6.   SPACES IN  COMPONENT NAMES  -   Spaces in component names are

not allowed  in V2.XX.   Any  components from  1.16 that have

spaces should  be renamed.  For example, "two part" should be

renamed "two_part".   Use  the Library  Rename  feature  (see

manual: p. 12-13).

Page 4