STATE SYNTAX

When designing MACH355/435/445/465 state machines, you should not

rely on power-up reset to establish a start-up state.  Asynchronous

initialization product term allocation may result in a start-up state

other than the one specified.  For this reason, the START_UP

command is not supported by the MACHXL software state machine

syntax.

In order to set up a known starting state, you must do the following

when using state machine syntax:

1. define the initial state; and

2. define the initialization transition.

1. Defining the initial state

The state machine needs to know which state bit values you intend as

the initial state.  If you do not specify this, the software will

assign them  for you, and they might not agree with what you want.

This is done by defining the state bit pattern for at least the

initial state using the following syntax:

INITIAL_STATE = BIT0*/BIT1*/BIT2*... ;AS APPROP

This syntax is defined in the MACHXL Software User's Manual.

2. Defining the initialization transition

There are a number of ways you can do this.  You can design either

synchronous or asynchronous initialization.  In the synchronous case,

you can use either the state machine section or boolean equations to

establish the starting state.  Asynchronous initialization uses boolean

equations.

Synchronous initialization

Synchronous initialization provides a reset that is synchronized with

the state machine clock.  Once the reset input condition is active, the

initialization will occur on the next clock edge.  The practical

implications of this are that when you power up, you need to assert the

reset condition and clock the device in order to start out in the right

state.

The most rigorous way to build initialization into your state machine

is to include it explicitly in the state machine definition.  This means

that for every state, a transition should be provided back to the initial

state.  This transition is to be taken if the reset input condition exists.

All other transitions should be qualified to make sure they don�t occur

when the reset input condition is active.  In addition, the best solution

is to define all unused states with a reset transition to ensure correct

initialization out of any illegal states.

As an example, suppose we have the following simple three-state

machine, implemented with three of the four combinations of two

state bits:

STATE

STATE1 := IN1 -> STATE2

+-> STATE3

STATE2 := VCC -> STATE3

STATE3 := VCC -> STATE1

Given a reset input called RST, and assuming we want our initial state

bit value to be [00], this machine can be modified as follows:

STATE

STATE1 = /Q0*/Q1  ;DEFINE THE INITIAL STATE BITS

STATE1 := COND1 -> STATE2

+ RST -> STATE1	;ADD RESET TRANSITION

+-> STATE3

STATE2 := RST -> STATE1   ;ADD RESET TRANSITION

+-> STATE3

STATE3 := VCC -> STATE1   ;NO EXTRA TRANSITION NEEDED

ILLEGAL1 := VCC -> STATE1  ;BEST, OR RST -> STATE1

CONDITION

COND1 = IN1*/RST          ;THIS TRANSITION ONLY IF NOT RESETING

Another possibility is to add some boolean equations following the

state machine section.  This requires the use of curly brackets ({}),

since you will be modifying equations that the software will generate

from the state machine section; you won�t know what those equations

will look like when you are writing the equations.

Assuming you are using Q0 and Q1 to implement the state bits, and

you wanted [00] as the initial state, you could solve the equations above

in the following way.

STATE

STATE1 = /Q0*/Q1  ;DEFINE THE INITIAL STATE BITS

STATE1 := IN1 -> STATE2

+-> STATE3

STATE2 := VCC -> STATE3

STATE3 := VCC -> STATE1

;WOULD STILL BE GOOD TO DEFINE ILLEGAL STATE

EQUATIONS

;AND ALL PRODUCT TERMS WITH /RST IF 0 DESIRED

Q0 := {Q0}*/RST

Q1 := {Q1}*/RST

;ADD RST PRODUCT TERM IF 1 WERE DESIRED

;Q0 := {Q0}

;    + RST

Asynchronous initialization

In this case, the asynchronous initializati

on product terms are used, so

that as soon as the reset condition is active, the intialization will occur

without waiting for the clock.

Boolean equations are simply written to define the initial values, as

shown below.

STATE

STATE1 = /Q0*/Q1  ;DEFINE THE INITIAL STATE BITS

STATE1 := IN1 -> STATE2

+-> STATE3

STATE2 := VCC -> STATE3

STATE3 := VCC -> STATE1

;WOULD STILL BE GOOD TO DEFINE ILLEGAL STATE

EQUATIONS

;AND ALL PRODUCT TERMS WITH /RST

Q0.RSTF := RST   ;OR SETF IF A 1 IS DESIRED

Q1.RSTF := RST   ;OR SETF IF A 1 IS DESIRED