MEMORY MAP INFORMATION FOR

HP 27247B PC LAN Adapter/16 TP Plus

HP 27252A PC LAN Adapter/16 TL Plus

The HP LAN Adapter/16 Plus cards can be configured to operate in one

of two modes -- I/O mapped mode or memory mapped mode.

In the I/O mapped mode of operation the card uses the standard I/O

channels to communicate with the computer.  In the memory mapped mode

the card uses a block of memory address space to communicate with the computer.

For ease of installation I/O mapped mode is recommended while for higher

performance memory mapped mode is recommended.

In general, memory mapped mode provides higher performance; but not all the

network operating system drivers can support memory mapped mode.  This file

describes which drivers support memory mapped mode and which portions of the

computer's memory address space each driver can use.

The HP LAN Adapter/16 Plus cards use a block of only 2 Kbytes of memory

addresses in the memory mapped mode.  Make sure that the block of addresses

allocated for the card is not used by anything else in the computer.

There are two regions of memory that can be safely used to allocate the

2 Kbytes of memory address space the HP LAN Adapter/16 Plus uses for

memory mapped mode:

Reserved Memory Area  - the 384 Kbyte of addresses above 640K and

below 1024K

Above Extended Memory - any unused addresses above the last byte of

extended memory and below 16 Mbyte

Reserved            Above

Driver                         Memory Area      Extended Memory

------------------------------    ---------------  -------------------

Novell 286 OS                        Supported         Supported

Novell 286 OS with Appletalk I       Supported         Supported

Novell 286 OS with Appletalk II      Supported         Supported

Novell 286 DOS                     Not Supported     Not Supported

Novell 386 OS (3.11)                 Supported       Not Supported

Novell 386 DOS ODI                 Not Supported     Not Supported

NDIS DOS                             Supported (a)   Not Supported

NDIS OS/2 1.X                        Supported         Supported

Packet Driver                        Supported (a)  Not Supported

----------------------------------------------------------------------

Memory mapped mode can only be activated through the HPLANSET

configurator.  See the installation manual for the HP LAN Adapter/16

Plus cards for information on memory mapped mode and instructions to

enable it.

To use an address in the reserved memory area, select one from the

list of addresses in the Memory Map Menu in the HPLANSET configurator.

HPLANSET tests the address you select for any conflicts with other

computer resources.

To use an address above extended memory, select the "enter memory map

address" option in the Memory Map Menu and enter an address.  For

example, in a PC that has 8 Mbytes of RAM installed (that is, one Mbyte

of base memory, plus 7 Mbytes of extended memory), enter a value of

80000h to put the 2 Kbytes of address space used by the card at a

memory location just above the last byte of installed RAM.

NOTE:

(a) If you are running extended memory manager on your computer, make sure

to use EXCLUDE statements in the CONFIG.SYS file to prevent the memory

manager from using the 2 Kbyte area allocated for the HP LAN Adapter/16

Plus cards.  If you are running Windows on your computer, make sure to

use the EXCLUDE statements in the SYSTEM.INI file.