GENERIC INSTALLATION PROCEDURE FOR A SCSI DRIVE

1. Install a 50 pin ribbon connector making sure pin 1, which

can be identified by the striped edge of the cable, is closest

to the power connector on the drive.

2. Install a DC power cable to the back of the drive.

3. Verify the SCSI ID selection.

Note: SCSI ID usually needs to be set to 0 to be the boot

drive. The last physical device on the SCSI bus must be

terminated. Any other drive or SCSI device connected between the

host and the device at the end of the bus must not be terminated.

4. Apply power to the computer.

4a. When memory test is complete go into your system's Standard

C-MOS set-up.

(Note:  There are various ways to get into C-MOS set-up,

please refer to system's manual for instuctions.)

4b. Set the drive type to "Not Installed". This intelligent drive

will identify itself to the SCSI host adapter.

5. Boot from a DOS diskette that has FDISK.EXE and FORMAT.COM on

it. At the A> prompt type in FDISK. At the menu options select

option 1 to create a DOS partition. Another menu will appear and

from those options choose 1 to create a Primary DOS partition.

Select yes to make 1 large partition and it will automatically

become active. Then escape from FDISK.

6. At the A> prompt type in FORMAT C:/S   This does a high-level

format on the drive and transfers the system files in order for

the drive to be bootable.

(Note: Maxtor SCSI drives are low-level formatted at the factory)

7. The drive is now bootable. As a test remove the DOS diskette

from A and press reset to re-boot the computer, a C> should be

displayed, the drive is now ready for operation.

Patty Barbee 01/18/94  Rev A