Voice Software Release Notes

System Release 4.01

for OS/2

Copyright 1994 Dialogic Corporation

All Rights Reserved

OS/2 is a trademark of International

Business Machines Corporation

This Release Note contains release information

not currently documented in the User's guide.

Releases documented: System Release 4.01

The following information applies to changes

that occurred from System Release 4.0 to

System Release 4.01.

Change Summary

System Release 4.01

Three missing members of the DL_CAP structure

have been added:

ca_pvdmxper -

Percent of the maximum sum (default = 19%)

ca_pvdszwnd - Length

of window in 10 ms increments (default = 7)

ca_pvddly - Delay

time, in 10 ms increments, prior to starting PVD for each non

silence (default = 1)

They are used to tune Positive Voice

Detection. Users of System Release 4.0 should

recompile their application if: A. They are

using enhanced call analysis. B. They are

using SIT detection. C. They intend to tune

PVD.

sb_rtrcvmt() function has been added to the

library. This function was not available in

System Release 4.0 even though it appeared in

the documentation.

Multiple D/2x boards are now supported

(previously only a single D/2x could be

supported in a system.) (Resolves PTR 1740)

The termination event priority has been

changed as follows: When two or more

termination conditions are satisfied

simultaneously, the library uses a specific

set of rules to select the termination event

that is returned to the application's DL_CSB.

Loop current drop now has the highest

priority. See list below:

Priority Before       In SR4.01

SR4.01

Highest T_DTMF       T_LOOP

T_TDIG       T_DTMF

T_TONEID     T_TDIG

T_MXDIG      T_TONEID

T_EOD        T_MXDIG

T_LOOP       T_EOD

T_PATRN      T_PATRN

T_MXSIL      T_MXSIL

T_MXNSIL     T_MXNSIL

T_IDIG       T_IDIG

T_MXSEC      T_MXSEC

T_STOP       T_STOP

T_ERR        T_ERR

T_NORML      T_NORML

Lowest

Compatibility Issues

On-hook audio path on the D/41E board:

The D/41E board receives ANI and other digits

while on-hook from the on-hook audio path.

When porting an application from a D/41D board

to a D/41E board, the ported application must

be modified to clear the buffer or retrieve

the digits before the state of the line

changes to off-hook and continues processing

the call.  If the application does not clear

the buffer, then the digits in the buffer may

terminate the play function so the user will

not hear the voice prompts.

Documentation Corrections

The following corrections should be made to

your Voice Software Installation Reference for

OS/2 (Part Number 05-0010-004):

In the Example paragraph of dx_addtone()

function (pages 7-135, 7-137), change the

code line

if (dx_addtone(channel, 0, 0)  == -1) {

To:

if (dx_addtone(cd, 0, 0)  == -1) {

In the dx_chgdur() section (page 7-167)

change the following:

Name: Change int dx_chgdur() to void

dx_chgdur()

Returns: Replace 0 if success, -1 if

failure with void.

In the dx_chgfreq() section (page 7-170)

change the following:

Name: Change int dx_chgfreq() to void

dx_chgfreq()

Returns: Replace 0 if success, -1 if

failure with void.

In the dx_chgrepcnt() section (page 7-174)

change the following:

Name: Change int dx_chgrepcnt() to

void dx_chgrepcnt()

Returns: Replace 0 if success, -1 if

failure with void.

In the DL_CSB data structure (page 8-8),

the current state is indicated in the

cs_state field. The possible values are:

CS_DIAL        Channel is dialing

CS-GTDIG       Channel is getting

digits

CS_IDLE        Channel in the idle

state

CS_PLAY        Channel in the play

state

CS_RECD        Channel in the record

state

CS_SHOOK       Channel is performing a

set hook

CS_PLAYTONE    Channel is

generating a GTG tone

CS_WTRNG       Channel is waiting for

rings